3GPP TSG-RAN WG4 Meeting #79 AH                                                                                   R4-79AH-0084

Hong Kong, China, 28 - 30 June, 2016

Agenda Item:	4.2

Source: 	Samsung

Title: 	Test case design for PDSCH demodulation test

Document for:	Discussion

Introduction

In last RAN4 meeting, such agreements were reached for demodulation performance requirements:

- DMRS configuration for target  UE

  - Fixed as port 11, nSCID=0, OCC=4

- Number of interference port

  - 1 port  with nSCID= 0,OCC =4

- Interference port selection: randomized interference port between port{7, 8,13}

  - Option 1:  per TTI  per RBG basis

  - Option 2: per TTI per RB basis

  - Option 3: per TTI basis

  - Decide set-up in next RAN4 meeting

- Detailed test parameters

  - Reusing existing test parameters for FRC, propagation channel, antenna correlation, and beam-forming mode as specified in 36.101 8.3.1.1 Test 2

- Performance requirements:

  - Bring simulation results and deciding performance requirements for FDD mode and  TDD mode in next RAN4 meeting.

In this contribution, we provide simulation results for PDSCH demodulation test and give proposals for open issues in this test.

Discussion

Simulation assumption

Detailed simulation assumptions were given in table below.

| parameter |  | Unit | FDD | TDD |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| System Bandwidth |  | MHz | 10 | 10 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.1 |
| MIMO channel |  |  | EPA5Hz, 2*2 Low | EPA5Hz, 2*2 Low |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset |  | Subframes | 5/2 | 5/4 |
| TCSI-RS / ∆CSI-RS |  |  |  |  |
| CSI reference signal configuration |  |  | 3 | 3 |
| Zero-power CSI-RS configuration ICSI-RS /ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 | 4/ 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Rel-13-DMRS-table |  |  | Yes | Yes |
| MCS and Rank |  |  | R.50 FDD | R.44 TDD |
| Simultaneous transmission |  |  | Yes | Yes |
| DMRS configuration for input signal |  |  | fixed as port 11, nSCID=0, OCC=4 | fixed as port 11, nSCID=0, OCC=4 |
| DMRS configuration for interfering signal |  |  | Random select between port {8,11,13} , nSCID=0, OCC=4 Option 1: per TTI per RBG basis Option 2: per TTI per RB basis Option 3: per TTI basis | Random select between port {8,11,13} ,nSCID=0, OCC=4 Option 1: per TTI per RBG basis Option 2: per TTI per RB basis Option 3: per TTI basis |
| PDSCH transmission mode |  |  | 9 | 9 |

Simulation result

Figure 1 and Figure 2 showed the absolute throughput performance for FDD mode and TDD mode. And in table 1, we summarized reference SNR points which corresponding to 70% relative throughput for alignment results.

Figure 1: Throughput vs. SNR for FDD mode

Figure 2: Throughput vs. SNR for TDD mode

Table 1: SNR at 70% relative throughput for alignment results

| SNR at 70% Relative Throughput | Interference port selection |  |  |
| --- | --- | --- | --- |
|  | Per PRG Per TTI | Per PB Per TTI | Per TTI |
| FDD mode | 18.3 | 18.3 | 18.3 |
| TDD mode | 18.1 | 18.1 | 18.1 |

Based on simulation results, we observed that there are no performance differences between different options for updated granularity of randomized interference port number i.e.  per PRG per TTI, per RB per TTI or per TTI. Considering realistic NW schedule for MU-MIMO UE paring and RB allocation, PRG most likely is a minimum scheduling granularity in typical scenarios. Furthermore, the purpose of PDSCH demodulation test case is to verify UE following DCI indication with new DMRS table configuration and correctly implementation CDM4 operation once OCC 4 was indicated.  All of these options can serve above test purpose well.  In current RAN4 specs, randomized precoder update granularity of TM9 demodulation is per PRG per TTI. Considering test complexity and consistent in specs, as a comprise, we propose:

Proposal1: DMRS configuration for interferencee UE: random select between port {8,11,13} as  per TTI per PRG basis, nSCID=0 (OCC=4)

Conclusion

In this contribution, we provide simulation results for PDSCH demodulation tests. Based on simulation results, we observed that under EPA5Hz, there are no performance differences between CDM2 and CDM4 over different interference port selection options. However, under EPA30Hz, we can discriminate UE behavior between OCC4 and OCC2 operation under interference port 11 or random selection interference port.

Based on above analysis and observations, we propose:

Proposal1: DMRS configuration for interferencee: random select between port {8,11,13} as  per TTI per PRG basis, nSCID=0 (OCC=4)

Reference

[1] R4-164859,” Way forward on UE performance requirements for FD-MIMO”, Samsung