3GPP TSG-RAN WG4 Meeting #79 AH                                                                                   R4-79AH-0086

Hong Kong, China, 28 - 30 June, 2016

Agenda Item:	4.3.3

Source: 	Samsung

Title: 	Simulation results for Class B K=1 PMI test

Document for:	Discussion

Introduction

In last RAN4 meeting, such agreements were reached for Class B PMI Test cases:

- Test Point: 70% of maximum throughput with following PMI

- MIMO channel correlation

  - ULA Low EPA5Hz  8*2 for TDD

  - ULA Low EPA5Hz 4*2 for FDD

- MCS & Rank: 16QAM ½ rank1

- Performance requirements

  - Bring simulation results for both FDD and TDD mode and deciding performance requirements in next RAN4 meeting

In this contribution, we provide simulation results for Class B K=1 PMI test cases of both FDD mode and TDD mode.

Discussion

Simulation assumption

Detailed simulation assumptions were given in table below.

| Parameter | Unit | FDD | TDD |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 | 10 |
| Transmission mode |  | 9 | 9 |
| Propagation channel |  | EPA5 | EPA5 |
| Precoding granularity (only for reporting and following PMI) |  | 50 | 50 |
| Correlation and antenna configuration |  | 4x2 ULA Low | 8x2 ULA Low |
| Beamforming model |  | [Annex B.4.3] | [Annex B.4.3] |
| Cell-specific reference signals |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  | Antenna ports | Antenna ports |
|  |  | 15,…,18 | 15,…,22 |
| Number of CSI-RS ports |  | 4 | 8 |
| CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS |  | 5/1 | 5/4 |
| CSI-RS reference signal configuration |  | 6 | 6 |
| eMIMO-Type |  | Class B | Class B |
| alternativeCodebookEnabledCLASSB_K1 |  | TRUE | TRUE |
| codebookSubsetRestriction-3 |  | 00 0000 0000 0000 1111 1111 | 0x 000 0000 0000 FFFF |
| Reporting mode |  | PUCCH 1-1 | PUCCH 1-1 |
| Reporting interval | ms | 5 | 5 |
| PMI delay | ms | 8 | 8 |
| Measurement channel |  | 16QAM 1/2 (R.45 FDD) | 16QAM 1/2 (R.45 TDD) |
| Rank Number of PDSCH |  | 1 | 1 |
| Max number of HARQ transmissions |  | 4 | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} | {0,1,2,3} |

Simulation result

Absolute throughput performances with several PMI adaption methods were evaluated:

- Case1:Following UE reporting PMI index n

- Case2:Random selection PMI index

- Case3:Fixed antenna pair selection with i=0 through CSR and following co-phasing selection

FDD Mode

Figure 1 below show absolute throughput performance with following PMI and Random PMI. Figure 2 show relative throughput ratios between following PMI and random PMI.

Figure 1: Absolute Throughput vs. SNR for FDD mode

Figure 2: Relative Throughput ratio vs. SNR for FDD mode

TDD mode

Figure 3 below show absolute throughput performance with following PMI for multiple PMI test with different CSS configurations. Figure 4 show relative throughput ratios between following PMI and random PMI.

Figure 3: Absolute Throughput vs. SNR for TDD mode

Figure 4: Relative Throughput ratio vs. SNR for TDD mode

Table 1: reference SNR points at 70% relative Throughput and TP ratio

| Duplex Mode | Reference SNR point [dB] | TP ratio |
| --- | --- | --- |
| FDD mode | 2.4 dB | 1.46 |
| TDD mode | 1.5 dB | 1.65 |

Based on simulation results, we observed that:

- For FDD mode with 4*2 ULA antenna: at SNR =2.4dB, throughput achieved 70% of maximum throughout with following PMI. Compared to random PMI, following PMI with antenna pair selection and co-phasing selection can bring 46% throughput gain, and antenna pair selection can bring additional 12% around gain compared to fixed antenna pair with following co-phasing.

- For TDD mode with 8*2 ULA antenna: at SNR =1.5dB, throughput achieved 70% of maximum throughout with following PMI. Compared to random PMI, following PMI with antenna pair selection and co-phasing selection can bring 65% throughput gain, and antenna pair selection can bring additional 40% around gain compared to fixed antenna pair with following co-phasing.

Proposal1: Defining performance requirements for FDD mode as [1.2], TDD mode as [1.3].

Conclusion

In this contribution, we provide simulation results for Class B K=1 PMI test cases of FDD mode and TDD mode.

Proposal1: Defining performance requirements for FDD mode as [1.2], TDD mode as [1.3].

Reference

[1] R4-164859,” Way forward on UE performance requirements for FD-MIMO”, Samsung