3GPP TSG-RAN WG4 Meeting #79-AH 	R4-79AH-0219

# Hong Kong, Hong Kong, 28-30 June, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | - | rev | 1 | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter-frequency event triggered reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2016-06-29 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Test cases for LAA are introduced in LTE_LAA-Perf work item. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases for Inter-frequency event triggered reporting are introduced. Event A4 for FDD/TDD PCell and neighbouring cell with FS3 in band 46 is used in the test cases. Detailed parameters are left TBD, because inter-frequency requirements are not yet agreed for LAA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-frequency event triggered reporting test cases for LAA will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections A.8.a and A.8.b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS3 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*Start of change*

## A.8.a	E-UTRAN FDD - FS3 Inter-frequency Measurements

### A.8.a.1	E-UTRAN FDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells

#### A.8.a.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A4 (Neighbour becomes better than threshold) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.a.1.1-1 and A.8.a.1.1-2 below. In this test, there are two cells on different carrier frequencies. Cell 1 is an FDD cell and neighbouring Cell 2 is an FS3 cell.

In the measurement control information it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.8.a.1.1-1: General test parameters for E-UTRAN FDD-FS3 inter-frequency event triggered reporting in fading propagation conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | One FDD carrier frequency is used for active cell and one FS3 carrier frequency is used for neighbour cell. |
| Active cell |  |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  |  | Cell 2 | Cell 2 is on RF channel number 2 |
| DMTC period |  |  | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration |  |  | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| Gap Pattern Id |  |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | [-105] | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | OFF |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | TBD |  |

Table A.8.a.1.1-2: Cell specific test parameters for E-UTRAN FDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Frame Structure |  | FDD |  | FS3 |  |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 5 MHz:NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  | 20 MHz: NRB,c = 100 |  |
| Uplink-downlink configuration |  | - |  | Note 5 |  |
| LBT model |  | - |  | TBD |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 (R.6 FDD) and A.3.1.1.6 (R.0 FS3) |  | 5 MHz: R.5 FDD 10 MHz: R.0 FDD 20 MHz: R.4 FDD |  | R.0 FS3 |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 (R.6 FDD, R.10 FDD, R.11 FDD) and A.3.1.2.3 (R.0 FS3) |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  | R.0 FS3 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD), A.3.2.1.15 (OP.15 FDD), A.3.2.1.11 (OP.11 FDD) and A.3.2.2.7 (OP.7 TDD) |  | 5 MHz: OP.15 FDD 10 MHz: OP.1 FDD 20 MHz: OP.11 FDD |  | OP.7 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | [-98] |  | [-98] |  |
| RSRP Note 4 | dBm/15 kHz | [-94] | [-94] | -inf | [-91] |
|  | dB | [4] | [4] | -inf | [7] |
| SCH_RP Note 4 | dBm/15 kHz | [-94] | [-94] | -inf | [-91] |
|  | dB | [4] | [4] | -inf | [7] |
| Io | dBm/Ch BW | [-64.76] +10log (NRB,c /50) | [-64.76] +10log (NRB,c /50) | [-70.22] | [-62.43] |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Downlink only configuration |  |  |  |  |  |

#### A.8.a.1.2	Test Requirements

The UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than TBD ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

*End of change*

*Start of change*

## A.8.b	E-UTRAN TDD - FS3 Inter-frequency Measurements

### A.8.b.1	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells

#### A.8.b.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A4 (Neighbour becomes better than threshold) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.b.1.1-1 and A.8.b.1.1-2 below. In this test, there are two cells on different carrier frequencies. Cell 1 is an TDD cell and neighbouring Cell 2 is an FS3 cell.

In the measurement control information it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.8.b.1.1-1: General test parameters for E-UTRAN TDD-FS3 inter-frequency event triggered reporting in fading propagation conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | One TDD carrier frequency is used for active cell and one FS3 carrier frequency is used for neighbour cell. |
| Active cell |  |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  |  | Cell 2 | Cell 2 is on RF channel number 2 |
| DMTC period |  |  | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration |  |  | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| Gap Pattern Id |  |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | [-105] | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | OFF |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | TBD |  |

Table A.8.b.1.1-2: Cell specific test parameters for E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Frame Structure |  | TDD |  | FS3 |  |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz:NRB,c = 100 |  | 20 MHz:NRB,c = 100 |  |
| Special subframe configuration |  | 6 |  | Note 5 |  |
| Uplink-downlink configuration |  | 1 |  | Note 5 |  |
| LBT model |  | - |  | TBD |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 (R.3 TDD) and A.3.1.1.6 (R.0 FS3) |  | R.3 TDD |  | R.0 FS3 |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 (R.6 TDD, R.10 TDD, R.11 TDD) and A.3.1.2.3 (R.0 FS3) |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | R.0 FS3 |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) , A.3.2.2.9 (OP.9 TDD) and A.3.2.2.7 (OP.7 TDD) |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  | OP.7 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | [-98] |  | [-98] |  |
| RSRP Note 4 | dBm/15 kHz | [-94] | [-94] | -inf | [-91] |
|  | dB | [4] | [4] | -inf | [7] |
| SCH_RP Note 4 | dBm/15 kHz | [-94] | [-94] | -inf | [-91] |
|  | dB | [4] | [4] | -inf | [7] |
| Io | dBm/Ch BW | [-64.76] +10log (NRB,c /50) | [-64.76] +10log (NRB,c /50) | [-70.22] | [-62.43] |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Downlink only configuration |  |  |  |  |  |

#### A.8.b.1.2	Test Requirements

The UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than TBD ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

*End of change*