3GPP TSG-RAN4 Meeting AH#2 	R4-1808657

# Montreal, Canada, 2nd to 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR37.843 Compact Antenna Test Range for OTA in channel selectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-0 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The text capturing detailed procedures in CATR for in channel selectivity |  |  |  |  |  |  |  |  |
| Summary of change: |  | All changes highlighted in yellow. Conformance test procedure and MU budget are included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Detailed test procedure descriptions not fully clear and correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 10.3.7.3	CATR

##### 10.3.7.3.1	General

This method measures the in channel selectivity in compact antenna test range with a sufficient separation between reference point of the AAS BS and the transmitting antenna by the range antenna reflector. The measurement system setup is as depicted in figure 10.3.7.3.1-1.

Figure 10.3.7.3.1-1: CATR measurement system setup for in channel selectivity

##### 10.3.7.3.2 	Calibration

Calibration shall be done with the procedure shown in 10.3.2.3.2 for the frequencies of wanted and interfering signals with power combiner/coupler. Calculate the calibration value for wanted and interfering signal from A to B as well as from A to D.

LWanted_cal, A→B:  Calibration value for wanted signal between A and B in figure 10.3.7.3.1-1.

Linterferer_cal, A→D:  Calibration value for interfering signal between A and D in figure 10.3.7.3.1-1.

##### 10.3.7.3.3 	Procedure

- Set up AAS BS in place of SGH from calibration stage. Align AAS BS with boresight of transmitting antenna.

- Configure RF signal generator for wanted signal to set it at the required level plus LA→B and at one side of the frequency center of reference measurement channel.

- Configure RF signal generator for interferer to set it at the required level plus LA→E and at the other side of the frequency center of reference measurement channel. The interferer should be adjacent to the wanted signal.

- Measure the throughput of the wanted signal according to annex E of the 3GPP TS 36.141 [??].

- Repeat the steps 2-4 by exchange the locations of wanted signal and interfering signal in frequency domain at the same channel.

- Repeat steps above per involved band for mulit-band RIB(s).

##### 10.3.7.3.4 	MU assessment

###### 10.3.7.3.4.1 	MU Budget

###### The MU budget for ICS is the same as that for RX ACS, described in section 10.3.5.3.4.1

###### 10.3.7.3.4.2 	MU Value

###### The MU value for ICS is the same as that for RX ACS, described in section 10.3.5.3.4.1.

#### 10.3.7.x	Summary

{editors note: Last section at this level summarises MU estimates for all chambers and concludes final agreed MU}