3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf	R4-1808677

Busan, Korea, 21 – 25 May 2018

Agenda Item:	5.1.4.3.6

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.141-2: NR BS OTA manufacturers declarations for radiated test requirements (4.6)

Document for:	Approval

# 1.	Introduction

The TPs to TS 38.141-2 [1] for Rel-15 NR BS OTA conformance testing have been approved in RAN4#86Bis and RAN4#87, and the draft CR to TS 37.145-2 for Rel-15 AAS BS OTA manufacturers declarations for radiated test requirements has been endorsed in RAN4#87 [2], while the corresponding manufacturers declarations in TS 38.141-2 have yet to be filled in. Moreover, according to the way forward agreed in RAN4#87 [3], manufacturer will declare the worst-case side of the AAS BS on which the co-location test antenna is placed and test will be done only on the declared side

This contribution provides a text proposal to fill in the manufacturers declarations for Rel-15 NR BS OTA manufacturers declarations for radiated test requirements in TS 38.141-2, based on the approved TPs for Rel-15 NR BS OTA conformance testing, the endorsed draft CR for Rel-15 AAS BS OTA manufacturers declarations for radiated test requirements, and the agreed way forward on co-location testing.

# 2.	Text proposal

## 4.6	Manufacturer’s declarations

The following manufacturer’s declarations are required for radiated requirements for BS type 1-H, BS type 1-O and BS type 2-O.

Table 4.6-1 Manufacturers declarations common for BS type 1-H, BS type 1-O and BS type 2-O

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D9.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the NR BS enclosure. |
| D9.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the NR BS enclosure. |
| D9.3 | Beam identifier | A unique title to identify a beam. e.g. a, b, c or 1,2,3. The vendor may declare any number of beams, the minimum requirement to declare for conformance are the beams with the highest intend EIRP for each of the beams widths below: 1) Narrowest intended BeWθ, narrowest intended BeWϕ (possible when narrowest intended BeWθ) at the reference beam direction. 2) Narrowest intended BeWϕ, narrowest intended BeWθ (possible when narrowest intended BeWϕ) at the reference beam direction. 3) Widest intended BeWθ, widest intended BeWϕ (possible when widest intended BeWθ at the reference beam direction. 4) Widest intended BeWϕ – widest intended BeWθ (possible when widest intended BeWϕ) at the reference beam direction. 5) BeWθ and BeWϕ which provide highest intended EIRP of all possible beams at the reference beam direction. NOTE 1: Depending on the capability of the system some of these beams may be the same. When selecting the above five beam widths for declaration, all beams that the NR BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. |
| D9.4 | Beam operating band support | NR operating band(s) supported by the beam, declared for every beam identified in D9.3. |
| D9.5 |  |  |
| D9.6 | Beam NR BS channel band width support | NR BS channel bandwidth supported for each beam and supported operating band identified in D9.3. |
| D9.7 | Reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam |
| D9.8 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam identified in D9.3. |
| D9.9 | Maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam identified in D9.3. |
| D9.10 | Rated beam EIRP | The rated EIRP level per carrier at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D9.9), as well as the reference beam direction pair (D9.7). Declared for every beam identified in D9.3. |
| D9.11 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam identified in D9.3. |
| D9.12 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D9.4-D9.11) made for the beams are identical and the transmitter unit and associated TAB connector(s), RDN and antenna array responsible for generating the beam are of identical design. |
| D9.13 | Parallel beams | List of beams which have been declared equivalent (D9.12) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. |
| D9.14 | Number of carriers at maximum EIRP | The number of carriers per operating band the NR BS is capable of generating at maximum EIRP declared for every beam identified in D9.3. |
| D9.15 | Multi-band transceiver units | Declared if an operating band is generated using Transceiver units supporting operation in multiple operating bands through common active RF components(s). |
| D9.16 | Operating bands with multi-band dependencies | List operating bands which are generated by multi-band transceiver units. Declared for each operating band for which multi-band transceiver units (D9.15) have been declared, |
| D9.15 | Maximum radiated Base Station RF Bandwidth | Largest Base Station RF Bandwidth in the operating band, declared for each supported operating band identified in D9.4. |
| D9.18 | Maximum radiated Base Station RF Bandwidth for contiguous operation. | Largest Base Station RF Bandwidth for contiguous spectrum operation, declared for each supported operating band identified in D9.4. |
| D9.19 | Maximum radiated Base Station RF Bandwidth for non- contiguous operation. | Largest Base Station RF Bandwidth for non-contiguous spectrum operation, declared for each supported operating band identified in D9.4. |
| D9.20 | Inter-band CA bands | Declared inter-band CA bands supported per operating band identified in D9.4. |
| D9.21 | CA only operation | Declared per operating band identified in D9.4. |
| D9.22 |  |  |
| D9.23 | Reduced number of supported carriers at maximum EIRP in multi-RAT operations | Declared for each supported operating band identified in D9.4. |
| D9.24 | Reduced maximum EIRP at the total number of supported carriers in multi-RAT operations | Declared for each supported operating band identified in D9.4. |
| D9.25 | Radiated capability set (RCSA) | The manufacturer shall declare the supported radiated capability set(s) according to table 4.9-1 for each supported operating band identified in D9.4. |
| D9.26 | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D9.16) |
| D9.27 | Total number of supported carriers for operating bands with multi-band dependencies | Total number of supported carriers for operating bands declared to have multi-band dependencies (D9.16). |
| D9.28 | OTA contiguous or non-contiguous spectrum support | Ability of NR BS to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier in an operating band. |
| D9.29 | OTA contiguous and non-contiguous parameters identical | Contiguous and non-contiguous operation is possible then parameters are the same. |
| D10.1 | OSDD identifier | A unique identifier for the OSDD. |
| D10.2 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD identified in D10.1. NOTE 2: As each identified OSDD has a declared minimum EIS value (D10.6), multiple operating band can only be declared if they have the same minimum EIS declaration. |
| D10.3 |  |  |
| D10.4 | OTA sensitivity supported NR BS channel BW | The NR BS channel BW's supported by each OSDD. |
| D10.5 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD. NOTE 3: Not applicable for BS type 2-O. |
| D10.6 | Minimum EIS | The minimum EIS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared for every supported RAT and E-UTRA supported channel BW for the OSDD. Declared per OSDD identified in D10.1. The lowest EIS value for all the declared OSDD’s is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. NOTE 4: If the BS (type 1-H or type 1-O) is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. |
| D10.7 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D10.9) for each OSDD. |
| D10.8 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. NOTE 5: Not applicable for BS type 2-O. |
| D10.9 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D10.8). NOTE 6: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. |
| D10.10 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D10.11). |
| D10.11 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. |

Table 4.6-2 Manufacturers declarations specific for BS type 1-O and BS type 2-O

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| Dxx.1 | The rated carrier OTA BS power, Pmax,c,TRP | Pmax,c,TRP, is declared as TRP OTA power per carrier, declared per supported operating band, per supported RAT. |
| Dxx.2 | NR BS requirements set | Declaration of either hybrid NR BS architecture conforming to the hybrid requirement set, or OTA NR BS architecture conforming to the OTA requirement set. |
| Dxx.3 | OTA coverage range | Declared as OTA NR BS common range of directions within which selected TX OTA requirements are intended to be met. [add description based on Conformance test directions declaration] NOTE: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error, TAE or EVM. |
| Dxx.4 | Worst-case side of the AAS BS on which the co-location test antenna is placed | Declare the worst-case side of the AAS BS on which the co-location test antenna is placed and test will be done only on the declared side. |
| Dxx.5 |  |  |
| Dxx.6 | [Equivalent passive antenna gain] | Editor’s note: name of this declaration (as well as related wording in the specification) are subject to further study. Declared value of the equivalent gain (to the OTA NR BS) of the passive antenna system, used for translation of regional regulatory requirements for unwanted emissions (i.e. EIRP value using an assumed passive antenna gain). |
| Dxx.7 | Spurious emission category | [To be defined based on its conduced version in D6.3] |
| Dxx.8 | Geographic area support | [To be defined based on its conduced version in D6.4] |
| Dxx.9 | Band 20 or Band XX support, operating in geographical areas allocated to broadcasting (DTT) | [To be defined based on its conduced version in D6.5] |
| Dxx.10 | Band 20 or Band XX support, emission level for channel N (PEM,N) | [To be defined based on its conduced version in D6.6] |
| Dxx.11 | Band 20 or Band XX support, Maximum output Power in 10 MHz (P10MHz) | [To be defined based on its conduced version in D6.7] |
| Dxx.12 | Band 32 or Band XXXII support, Declared emission level in Band 32/XXXII (PEM,B32,ind) | [To be defined based on its conduced version in D6.8] |
| Dxx.13 | Co-existence with other systems | [To be defined based on its conduced version in D6.10] |
| Dxx.14 | Co-location with other base stations | [To be defined based on its conduced version in D6.11] |
| Dxx.15 | Single-band RIB or multi-band RIB | [To be defined based on its conduced version in D6.13] |
| Dxx.16 | Single or multiple carrier | [To be defined based on its conduced version in D6.23] |
| Dxx.17 | Maximum number of supported carriers per band | [To be defined based on its conduced version in D6.24] |
| Dxx.18 | Total maximum number of supported carriers | [To be defined based on its conduced version in D6.25] |
| Dxx.19 | Other band combination multi-band restrictions | [To be defined based on its conduced version in D6.28] |
| Dxx.20 | Ncells | [To be defined based on its conduced version in D6.38] |
| Dxx.21 | Maximum supported power difference between carriers | [To be defined based on its conduced version in D6.39] |
| Dxx.22 | Maximum supported power difference between carriers is different operating bands | [To be defined based on its conduced version in D6.40] |
| Dxx.23 | OTA NR BS operating band combination support | [To be defined based on its conduced version in D6.41] |
| Dxx.24 | UTRA FDD MIMO support | [To be defined based on its conduced version in D6.51] |
| Dxx.25 | UTRA Inner loop power control dynamic range | [To be defined based on its conduced version in D6.57] |
| Dxx.26 | Inter-band CA or HSDPA bands | [To be defined based on its conduced version in D6.59] |
| Dxx.27 | Intra-band contiguous CA or HSDPA | [To be defined based on its conduced version in D6.60] |
| Dxx.28 | Intra-band non-contiguous CA or HSDPA | [To be defined based on its conduced version in D6.61] |
| Dxx.29 | OTA REFSENS RoAoA | The REFSENS RoAoA associated with the receiver target reference direction (Dxx.30). |
| Dxx.30 | OTA REFSENS receiver target reference direction | An associated direction inside the OTA REFSENS RoAoA (Dxx.29). |
| Dxx.31 | OTA REFSENS conformance test directions | Four conformance test directions for the OTA REFSENS: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. |

# References

[1]	R4-180xxxx, “TS 38.141-2 v0.2.0”, Huawei.

[2]	R4-1808386, “Draft CR to TS 37.145-2: manufacturers declarations (4.10), Rel-15”, Huawei.

[3]	R4-1808421, “WF on co-location testing”, Nokia, Nokia Shanghai Bell.