3GPP TSG-RAN WG4 Meeting AH-1807                	R4-1808778

Montreal, Canada, July 2 – 6 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on MG sharing

Agenda item:	5.2.3.4

Document for:	Discussion

Introduction

RAN4 is discussing the UE behavior for different cases of collision among MG, SMTC and RLM-RS, and the corresponding UE measurement requirements. In RAN4#86bis, WF [1] is agreed and the UE behavior for most of the cases are defined. In RAN4#87, UE measurement behavior in all the remaining cases are agreed. Following are agreements from RAN4#87 Chairman Notes.

- UE behaviour for type A/B measurements - FR1 - When FR1 type A/B measurements and measurement gap are fully colliding, the type A/B measurement is treated in the same way as a type C measurement (i.e., gap sharing) - FR2 - For scenario 2b, Option 1-1: “Intra frequency measurement could be conducted only outside of MG” is selected - When FR2 type B measurements and measurement gap are fully colliding, the type B measurement is treated in the same way as a type C measurement (i.e., gap sharing)

However, one major issue that remains open is the exact MG sharing scheme. In RAN4#87, RAN4 confirms the earlier agreement that the sharing scheme is network configurable and asks RAN2 to introduce related signaling support [2]. In addition, RAN4 also agrees that when per FR gaps are configured, sharing parameters can be configured independently for each FR. In RAN4#87, the 4 sharing schemes are also agreed [3] although how the configured sharing impacts the performance requirements of gap based measurements are FFS.

In this paper, we will provide our views on MG sharing and the impact to gap based measurement requirements.

Discussion

In [3] it is specified that

Measurement gap sharing shall be applies when UE requires measurement gaps to identify and measure intra-frequency cells or when SMTC configured for intra-frequency measurement are fully overlapping with measurement gaps, and when UE is configured to identify and measure cells on inter-frequency carriers and inter-RAT carriers. When network signals “01”, “10” or “11”, where X is a signalled RRC parameter TBD and is defined as in Table 9.1.2-5, - the performance of intra-frequency measurements with no measurement gaps as specified in section 9.2.5, when SMTC configured for intra-frequency measurement are fully overlapping with measurement gaps, shall consider the factor Kintra = 1 / X * 100, - the performance of intra-frequency measurements with measurement gaps as specified in section 9.2.6 shall consider the factor Kintra = 1 / X * 100, - the performance of inter-frequency measurement as specified in section 9.3 and the performance of inter-RAT measurement as specified in section 9.4 shall consider the factor Kinter = 1 / (100 – X) * 100, When network signals “00” indicating equal splitting gap sharing, X is not applied and the performance of intra-frequency measurements as specified in section 9.2.5 and section 9.2.6, the performance of inter-frequency measurement as specified in section 9.3 and the performance of inter-RAT measurement as specified in section 9.4 are FFS.

The condition when MG sharing applies is clear:

- UE is configured with inter-frequency measurement or inter-RAT measurement, and

- UE is configured with one of the following gap based intra-frequency measurement

  - Intra-frequency measurement requiring MG

  - Intra-frequency measurement with SMTC fully overlapping with MG

What is unclear is how MG sharing impacts the performance requirements of respectively measurements. For example, when X = 25%, 50% or 75% is indicated (corresponding to “01”, “10” or “11”) the sharing factor K shall be considered in the requirements, but how it should be considered needs further discussion. Also, when “equal split” is indicated (corresponding to “00”), the requirements are left as FFS.

In order to decide how MG sharing impacts the performance requirements of respectively measurements, one important issue is in which MG occasions the MG sharing applies. For a specific MG occasion, there could be Y gap based intra-frequency layers whose SMTC are present in the occasion, and Z inter-frequency and inter-RAT layers whose SMTC are present in the occasion. Due to different SMTC on intra- and inter-frequency/RAT layers, Y and Z can each range from zero to the maximum number of intra-frequency and inter-frequency/RAT layers that can be configured to UE. There are two options:

- Option 1: MG sharing applies to all MG occasions

- Option 2: MG sharing applies to MG occasions where Y>0 and Z>0

Option 2 is clearly more reasonable since if in a MG occasion there is no intra-frequency or inter-frequency/RAT layer with SMTC present in the occasion, UE can dedicate 100% of the occasion for the other type of measurement.

MG sharing applies only in MG occasions where there is at least one intra-frequency layer (with gap based measurement) and one inter-frequency or inter-RAT layer with SMTC present in the occasion.

The next issue is how the MG sharing is impacting the intra- and inter-frequency measurement requirements. It of course depends on how intra- and inter-frequency measurement requirements are defined without considering MG sharing, which will be addressed in our companion paper [4].

On the other hand, a high-level principle can be defined following the same principle as in LTE eMTC. In a MG occasion where MG sharing applies, if X is numerical number (25%, 50% or 75%), all intra-frequency layers will equally share X percent of this MG occasion, and all inter-frequency and inter-RAT layers will equally share (1-X) of this MG occasion; if X is ‘equal split’, all intra- and inter-frequency and inter-RAT layers will equally share the 100% of this MG occasion.

In a MG occasion where MG sharing applies,

- if X is numerical number (25%, 50% or 75%), all intra-frequency layers (with gap based measurement) will equally share X percent of this MG occasion, and all inter-frequency and inter-RAT layers will equally share (1-X) of this MG occasion;

- if X is ‘equal split’, all intra- and inter-frequency and inter-RAT layers will equally share the 100% of this MG occasion.

Conclusion

In this paper, we provided our views on MG sharing and the impact to gap based measurement requirements.

Proposal 1: MG sharing applies only in MG occasions where there is at least one intra-frequency layer (with gap based measurement) and one inter-frequency or inter-RAT layer with SMTC present in the occasion.

Proposal 2: In a MG occasion where MG sharing applies,

- if X is numerical number (25%, 50% or 75%), all intra-frequency layers (with gap based measurement) will equally share X percent of this MG occasion, and all inter-frequency and inter-RAT layers will equally share (1-X) of this MG occasion;

- if X is ‘equal split’, all intra- and inter-frequency and inter-RAT layers will equally share the 100% of this MG occasion.

References

- R4-1805541, WF on collision issue among RLM-RS, SMTC and measurement gap, NTT DOCOMO

- R4-1807972, LS on signalling for gap sharing, RAN4

- R4-1808435, CR on gap sharing, Huawei, HiSilicon

- R4-180xxxx, Discussion on inter-frequency measurement requirements, Nokia, Nokia Shanghai Bell