# GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf                                     R4-1808820

# Montreal, Canada, 2th – 6th July, 2018

Source: 	ZTE Corporation

Title: 	TP to TS 38.141-1: Test configuration(Sections 4.7)

Agenda Item:	5.1.3

Document for:	Approval

1	Introduction

This contribution provide a TP to TS38.141-1[1] for test configuration in section 4.7 based on [2] and [3].

2	Reference

[1] 38.141-1 v0.3.0

[2] R4-1808818, On typical channel bandwidth for NR test configuration, ZTE Corporation

[3] R4-1808819, On NR OBW test configuration for Contiguous CA, ZTE Corporation

--------------Start of TP  -------------

4.7	Test configurations

The test configurations shall be constructed using the methods defined below, subject to the parameters declared by the manufacturer for the supported RF configurations as listed in subclause 4.6. The test configurations to use for conformance testing are defined for each supported RF configuration in subclause 4.8.3 and 4.8.4.

The applicable test models for generation of the carrier transmit test signal are defined in subclause 6.1.1.

The values of typical bandwidth for the test signal configurations are defined in table 2.2-1 for the NR operating bands.

Table 4.7-1: Typical bandwidth values

| Operating band characteristics | Typical bandwidth [MHz] |
| --- | --- |
| FDL_high – FDL_low < 100 MHz | 5 |
| FDL_high – FDL_low ≥100 MHz | 20 |

4.7.1		NRTC1: NR Contiguous spectrum operation

The purpose of test configuration NRTC1 is to test all BS requirements excluding CA occupied bandwidth.

For NRTC1 used in receiver tests only the two outermost carriers within each supported operating band need to be generated by the test equipment.

4.7.1.1		NRTC1 generation

NRTC1 shall be constructed on a per band basis using the following method:

-	Declared maximum Base Station RF Bandwidth supported for contiguous spectrum operation of the single-band connector shall be used;

-	Select the narrowest supported carrier with the smallest supported SCS and place it adjacent to the lower Base Station RF Bandwidth edge. Place a typical bandwidth carrier with the smallest supported SCS adjacent to the upper Base Station RF Bandwidth edge.

-	For transmitter tests, select as many typical bandwidth carriers with the smallest supported SCS that the BS supports within a band, and fit in the rest of the declared maximum Base Station RF Bandwidth. Place the carriers adjacent to each other starting from the upper Base Station RF Bandwidth edge. The nominal carrier spacing shall apply;

-	If the typical bandwidth carriers are not supported by the BS the narrowest supported BS channel bandwidth shall be selected instead.

The test configuration should be constructed on a per band basis for all component carriers of the inter-band CA bands declared to be supported by the single-band connector  and are transmitted using the same antenna port. All configured component carriers are transmitted simultaneously in the tests where the transmitter should be on.

4.7.1.2		NRTC1 power allocation

For a BS declared to support MC operation,

Set the power of each carrier to the same level so that the sum of the carrier powers equals the rated total output power Prated,t according to the manufacturer’s declaration.

For a single-band connector declared to support only CA operation,

Set the power spectral density of each carrier to the same level so that the sum of the carrier powers equals the rated total output power Prated,t according to the manufacturer’s declaration.

4.7.2			NRTC2: Contiguous CA occupied bandwidth

NRTC2 in this subclause is used to test CA occupied bandwidth.

4.7.2.1		NRTC2 generation

The CA specific test configuration should be constructed on a per band basis using the following method:

-	All component carrier combinations supported by the single-band connector, which have different sum of  BS channel bandwidth of component carrier, shall be tested. For all component carrier combinations which have the same sum of BS channel bandwidth of component carriers, only one of the component carrier combinations shall be tested.

-	Of all component carrier combinations which have same sum of BS channel bandwidth of component carrier, select those with the narrowest carrier with the smallest supported SCS at the lower Base Station RF Bandwidth edge.

-	Of the combinations selected in the previous step, select one with the narrowest carrier with the smallest supported SCS at the upper Base Station RF Bandwidth edge.

-	If there are multiple combinations fulfilling previous steps, select the one with the smallest number of component carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier with the smallest supported SCS being adjacent to the lowest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier with the smallest supported SCS being adjacent to the highest carrier

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier with the smallest supported SCS being adjacent to the carrier which has been selected in the previous step.

-	If there are multiple combinations fulfilling previous steps, repeat the previous step until there is only one combination left.

-	The nominal carrier spacing shall apply.

4.7.2.2		NRTC2 power allocation

Set the power spectral density of each carrier to be the same level so that the sum of the carrier powers equals the rated total output power Prated,t for NR according to the manufacturer’s declaration.

4.7.3			NRTC3: NR Non-contiguous spectrum operation

The purpose of NRTC3 is to test all BS requirements excluding CA occupied bandwidth.

For NRTC3 used in receiver tests, outermost carriers for each sub-block need to be generated by the test equipment.

4.7.3.1		NRTC3 generation

NRTC3 is constructed on a per band basis using the following method:

-	The Base Station RF Bandwidth shall be the maximum Base Station RF Bandwidth supported for non-contiguous spectrum operation of the single-band connector. The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum supported Base Station RF Bandwidth.

-	For transmitter tests, place a typical bandwidth carrier with the smallest supported SCS  adjacent to the upper Base Station RF Bandwidth edge and a typical bandwidth carrier with the smallest supported SCS adjacent to the lower Base Station RF Bandwidth edge. If the typical bandwidth carriers are not supported by the BS, the narrowest supported BS channel bandwidth shall be selected instead.

-	For receiver tests, place a typical bandwidth carrier with the smallest supported SCS adjacent to the upper Base Station RF Bandwidth edge and a typical bandwidth carrier with the smallest supported SCS  adjacent to the lower Base Station RF Bandwidth edge.  If the typical bandwidth carriers are not supported by the BS, the narrowest supported BS channel bandwidth shall be selected instead.

-	For single-band operation receiver tests, if the remaining gap is at least 15 MHz plus two times the BS channel bandwidth for the typical bandwidth of 5MHz or 60MHz plus two times the BS channel bandwidth for the typical bandwidth of 20MHz used in the previous step and the single-band connector supports at least 4 NR carriers, place a NR carrier of this BS channel bandwidth with the lowest supported SCS adjacent to each already placed carrier for each sub-block. The nominal carrier spacing shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset for the carrier adjacent to the sub-block gap.

4.7.3.2		NRTC3 power allocation

Set the power of each carrier to the same level so that the sum of the carrier powers equals the rated total output power Prated,t according to the manufacturer’s declaration.

4.7.4			NRTC4: NR multi-band test configuration for full carrier allocation

The purpose of NRTC4 is to test multi-band operation aspects considering maximum supported number of carriers.

4.7.4.1		NRTC4 generation

NRTC4 is based on re-using the existing test configuration applicable per band on multi-band connector. It is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth of the multi-band connector.

-	The number of carriers of each supported operating band shall be the declared maximum number of supported carriers of the multi-band connector. Carriers shall first be placed at the outermost edges of the declared Maximum Radio Bandwidth for outermost bands and at the Base Station RF Bandwidths edges for middle band(s) if any. Additional carriers shall next be placed at the Base Station RF Bandwidths edges, if possible.

-	The allocated Base Station RF Bandwidth of the outermost bands shall be located at the outermost edges of the declared Maximum Radio Bandwidth.

-	Each concerned band shall be considered as an independent band and the carrier placement in each band shall be according to NRTC1, where the declared parameters for multi-band operation shall apply. The mirror image of the single-band test configuration shall be used in each alternate band(s) and in the highest band being tested for the multi-band connector to ensure a narrowband carrier being placed at both edges of the Maximum Radio Bandwidth.

-	If only one carrier can be placed for the concerned band(s), the carrier(s) shall be placed at the outermost edges of the declared maximum radio bandwidth for outermost band(s) and at one of the outermost edges of the supported frequency range within the Base Station RF Bandwidths for middle band(s) if any.

-	If the sum of the maximum Base Station RF Bandwidths of each supported operating bands is larger than the declared Total RF Bandwidth of transmitter and receiver for the declared band combinations of the multi-band connector, repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the Total RF Bandwidth BWtot of transmitter and receiver is not exceeded and vice versa.

-	If the sum of the maximum number of supported carrier of each supported operating bands of the multi-band connector  is larger than the declared total number of supported carriers for the declared band combinations of the NR BS, repeat the steps above for test configurations where in each test configuration the number of carriers of one of the operating band shall be reduced so that the total number of supported carriers is not exceeded and vice versa.

4.7.4.2		NRTC4 power allocation

Unless otherwise stated, set the power of each carrier in all supported operating bands to the same power so that the sum of the carrier powers equals the total output power according to the manufacturer’s declaration.

If the allocated power of a supported operating band(s) exceeds the declared rated total output power Prated,t of the operating band(s) in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the power allocated for a carrier exceeds the rated output power declared for that carrier, the exceeded power shall, if possible, be reallocated into the other carriers.

4.7.5			NRTC5: NR multi-band test configuration with high PSD per carrier

The purpose of NRTC5 is to test multi-band operation aspects considering higher PSD cases with reduced number of carriers and non-contiguous operation (if supported) in multi-band mode.

4.7.5.1		NRTC5 generation

NRTC5 is based on re-using the existing test configuration applicable per band on multi-band connector operation. It is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth of the multi-band connector.

-	The allocated Base Station RF Bandwidth of the outermost bands shall be located at the outermost edges of the declared Maximum Radio Bandwidth.

-	The maximum number of carriers is limited to two per band.  Carriers shall first be placed at the outermost edges of the declared Maximum Radio Bandwidth for outermost bands and at the Base Station RF Bandwidths edges for middle band(s) if any. Additional carriers shall next be placed at the Base Station RF Bandwidths edges, if possible.

-	Each concerned band shall be considered as an independent band and the carrier placement in each band shall be according to NRTC3, where the declared parameters for multi-band operation shall apply. Narrowest supported NR BS channel bandwidth shall be used in the test configuration.

-	If only one carrier can be placed for the concerned band(s), the carrier(s) shall be placed at the outermost edges of the declared maximum radio bandwidth for outermost band(s) and at one of the outermost edges of the supported frequency range within the Base Station RF Bandwidths for middle band(s) if any.

-	If the sum of the maximum Base Station RF Bandwidth of each supported operating bands is larger than the declared Total RF Bandwidth BWtot of transmitter and receiver for the declared band combinations of the NR BS, repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the Total RF Bandwidth BWtot of transmitter and receiver is not exceeded and vice versa.

4.7.5.2		NRTC5 power allocation

Unless otherwise stated, set the power of each carrier in all supported operating bands to the same power so that the sum of the carrier powers equals the total output power according to the manufacturer’s declaration.

If the allocated power of a supported operating band(s) exceeds the declared rated total output power Prated,t of the operating band(s) in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the power allocated for a carrier exceeds the rated output power declared for that carrier, the exceeded power shall, if possible, be reallocated into the other carriers.

--------------End of TP ------------