3GPP TSG-RAN WG4 Meeting #AH1807                     	  					R4-1808842

Montreal, Canada, July 2nd –6th, 2018

Title:	[DRAFT] LS on missing SSB information

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG2

CC:

Contact Person:

Name:	Tsang-wei Yu

E-mail Address:	ato.yu@mediatek.com

Attachments:

1. Overall Description:

RAN4 would like to inform RAN2 that some of the SSB-related assistance information are missing in current TS 36.331 spec and TS 38.331 spec, as well as RAN4’s suggestion on whether they should be mandatory or optional:

- In TS 38.331,

  - For NR PSCell/SCell addition and handover procedure,

    - The assistance information about SSB offset should be provided in ServingCellConfigCommon.

    - The SCS, periodicity and offset of SSB should be mandatory in ServingCellConfigCommon.

  - For RRC Release with redirection to an inter-frequency NR cell,

    - The SCS of SSB, SMTC periodicity and SMTC offset should be mandatory provided in RedirectedCarrierInfo.

    - SMTC duration can be also optionally provided in RedirectedCarrierInfo to reduce UE power consumption.

- In TS 36.331

  - For RRC Release with redirection to an inter-RAT NR cell,

    - The SCS of SSB, SMTC periodicity and SMTC offset should be mandatory provided in RedirectedCarrierInfo.

    - SMTC duration can be also optionally provided in RedirectedCarrierInfo to reduce UE power consumption.

2. Actions:

To RAN WG2

ACTION: 	RAN4 respectfully asks RAN2 to consider above suggestions in modifying RRC signalling specs.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #88		Aug 20 – 24, 2018			             Gothenburg, Sweden

TSG RAN WG4 Meeting #88b		October 8 – 12, 2018				        Chengdu, China