3GPP TSG-RAN4 Meeting AH-1807 	R4-1809094

# Montreal, Canada, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of sparsity factor for TRP measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-06-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Sparse grids can be used for test time reduction, given a TRP systematic correction factor that needs to be considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduction of sparsity factor |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR does not include possibility to use sparse grid for faster TRP evaluation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 10.9.2	Spherical equal angle grids

#### 10.9.2.1	General

With the spherical equal angle grid, the grid spacing is uniform in the θ and ϕ directions. The range of θ angles from 0 to π is divided into N equally spaced subintervals and the range of ϕ angles from 0 to 2π is divided into M equally spaced subintervals. The width of each subinterval in the θ- and ϕ-angle is given as

Δθ= πN  and ∆ϕ= 2πM

The total number of angular sampling points is equal to (N-1)×M.

Let n and m be the indices used to denote the nth θ and mth ϕ angles, respectively. In pactice, discrete samples of EIRP are measured at each sample point (θn, ϕm) by measuring its two orthogonally polarized components, EIRPp1(θn,ϕm) and EIRPp2(θn,ϕm). The EIRP sample are then used to approximate the definite integral for TRPReference as

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn.

The total number of angular sampling points is equal to (N-1)×M.

Note TRPEstimate = TRPReference as N and M approach ∞.

There is a trade-off between the accuracy of the TRPEstimate and the total number of sampling points. A large number of sampling points leads to long measurement time. Thus, it is important to achieve short measurement time and fulfilling the minimum TRP systematic error. Subclauses 10.9.3.1 and 10.9.3.2 outline the criteria for determining the minimum number of sampling points to characterize. Other means for set the number of sampling points are not precluded.

For each emission frequency, the reference angular steps   Δϕ and Δθ in radians are calculated as:

Δϕref=λ/Dcyl

Δθref=λ/D

, where D and Dcyl are defined in subclause 10.9.2.1.

According to [x] and [y] using a grid with reference angular step is enough for accurate error-free TRP estimation. The reference angular steps are dependent only on the electrical size of the radiation source (i.e., physical size and the emission frequency) and is not affected by the correlation level. When sampling with the reference angular step, fine details of the radiation pattern are not observed, but the TRP value is not affected.

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the sparsity of the grid is defined as

SF=maxΔϕΔϕref,ΔθΔθref

Where Δϕ and Δθ are the actual angular steps used in the measurement.

End of change