3GPP TSG-RAN WG4 Meeting AH-1807	R4-1809167

# Montreal, CA, 2nd – 6th July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | CRNum | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding tables for test tolerance and derivation of test requirements in Annex C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C |  |  |  |  |  |  |  |  |  |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For adding tables for derivation of test requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add tables for derivation of test requirements of transmitter and receiver tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Values of test tolerances for test requirements are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

C.2	Measurement of transmitter

Table C.2-1: Derivation of Test Requirements (Transmitter tests)

| Test | Minimum Requirement in TS 37.105 | Test Tolerance (TT) | Test Requirement in TS 37.145-2 |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | In normal conditions: within ±2.2 dB of manufacturer's rated output power of EIRP In extreme conditions: within ±2.7 dB of manufacturer's rated output power of EIRP | Normal conditions: 1.0 dB, f ≤3.0 GHz 1.2 dB, 3.0 GHz < f ≤3.0 Extreme conditions: x.x dB, f ≤3.0 GHz x.x dB, 3.0 GHz < f ≤3.0 | Formula: Upper limit + TT, Lower limit - TT In normal conditions: within +3.2 dB and -3.2 dB of the manufacturer's rated output power, f ≤ 3.0GHz within +3.4 dB and -3.4 dB of the manufacturer's rated output power, 3.0GHz < f ≤ 4.2GHz In extreme conditions: within +x.x dB and -x.x dB of the manufacturer's rated output power, f ≤ 3.0GHz within +x.x dB and -x.x dB of the manufacturer's rated output power, 3.0GHz < f ≤ 4.2GHz |
| 6.3 OTA Base station maximum output power | Within ±2.0 dB of manufacturer's rated output power as TRP | x.x dB | Formula: Upper limit + TT, Lower limit - TT within +x.x dB and -x.x dB of the manufacturer's rated output power as TRP, f ≤ 3.0GHz within +x.x dB and -x.x dB of the manufacturer's rated output power as TRP, 3.0GHz < f ≤ 4.2GHz |
| 6.3.3 OTA DL RS power | DL RS power shall be within 2.1 dB | 1.3 dB, f ≤ 3.0GHz 1.5 dB, 3.0GHz < f ≤ 4.2GHz | Formula: Upper limit + TT Lower limit - TT DL RS power shall be within 3.4 dB, f ≤ 3.0GHz DL RS power shall be within 3.6 dB, 3.0GHz < f ≤ 4.2GHz |
| 6.4.4 OTA Total power dynamic range | Total power dynamic range (dB): 1.4 MHz E-UTRA: 7.7 3 MHz E-UTRA: 11.7 5 MHz E-UTRA: 13.9 10 MHz E-UTRA: 16.9 15 MHz E-UTRA: 18.7 20 MHz E-UTRA: 20 | 0.4 dB | Formula: Total power dynamic range – TT (dB) 1.4 MHz E-UTRA: 7.3 3 MHz E-UTRA: 11.3 5 MHz E-UTRA: 13.5 10 MHz E-UTRA: 16.5 15 MHz E-UTRA: 18.3 20 MHz E-UTRA: 19.6 |
| 6.5.2 OTA Transmitter OFF power | -106 dBm/MHz | x.x dB, f ≤ 3.0GHz x.x dB, 3.0GHz < f ≤ 4.2GHz | Formula: Minimum Requirement + TT x.x dB, f ≤ 3.0GHz x.x dB, f ≤ 4.2GHz |
| 6.5.3 OTA Transmitter transient period | Transmitter transient period : off to on: 17 us on to off: 17 us | N/A | Minimum Requirement |
| 6.6.2 OTA Frequency error | Frequency error limit ±0.05 ppm | 12 Hz | Formula: Frequency Error limit + TT 0.05 ppm + 12 Hz |
| 6.6.3 OTA Time alignment error | Time alignment error within 65 ns | 25 ns | Formula: Time alignment error limit + TT 90 ns |
| 6.6.4 OTA Modulation quality (EVM) | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % 64QAM: 8 % 256QAM: 3.5% | 1 % | Formula: EVM limit + TT QPSK: 18.5 % 16QAM: 13.5 % 64QAM: 9 % 256QAM: 4.5% |
| 6.7.2 OTA occupied bandwidth | 1.4 MHz 3 MHz 5 MHz 10 MHz 15 MHz 20 MHz Standalone NB-IoT: 200 kHz | 0 kHz | Formula: Minimum Requirement + TT |
| 6.7.5 OTA Operating band unwanted emissions | For Wide Area BS: Category A, bands < 1GHz For 1.4MHz BW: Offsets < 2.8MHz 8dBm to -2dBm / 100kHz Offsets ≥ 2.8MHz -4dBm / 100kHz For 3MHz BW: Offsets < 3MHz 4dBm to -6dBm / 100kHz Offsets ≥ 3MHz -4dBm / 100kHz For 5, 10, 15, 20MHz BW: Offsets < 10MHz 2dBm to -5dBm / 100kHz Offsets ≥ 10MHz -4dBm / 100kHz Category A, bands > 1GHz For 1.4MHz BW: Offsets < 2.8MHz 8dBm to -2dBm / 100kHz Offsets ≥ 2.8MHz -4dBm / 1MHz For 3MHz BW: Offsets < 6MHz 4dBm to -6dBm / 100kHz Offsets ≥ 6MHz -4dBm / 1MHz For 5, 10, 15, 20MHz BW: Offsets < 10MHz 2dBm to -5dBm / 100kHz Offsets ≥ 10MHz -4dBm / 1MHz Category B, bands < 1GHz For 1.4MHz BW: Offsets < 2.8MHz 8dBm to -2dBm / 100kHz Offsets ≥ 2.8MHz -7dBm / 100kHz For 3MHz BW: Offsets < 6MHz 4dBm to -6dBm / 100kHz Offsets ≥ 6MHz -7dBm / 100kHz For 5, 10, 15, 20MHz BW: Offsets < 10MHz -2dBm to -5dBm / 100kHz Offsets ≥ 10MHz -7dBm / 100kHz Category B, bands > 1GHz For 1.4MHz BW: Offsets < 2.8MHz 8dBm to -2dBm / 100kHz Offsets ≥ 2.8MHz -6dBm / 1MHz For 3MHz BW: Offsets < 6MHz 4dBm to -6dBm / 100kHz Offsets ≥ 6MHz -6dBm / 1MHz For 5, 10, 15, 20MHz BW: Offsets < 10MHz 2dBm to -5dBm / 100kHz Offsets ≥ 10MHz -6dBm / 1MHz For Local BS: Category A and B: For 1.4MHz BW: Offsets < 2.8MHz -12dBm to -22dBm / 100kHz Offsets ≥ 2.8MHz -22 dBm/ 100kHz For 3MHz BW: Offsets < 6MHz -16dBm to -26dBm / 100kHz Offsets ≥ 6MHz -26dBm/ 100kHz For 5, 10, 15, 20MHz BW: Offsets < 10MHz -21dBm to -28dBm / 100kHz Offsets ≥ 10MHz -28dBm/ 100kHz For Band 46 for 10 MHz BW: Additional Reqts, bands < 1GHz All BWs: Additional Reqts, bands > 1GHz All BWs: Additional Reqts bands 12,13,14 All BWs: Additional Reqts bands 20 All BWs: | [1.5dB] 0dB [1.5dB] 0dB [1.5dB] 0dB [1.5dB], f ≤ 3.0GHz [1.8dB], 3.0GHz < f ≤ 4.2GHz 0dB [1.5dB], f ≤ 3.0GHz [1.8dB], 3.0GHz < f ≤ 4.2GHz 0dB [1.5dB], f ≤ 3.0GHz [1.8dB], 3.0GHz < f ≤ 4.2GHz 0dB [1.5dB] 0dB [1.5dB] 0dB [1.5dB] 0 dB [1.5dB], f ≤ 3.0GHz [1.8dB], 3.0GHz < f ≤ 4.2GHz 0dB [1.5dB], f ≤ 3.0GHz [1.8dB], 3.0GHz < f ≤ 4.2GHz 0dB [1.5dB], f ≤ 3.0GHz [1.8dB], 3.0GHz < f ≤ 4.2GHz 0dB [1.5dB], f ≤ 3.0GHz [1.8dB], 3.0GHz < f ≤ 4.2GHz 0dB [1.5dB], f ≤ 3.0GHz [1.8dB], 3.0GHz < f ≤ 4.2GHz 0dB [1.5dB], f ≤ 3.0GHz [1.8dB], 3.0GHz < f ≤ 4.2GHz 0dB 0dB 0dB 0dB 0dB | Formula: Minimum Requirement + TT |
| 6.7.6 OTA Transmitter spurious emissions, Mandatory Requirements | Category A 9 kHz f < 150 kHz: -13dBm / 1kHz 150 kHz f < 30 MHz: -13dBm / 10 kHz 30 MHz f < 1 GHz: -13dBm / 100 kHz 1 GHz f < 12.75 GHz: -13dBm / 1 MHz | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.6 OTA Transmitter spurious emissions, Mandatory Requirements | Category B 9 kHz f < 150 kHz: -36dBm / 1 kHz 150 kHz f < 30 MHz: -36dBm / 10 kHz 30 MHz f < 1 GHz: -36dBm / 100 kHz 1 GHz f < 12.75 GHz: -36dBm / 1 MHz | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.6 OTA Transmitter spurious emissions, Protection of BS receiver | -96dBm / 100 kHz |  | Formula: Minimum Requirement + TT |
| 6.7.6 OTA Transmitter spurious emissions, Additional spurious emissions requirements | Levels from -61dBm to -41dBm Bandwidths from 6.25 kHz to 1MHz See TS 36.104 [2] for details |  | Formula: Minimum Requirement + TT |
| 6.7.6 OTA Transmitter spurious emissions, Co-location | Levels from -98dBm to -96dBm Bandwidth 100 kHz See TS 36.104 [2] for details |  | Formula: Minimum Requirement + TT |
| 6.8 OTA Transmitter intermodulation (interferer requirements) This tolerance applies to the stimulus and not the measurements defined in 9.7.3, 9.7.5 and 9.7.6. | [interferer level = PratedC,TRP] | [0 dB] | Formula: [Interferer level + TT] |

C.3	Measurement of receiver

Table C.3-1: Derivation of Test Requirements (Receiver tests)

| Test | Minimum Requirement in TS 37.105 | Test Tolerance (TT) | Test Requirement in TS 37.145-2 |
| --- | --- | --- | --- |
| 7.2 OTA sensitivity | See 3GPP TS37.105 [x], subclause 10.2 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Declared Minimum EIS + TT |
| 7.3 OTA Reference sensitivity level | Reference sensitivity power level: For E-UTRA: -106.8 dBm - OTAREFSENS for 1.4 MHz BW -103.0 dBm - OTAREFSENS for 3 MHz BW -101.5 dBm - OTAREFSENS for 5 MHz BW -101.5 dBm - OTAREFSENS for 10 MHz BW -101.5 dBm - OTAREFSENS for 15 MHz BW -101.5 dBm - OTAREFSENS for 20 MHz BW | f ≤ 3.0GHz 1.3 dB 3.0GHz < f ≤ 4.2GHz 1.4 dB | Formula: Reference sensitivity power level + TT |
| 7.4 OTA Dynamic range | Wanted signal power for Wide Area BS: For E-UTRA: -76.3 dBm - OTAREFSENS for 1.4 MHz BW -72.4 dBm - OTAREFSENS for 3 MHz BW -70.2 dBm - OTAREFSENS for 5 MHz BW -70.2 dBm - OTAREFSENS for 10 MHz BW -70.2 dBm - OTAREFSENS for 15 MHz BW -70.2 dBm - OTAREFSENS for 20 MHz BW Wanted signal power for Local Area BS: -68.3 dBm - OTAREFSENS for 1.4 MHz BW -64.4 dBm - OTAREFSENS for 3 MHz BW -62.2 dBm - OTAREFSENS for 5 MHz BW -62.2 dBm - OTAREFSENS for 10 MHz BW (FRC A2-3) -65.3 dBm - OTAREFSENS for 10 MHz BW (FRC A2-4) -62.2 dBm - OTAREFSENS for 15 MHz BW -62.2 dBm - OTAREFSENS for 20 MHz BW (FRC A2-3) -65.3 dBm - OTAREFSENS for 10 MHz BW (FRC A2-5) |  | Formula: Wanted signal power + TT |
| 7.5 OTA Adjacent channel selectivity (ACS) and narrow-band blocking | Narrowband blocking: Wanted signal power For E-UTRA, all BWs: (EISminSENS + 6 dB) (EISREFSENS + 6 dB) Interferer signal power, all BWs: -49 dBm – (minSENS or OTAREFSENS) for Wide Area BS -44 dBm – (minSENS or OTAREFSENS) for Medium Range BS -41 dBm – (minSENS or OTAREFSENS) for Local Area BS Adjacent channel selectivity: Wanted signal power For E-UTRA,1.4 MHz BW: (EISminSENS + 11 dB) For E-UTRA,3 MHz BW: (EISminSENS + 8 dB) For E-UTRA,5 MHz, 10MHz, 15MHz and 20MHz BW: (EISminSENS + 6 dB) -52dBm – minSENS for Wide Area BS -47 dBm – minSENS for Medium Range BS -44 dBm – minSENS for Local Area BS |  | Formula: Wanted signal power + TT |
| 7.6.2 OTA Blocking (General requirements) | In-band blocking Wanted signal power, all BWs: EISminSENS + x dB EISOTAREFSENS + x dB x is 6 for Wide Area BS and all BS classes of UTRA. 9 for E-UTRA Medium Range BS. 11 for E-UTRA Local Area BS. Interferer signal power, all BWs: -40 - (minSENS or OTAREFSENS) for Wide Area BS -35 - (minSENS or OTAREFSENS) for Medium Range BS -30 - (minSENS or OTAREFSENS) for Local Area BS Out of band blocking Wanted signal power, all BWs: (EISminSENS + 6 dB) Interferer signal power, all BWs: 0.36 V/m CW |  | Formula: Wanted signal power + TT, all BWs: (PREFSENS + 6 dB) Interferer signal power unchanged T-put limit unchanged |
| 7.6.3 OTA Blocking (Co-location with other base stations) | Co-located blocking Wanted signal power, all BWs: (EISminSENS + 6 dB) Interferer signal power, all BWs: +46dBm (Wide Area BS), +38 dBm (Medium Range BS), and +24 dBm (Local Area BS). |  | Formula: Wanted signal power + TT, all BWs: |
| 7.7 OTA Receiver spurious emissions | Frequency range: 30 MHz-1 GHz -48dBm / 100 kHz Frequency range: 1 GHz- -38 dBm/ 1MHz | 0 dB | Formula: Minimum Requirement + TT Emission requirements unchanged |
| 7.8 OTA Receiver intermodulation | Wanted signal power, all BWs: EISminSENS + x dB EISOTAREFSENS + x dB x is 6 for Wide Area BS and all BS classes of UTRA. 9 for E-UTRA Medium Range BS. 12 for E-UTRA Local Area BS. Interferer signal power, all BWs: -48 - (minSENS or OTAREFSENS) for Wide Area BS -44 - (minSENS or OTAREFSENS) for Medium Range BS -38 - (minSENS or OTAREFSENS) for Local Area BS |  | Formula: Wanted signal power + TT, |
| 10.9 OTA In-channel selectivity | Wanted signal power: For E-UTRA: -106.9 dBm – minSENS for 1.4 MHz BW -102.1 dBm – minSENS for 3 MHz BW -100.0 dBm – minSENS for 5 MHz BW -98.5 dBm – minSENS for 10 MHz BW -98.5 dBm – minSENS for 15 MHz BW -98.5 dBm – minSENS for 20 MHz BW |  | Formula: Wanted signal power + TT |