3GPP TSG-RAN WG4 Meeting #AH1807                	R4-1809209

Montreal, Canada, Jul 2nd – 6th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NR PSCell addition delay

Agenda item:	5.2.8.3

Document for:	Discussion

Introduction

The NR PSCell addition delay has been discussed in the past several RAN4 meetings. In RAN4 #87 meeting, Requirement on NR PSCell addition delay was agreed in [1]. However, it has still some open issues for the requirement. In this contribution, we will provide our view on this delay requirement.

Discussion

PSCell addition delay

In the Busan meeting, RAN4 has agreed the requirement on NR PSCell addition delay as below:

Upon receiving NR PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards NR PSCell no later than in subframe n + Tconfig PSCell: Where: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU TRRC_delay is the RRC procedure delay as specified in [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20ms if PSCell is in FR1 or in FR2 and UE provides the measurement report within the last [TBD] ms for the PSCell before the PSCell is added. Otherwise Tprocessing = 40ms. Tsearch is the time for AGC settling and PSS/SSS detection. - For PSCell in FR1: if the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = SMTC periodicity + 5 ms. If the target cell is an unknown inter-frequency cell and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = [TBD*SMTC periodicity + 5] ms; - For PSCell in FR2: if the target cell is an unknown intra-frequency cell and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = [N1*SMTC periodicity + 5] ms. If the target cell is an unkown inter-frequency cell and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = [N1*TBD* SMTC periodicity + 5] ms, else Tsearch = [TBD* SMTC periodicity + 5] ms. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = SMTC periodicity if the PSCell is in FR1. Otherwise T∆ = [TBD]* SMTC periodicity. TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the NR PSCell. TPSCell_ DU is up to x*10 +10 ms. x is defined in the table 6.3.3.2-2 of [6].

PSCell addition procedure is similar as NR handover procedure, since Handover will often happen when is at cell edge, we assume PSCell addition procedure will have better conditions than Handover. Considering this fact and handover requirement, we propose that  Tsearch = 2*SMTC periodicity + 5 ms for unknown PSCell in FR1, and Tsearch = N1*2*SMTC periodicity + 5 ms for unknown PSCell in FR2.

Tsearch = 2*SMTC periodicity + 5 ms for unknown PSCell in FR1

Tsearch = N1*2*SMTC periodicity + 5 ms for unknown PSCell in FR2

Since the time for fine time tracking and acquiring full timing information of the target cell should be no difference in FR1 and FR2, we propose to use the FR1 requirement for FR2.

T∆ = SMTC periodicity in FR2

Proposals are included in our CR [3].

PSCell known condition

In the Busan meeting, RAN4 has agreed the requirement on NR PSCell known condition as below:

In FR1, the NR PSCell is known if it has been meeting the following conditions: During the last 5 seconds before the reception of the NR PSCell configuration command: - the UE has sent a valid measurement report for the NR PSCell being configured and - the NR PSCell being configured remains detectable according to the cell identification conditions specified in section 9.3 of TS 38.133 [50], - NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 9.3 of TS 38.133 [50]. otherwise it is unknown.

In the agreement, the PSCell known condition in FR1 is almost ready, but in FR2, it is left as FFS.

For FR2, since there are some additional consideration as beamforming used in Tx and Rx. We could use the definition in FR1 as baseline and introduce to identify beam issue when a PSCell remains detectable in the cell known condition.

As UE Rx beam forming is related to UE implementation and not testable from radio side, UE Rx beam should not be part of the UE PSCell addition delay requirements. As mentioned above, while the PSCell is configured the UE will continuously measure the PSCell according to the requirements. Such measurements include measuring using UE Rx beam sweeping. I.e. the UE should be able to track the PSCell and which Rx beam to use once the PSCell is being added. For requirements point of view, RAN4 can include a condition that the UE can assume that a Tx beam that transmitted SSB when the UE reported the cell has not changed, and under such condition the cell is and remains detectable.

Exactly how to capture such requirement would need discussions in RAN4. E.g. it does not seem reasonable to declare an PSCell unknown just because UE Rx beam changes between reporting and configuration an additional delay. Another aspect that the requirement should be testable, which may not the case with which UE Rx beam selection.

We propose to extend the definition of known cell in FR1 by assuming that the known cell can be identified using the same Tx beam.

- NR PSCell known condition in FR2 could be:

During the last 5 seconds before the reception of the NR PSCell configuration command:

-	the UE has sent a valid measurement report for the NR PSCell being configured and

-	the NR PSCell being configured remains detectable according to the cell identification conditions specified in section 9.3 of TS 38.133 [50] by any UE Rx beam,

- PSCell remains detectable by any Tx beam being used by the UE

-	NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 9.3 of TS 38.133 [50].

Proposals are included in our CR [3].

Conclusion

In this contribution we have discussed NR PSCell addition delay requirement and PSCell known condition. We have made the following proposals:

- Tsearch = 2*SMTC periodicity + 5 ms for unknown PSCell in FR1

- Tsearch = N1*2*SMTC periodicity + 5 ms for unknown PSCell in FR2

- T∆ = SMTC periodicity in FR2

- NR PSCell known condition in FR2 could be:

During the last 5 seconds before the reception of the NR PSCell configuration command:

-	the UE has sent a valid measurement report for the NR PSCell being configured and

-	the NR PSCell being configured remains detectable according to the cell identification conditions specified in section 9.3 of TS 38.133 [50] by any UE Rx beam,

- PSCell remains detectable by any Tx beam being used by the UE

-	NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 9.3 of TS 38.133 [50].

References

- R4-1807989, CR on TS36.133 for NR PSCell addition delay, Huawei, Hisilicon

- R4-1807894, Discussion on NR SCell known condition, Nokia, Nokia Shanghai Bell

- R4-1809210, CR 36.133 NR PSCell addition, Nokia, Nokia Shanghai Bell