3GPP TSG-RAN WG4 AH-1807 LTE/NR Perf	R4-1809245

Montreal, Canada, July 2-6, 2018

Agenda item:	  5.3.1

Source: 	Qualcomm Incorporated

Title: 	SS SINR Measurements Requirements in NR

Document for:	Discussion

# 1. 	Introduction

In this contribution, we present our analysis and related simulation results for SS SINR measurements for both sub-6 and mmWave bands, similarly to what done in [1] for SS-RSRP and reusing the same simulation assumptions as in [2].

# 2. 	Assumptions

SS SINR measurements simulations have been done according to the assumption in [2] and with these particular cases:

- Synchronous deployment (relative delay of CP/2)

- 1 TX antenna with cross-polarization and 2 RX antennas with ideal receiver beamforming (Rx beam is pointing towards the Rx signal and no beam sweep is simulated)

- Serving cell: SNR = 6 dB and Measured cell SNR = 1 dB

- Single transmitted SS block per SS burst set (K=1)

The selected parameters for subcarrier spacing and propagation conditions are listed directly in Table 1 and Table 2 with the results, as well as reported in the included CDF figures. The SINR value is measured at the baseband (detector) and no implementation margin nor RF impairments have been simulated.

# 3.	Results for 4 GHz

The reported results refer to NR-SSS SINR measurements. It can be observed that for both subcarrier spacing and variable propagation conditions, an accuracy better than ±2dB can be achieved with NR-SSS measurements and a single sample in AWGN channel conditions, while in fading channel conditions up to ±4dB can be achieved with single sample.

Table 1. NR-SSS SINR measurement accuracy for 4 GHz propagation conditions and various average

| Channel | Averaged number of samples | Distribution of delta SINR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 15 kHz |  |  | SCS = 30 kHz |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -1.9 | -0.1 | 1.3 | -1.8 | -0.1 | 1.2 |
|  | 2 | -1.5 | -0.1 | 0.9 | -1.3 | -0.2 | 0.9 |
|  | 3 | -1.4 | -0.1 | 0.8 | -1.2 | -0.2 | 0.8 |
|  | 4 | -1.4 | -0.1 | 0.7 | -1.2 | -0.1 | 0.7 |
|  | 5 | -1.4 | -0.1 | 0.7 | -1.1 | -0.2 | 0.6 |
| EPA5 | 1 | -3.5 | -0.3 | 2.4 | -3.8 | -0.2 | 2.1 |
|  | 2 | -2.9 | -0.3 | 1.8 | -3.0 | -0.3 | 1.5 |
|  | 3 | -2.6 | -0.4 | 1.5 | -2.8 | -0.3 | 1.4 |
|  | 4 | -2.4 | -0.3 | 1.4 | -2.6 | -0.4 | 1.1 |
|  | 5 | -2.2 | -0.4 | 1.3 | -2.4 | -0.4 | 1.1 |
| ETU30 | 1 | -4.3 | -0.5 | 1.8 | -5.1 | -0.6 | 1.9 |
|  | 2 | -3.5 | -0.6 | 1.3 | -4.2 | -0.8 | 1.2 |
|  | 3 | -3.0 | -0.7 | 0.9 | -3.8 | -0.8 | 0.8 |
|  | 4 | -2.9 | -0.7 | 0.7 | -3.3 | -0.9 | 0.7 |
|  | 5 | -2.5 | -0.7 | 0.6 | -3.1 | -0.9 | 0.5 |
| ETU70 | 1 | -4.2 | -0.5 | 2.0 | -4.8 | -0.6 | 2.0 |
|  | 2 | -3.6 | -0.5 | 1.3 | -4.1 | -0.8 | 1.3 |
|  | 3 | -3.0 | -0.6 | 0.9 | -3.6 | -0.8 | 1.0 |
|  | 4 | -2.7 | -0.6 | 0.9 | -3.3 | -0.8 | 0.7 |
|  | 5 | -2.6 | -0.6 | 0.7 | -3.1 | -0.8 | 0.6 |
| TDL-A | 1 | -3.5 | -0.3 | 2.7 | -3.2 | -0.2 | 2.5 |
|  | 2 | -2.7 | -0.3 | 2.2 | -2.6 | -0.2 | 2.1 |
|  | 3 | -2.3 | -0.3 | 1.9 | -2.1 | -0.3 | 1.9 |
|  | 4 | -2.2 | -0.3 | 1.9 | -2.0 | -0.3 | 1.6 |
|  | 5 | -2.1 | -0.3 | 1.6 | -1.9 | -0.2 | 1.7 |
| TDL-B | 1 | -4.0 | -0.2 | 2.4 | -3.4 | -0.2 | 2.3 |
|  | 2 | -3.3 | -0.3 | 2.2 | -2.6 | -0.2 | 1.9 |
|  | 3 | -3.1 | -0.3 | 1.8 | -2.3 | -0.3 | 1.7 |
|  | 4 | -2.7 | -0.3 | 1.7 | -2.2 | -0.2 | 1.7 |
|  | 5 | -2.7 | -0.3 | 1.6 | -2.1 | -0.3 | 1.4 |
| TDL-C | 1 | -3.5 | -0.2 | 2.6 | -3.6 | -0.3 | 2.4 |
|  | 2 | -3.0 | -0.3 | 2.3 | -2.9 | -0.3 | 2.1 |
|  | 3 | -2.5 | -0.3 | 2.0 | -2.5 | -0.3 | 1.6 |
|  | 4 | -2.4 | -0.3 | 1.9 | -2.2 | -0.3 | 1.7 |
|  | 5 | -2.1 | -0.3 | 1.7 | -2.0 | -0.3 | 1.4 |

# 4.	Results for 30 GHz

The reported results refer to NR-SSS SINR measurements. Also for this band, for both subcarrier spacing and variable propagation conditions, an accuracy better than ±4dB can be achieved with NR-SSS measurements and a single sample.

Table 2. NR-SSS SINR measurement accuracy for 30 GHz propagation conditions and various average

| Channel | Averaged number of samples | Distribution of delta SINR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 120 kHz |  |  | SCS = 240 kHz |  |  |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -1.6 | -0.1 | 1.2 | -1.6 | -0.1 | 1.3 |
|  | 2 | -1.2 | -0.1 | 0.8 | -1.2 | -0.1 | 0.9 |
|  | 3 | -1.1 | -0.1 | 0.7 | -1.1 | -0.1 | 0.8 |
|  | 4 | -1.0 | -0.1 | 0.7 | -0.9 | -0.1 | 0.7 |
|  | 5 | -0.9 | -0.1 | 0.6 | -0.9 | -0.1 | 0.6 |
| CDL-A | 1 | -3.7 | -0.2 | 2.4 | -3.5 | -0.1 | 2.6 |
|  | 2 | -2.8 | -0.2 | 1.6 | -2.6 | -0.1 | 1.9 |
|  | 3 | -2.4 | -0.2 | 1.6 | -2.4 | -0.1 | 1.6 |
|  | 4 | -2.1 | -0.2 | 1.3 | -2.2 | -0.2 | 1.6 |
|  | 5 | -1.9 | -0.3 | 1.1 | -1.9 | -0.2 | 1.2 |
| CDL-B | 1 | -4.0 | -0.2 | 2.3 | -3.9 | -0.2 | 2.3 |
|  | 2 | -3.1 | -0.3 | 1.6 | -2.9 | -0.3 | 1.6 |
|  | 3 | -2.8 | -0.3 | 1.3 | -2.5 | -0.3 | 1.3 |
|  | 4 | -2.5 | -0.3 | 1.0 | -2.2 | -0.4 | 1.1 |
|  | 5 | -2.3 | -0.3 | 0.9 | -2.1 | -0.4 | 0.9 |
| CDL-C | 1 | -3.9 | -0.1 | 2.3 | -3.6 | -0.1 | 2.5 |
|  | 2 | -3.1 | -0.2 | 1.8 | -2.8 | -0.2 | 1.7 |
|  | 3 | -2.6 | -0.3 | 1.5 | -2.3 | -0.2 | 1.4 |
|  | 4 | -2.4 | -0.2 | 1.3 | -2.1 | -0.2 | 1.2 |
|  | 5 | -2.5 | -0.2 | 1.2 | -2.0 | -0.2 | 1.0 |
| TDL-A | 1 | -3.2 | -0.1 | 2.4 | -3.1 | -0.1 | 2.5 |
|  | 2 | -2.7 | -0.1 | 2.1 | -2.6 | -0.1 | 2.3 |
|  | 3 | -2.3 | -0.2 | 1.8 | -2.2 | -0.1 | 2.0 |
|  | 4 | -2.2 | -0.2 | 1.8 | -1.9 | -0.1 | 1.7 |
|  | 5 | -1.9 | -0.2 | 1.3 | -2.1 | -0.1 | 1.7 |
| TDL-B | 1 | -3.1 | -0.1 | 2.6 | -2.7 | -0.1 | 2.6 |
|  | 2 | -2.5 | -0.1 | 2.1 | -2.2 | -0.1 | 2.2 |
|  | 3 | -2.1 | -0.2 | 1.9 | -1.8 | -0.2 | 1.9 |
|  | 4 | -2.2 | -0.2 | 1.8 | -1.6 | -0.1 | 1.7 |
|  | 5 | -1.7 | -0.2 | 1.5 | -1.4 | -0.1 | 1.5 |
| TDL-C | 1 | -3.1 | -0.1 | 3.1 | -2.7 | -0.1 | 2.9 |
|  | 2 | -2.4 | -0.1 | 2.7 | -2.1 | -0.1 | 2.6 |
|  | 3 | -2.0 | -0.1 | 2.6 | -1.9 | -0.1 | 2.5 |
|  | 4 | -1.8 | -0.1 | 2.2 | -1.8 | -0.1 | 2.1 |
|  | 5 | -1.7 | -0.1 | 1.8 | -1.5 | -0.1 | 1.9 |

# 5. 	Discussion and Conclusions

In this paper, we provided link level simulation results for SS-based SINR measurements in NR bands, for a number of parameter combinations according to the simulation assumptions agreed in [2], with focus on SSS-SINR.

Observation 1: For both bands, an accuracy better than ±2dB can be achieved for SS-based SINR measurements with NR-SSS measurements and a single sample in AWGN propagation conditions.

Observation 2: For both bands, an accuracy better than ±4dB can be achieved for SS-based SINR measurements with NR-SSS measurements and a single sample in fading propagation conditions.

RAN4 is recommended to take the above results and observations into account, when discussing and defining SS-SINR accuracy requirements for NR.

# 6. 	References

- R4-1804673, “SS RSRP Measurements Requirements in NR”, Qualcomm

- R4-1806551, “Link-level simulation assumptions for SSB-based measurement accuracy”, Ericsson