3GPP TSG-RAN WG4 Meeting #AH-1807	R4-1809270

Montreal, Quebec, Canada, 2nd – 6th July 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On TRP systematic errors for in band TRP test requirements

Agenda item:	4.2.1.1.4.1

Document for:	Discussion

# 1	Introduction

During the RAN4#87 meeting, the WF on TRP systematic error and MU [1] for both OTA AAS and NR base stations was approved, which outlined a list of open issues to be addressed at the forthcoming meetings.

This document discusses the open issue concerning the TRP systematic error for in band requirements. We provide a comprehensive analysis of TRP systematic error for a different number of measurement points. The analysis is based on measurement results obtained from experiments conducted in a CATR chamber using an AAS base station. We conclude with our observations and findings.

# 2	Background

From Rel-15 TS 37.105, the in band TRP requirements are:

- OTA BS maximum output power

- OTA ACLR

- OTA spectrum emission mask

- OTA operating band unwanted emissions

Referring to Rel-15 TR 37.843, the following TRP terms are defined:

| Term | Definition |
| --- | --- |
| TRPReference | theoretical TRP |
| TRPEstimate | numerically approximated TRP |
| TRP systematic error | the relative difference between the theoretical TRP and numerically estimated TRP |

# 3	Discussion

In the far field region, TRPReference can be expressed as

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ															(1)

With the spherical equal angle sampling grid, the elevation angle interval [0, π] and azimuth angle interval 0,2π are discretized into equally spaced angular subintervals. The numerical approximation of Equation (1) is

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn																(2)

where N and M are the number of equally spaced angular subintervals in the elevation and azimuth angular directions, respectively. The subinterval width in the elevation (θ)  and azimuth (ϕ) angles is defined as

Δθ= π-0N     																											(2a)

∆ϕ= 2π-0M  																											(2b)

From Rel-15 TR 37.843, the TRP systemtatic error is defined as

TRPsyserr=TRPEstimate- TRPReference																		(3)

Figures 1 and 2 show the normalized H- and E-plane radiation pattern plots, respectively. The normalized EIRP plots show a beam pattern obtained from OTA measurements in a CATR chamber of an AAS base station. In the OTA measurement, the spherical equal angle sampling grid was used to collect EIRP data samples.

Figure 1: Normalized H-plane EIRP versus ϕ for θ=83°

Figure 2: Normalized E-plane EIRP versus θ for ϕ= 180°

Table 1 shows the TRPEstimate for different azimuth angular subintervals, ∆ϕ while the elevation angular subinterval, ∆θ, was set to 1°.

Table 1: TRP measurements with varying azimuth angular subintervals, ∆ϕ

| ∆ϕ | ∆θ | Number of measurement points (N-1)×M | Normalized TRPEstimate (dBm) | TRP Systematic Error (dB) |
| --- | --- | --- | --- | --- |
| 1º | 1º | 64440 | -22.41 | 0.59 |
| 2° | 1° | 32220 | -22.40 | 0.58 |
| 6° | 1º | 10740 | -22.41 | 0.59 |
| 10° | 1° | 6444 | -22.44 | 0.62 |
| 18° | 1° | 3580 | -22.52 | 0.70 |
| 22° | 1° | 3040 | -22.69 | 0.87 |
| 26° | 1° | 2506 | -22.35 | 0.53 |
| 30° | 1° | 2148 | -21.69 | 0.13 |
| 34° | 1° | 1969 | -22.29 | 0.47 |
| 38° | 1° | 1790 | -40.25 | 2.07 |
| 42° | 1° | 1611 | -21.99 | 0.17 |
| 50° | 1° | 1432 | -31.45 | 9.63 |

Table 2 shows the TRPEstimate for different elevation subintervals ∆θ while the azimuth angular subinterval, ∆ϕ, was set 1°.

Table 2: TRP measurements with varying elevation angular subintervals, ∆θ

| ∆θ | ∆ϕ | Number of measurement points (N-1)×M | Normalized TRPEstimate (dBm) | TRP Systematic Error (dB) |
| --- | --- | --- | --- | --- |
| 1° | 1° | 64440 | -32.63 | 0.59 |
| 2° | 1° | 32040 | -24.86 | 1.82 |
| 4° | 1° | 15840 | -26.37 | 3.33 |
| 8° | 1° | 7920 | -31.83 | 8.79 |

For simplicity, let us consider the reference TRP (TRPReference) to be equal to the normalized conducted output of the AAS base station for the computation of TRP systematic error, assuming the frequency-dependent loss factor is negligible. This is,

TRPReference = 	normalized conducted power

By comparing Figures 1 and 2, the elevation beamwidth is narrower than the azimuth beamwidth.  As shown in Table 2, the TRP systematic error is increased by approximately two-fold as ∆θ is doubled as compared with Table 1.

Observation 1: The impact of narrow beamwidth on the TRP systematic error is greater than wide beamwidth for the same change (increase) in the angular subinterval.

For wide beams, TRP systematic error will only become significant when the angular subinterval is greater than 30° as observed in Table 1.

Observation 2: TRP systematic error is dependent on beam patterns for a given azimuth and elevation angular subinterval.

Observation 3: Narrow beams would require more TRP measurement points than wide beams in order to achieve the same TRP systematic error.

Ideally, TRP systematic error should be derived from OTA measurements by generating a large number of beam patterns with different beamwidths and operating frequencies in which the maximum azimuth and elevation angular subinterval can be determined using the sampling criteria outlined in Section 10.9.2, TR 37.843. However, this approach is not practical because it consumes a significant amount of test time.

Observation 4: Sampling criteria outlined in Section 10.9.2, TR 37.843 can be used to determine the maximum azimuth and elevation angular subintervals for computing the TRP systematic error.

From Tables 1 and 2, we find the following maximum angular subintervals, which satisfy the sampling criteria, are reasonable for computing the TRP systematic error:

∆θ = 10° and

∆ϕ = 2°

This gives TRPEstimate = -23.70 dBm and TRP systematic error = 1.9 dB for operating frequency: f ≤ 3 GHz and 3 GHz < f ≤ 4.2 GHz.

# 3	Conclusions

We have discussed and analysed TRP systematic error using a set of measurements results which were obtained from OTA measurements of an AAS base station in a CATR chamber. Based on our analyses, the following observations can be made:

Observation 1: The impact of narrow beamwidth on the TRP systematic error is greater than wide beamwidth for the same change (increase) in the angular subinterval.

Observation 2: TRP systematic error is dependent on beam patterns for a given azimuth and elevation angular subinterval.

Observation 3: Narrow beams would require more TRP measurement points than wide beams in order to achieve the same TRP systematic error.

Observation 4: Sampling criteria outlined in Section 10.9.2, TR 37.843 can be used to determine the maximum azimuth and elevation angular subintervals for computing the TRP systematic error.

From the outcome of our analyses, a suggested TRP systematic error of 1.9 dB seems to be a reasonable value for the CATR environment operating in the frequencies: f ≤ 3 GHz and 3 GHz < f ≤ 4.2 GHz.

# References

- R4-1808377, WF on OTA TRP conformance requirements, Nokia, Nokia Shanghai Bell, NTT DoCoMo