3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf               	R4-1809365

Montreal, Canada, 2 July – 6 July 2018

Agenda item:	5.4.2.2

Title:	Discussion and Simulation results for NR PUSCH

Source:	Samsung

Document for:	Discussion

# 1	Introduction

During RAN4#87 meeting, the NR BS demodulation performance requirements are further discussed on the general issue and performance requirements test. Some general issue about NR PUSCH are captured and agreed in the WF[1] as follows:

- Transmission scheme

  - Option 1: performance requirements are defined only for 1Tx transmission schemes

  - Option 2: performance requirements are defined only based on codebook-based transmission schemes

- Number of layers for 2Tx CP-OFDM based PUSCH tests

- FFS both 1 layer and 2 layer are to be tested

- DMRS

  - Single-symbol DMRS configuration are tested in Rel-15

- Option 1

- Only 1 front –loaded symbol;

- Option 2

- 1 front-loaded symbol + one additional DMRS symbol

- Option 3

- Only 1 front-loaded symbol, and

- 1 front-loaded symbol + one additional DMRS symbol

- Option 4

- Only 1 front-loaded symbol, and

- 1 front-loaded symbol + one additional DMRS symbol, and

- 1 front-loaded symbol+ two additional DMRS symbols

  - FFS DMRS type (type1 and type2) are to be tested

- PTRS

  - Not introduced PTRS for FR1 tests

  - FFS for FR2

- SRS

  - Not modeled in the tests

- Time domain resource allocation

  - For FR1,slot based transmission is tested, FFS not –slot based transmission

  - For FR2, FFS for slot-based or non-slot based transmission

  - FFS resource mapping type (Type A or type B)

- MCS

  - Selected from QPSK, 16QAM and 64 QAM

  - FFS pi/2-BPSK

- Other

  - Code block group based PUSCH is disable in the tests

  - Frequency hoping is disable in the tests

- FFS: whether to introduce specific test case for frequency hopping

  - Limited buffer rate matching is disable in the tests

- FFS: whether to introduce specific test case for limited buffer rate matching

  - Number of HARQ transmission is 4

- Testing Metric

  - SNR at 70% of maximum throughput of  the FRC

  - Other metric is not precluded for some specific test cases

In order to facilitate simulation alignment of NR BS PUSCH performance between each company, some test cases are introduced for initial alignment. In this contribution, we provide our view on the remained issue of PUSCH performance requirement. Also, the initial simulation results are provided for alignment

# 2	Discussion

## 2.1	UE feature

NR defined the UE feature list. Some UE features are mandatory with / without capability signaling, while some UE features are optional. In this contribution, we just capture the lasted UE feature list related with PUSCH performance requirement, which is agreed in RAN#80 plenary meeting [2], as shown in Table 1

Table 1: UE feature list related with PUSCH

| WI | Feature group | Components | Type | TSG-RAN decision |
| --- | --- | --- | --- | --- |
| System Parameters | 256QAM for PUSCH | 256QAM for PUSCH | Type1 for FR1 Type2 for FR2 | Optional for FR1 Optional for FR2 |
|  | 64QAM for PUSCH | 64QAM for PUSCH | N.A | Mandatory without capability signaling |
|  | Pi/2- BPSK for PUSCH | Pi/2- BPSK for PUSCH | Type 4 | Optional for FR1 Mandatory with capability for FR2 |
|  | Pi/2-BPSK for PUCCH format 3/4 | Pi/2-BPSK for PUCCH format 3/4 | Type 4 | Optional for FR1 Mandatory with capability for FR2 |
|  | CP-OFDM for UL | CP-OFDM for UL | N.A | Mandatory without capability signaling |
|  | DFT-s-OFDM for UL | DFT-s-OFDM for UL | N.A | Mandatory without capability signaling |
| PUSCH | Basic PUSCH transmission | Data RE mapping Single layer (single Tx) transmission | N.A | Mandatory without capability signaling |
|  | Basic uplink DMRS (uplink) for scheduling type A | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbols 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols | N.A | Mandatory without capability signaling |
|  | Basic uplink DMRS for scheduling type B | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol |  | Mandatory without capability signaling |
|  | Support DMRS type (uplink) | 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port | Type 4 | Mandatory with capability signaling |
|  | Support DMRS type (uplink) | Support DMRS {type 1, both type 1 and type 2 } | Type 4 | Support both type 1 and type 2 are mandatory with capability signaling |
|  | Supported 2 symbols front-loaded DMRS (uplink) | 1. Support 2 symbols FL-DMRS | Type 4 | Mandatory with capability signaling |
|  | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) | 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS | Type 4 | Mandatory with capability signaling |
|  | Support 1+3 uplink DMRS symbols(uplink) | 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols | Type 4 | Optional with capability signaling |

In existed LTE 36104, PUSCH performance requirements are defined serval categories. Compared with LTE, there are several categories. In our view, we should focus on the essential performance, where the performance requirement of mandatory UE feature without capability signaling should be defined firstly in Rel-15.

Proposal 1: The performance requirement of NR PUSCH should be prioritized to support UE feature of mandatory with capability signaling in Rel-15.

## 2.2	waveform

As for UL waveform, NR defines two kinds of waveforms, i.e. CP-OFDM and SC-FDM, where UL waveform can be semi-statically configured by the network. From the RAN1 discussion, CP-OFDM is main stream wave for NR PUSCH in FR1. In the table2, we just summary the pros and cons of NR UL waveforms with serval aspects:

Table 2: Comparison between CP-OFDM and DFT-S-OFDM

|  | CP-OFDM | DFT-S-OFDM |
| --- | --- | --- |
| PA Efficiency | High PAPR/CM | Low PAPR/CM |
| MIMO support | Easy (per-tone processing, ML) | Hard (post-IDFT processing ,MMSE-IRC) |
| Diversity Support | Better performance at low code-rate (coding gain at frequency selective channel) | Better performance at high code-rate (spreading gain at frequency selective channel) |
| RS/Data Multiplexing | FDM/TDM | TDM(for maintaining low PAPR) |

From this table, there is no obvious benefit of DFT-S-OFDM over CP-OFDM in FR1.From the view of implementation, additional implementation complexity will be expected due to IDFT operation.

Regarding to UL coverage, with FR1, NR PRACH is nearly same with LTE PRACH, especially for format0 and format1 (LTE format 3). And NR PUCCH with long format and single RB is also similar with LTE PUCCH. So, our view the coverage issue of CP-OFDM is not serious for the control channel. For the coverage of date channel, our understanding the coverage issue of traffic channel is second priority compared with other control channel.  For the PUSCH, it is almost same except for DMRS pattern. So, our view is the performance of DFT-s-OFDM is not much different compared with LTE in SIMO transmission.

Based on above analysis, our view is that CP-OFDM has more advantage than DFT. In terms of UE feature list, both CP-OFDM and DFT-s-OFDM for UL are mandatory UE feature without capability signaling. Compared with LTE UL, CP-OFDM is new feature for PUSCH, it is reasonable to define the performance requirements with CP-OFDM. Considered that the performance of DFT-s-OFDM may be not too much different with LTE, also if both CP-OFDM and DFT-s-OFDM performance requirement are defined in Rel-15, which will result in the test case double. Our preference is that introduce the performance requirement later. We propose the PUSCH performance requirement defined for CP-OFDM firstly in order to avoid too many test cases with meeting the deadline of NR BS performance requirement in Re-15. If there is coverage issue with low SNR region for CP-OFDM, limited test cases for the performance requirement of DFT-s-OFDM can be defined.  As for the test cases defined for DFT-s-OFDM, it should be further studied.

Proposal 2: For UL PUSCH waveform, CP-OFDM should be introduced to performance test firstly in Rel-15. Deprioritize the performance requirement of DFT-s-OFDM in Rel-15.

## 2.3	RS

As agreed that in WF [2], single –symbol DMRS configuration is tested in Rel-15. There are four options for the number of DMRS symbols. As indicated in the UE feature list, for resource allocation type A, 1 symbol front-loaded DMRS without additional symbol, 1 symbol front-loaded DMRS and 1 additional DMRS symbols, 1 symbol front-loaded DMRS and 2 additional DMRS symbols are UE mandatory feature without capability signaling; for resource allocation type B, 1 symbol front-loaded DMRS without additional symbol, 1 symbol front-loaded DMRS with 1 additional DMRS symbol are UE mandatory feature without capability signaling. Since we focus on the most essential normal mode in Rel -15, our view is that front-loaded DMRS is enough for performance test, due to no requirement for high speed scenario with good channel estimation performance requirement. Considering the overheard of DMRS, we prefer that only 1 front –loaded with at most 1 additional DMRS symbol is defined the performance requirement for FR1, and only 1 front-loaded DMRS symbol for FR2.

For DMRS, NR supports 2 types, i.e, type 1nad type2. In our view, type 1 can be considered as the default configuration, type 2 is for MU-MIMO case, since we are only focus on the single UE case, and thus, there is no need to introduce the performance test with type-2.

For PTRS, it is only available for FR2. As indicated in UE feature list, it is mandatory UE feature with UE capability signaling in FR2. So, our preference is that we should deprioritize the performance requirement of PTRS in Rel-15.

Proposal 3: For UL PUSCH RS, only Type1 DRMS is introduced to performance test in Rel-15. Number of DMRS with included 1 front loaded should be limited with 2 for FR1, and only 1 front-loaded for FR2. Deprioritize the performance requirement of PTRS for FR2 in Rel-15.

## 2.4	Time domain resource allocation

NR defines two kinds of time domain resource allocation, type A and type B. The start symbol and symbol length can be configured in time domain as follows:

Table 3: time domain resource allocation

|  | Start symbol | length |
| --- | --- | --- |
| Type A | 0 | {4,…,14} |
| Type B | {0,13} | {1,….14} |

Since the number of DMRS and location is different for each resource allocation, we should cover the performance requirement for both type A and type B. From the implementation point view, the processing delay for  data is less in type B, due to that DMRS is located within the first symbol with related the scheduled data.  So, our preference is that

For scheduling, both slot-based scheduling and not slot-based scheduling are supported. In current stage, we recommend that the performance requirement of slot-based scheduling should be defined firstly.

## 2.4	MCS

As indicated in the UE feature list, QPSK, 16QAM, and 64QAM are mandatory without capacity singling. Definitely, they should be defined performance report as test cases. For pi/2-BPSK, it is only available for DFT-s-OFDM waveform, and it is optional feature for FR1, and mandatory without capability signaling in FR2. Since our preferred since we are priority CP-OFDM, we should focus on mandatory firstly. So, our preference is that there is no performance requirement for pi/2-BPSK in Rel-15

Proposal 4: Not define the performance requirement for pi/2-BPSK in Rel-15

# 3	Simulation Results

In this section, based on our preferred, the initial simulation results of PUSCH are provided as table 5. As agreed in WF [2] last meeting, some tests are introduced to alignment purpose. Here, we just list the main simulation assumption as follows:

- SCS and BW

- 15KHz, 10MHz

- 30KHz, 20/40/100MHz

- 60KHz(FR2): 100MHz

- 120KHz: 100MHz

- DMRS type

- Type 1, Type 2 FFS in both FR1 and FR2

- Number of DMRS symbols

- Case 1: 1+1 (symbol #2 and symbol #11) in both FR1 and FR2

- Case 2: 1 front-loaded in both FR1 and FR2

- Time domain resource allocation type

- Type A and/or Type B in both FR1 and FR2

- PUSCH symbol length

- FR1

- 14 OFDM symbol

- FR2

- Resource allocation type A: [14] OFDM symbols

- Resource allocation type B: [11] OFDM symbols

- MCS index

- QPSK: [2 or 4] in both FR1 and FR2

- 16QAM: [13 or 16] in both FR1 and FR2

- 64QAM: [25] in both FR1 and FR2

Generally, considering the combination of SCS and BW, different RS configuration, time domain resource allocation scheme, the number of test cases to be cover in this meeting is still very huge. In this paper, we just down select some cases for alignment only in this meeting as shown in the Table 4.  We will provide other cases later. The FRC tables for test cases are shown in Table 6, Table 7, and Table 8, respectively

Table 4: Simulation Assumption for PUSCH with preferred test cases

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Number of Tx antennas | 1 | 1 |
| Number of Rx antennas | 2 | 2 |
| AWGN | AWGN | AWGN |
| Cyclic prefix | Normal | Normal |
| Reference receiver | MMSE | MMSE |
| Channel bandwidth | 10M | 100M |
| SCS | 15KHZ | 120KHz |
| Number of layers | 1 | 1 |
| DMRS type | Type1 | Type2 |
| DMRS mapping | DMRS symbol index 0 for 1 front –loaded configuration for type B DMRS symbol index 0 and 10 for 1+1 DMRS configuration for type B | DMRS symbol index 2 for 1 front –loaded configuration for type A |
| Number of DMRS symbol | Case 1: 1+1 Case 2: 1 front-loaded | Case 2: 1 front-loaded DMRS |
| Time domain Resource allocation type | Type B | Type A |
| PUSCH symbol length | 14 | 14 |
| Test metric | SNR @ 70 of maximum through put | SNR @ 70 of maximum through put |

Table5: The target SNR performance for NR PUSCH (To be updated)

| Case | SCS | Number of DMRS symbol | BW | Target SNR for MCS2 (dB) | Target SNR for MCS4 (dB) | Target SNR for MCS13 (dB) | Target SNR for MCS16 (dB) | Target SNR for MCS25 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 1 | 10 | -4.582 | -2.858 | 4.412 | 7.482 | 15.095 |
| 1 | 15 | 2 | 10 | -5.140 | -3.283 | 3.658 | 7.002 | 14.51 |
| 2 | 120 | 1 | 100 | -4.625 | -2.823 | 4.440 | 7.480 | 15.053 |

# 3	Conclusion

In this contrition, based on the agreement of WF on the NR BS PUSCH demodulation, we provide our view about the remained issue of NR PUSCH demodulation requirement.

Proposal 1: The performance requirement of NR PUSCH should be prioritized to support UE feature of mandatory with capability signaling in Rel-15.

Proposal 2: For UL PUSCH waveform, CP-OFDM should be introduced to performance test firstly in Rel-15. Deprioritize the performance requirement of DFT-s-OFDM in Rel-15.

Proposal 3: For UL PUSCH RS, only Type1 DRMS is introduced to performance test in Rel-15. Number of DMRS with included 1 front loaded should be limited with 2 for FR1, and only 1 front-loaded for FR2. Deprioritize the performance requirement of PTRS for FR2 in Rel-15.

Proposal 4: Not define the performance requirement for pi/2-BPSK in Rel-15

# References

RP-181472  RAN4 Rel-15 NR UE feature list

R4-1808024, WF on NR PUSCH demodulation requirements in Rel-15, Nokia

# Appendix

Table 6: FRC for FR1 with SCS 15 kHz and 1 symbol front-load DM-RS 10M

| CBW(MHz) | 10MHz | 10MHz | 10MHz | 10MHz | 10MHz |
| --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 15 | 15 | 15 | 15 | 15 |
| RB | 52 | 52 | 52 | 52 | 52 |
| Modulation order | 2 | 2 | 4 | 4 | 6 |
| MCS index | 2 | 4 | 13 | 16 | 25 |
| Code Rate | 1/5 | 1/3 | 1/2 | 2/3 | 4/5 |
| Num of DMRS | 1 | 1 | 1 | 1 | 1 |
| Channel bits | 16224 | 16224 | 32448 | 32448 | 48672 |
| Final TBS (A) | 3104 | 4864 | 15624 | 21000 | 38936 |
| CRC for TB (A>3824, 24; otherwise 16) | 16 | 24 | 24 | 24 | 24 |
| C (Num of CB) | 1 | 1 | 2 | 3 | 5 |
| Base Graph Type | BG2 | BG1 | BG1 | BG1 | BG1 |

Table 7: FRC for FR1 with SCS 15kHz and （1+1） DM-RS  10M

| CBW(MHz) | 10MHz | 10MHz | 10MHz | 10MHz | 10MHz |
| --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 15 | 15 | 15 | 15 | 15 |
| RB | 52 | 52 | 52 | 52 | 52 |
| Modulation order | 2 | 2 | 4 | 4 | 6 |
| MCS index | 2 | 4 | 13 | 16 | 25 |
| Code Rate | 1/5 | 1/3 | 1/2 | 2/3 | 4/5 |
| Num of DMRS | 1 | 1 | 1 | 1 | 1 |
| Channel bits | 14976 | 14976 | 29952 | 29952 | 44928 |
| Final TBS (A) | 2856 | 4480 | 14344 | 19464 | 35856 |
| CRC for TB (A>3824, 24; otherwise 16) | 16 | 24 | 24 | 24 | 24 |
| C (Num of CB) | 1 | 1 | 2 | 3 | 5 |
| Base Graph Type | BG2 | BG1 | BG1 | BG1 | BG1 |

Table 8: FRC for FR2 with SCS 120kHz and 1 symbol front-load DM-RS  100M

| CBW(MHz) | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 120 | 120 | 120 | 120 | 120 |
| RB | 66 | 66 | 66 | 66 | 66 |
| Modulation order | 2 | 2 | 4 | 4 | 6 |
| MCS index | 2 | 4 | 13 | 16 | 25 |
| Code Rate | 1/5 | 1/3 | 1/2 | 2/3 | 4/5 |
| Num of DMRS | 1 | 1 | 1 | 1 | 1 |
| Channel bits | 20592 | 20592 | 41184 | 41184 | 61776 |
| Final TBS (A) | 3848 | 6144 | 19464 | 26632 | 49176 |
| CRC for TB (A>3824, 24; otherwise 16) | 24 | 24 | 24 | 24 | 24 |
| C (Num of CB) | 1 | 1 | 3 | 4 | 6 |
| Base Graph Type | BG1 | BG1 | BG1 | BG1 | BG1 |