3GPP TSG-RAN WG4 Meeting #AH1807                     	  					R4-1809409

Montreal, Canada, July 2nd –6th, 2018

Title:	[DRAFT] LS on clarification of useServingCellTimingForSync

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG1, RAN WG2

CC:

Contact Person:

Name:	Dinhwa Huang

E-mail Address:	Althea.huang@mediatek.com

Attachments:

1. Overall Description:

RAN4 would like to inform to RAN1 and RAN2 that RAN4 has discussed further on the definition of useServingCellTimingForSync indication and concluded on the following:

- When useServingCellTimingForSync is set to TRUE for the carrier, it means the following:

  - UE can assume frame boundary alignment (including half frame/subframe/slot boundary alignment) across cells on the same frequency carrier, within a tolerance of less than

min(2 SSB symbols, 1 data symbol).

  - UE can assume the SFN numbers in all cells on the same frequency carrier are the same.

2. Actions:

To RAN WG1 and WG2

ACTION: 	RAN4 respectfully asks RAN1 and RAN2 to consider the above conclusion in the further work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #88		Aug 20 – 24, 2018			     Gothenburg, Sweden

TSG RAN WG4 Meeting #88b		October 8 – 12, 2018				Chengdu, China