3GPP TSG-RAN WG4 AH1807	R4- 1809521

Montreal, Canada 2nd –6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843 OTA Occupied Bandwidth measurement in Near Field Test Range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding text for OTA Occupied Bandwidth measurement in Near Field Test setup |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for OTA Occupied Bandwidth measurement for directional requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text is needed for OTA Occupied Bandwidth requirement. If not included, the current text doesn’t reflect what it is measured in Near Field Test Range |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.7.4.2, 10.2.7.4.3, 10.2.7.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 10.2.7.4	Near field

##### 10.2.7.4.1	General

The system is depicted in section 10.2.2.4.1. In case of OTA Occupied BW type of measurements, NF to FF transform is not needed. Occupied BW is measured in Near Field for the declared direction

##### 10.2.7.4.2 	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in 10.2.2.4.2. to ensure that the SNR at the measurement equipment input is appropriate and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.7.4.3 	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

- Align the AAS BS with (Theta,Phi) angles corresponding to the declared beam direction to be measured

- Configure TX branch and carrier at a time according to the manufacturer's declared rated output power

- Set the AAS BS to transmit the test signal according to E-TM3.1 at 5 MHz bandwidth configuration.

- Measure Occupied BW of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument).

- Repeat steps 3-4 for all conformance test directions as described in 3GPP TS 37.145-2 [24], subclause 6.2.

- Repeat steps 3-5 for E-TM3.2, E-TM3.3 and E-TM2.

For conformance tests, Occupied BW shall be measured at maximum power setting.

##### 10.2.7.4.4 	MU assessment

###### 10.2.7.4.4.1	MU Budget

#### 10.2.7.4.4.2 	MU Value