3GPP TSG-RAN WG4 AH1807	R4- 1809522

Montreal, Canada 2nd –6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.843 –Section 10.9 - General |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding text for clause 10.9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for TRP measurement grids |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text is needed in order to introduce Near Field Power density measurement for TRP estimation. If the text is not approved, in-band TRP will be only estimated based on EIRP measurements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.9	TRP measurement grids

### 10.9.1	General

The TRP or the radiated power is simply the total power radiated by an antenna/EUT. Due to energy conservation, TRP is independent of choice of test distance, apart from measurement errors.

TRPReference is the total power radiated by the antenna of a transmitting device, when non-idealities such as mismatch and losses in the antenna are taken into account. In theory, TRPReference is defined as the integral of the antenna’s  radiation pattern over a spherical surface, that is

TRPReference= θ=0πϕ=02πUθ,ϕ sinθdθdϕ

TRPReference=sE×H.dS

U(θ,ϕ)

- Far Fieldregions:

Uθ,ϕ=EIRP(θ,ϕ)4π

Thus the definite integral for TRPReference becomes

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

- Near Field regions

Uθ,φ=r2PDr,θ,φ

PDr,θ,φ

The near field region consists of reactive field and radiating near field. In the radiating near field, the definite integral for TRPReference becomes:

[Based on experimental findings, it is possible to measure power density in near field with an acceptable level of accuracy. Thus equation TRPReference can be expressed as

TRPReference= θ=0πϕ=02πr2PDr,θ,ϕ sinθdθdϕ

FFS]

The range of radiating near field is defined as

0.62D3λ<R<2D2λ

Where D is the maximum dimension of  the DUT antenna aperture.

TRPReference= θ=0πϕ=02πr2PDr,θ,ϕ sinθdθdϕ

In practice, discrete samples of EIRP, and power density are measured at different directions (θ,ϕ) over the entire sphere, which are used to numerically approximate the surface TRPReference integral.  The obtained value, TRPEstimate , is an approximation of TRPReference and the difference between then is defined as the TRP systematic error.

TRPsyserr = | TRPEstimate – TRPreference |   (in dB)

When measuring radiated power, at each measurement point for both far field and near field, two partial results for two orthogonal polarizations needs to be measured. These can be the θ (theta) and φ (phi) polarizations or any pair of orthogonal polarizations.

Power measurements can be performed at distance less than 2D^2/λ (the traditional far-field criteria). In following subclauses we describe several spherical sampling grids which can be applied to Power density, and EIRP measurements.