3GPP TSG-RAN5 Meeting #87-e		R5-202117
Electronic Meeting, 18 May – 29 May 2020

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On UBF for Intra-Band Carrier Aggregation

Document for:	Discussion for Endorsement

# Introduction

This contribution is discussing the need to further clarify the UBF, defined in [1], and the need to augment the applicability of the UBF for Carrier Aggregation (CA).

# Discussion

For non-CA test cases, the current UBF definition in [1] is sufficient. Most TX test cases activate the UBF, i.e., for TRP-based test cases, the UBF allows the 3D scan of the pattern with a combined link and measurement antenna while for EIRP-based test cases, activating the UBF allows for a more stable measurement. Only one RX test case, i.e., Maximum Input Level, currently utilizes the UBF.

In Rel-16 CA discussions, specifically inter-band FR2+FR2 CA [2],

- UE is assumed to be feasible to have independent beam management for the bands that are part of supported band configuration in inter-band CA for 28 GHz + 39 GHz combinations. - “28GHz” stands for a band group includes n257, n258, n261 - “39GHz” stands for a band group includes n259, n260

independent beam management was discussed. Whether measurements need to be performed from the same direction simultaneously or not will be part of a separate Rel-17 SI in RAN4 [3]

- Spherical coverage requirements for inter-band CA are tested from single AoA for Rel-16 if the following testability solution can be provided. - Testability SI will study the TE capability of transmitting 28 GHz + 39 GHz, 28 GHz + 28 GHz, or 39 GHz + 39GHz from same direction simultaneously.

Considering the independent beam management discussions and the potential need to lock the UL and/or DL beams independently, an extension of the UBF might eventually be required that allows the locking of beams for different CCs independently.

Observation 1: For testing beyond Rel-15, an extension of the UBF might be required that allows the locking of beams for different CCs independently

For Intra-Band Contiguous CA test cases, it is assumed that tests are performed at the beam peak of the aggregated CBW, e.g., as stated in [4]

3.6.3 Applicability for intra-band FR2 For the requirements in RRC connected state specified in this version of the specification, UE shall assume that the transmitted signals from the serving cells should have the same downlink spatial domain transmission filter on one OFDM symbol in the same band in FR2. Otherwise, the UE is not supposed to satisfy any requirements for SCell.

For these test cases to be executed accurately and reliably, it needs to be guaranteed that the UBF activation is applicable to all component carriers (CCs). It is therefore proposed for the OEMs and chipset vendors to confirm that upon UBF activation, the TX, RX, and TX&RX beams for all aggregated CCs are locked for intra-band combinations (either contiguous or non-contiguous).

Proposal 1: OEMs and chipset vendors to confirm that upon UBF activation, the TX, RX, and TX&RX beams for all aggregated CCs are locked for intra-band combinations (either contiguous or non-contiguous).

Upon confirmation, it is proposed to augment the UBF definition in [1], e.g., by adding the following statement

5.4.1 General The UE Beamlock test function is intended for making the UE to lock the UE antenna pattern once it has formed a beam towards the base station (SS) direction following the cell identification procedure in preparation for subsequent test procedures. Activating the UBF shall lock the antenna pattern of all active intra-band component carriers.

Proposal 2: Augment the UBF definition in [1] to highlight that the UBF activation applies to all intra-band CCs

# Conclusion

The following observations and proposals were made in this contribution

Observation 1: For testing beyond Rel-15, an extension of the UBF might be required that allows the locking of beams for different CCs independently

Proposal 1: OEMs and chipset vendors to confirm that upon UBF activation, the TX, RX, and TX&RX beams for all aggregated CCs are locked for intra-band combinations (either contiguous or non-contiguous).

Proposal 2: Augment the UBF definition in [1] to highlight that the UBF activation applies to all intra-band CCs

# References

- TS 38.509, Special conformance testing functions for User Equipment (UE), V15.7.0 (2020-03)

- R4-1916024, WF on FR2 inter-band DL CA, Nokia, 3GPP TSG-RAN WG4 Meeting #93, November 2019

- RP-192322, New SID: Study on enhanced test methods for FR2, Apple Inc., CAICT, 3GPP TSG-RAN Meeting #85, September 2019

- TS 38.133, Requirements for support of radio resource management, V16.3.0 (2020-03)