# OTA Testing of IoT Device

# Version 0.1

# [Publication Date]

This is a Non-binding Permanent Reference Document of the GSMA

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is confidential to the Association and is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2020 GSM Association

Disclaimer

The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Antitrust Notice

The information contain herein is in full compliance with the GSM Association’s antitrust compliance policy.

Table of Contents

1	3

1	Introduction	4

1.1	Overview	4

1.2	Scope	4

1.3	Abbreviations	4

1.4	References	5

1.5	Conventions	5

2	Overview of Access Technology of Cellular IoT	6

3	Test Set-up	6

3.1	General	6

3.1.1	Anechoic Chamber	7

3.1.2	Reverberation Chamber	7

4	Total Radiated Power	7

4.1	Test Method of Total Radiated Power	7

4.1.1	General requirement for Power measurement	8

4.1.2	Test method of Total Radiated Power in Anechoic Chamber	8

4.1.3	Test method of Total Radiated Power in Reverberation Chamber	8

4.1.4	Calculation method of Total Radiated Power	8

4.2	Total Radiated Power for NB-IoT UE	10

4.2.1	Test Procedures	10

4.2.2	Performance requirement	10

4.3	Total Radiated Power for LTE-M UE	12

4.3.1	Test Procedures	12

4.3.2	Performance requirement	15

4.4	Total Radiated Power for Cat1/Cat1bis	16

4.4.1	Test Procedures	17

4.4.2	Performance requirement	17

5	Total Radiated Sensitivity	17

5.1	Test Method of Total Radiated Sensitivity	17

5.1.1	General requirement for receiver performance measurement	17

5.1.2	Test method of Total Radiated Sensitivity in Anechoic Chamber	18

5.1.3	Test method of Total Radiated Sensitivity in Reverberation Chamber	18

5.1.4	Calculation method of Total Radiated Sensitivity	18

5.2	Total Radiated Sensitivity for NB-IoT UE	19

5.2.1	Test Procedures	19

5.2.2	Performance requirement	20

5.3	Total Radiated Sensitivity for LTE-M UE	22

5.3.1	Test Procedures	22

5.3.2	Performance requirement	25

5.4	Total Radiated Sensitivity for Cat1/Cat1bis	27

5.4.1	Test Procedures	27

5.4.2	Performance requirement	27

Annex A	Classification of DUT Catagory	28

Annex B	Document Management	29

B.1	Document History	29

B.2	Other Information	29

# Introduction

## Overview

OTA (Over the Air) testing is the key method to evaluate the antenna performance and radiated performance of wireless devices. Poor OTA performance would result in high Block Error Rate (BLER) in communication and potential access failure.

The number of IoT connected devices has experienced exceptional growth in recent years. Some of these IoT devices suffer from very poor radiated antenna performance and cause a bad user experience.

Unlike traditional devices, IoT devices come in various sizes, shapes, materials and work in different environments. This presents lots of challenges for OTA testing of IoT devices. CTIA have set up an IoT working group and have released OTA test methods for LTE-M devices, however the details about working scenarios and benchmarks have not been defined.

Given the fast development of the IoT industry, the importance of OTA measurements and the existing standardization status, the GSMA is developing the specification of OTA Testing of IoT devices, which includes test set-up, test method, test procedures and performance requirements.

To measure the radiation power and receiver performance of IoT devices, this document provides test solutions for measuring the Total Radiation Power (TRP) and Total Radiated Sensitivity (TRS), which can be carried out using two test systems: An Anechoic Chamber (AC) and a Reverberation Chamber (RC).

## Scope

This document defines the OTA Testing of cellular IoT devices, including test set-up, test method, test procedure and performance requirements of Total Radiated Power and Total Radiated Sensitivity for different types of cellular IoT devices.

The cellular IoT devices covered by this document are those using 3GPP communication technology; for example NB-IoT, LTE-M, Cat1/Cat1bis, etc.

## Abbreviations

| Term | Description |
| --- | --- |
| AC | Anechoic Chamber |
| BER | Bit Error Rate |
| BLER | Block Error Rate |
| DUT | Device Under Test |
| FS | Free Space |
| HARQ | Hybrid automatic repeat request |
| NPDCCH | Narrowband Physical Downlink Shared Channel |
| PUMAX | The measured configured maximum UE output power |
| QPSK | Quadrature Phase Shift Keying |
| RC | Reverberation chamber |
| RF | Radio Frequency |
| TIS | Total Isotropic Sensitivity |
| TRP | Total Radiated Power |
| TRS | Total Radiated Sensitivity |
| UE | User Equipment |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | CTIA | Cellular Telecommunications & Internet Association OTA Test Plan |
|  | 3GPP | TS 34.114, TS 37.544 v14.7.0, TS 36.521-1 V16-1.0, TS 36.101 latest version |
|  | CTIA | CTIA Test plan for wireless device over the air performance ver 3.8.2 |
|  | 3GPP TS 36.521-1 | User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing V16.1.0 |
|  | 3GPP TS 36.508 | Common test environments for User Equipment (UE) conformance testing V16.1.0 |
|  | CTIA | Test Plan for Wireless Large Form Factor Device Over the Air Performance Version 1.2 |
|  | RFC 2119 | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997. Available at http://www.ietf.org/rfc/rfc2119.txt |
|  | RFC 8174 | Ambiguity of Uppercase vs Lowercase in RFC 2119 Key Words |

## Conventions

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "NOT RECOMMENDED", "MAY", and "OPTIONAL" in this document are to be interpreted as described in BCP 14 (RFC2119) [7] (RFC8174) [8] when, and only when, they appear in all capitals, as shown here.

# Overview of Access Technology of Cellular IoT

This section compares Cat1/Cat1bis, LTE-M & NB-IoT in terms of various performance requirements against a number of critreia (bandwidth, coverage, bitrate, mobility and voice), as outlined in the following Table.

|  |  | NB-IoT | LTE-M |  | Cat1/Cat1bis |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LTE TDD Cat. M | LTE FDD Cat. M |  |
| Bandwidth |  | 200kHz | 1.4MHz | 1.4MHz | 10MHz/20MHz |
| Coverage (NOTE 1) |  | About 164dB | Mode A: about 148 dB Mode B: about 156 dB | Mode A: about 148 dB Mode B: about 156 dB | About 130dB |
| Bitrate | Uplink | R13:62kbit/s(MT) R14:150kbit/s(MT) （NOTE2） | R13:200kbit/s R14:655kbit/s | R13: FD:1Mbit/s HD:375kbit/s R14: FD:2.98Mbit/s HD:1.11Mbit/s | 5Mbilt/s |
|  | Downlink | R13:21kbit/s R14:120kbit/s | R13:750kbit/s R14:750kbit/s | R13: FD:800kbit/s HD:300kbit/s R14: FD:1Mbit/s HD:588kbit/s | 10Mbit/s |
| Mobility |  | Support low speed mobility; Support relocation; Not support handover; | Support high speed mobility; Support relocation; Support handover; | Support high speed mobility; Support relocation; Support handover; | Support high speed mobility; Support relocation; Support handover; |
| Voice |  | NOT Support | Support | Support | Support |
| NOTE 1: Coverage refers to the ideal coverage capacity in the scene without interference. Coverage shrinks when there is significant interference in the system. NOTE2: For peak bitrate on uplink NB is used for multi-frequency transmission. If single frequency transmission is used, the rate will be reduced. |  |  |  |  |  |

: Comparison of NB-IoT, LTE-M and Cat1/Cat1bis

# Test Set-up

## General

The Device Under Test (DUT) can be evaluated in either an Anechoic Chamber or Reverberation Chamber already specified in both 3GPP and CTIA specifications. Test equipment is described below.

### Anechoic Chamber

DUT is set in an Anechoic Chamber at a distance which will allow a quiet zone from the measurement antenna. A rotation system shall be used so that the device will describe a full sphere rotation with 30° step angle. The antenna measurement can switch for horizontal and vertical polarization or a dual polarization antenna could also be used.

DUT

measurement antenna

: Anechoic chamber

### Reverberation Chamber

DUT can be set in a reverberating chamber which allows multipath signal.

Stirrers can be moved continuously or step by step to create a uniform distribution of waves around the DUT.

The number of sample measurements need to be set at least for 200 for TRP (Total Radiated Power) measurement and 300 for TIS (Total Isotropic Sensitivity) measurement.

Network simulator

: Reverberating chamber

# Total Radiated Power

## Test Method of Total Radiated Power

The test method defined herein will:

  - clarify general requirements for power measurement;

  - define test methods of TRP in both an anechoic chamber and reverberation chamber.

The radiated Radio Frequency (RF) performance of the DUT is measured by sampling the radiated transmit power of the mobile at various locations surrounding the device. All of the measured power values will be integrated to give a single figure of merit referred to as TRP.

Due to the diversity working scenarios of IoT devices, tests shall be configured according to the DUT’s working scenarios, i.e., free space configurations to metering devices, forearm phantom to wrist worn devices.

Coordinating systems and positioning requirements for all configurations are described in Appendix A of CTIA Test plan for wireless device over the air performance [3]. The requirements for the wrist worn devices are also defined in Appendix Q of ref [3].

It is the responsibility of the manufacturer to define which options represent the “baseline” configuration and to provide adequate supporting data that validates assumptions for reduced testing of the remaining options. At a minimum, a complete set of measurements is required for the baseline DUT configuration.

### General requirement for Power measurement

The LTE system simulator and the DUT shall be configured as per Section 6.2 of ref [4] 3GPP TS 36.521-1, using the default settings specified in ref [4] 3GPP TS 36.521-1 and [5] 3GPP TS 36.508 as applicable. The power radiated by the DUT shall be measured using a calibrated and accurate RF measuring instrument.

For NB-IoT type devices, since NB-IoT is a separate system different from LTE, a separate parameter set is required in the system configuration, and the test configuration is different.

For LTE-M type devices, the OTA test methodology is fundamentally same as the LTE devices.

In the maximum transmission power test, since the NB-IoT has no closed loop power control and only open loop power control, it is not possible to adjust the DUT to the maximum transmit power through closed loop power control. The DUT can reach the maximum transmit power by setting a higher open-loop target power. At the same time, unless it is demonstrated that there is negligible impact to performance with different options, tests shall be performed on all possible DUT configurations such as battery pack configurations, and different working scenarios. The test can be executed either using and Anechoic Chamber or a Reverberation Chamber

### Test method of Total Radiated Power in Anechoic Chamber

CTIA have provided two test methods of scanning the DUT, the “conical” cut method and the “great circle” cut method, which can be found in section 2 of ref [3].

### Test method of Total Radiated Power in Reverberation Chamber

CTIA have provided test methods of transmitter performance assessment using the RC, which can be found in the CTIA Test Plan section 2 of ref [6].

### Calculation method of Total Radiated Power

The calculation method of TRP for EUT is shown in formulas below, which is described in 3GPP TS 34.114 of ref [2]

(Formula 4.1.4-1)

Where  is the solid angle describing the direction,  is frequency.  and  are the orthogonal polarizations.  and  are the actually transmitted power-levels in corresponding polarizations.

(Formula 4.1.4-2)

In these formulas  and  are the number of sampling intervals for theta and phi.  and  are the measurement angles.

When using

(Formula 4.1.4-3)

Where  is the reference power transfer function for fixed measurement antenna n,  is the reflection coefficient for fixed measurement antenna n and  is the path loss in the cables connecting the measurement receiver to fixed measurement antenna n.

is the average power measured by fixed measurement antenna n and can be calculated using the following expression:

(Formula 4.1.4-4)

Where  is sample number m of the complex transfer function measured with fixed measurement antenna n and  is the total number of samples measured for each fixed measurement antenna.

## Total Radiated Power for NB-IoT UE

Category NB1 and NB2 are designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 25, 26, 28, 31, 41, 66, 70, 71, 72, 73, 74 and 85. Category NB1 and NB2 systems operate in HD-FDD duplex mode or in TDD mode.

### Test Procedures

The test procedure is based in principle on CTIA [1] and 3GPP [2] methodologies.

The Total radiated power is measure with the following steps:

- Set the initial conditions as per Table 6.2.2F.4.1-1 test ID3 in sub clause 6.2.2F UE Maximum Output Power for category NB1 and NB2 of 3GPP TS 36.521-1 and they are specified in the table below:

| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
| --- | --- | --- | --- | --- |
| 1 | N/A | Modulation | Ntones | Subcarrier spacing |
|  |  | QPSK | 1@0 | 15kHz |

: UE Maximum Output Power for category NB1 and NB2

- System simulator sends uplink scheduling information for each UL HARQ process via NPDCCH as per 3GPP TS 36.521-1，6.2.2F to make sure UE transmit PUMAX after Initial Conditions setting.

- When using Anechoic chamber Rotate the device in 3D dimensions using 15° step (when using RC, 200 samples need to be measures at least).

- Measure the  and   with a sample step of 15° in theta () and phi () directions.

- Calculate TRP with formula detailed in section 4.1.4-2 when using anechoic chamber and calculate TRP with formula detailed in section 4.1.4-3 when using reverberation chamber.

### Performance requirement

The average TRP of Low, Mid and High channels should be higher than the test performance requirements in Table 3. UE category is shown in Annex A.

Category A NB-IoT TRP:

Stationary IoT devices-(Free Space): Devices will not perform handover or extreme cell edge radio conditions for example: smoke detectors, temperature probes, etc.

|  | Power Class | Band | UE maximum output power (specified in sub clause 6.2.2F of 3GPP TS.36.101) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRP (dBm) |
| --- | --- | --- | --- | --- | --- |
| NB-IoT | 3 | 1, 3 | 23dBm | Less than or equal to 5cm | 14 |
|  |  |  |  | Greater than 5cm | 18 |
|  |  | 5, 8, 20 | 23dBm | Less than or equal to 5cm | 12 |
|  |  |  |  | Greater than 5cm | 18 |
|  | 5 | 1, 3 | 20dBm | Less than or equal to 5cm | 11 |
|  |  |  |  | Greater than 5cm | 15 |
|  |  | 5, 8, 20 | 20dBm | Less than or equal to 5cm | 9 |
|  |  |  |  | Greater than 5cm | 15 |

: Average TRP test requirement for NB-IoT UE Category A

Category B NB-IoT TRP:

Mobile IoT devices-(Body Phantoms): The testing condition should be similar to the usage, e.g. with wrist phantom hand or torso etc. However, given that the phantom torso is not yet standardized, the requirements defined will cover wrist phantom hand only.

|  | Power Class | Band | UE maximum output power (specified in subclause 6.2.2F of 3GPP TS.36.101) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRP (dBm) |
| --- | --- | --- | --- | --- | --- |
| NB-IoT | 3 | 1, 3 | 23dBm | Less than or equal to 5cm | 11 |
|  |  |  |  | Greater than 5cm | 15 |
|  |  | 5, 8, 20 | 23dBm | Less than or equal to 5cm | 9 |
|  |  |  |  | Greater than 5cm | 15 |
|  | 5 | 1, 3 | 20dBm | Less than or equal to 5cm | 8 |
|  |  |  |  | Greater than 5cm | 12 |
|  |  | 5, 8, 20 | 20dBm | Less than or equal to 5cm | 6 |
|  |  |  |  | Greater than 5cm | 12 |

: Average TRP test requirement for NB-IoT UE Category B

Category C NB-IoT TRP:

Mobile IoT devices-(Free Space): those devices can be in a handover process in the live network or in extreme radio conditions.

|  | Power Class | Band | UE maximum output power (specified in subclause 6.2.2F of 3GPP TS.36.101) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRP (dBm) |
| --- | --- | --- | --- | --- | --- |
| NB-IoT | 3 | 1, 3 | 23dBm | Less than or equal to 5cm | 14 |
|  |  |  |  | Greater than 5cm | 18 |
|  |  | 5, 8, 20 | 23dBm | Less than or equal to 5cm | 12 |
|  |  |  |  | Greater than 5cm | 18 |
|  | 5 | 1, 3 | 20dBm | Less than or equal to 5cm | 11 |
|  |  |  |  | Greater than 5cm | 15 |
|  |  | 5, 8, 20 | 20dBm | Less than or equal to 5cm | 9 |
|  |  |  |  | Greater than 5cm | 15 |

: Average TRP test requirement for NB-IoT UE Category C

## Total Radiated Power for LTE-M UE

UE category M1 (and M2) is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 18, 19, 20, 21, 25, 26, 27, 28, 31, 66, 71, 72, 73, 74 and 85 in both half duplex FDD mode and full-duplex FDD mode, and in band 39, 40 and 41 in TDD mode.

### Test Procedures

The test procedure is based in principle on CTIA [1] and 3GPP [2] methodologies

The test can be executed using an anechoic chamber or reverberation chamber

The power can be measured using a spectrum analyser, a receiver of test equipment, or power meter.

The total radiated power is measured with the following steps:

- Set the Network simulator as described in the table below.

- Send continuously Up power control commands to the UE.

- When using Anechoic chamber Rotate the device in 3D dimensions using 15° step

- (when using RC, 200 samples need to be measured at least).

- Measure the  and   with a sample step of 15° in theta () and phi () directions.

- Calculate TRP with formula in section 4.1.4-2 when using anechoic chamber and calculate TRP with formula in section 4.1.4-3 when using a reverberation chamber.

| Band | Channel Bandwidth (MHz) | Channel | Power Class 3 | Power Class 5 | DL RB Allocation |
| --- | --- | --- | --- | --- | --- |
|  |  |  | UL RB allocation | UL RB allocation |  |
| 1 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 2 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 3 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 4 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 5 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 7 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 8 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 11 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 12 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 13 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 14 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 17 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 18 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 19 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 20 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 21 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 25 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 26 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 28 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 31 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 41 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 66 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 70 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 71 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 72 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 73 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 74 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |
| 85 | 10 | Low | 4 | 5 | 6 |
|  | 10 | Mid | 4 | 5 | 6 |
|  | 10 | High | 4 | 5 | 6 |

: Network Simulator configuration

### Performance requirement

The average TRP of Low, Mid and High channels should be higher than the test performance requirements in Table 6. UE category is shown in Annex A.

Category A LTE-M TRP:

Stationary IoT devices-(Free Space): such device will not perform handover or extreme cell edge radio condition in example: smoke detector, temperature probe, etc.

|  | Power Class | Band | UE maximum output power (specified in subclause 6.2.2F of 3GPP TS.36.101) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRP (dBm) |
| --- | --- | --- | --- | --- | --- |
| LTE-M | 3 | 1, 3, 7 | 23dBm | Less than or equal to 5cm | 14 |
|  |  |  |  | Greater than 5cm | 18 |
|  |  | 8, 20 | 23dBm | Less than or equal to 5cm | 12 |
|  |  |  |  | Greater than 5cm | 18 |
|  | 5 | 1, 3, 7 | 20dBm | Less than or equal to 5cm | 11 |
|  |  |  |  | Greater than 5cm | 15 |
|  |  | 8, 20 | 20dBm | Less than or equal to 5cm | 9 |
|  |  |  |  | Greater than 5cm | 15 |

: Average TRP test requirement for LTE-M UE Category A

Category B LTE-M TRP:

Mobile IoT devices-(Body Phantoms): The testing condition should be similar to the usage, e.g. with a wrist phantom hand or torso, etc. However, given that the phantom torso is not yet standardized, the requirements defined will cover wrist phantom hand only.

|  | Power Class | Band | UE maximum output power (specified in subclause 6.2.2F of 3GPP TS.36.101) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRP (dBm) |
| --- | --- | --- | --- | --- | --- |
| LTE-M | 3 | 1, 3, 7 | 23dBm | Less than or equal to 5cm | 11 |
|  |  |  |  | Greater than 5cm | 15 |
|  |  | 8, 20 | 23dBm | Less than or equal to 5cm | 9 |
|  |  |  |  | Greater than 5cm | 15 |
|  | 5 | 1, 3, 7 | 20dBm | Less than or equal to 5cm | 8 |
|  |  |  |  | Greater than 5cm | 12 |
|  |  | 8, 20 | 20dBm | Less than or equal to 5cm | 6 |
|  |  |  |  | Greater than 5cm | 12 |

: Average TRP test requirement for LTE-M UE Category B

Category C LTE-M TRP:

Mobile IoT devices-(Free Space): These devices can be in the handover process in the live network or in extreme radio condition.

|  | Power Class | Band | UE maximum output power (specified in sub clause 6.2.2F of 3GPP TS.36.101) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRP (dBm) |
| --- | --- | --- | --- | --- | --- |
| LTE-M | 3 | 1, 3, 7 | 23dBm | Less than or equal to 5cm | 14 |
|  |  |  |  | Greater than 5cm | 18 |
|  |  | 8, 20 | 23dBm | Less than or equal to 5cm | 12 |
|  |  |  |  | Greater than 5cm | 18 |
|  | 5 | 1, 3, 7 | 20dBm | Less than or equal to 5cm | 11 |
|  |  |  |  | Greater than 5cm | 15 |
|  |  | 8, 20 | 20dBm | Less than or equal to 5cm | 9 |
|  |  |  |  | Greater than 5cm | 15 |

: Average TRP test requirement for LTE-M UE Category C

## Total Radiated Power for Cat1/Cat1bis

For Future Study

### Test Procedures

For Future Study

### Performance requirement

For Future Study

# Total Radiated Sensitivity

## Test Method of Total Radiated Sensitivity

The test method defined herein will:

  - define general requirement for receiver performance measurement;

  - define test method of TRS in both AC and RC.

Receiver sensitivity measurements shall be performed using data throughput as the measurement metric. The DUT's receiver sensitivity corresponds to the minimum downlink signal power required to provide a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel.

The receiver performance of the DUT is measured utilizing BLER, or other error criteria. A three-dimensional characterization of the receiver performance of the DUT is pieced together by analyzing the data from the spatially distributed measurements. All of the measured sensitivity values for each DUT test condition will be integrated to give a single figure of merit referred to as Total Radiated Sensitivity.

For the TRS test, tests shall be configured according to DUT’s working scenarios, i.e., free space configurations to metering devices, forearm phantom to wrist worn devices.

Coordinate systems and positioning requirements for all configurations are described in Appendix A of ref [3]. The requirements for the wrist worn devices are defined in Appendix Q of ref [3].

### General requirement for receiver performance measurement

The LTE system simulator and DUT shall be configured per 3GPP TS 36.521-1Section 7.3 of ref [4], using the default settings specified in 3GPP TS 36.521-1 ref [4] and 3GPP TS 36.508 [5] as applicable. The power radiated by the DUT shall be measured using a calibrated and accurate RF measuring instrument. If the DUT has multiple receivers, the receiver sensitivity measurements should be performed independently, and all receivers in the DUT other than the receiver under test should be disabled.

For NB-IoT type devices, since NB-IoT is a separate system different from LTE, a separate parameter set is required in the system configuration, and all frequencies and resource block (RB) allocations should be tested.

For LTE-M type devices, the OTA test methodology is the same as LTE devices.

Total Radiated Sensitivity shall be fully measured and calculated pursuant to ref [3] [6] for the channel(s) specified in Section 4.2 and 4.3 for all frequency bands supported by the DUT.

Since the process of measuring effective radiated receiver sensitivity is typically less accurate than measuring transmitter radiated power, this test specification allows for a relaxation in the spatial separation to 30 degrees in the theta (ɵ) and in the phi (ɸ) axis for TRS. The test can be executed either using AC or RC.

### Test method of Total Radiated Sensitivity in Anechoic Chamber

Receiver Sensitivity procedures will utilize the same spherical measurement procedure as specified for the Transmitter power measurements in this document Section 4.1.2 with the exception of angular step size.  Receiver Performance measurements may be made simultaneously with tests performed in this document Section 4.1.2.

CTIA have provided two test methods of scanning the DUT are proposed, the “conical” cut method and the “great circle” cut method, which can be found in sections 2 of ref [3].

### Test method of Total Radiated Sensitivity in Reverberation Chamber

CTIA have provided test methods of receiver performance assessment using the RC, which can be found in sections 2 of ref [6].

### Calculation method of Total Radiated Sensitivity

The calculation method of TRS for EUT is shown in formulas below, which is described in 3GPP TS 34.114 of ref [2]:

The TRS is a measure of the minimum power required to achieve a specified Block error rate (BLER). The TRS is defined as:

(Formula 5.1.4-1)

Where the effective isotropic sensitivity (EIS) is defined as the power available at the antenna output such as the sensitivity threshold is achieved for each polarization.  is the solid angle describing the direction,  is frequency.  and  are the orthogonal polarizations.

(Formula 5.1.4-2)

In these formulas  and  are the number of sampling intervals for theta and phi.  and  are the measurement angles.

The TRS can also be calculated from measurements in a Rayleigh fading 3 dimensional isotropic environment with in average uniform elevation and azimuth distribution. The calculation of the TRS is in this case based on searching for the lowest power received by the UE/MS for a discrete number of field combinations in the chamber that gives a BLER that is better than 5% (BLER <5%). By calibrating the average power transfer function, an absolute value of the TRS can be obtained. The following expression can be used to find the TRS.

(Formula 5.1.4-3)

where  is the reference power transfer function for fixed measurement antenna n,  is the reflection coefficient for fixed measurement antenna n and  is the path loss in the cables connecting the measurement receiver to fixed measurement antenna n. These parameters are calculated from the calibration measurement.  is calculated by using the following equation:

(Formula 5.1.4-4)

where  is the m:th value of the transfer function for fixed measurement antenna n, which gives 5% BLER threshold.  is the total number of values of the BLER threshold power measured for each fixed measurement antenna.

## Total Radiated Sensitivity for NB-IoT UE

### Test Procedures

The test procedure is based in principle on CTIA [1] and 3GPP [2] methodologies.

The power can be measure using, receiver of test equipment.

The Total Isotropic is measure with the following steps:

- Set the initial conditions as per Table 7.3F.1.4.1-1 in sub-clause 7.3F.1 of 3GPP TS 36.521-1 and they are specified in the table below:

| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
|  | QPSK | 12 | BPSK | 1@0 | 15kHz |

: Initial Conditions

- Measure the receiver sensitivity by adjusting the downlink signal level to 95 % throughput of the maximum throughput of the reference channel (maximum throughput is per Annex A of 3GPP TS 36.521-1).

- When using Anechoic chamber repeat Step 1) with 3-D sampling grid specified in Sub clause 4.4 of 3GPP TS 37.544. The minimum RF power level resulting in a data throughput greater than or equal to 95 % throughput of the maximum throughput for each test shall be recorded for integration pursuant to Sub clause 7.1.10.1 of 3GPP TS 37.544 to calculate TRS.

- When using Reverberation chamber Repeat Step 1) for a long enough time to get the statistic result. The minimum RF power level resulting in a data throughput greater than or equal to 95 % throughput of the maximum throughput.

- Repeat the measurement of the DUT on low, mid and high channels.

- Calculate the linear average and minimum TRS.

### Performance requirement

The average TRS of Low, Mid and High channels should be lower than the test performance requirements in Table 11. UE category is shown in Annex A.

Category A NB-IoT TRS:

Stationary IoT devices-(Free Space): Devices will not perform handover or extreme cell edge radio conditions for example: smoke detectors, temperature probes, etc.

|  | Power Class | Band | Reference sensitivity (specified in subclause 7.3.1F.1 of 3GPP TS.36.101) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRS (dBm) |
| --- | --- | --- | --- | --- | --- |
| NB-IoT | 3 | 1, 3 | -108.2dBm | Less than or equal to 5cm | -99 |
|  |  |  |  | Greater than 5cm | -103 |
|  |  | 5, 8, 20 | -108.2dBm | Less than or equal to 5cm | -97 |
|  |  |  |  | Greater than 5cm | -103 |
|  | 5 | 1, 3 | -108.2dBm | Less than or equal to 5cm | -99 |
|  |  |  |  | Greater than 5cm | -103 |
|  |  | 5, 8, 20 | -108.2dBm | Less than or equal to 5cm | -97 |
|  |  |  |  | Greater than 5cm | -103 |

: Average TRS test requirement for NB-IoT UE Category A

Category B NB-IoT TRS:

Mobile IoT devices-(Body Phantoms): The testing condition should be similar to the usage, e.g. with wrist phantom hand or torso, etc. However, given that the phantom torso is not yet standardized, the requirements defined will cover wrist phantom hand only.

|  | Power Class | Band | Reference sensitivity (specified in subclause 7.3.1F.1 of 3GPP TS.36.101) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRS (dBm) |
| --- | --- | --- | --- | --- | --- |
| NB-IoT | 3 | 1, 3 | -108.2dBm | Less than or equal to 5cm | -96 |
|  |  |  |  | Greater than 5cm | -100 |
|  |  | 5, 8, 20 | -108.2dBm | Less than or equal to 5cm | -94 |
|  |  |  |  | Greater than 5cm | -100 |
|  | 5 | 1, 3 | -108.2dBm | Less than or equal to 5cm | -96 |
|  |  |  |  | Greater than 5cm | -100 |
|  |  | 5, 8, 20 | -108.2dBm | Less than or equal to 5cm | -94 |
|  |  |  |  | Greater than 5cm | -100 |

: Average TRS test requirement for NB-IoT UE Category B

Category C NB-IoT TRS:

Mobile IoT devices-(Free Space): those devices can be in handover process in the live network or in extreme radio condition.

|  | Power Class | Band | Reference sensitivity (specified in subclause 7.3.1F.1 of 3GPP TS.36.101) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRS (dBm) |
| --- | --- | --- | --- | --- | --- |
| NB-IoT | 3 | 1, 3 | -108.2dBm | Less than or equal to 5cm | -99 |
|  |  |  |  | Greater than 5cm | -103 |
|  |  | 5, 8, 20 | -108.2dBm | Less than or equal to 5cm | -97 |
|  |  |  |  | Greater than 5cm | -103 |
|  | 5 | 1, 3 | -108.2dBm | Less than or equal to 5cm | -99 |
|  |  |  |  | Greater than 5cm | -103 |
|  |  | 5, 8, 20 | -108.2dBm | Less than or equal to 5cm | -97 |
|  |  |  |  | Greater than 5cm | -103 |

: Average TRS test requirement for NB-IoT UE Category C

## Total Radiated Sensitivity for LTE-M UE

### Test Procedures

The test procedure is based in principle on CTIA [1] and 3GPP [2] methodologies with specific configuration which will be indicated in this document whenever needed.

| Band | Channel Bandwidth (MHz) | Channel | Power Class 3 | Power Class 5 | UL RB Allocation |
| --- | --- | --- | --- | --- | --- |
|  |  |  | DL RB allocation | DL RB allocation |  |
| 1 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 2 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 3 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 4 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 5 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 7 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 8 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 11 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 12 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 13 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 14 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 17 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 18 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 19 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 20 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 21 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 25 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 26 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 28 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 31 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 41 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 66 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 70 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 71 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 72 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 73 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 74 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |
| 85 | 10 | Low | 4 | 4 | 6 |
|  | 10 | Mid | 4 | 4 | 6 |
|  | 10 | High | 4 | 4 | 6 |

: Table 5.3.1-1: Network Simulator configuration

### Performance requirement

The average TRS of Low, Mid and High channels should be lower than the test performance requirements in Table 5.3.2-1. UE category is shown in Annex A.

Category A LTE-M TRS:

Stationary IoT devices-(Free Space): such device will not perform handover or extreme cell edge radio condition in example: smoke detector, temperature probe, etc.

|  | Power Class | Band | Reference sensitivity (NOTE 1) | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRS (dBm) |
| --- | --- | --- | --- | --- | --- |
| LTE-M | 3 | 1, 3, 7 | -101dBm | Less than or equal to 5cm | -92 |
|  |  |  |  | Greater than 5cm | -96 |
|  |  | 8, 20 | -101dBm | Less than or equal to 5cm | -90 |
|  |  |  |  | Greater than 5cm | -96 |
|  | 5 | 1, 3, 7 | -101dBm | Less than or equal to 5cm | -92 |
|  |  |  |  | Greater than 5cm | -96 |
|  |  | 8, 20 | -101dBm | Less than or equal to 5cm | -90 |
|  |  |  |  | Greater than 5cm | -96 |
| NOTE 1: Reference sensitivity for the test bands are set to -101dBm in GSMA referring to sub clause 7.3.1E of 3GPP TS.36.101. |  |  |  |  |  |

: Average TRS test requirement for LTE-M UE Category A

Category B LTE-M TRS:

Mobile IoT devices-(Body Phantoms): the testing condition should be similar to the usage, e.g. with wrist phantom hand or torso, etc. However, given that the phantom torso is not yet standardized, the requirements defined will cover wrist phantom hand only.

|  | Power Class | Band | Reference sensitivity | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRS (dBm) |
| --- | --- | --- | --- | --- | --- |
| LTE-M | 3 | 1, 3, 7 | -101dBm | Less than or equal to 5cm | -89 |
|  |  |  |  | Greater than 5cm | -93 |
|  |  | 8, 20 | -101dBm | Less than or equal to 5cm | -87 |
|  |  |  |  | Greater than 5cm | -93 |
|  | 5 | 1, 3, 7 | -101dBm | Less than or equal to 5cm | -89 |
|  |  |  |  | Greater than 5cm | -93 |
|  |  | 8, 20 | -101dBm | Less than or equal to 5cm | -87 |
|  |  |  |  | Greater than 5cm | -93 |

: Table 5.3.2-2: Average TRS test requirement for LTE-M UE Category B

Category C LTE-M TRS:

Mobile IoT devices-(Free Space): those devices can be in handover process in the live network or in extreme radio condition.

|  | Power Class | Band | Reference sensitivity | Final DUT Size (The largest dimension in any direction should be within these limits) | Reference Average TRS (dBm) |
| --- | --- | --- | --- | --- | --- |
| LTE-M | 3 | 1, 3, 7 | -101dBm | Less than or equal to 5cm | -92 |
|  |  |  |  | Greater than 5cm | -96 |
|  |  | 8, 20 | -101dBm | Less than or equal to 5cm | -90 |
|  |  |  |  | Greater than 5cm | -96 |
|  | 5 | 1, 3, 7 | -101dBm | Less than or equal to 5cm | -92 |
|  |  |  |  | Greater than 5cm | -96 |
|  |  | 8, 20 | -101dBm | Less than or equal to 5cm | -90 |
|  |  |  |  | Greater than 5cm | -96 |

: Average TRS test requirement for LTE-M UE Category C

## Total Radiated Sensitivity for Cat1/Cat1bis

For Future Study

### Test Procedures

For Future Study

### Performance requirement

For Future Study

Classification of DUT Catagory

This specification requires the classification of the DUT in accordance with the table below. New categories could be added to this table in future versions.

| UE Catagory | Description |
| --- | --- |
| UE Catagory A | Stationary IoT devices-(Free Space) |
| UE Catagory B | Mobile IoT devices-(Body Phantoms) |
| UE Catagory C | Mobile IoT devices-(Free Space) |

: Classification of DUT

Document Management

Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 1.0 | March 2020 | New PRD TS.51 | TSG#38 TG#20 | Anni Wei / China Mobile |

Other Information

| Type | Description |
| --- | --- |
| Document Owner | Terminal Steering Group |
| Editor / Company | Anni Wei / China Mobile |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com

Your comments or suggestions & questions are always welcome.

nts or suggestions & questions are always welcome.