3GPP TSG-RAN5 Meeting #90-e	R5-211244r1

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0392 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference sensitivity TP analysis for DC_7A-20A_n1A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test points are not analyzed for DC_7A-20A_n1A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added DC_7A-20A_n1A in Table 4.3.3.1-2 to indicate that the anaysis is completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to test DC_7A-20A_n1A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR .38.521-3.. CR ..0934. |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | <CR implementation guideline: Table rows to be inserted in Table 4.3.3.1-1 and Table 4.3.3.2-1 after implementation of CR #0359. TR rapporteur to guide MCC in order to implement the CRs impacting the new Table format> R5-211244r1: Revised to show content from CR R5-210941 (CR #0359) and R5-210942 (CR #0359) with revision marks and grey highlight. Updated CR implementation guidline |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## [Start of changes]

#### 4.3.3.1	Reference sensitivity test cases for EN-DC

This clause contains information on test point analysis and status for FR1 NR CA test cases in TS 38.521-3 [4] clause 7. The analyses are performed per CA configuration in Table 4.3.3.1-1, Table 4.3.3.1-2 and Table 4.3.3.1-3.

Table 4.3.3.1-1: Reference Sensitivity test cases per EN-DC configuration (2CC)

| Band or band configuration |  | Single UL | Refsens exception, victim band (Note 2) | Requirement coverage (Note 2) | Comments |
| --- | --- | --- | --- | --- | --- |
| DC_(n)XAA |  |  |  |  |
| 5 |  | Only single UL requirements defined | 5 |  |  |
| 12 |  | Only single UL requirements defined | 12 |  |  |
| 38 |  | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 41 |  | Yes | N/A | NE | Added at RAN5#88-e |
| 48 |  | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 71 |  | Yes | 71 | Exception | Added at RAN5#88-e |
| DC_XA_nXA |  |  |  |  |
| DC_2A_n2A |  | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_3A_n3A |  | Yes | 3 | Exception | Added at RAN5#88-e |
| DC_5A_n5A |  | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_7A_n7A |  | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_41A_n41A |  | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_48(n)n48AA |  | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_66A_n66A |  | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_XA-nYA (Note 1) |  |  |  |  |
| DC_1A_n3A |  | Yes | 1 (IMD3) CBI | NE (Note 1) IMD3 CBI | Added at RAN5#89-e |
| DC_1A_n78A |  | No | 1 (IMD4) | IMD4 | Added at RAN5#88-e |
| DC_3A_n1A |  | Yes | 1 (IMD3) CBI | NE (Note 1) IMD3 CBI | Added at RAN5#89-e |
| DC_3A_n78A |  | Yes | 3 (IMD2) 3 (IMD4) 3 (HD) n78 (HM) | NE (Note 1) IMD2 PC2&PC3 IMD4 PC2&PC3 HD HM | Added at RAN5#89-e |
| DC_7A_n78A |  | No | N/A | NE | Added at RAN5#89-e |
| DC_20A_n3A |  | No | n3 (IMD4), 20 (IMD4) | IMD4, IMD4 | Added at RAN5#89-e |
| DC_20A_n78A |  | No | 20 (IMD4), n78A (HD) | IMD4, HD | Added at RAN5#88-e |
| DC_40A_n1A |  | No | N/A | NE | Added at RAN5#89-e |
| DC_40A_n78A |  | No | 40 (HM), | HM | Added at RAN5#89-e |
| NOTE 1: If single UL is allowed in a DC_XA-nYA configuration the IMD requirements does not apply for a single UL UE and non-exception requirements are tested. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |

Table 4.3.3.1-2: Reference Sensitivity test cases per EN-DC configuration (3CC)

| Band or band configuration |  | Single UL |  | UL config |  | Refsens exception, victim band |  | Requirement coverage (Note 2) |  | Fallback CA configurations |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XCA |  |  |  |  |  |  |
| 41 |  | Yes |  | DC_(n)41AA |  | N/A |  | None |  | DC_(n)41AA DC_41A_n41A |  | Added at RAN5#89-e |  |
| 41 |  | Yes |  | DC_41A_n41A |  | N/A |  | Non-exception |  | DC_(n)41AA DC_41A_n41A |  | Added at RAN5#89-e |  |
| 48 |  | Only single UL requirements defined |  | DC_(n)48AA |  | N/A |  | None |  | DC_(n)48AA DC_48A_n48A |  | Added at RAN5#89-e |  |
| 48 |  | Only single UL requirements defined |  | DC_48A_n48A |  | N/A |  | Non-exception |  | DC_(n)48AA DC_48A_n48A |  | Added at RAN5#89-e |  |
| DC_XA-nYC |  |  |  |  |  |  |
| DC_XA-nY(2A) |  |  |  |  |  |  |
| DC_XA-nYA-nZA |  |  |  |  |  |  |
| DC_XA-YA-nZA |  |  |  |  |  |  |
| DC_1A-7A_n78A |  | No |  | DC_1A_n78A |  | 7 (3 band IMD4) |  | IMD4 |  |  |  |  |  |
|  |  | No |  | DC_7A_n78A |  | 1 (3 band IMD4) |  | IMD4 |  |  |  |  |  |
| DC_1A-20A_n3A |  | Yes |  | DC_1A_n3A |  | No 3CC exception |  | No test required |  |  |  |  |  |
|  |  | No |  | DC_20A_n3A |  |  |  |  |  |  |  |  |  |
| DC_1A-20A_n78A |  | No |  | DC_1A_n78A |  | 20 (IMD5 ¦ |  | IMD5 |  |  |  |  |  |
|  |  | No |  | DC_20A_n78A |  | 1 (IMD3 |  | IMD3 |  |  |  |  |  |
| DC_3A-20A_n78 |  | Yes |  | DC_3A_n78A |  | No 3CC exception |  |  |  |  |  |  |  |
|  |  | No |  | DC_20A_n78A |  | 3 (3 band IMD3) |  | IMD3 |  |  |  |  |  |
| DC_3A-40A_n1A |  | Yes |  | DC_3A_n1A |  | 40 (3 band IMD5) |  | IMD5 if UE supporting dual UL |  |  |  |  |  |
|  |  | No |  | DC_40A_n1A |  | No 3CC exception |  |  |  |  |  |  |  |
| DC_7A-20A_n1A |  | No |  | DC_20A_n1A |  | 20 (3 band IMD5) |  | IMD5 |  |  |  | Added at RAN5#90-e |  |
|  |  | No |  | DC_7A_n1A |  | No 3CC exception |  | _ |  |  |  |  |  |
| DC_7A-20A_n78A |  | No |  | DC_7A_n78A |  | 20 (3 band IMD2) 20 (3 band IMD5) |  | IMD2, IMD5 |  |  |  |  |  |
|  |  | No |  | 2 DC_0A_n78A |  | 7 (3 band IMD2) |  | IMD2 |  |  |  |  |  |
| DC_(n)XAA-nYA |  |  |  |  |  |  |
| DC_XA_nXA_nYA |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA-nYA configuration the IMD requirements does not apply for a single UL UE NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |

Table 4.3.3.1-3: Reference Sensitivity test cases per EN-DC configuration (4CC)

| Band or band configuration | Single UL | UL config | Refsens exception, victim band | Requirement coverage (Note 2) | Fallback CA configurations | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XCA |  |  |  |  |  |  |
| 41 | Yes | DC_(n)41AA | N/A | None | DC_(n)41CA DC_41C_n41A | Added at RAN5#89-e |  |
| 41 | Yes | DC_41A_n41A | N/A | Non-exception | N/A | Added at RAN5#89-e |  |
| 48 | Only single UL requirements defined | DC_(n)48AA | N/A | None | DC_(n)48CA DC_48C_n48A | Added at RAN5#89-e |  |
| 48 | Only single UL requirements defined | DC_48A_n48A | N/A | Non-exception | N/A | Added at RAN5#89-e |  |
| DC_XA-nY(2A)-nZA |  |  |  |  |  |  |
| DC_XA-nYA-nZC |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA-nYA configuration the IMD requirements does not apply for a single UL UE. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |

[End of change]