3GPP TSG-RAN5 Meeting #91-e	R5-212439

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5017 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE TC 6.1.1.2a-Automatic PLMN selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, Cell 2 in TC 6.1.1.2a are replaced with Cell 11 to avoid SIB1 overlapping (Corresponding CR R5-210681). However, "Cell 2" in test procedure step 22-26 is still left. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. "Cell 2" in test procedure step 22-26 is replaced with "Cell 11". 2. Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.1.2a	PLMN selection of "Other PLMN/access technology combinations" / Automatic mode / Single Frequency operation

6.1.1.2a.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and EHPLMN and UPLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the EHPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(2)

with { UE in Automatic network selection mode and UPLMN and OPLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the UPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(3)

with { UE in Automatic network selection mode and OPLMN and other PLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the OPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(4)

with { UE in Automatic network selection mode and other PLMN/access technology combinations cells not belonging to any of EHPLMN, UPLMN or OPLMN available }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of other PLMN/access technology combinations and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

6.1.1.2a.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.2.2

6.1.1.2a.3	Test description

6.1.1.2a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency multi-PLMN cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.2a.3.1-1.

Table 6.1.1.2a.3.1–1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 11 | PLMN2 |
| 4 | PLMN3 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.2a.3.1-2.

Table 6.1.1.2a.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEHPLMN | 1 | PLMN1 Remaining mandatory entries use default values |  |
| EFPLMNwAcT | 1 | PLMN2 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 | PLMN3 Remaining defined entries use default values | E-UTRAN |
| EFUST |  | Services 20, 42 and 71 are supported. |  |

Preamble:

-	The UE is in state Switched OFF (State 1).

6.1.1.2a.3.2	Test procedure sequence

Table 6.1.1.2a.3.2 – 1 shows the cell configurations used during the test. Subsequent configurations marked “T1”, “T2” “T3” “T4”etc are applied at the points indicated in the Main behaviour description in Table 6.1.1.2a.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.2a.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 4 | Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -79 | "Off" | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | “Off” | -85 | -79 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T3 | RS EPRE | dBm/15kHz | “Off” | -79 | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T4 | RS EPRE | dBm/15kHz | “Off” | ”Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T5 | RS EPRE | dBm/15kHz | -79 | "Off" | "Off" | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.2a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4-18 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | Check: Is PLMN1 indicated by the UE? | - | - | 1 | P |
| 20 | SS adjusts cell levels according to row T2 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 21 | Check: Does the UE send an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 2 | P |
| 22-26 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 27 | Void | - | - | - | - |
| - | EXCEPTION: Step 27Aa1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 27Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 27 | Check: Is PLMN2 indicated by the UE? | - | - | 2 | P |
| 27A | SS adjusts cell levels according to row T5 of table 6.1.1.2a.3.2-1 and change PLMN identifiers of Cell 11 to PLMN4 | - | - | - | - |
| 27B | The generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. |  |  |  |  |
| 28 | SS adjusts cell levels according to row T3 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 29 | Check: Does the UE send an RRCConnectionRequest on Cell 4? | --> | RRCConnectionRequest | 3 | P |
| 30-34 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 4. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 35 | Check: Is PLMN3 indicated by the UE? | - | - | 3 | P |
| 36 | SS adjusts cell levels according to row T4 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 37 | Check: Does the UE send an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 4 | P |
| 38-42 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 43 | Check: Is PLMN4 indicated by the UE? | - | - | 4 | P |

6.1.1.2a.3.3	Specific message contents

None

<End of modified section 1>