3GPP TSG-RAN5 Meeting #91-e	R5-212633

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5301 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding PC6 in efeMTC Tx test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the WP, PC6 should be added to existing Tx test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PC6 to existing Tx test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | efeMTC WI cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1EA.1, 6.5.2.1EC.1, 6.6.2.1EC, 6.6.2.2EC, 6.6.2.3EC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

##### 6.5.2.1EA.1	Error Vector Magnitude (EVM) for UE category M1

6.5.2.1EA.1.1	Test Purpose

Same test purpose as in clause 6.5.2.1.1.

6.5.2.1EA.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

6.5.2.1EA.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3.

6.5.2.1EA.1.4	Test description

6.5.2.1EA.1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands 	. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1EA.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1EA.1.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for PUSCH EVM testing | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz |  | QPSK | 6 | 6 | 0 |
| 5MHz |  | QPSK | 1 (Note 3), 3 (Note 4) | 1 (Note 3), 3 (Note 4) | 0 |
| 5MHz |  | 16QAM | 5 | 5 | 0 |
| 5MHz |  | 16QAM | 1 (Note 3), 3 (Note 4) | 1 (Note 3), 3 (Note 4) | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (6 - RB allocation) of the channel bandwidth. Note 3: Only applicable for Power class 3 Note 4: Only applicable for Power class 5 and Power class 6 |  |  |  |  |  |

Table 6.5.2.1EA.1.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  |  | FDD and HD-FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD | Narrowband index2 |  |
| 5MHz | QPSK | 4 | 4 | 0 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 3: The RBstart of partial RB allocation can be either RB#0 or RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.5.2.1EA.1.4.1-3: Test Configuration for PRACH

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) | NC |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) | 5MHz, 15MHz |  |
| PRACH preamble format |  |  |
|  | FDD and HD-FDD | TDD |
| PRACH Configuration Index | 4 | 53 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 | -63 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 | -78 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.1EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.2.1EA.4.3.

6.5.2.1EA.1.4.2	Test procedure

Test procedure for PUSCH:

1.1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.5.2.1EA.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.2	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

1.3	Measure the EVM and  using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

1.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.5	Measure the EVM and  using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

Test procedure for PUCCH:

2.1	PUCCH are set according to Table 6.5.2.1EA.4.1-2.

2.2	SS transmits PDSCH via MPDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 6.5.2.1EA.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH. There is no PUSCH transmission.

2.3	SS send appropriate TPC commands for PUCCH to the UE until the UE transmit PUCCH at PUMAX level.

2.4	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

2.5	Send the appropriate TPC commands for PUCCH to the UE until the UE transmits PUCCH at -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.6	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

Test procedure for PRACH:

3.1	The SS shall set RS EPRE according to Table 6.5.2.1EA.4.1-3.

3.2	PRACH is set according to Table 6.5.2.1EA.4.1-3.

3.3	The SS shall signal a Random Access Preamble ID via a MPDCCH order to the UE and initiate a Non-contention based Random Access procedure

3.4	The UE shall send the signalled preamble to the SS.

3.5	In response to the preamble, the SS shall transmit a random access response not corresponding to the transmitted random access preamble, or send no response.

3.6	The UE shall consider the random access response reception not successful then re-transmit the preamble with the calculated PRACH transmission power.

3.7	Repeat step 5 and 6 until the SS collect enough PRACH preambles (2 preambles for format 0 and 10 preambles for format 4). Measure the EVM in PRACH channel using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

6.5.2.1EA.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and the following exceptions:

Table 6.5.2.1EA.4.3-1: PRACH-Config-DEFAULT: PRACH EVM measurement for FDD

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-7 PRACH-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 4 |  |  |

Table 6.5.2.1EA.4.3-2: PRACH-ConfCommonDEFAULT: PRACH EVM measurement for TDD

| Derivation Path: TS 36.508 [7] clause 5.3.1, Table 5.3.1-3 PRACH-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 53 |  |  |

Table 6.5.2.1EA.4.3-4: RACH-ConfigCommon-DEFAULT: PRACH EVM measurement

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-12 RACH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleInfo SEQUENCE { |  |  |  |
| numberOfRA-Preambles | n52 |  |  |
| preamblesGroupAConfig SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| powerRampingParameters SEQUENCE { |  |  |  |
| powerRampingStep | dB0 |  |  |
| preambleInitialReceivedTargetPower | dBm-120 Test point 1 dBm-90 Test point 2 |  |  |
| } |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n10 |  | FDD |
|  | n20 |  | TDD |
| ra-ResponseWindowSize | Sf10 |  |  |
| mac-ContentionResolutionTimer | sf48 |  |  |
| } |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |

Table 6.5.2.1EA.4.3-5: TDD-Config-DEFAULT: PRACH EVM measurement for TDD

| Derivation Path: TS 36.508 [7] clause 5.3.1, Table 5.3.1-1: TDD-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| subframeAssignment | sa1 |  |  |
| specialSubframePatterns | ssp5 | To enable two symbol UpPTS, and to have 9 symbols GP. |  |
| } |  |  |  |

6.5.2.1EA.1.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed [17,5 %] when embedded with data symbols of QPSK and BPSK, [12,5%] for 16 QAM.

The PUCCH EVM and derived in E.5.9.2 shall not exceed 17,5 %.

The PRACH EVM derived in FFS shall not exceed 17.5%.

<< Clauses skipped here>>

##### 6.5.2.1EC.1	Error Vector Magnitude (EVM) for UE category M2

6.5.2.1EC.1.1	Test Purpose

Same test purpose as in clause 6.5.2.1.1.

6.5.2.1EC.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

6.5.2.1EC.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3.

6.5.2.1EC.1.4	Test description

6.5.2.1EC.1.4.1		Initial conditions

Same as in clause 6.5.2.1EA.1.4.1 with the following exceptions:

-	Instead of Table 6.5.2.1EA.1.4.1-1  use Table 6.5.2.1EC.1.4.1-1

-	Instead of Table 6.5.2.1EA.1.4.1-2  use Table 6.5.2.1EC.1.4.1-2

-	Instead of Table 6.5.2.1EA.1.4.1-3  use Table 6.5.2.1EC.1.4.1-3

Table 6.5.2.1EC.1.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for PUSCH EVM testing | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz |  | QPSK | 24 | 24 | 0 |
| 5MHz |  | QPSK | 1 (Note 3), 3 (Note 4) | 1 (Note 3), 3 (Note 4) | 0 |
| 5MHz |  | 16QAM | 24 | 24 | 0 |
| 5MHz |  | 16QAM | 1 (Note 3), 3 (Note 4) | 1 (Note 3), 3 (Note 4) | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 3: Only applicable for Power class 3 Note 4: Only applicable for Power class 5 and Power class 6 |  |  |  |  |  |

Table 6.5.2.1EC.1.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  |  | FDD and HD-FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD | Narrowband index2 |  |
| 5MHz | QPSK | 16 | 16 | 0 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

Table 6.5.2.1EC.1.4.1-3: Test Configuration for PRACH

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) | NC |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) | 5MHz, 15MHz |  |
| PRACH preamble format |  |  |
|  | FDD and HD-FDD | TDD |
| PRACH Configuration Index | 4 | 53 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 | -63 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 | -78 |

6.5.2.1EC.1.4.3	Message contents

Same as in clause 6.5.2.1EA.1.4.3.

6.5.2.1EC.1.5	Test requirement

Same as in clause 6.5.2.1EA.1.5.

<< Clauses skipped here>>

#### 6.6.2.1EC	Spectrum Emission Mask for UE category M2

##### 6.6.2.1EC.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

##### 6.6.2.1EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.6.2.1EC.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1EC.3-1, the spurious requirements in clause 6.6.3EC are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1EC.3-1 for the specified channel bandwidth.

Table 6.6.2.1EC.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.1.

##### 6.6.2.1EC.4	Test description

6.6.2.1EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1EC.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 0 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 0 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 0 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 0 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 0 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 0 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 0 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 0 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 0 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 0 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 0 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 0 |
| 24 | 15MHz |  | QPSK | 16 | 16 | 0 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 0 |
| 26 | 15MHz |  | 16QAM | 16 | 16 | 0 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 0 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 0 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 0 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 0 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 0 |
| High range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 1 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 3 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 3 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 3 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 3 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 3 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 7 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 7 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 4 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 7 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 7 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 4 |
| 24 | 15MHz |  | QPSK | 15 | 15 | 9 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 8 |
| 26 | 15MHz |  | 16QAM | 15 | 15 | 9 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 8 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 13 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 12 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 13 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 or power class 6 UE |  |  |  |  |  |  |

Table 6.6.2.1EC.4.1-2: Test Configuration Table, subPRB allocation

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, 15 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 5MHz |  | π/2-BPSK | ¼ | 0 |  |
| 2 | 5MHz |  | QPSK | ¼ | 0 |  |
| 3 | 5MHz |  | QPSK | ½ | 0 |  |
| 4 | 10MHz |  | π/2-BPSK | ¼ | 0 |  |
| 5 | 10MHz |  | π/2-BPSK | ¼ | 4 |  |
| 6 | 10MHz |  | QPSK | ¼ | 0 |  |
| 7 | 10MHz |  | QPSK | ¼ | 4 |  |
| 8 | 10MHz |  | QPSK | ½ | 0 |  |
| 9 | 10MHz |  | QPSK | ½ | 4 |  |
| 10 | 15MHz |  | π/2-BPSK | ¼ | 0 |  |
| 11 | 15MHz |  | π/2-BPSK | ¼ | 8 |  |
| 12 | 15MHz |  | QPSK | ¼ | 0 |  |
| 13 | 15MHz |  | QPSK | ¼ | 8 |  |
| 14 | 15MHz |  | QPSK | ½ | 0 |  |
| 15 | 15MHz |  | QPSK | ½ | 8 |  |
| High range |  |  |  |  |  |  |
| 16 | 5MHz |  | π/2-BPSK | ¼ | 0 |  |
| 17 | 5MHz |  | QPSK | ¼ | 0 |  |
| 18 | 5MHz |  | QPSK | ½ | 0 |  |
| 19 | 10MHz |  | π/2-BPSK | ¼ | 0 |  |
| 20 | 10MHz |  | π/2-BPSK | ¼ | 4 |  |
| 21 | 10MHz |  | QPSK | ¼ | 0 |  |
| 22 | 10MHz |  | QPSK | ½ | 4 |  |
| 23 | 10MHz |  | QPSK | ¼ | 0 |  |
| 24 | 10MHz |  | QPSK | ½ | 4 |  |
| 25 | 15MHz |  | π/2-BPSK | ¼ | 0 |  |
| 26 | 15MHz |  | π/2-BPSK | ¼ | 8 |  |
| 27 | 15MHz |  | QPSK | ¼ | 0 |  |
| 28 | 15MHz |  | QPSK | ¼ | 8 |  |
| 29 | 15MHz |  | QPSK | ½ | 0 |  |
| 30 | 15MHz |  | QPSK | ½ | 8 |  |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1EC.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.1EC.4.3.

6.6.2.1EC.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.2.1EC.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.3EC.5-1 to 6.2.3EC.5-3. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard sub frame are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1EC.5-1, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

5.	For UE supporting subPRB allocation, repeat step 1-4 with UL RMC according to Table 6.6.2.1EC.4.1-2.

6.6.2.1EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.6.2.1EC.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.3EC.5-1 to 6.2.3EC.5-3 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1EC.5-1, as applicable.

Table 6.6.2.1EC.5-1: General E-UTRA spectrum emission mask

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth

<< Clauses skipped here>>

#### 6.6.2.2EC	Additional Spectrum Emission Mask for UE category M2

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- NS_03 (B2, B4), NS_04 (B41) and NS_07 (B13): Test configuration table missing due to minimum requirements for A-MPR in 36.101 are incomplete.

##### 6.6.2.2EC.1	Test purpose

Same test purpose as in clause 6.6.2.2.1.

##### 6.6.2.2EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.6.2.2EC.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2.3

##### 6.6.2.2EC.4	Test description

6.6.2.2EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2EC.4.1-1 through 6.6.2.2EC.4.1-5. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2EC.4.1-1: Test Configuration Table (network signalled value "NS_03")

TBD

Table 6.6.2.2EC.4.1-2: Test Configuration Table (network signalled value "NS_04")

TBD

Table 6.6.2.2EC.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 (Note 4) | 5MHz |  | QPSK | 1 | 0 |
| 7 (Note 5) | 5MHz |  | QPSK | 3 | 0 |
| 8 | 5MHz |  | QPSK | 24 | 0 |
| 9 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 10 | 5MHz |  | 16QAM | 3 | 0 |
| 11 (Note 5) | 5MHz |  | 16QAM | 5 | 0 |
| 12 | 5MHz |  | 16QAM | 24 | 0 |
| 13 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 14 (Note 5) | 10MHz |  | QPSK | 5 | 0 |
| 15 | 10MHz |  | QPSK | 24 | 0 |
| 16 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 17 | 10MHz |  | 16QAM | 5 | 0 |
| 18 | 10MHz |  | 16QAM | 24 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 (Note 4) | 5MHz |  | QPSK | 1 | 3 |
| 7 (Note 5) | 5MHz |  | QPSK | 3 | 3 |
| 8 | 5MHz |  | QPSK | 24 | 0 |
| 9 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 10 | 5MHz |  | 16QAM | 3 | 3 |
| 11 (Note 5) | 5MHz |  | 16QAM | 5 | 3 |
| 12 | 5MHz |  | 16QAM | 24 | 0 |
| 13 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 14 (Note 5) | 10MHz |  | QPSK | 5 | 7 |
| 15 | 10MHz |  | QPSK | 24 | 4 |
| 16 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 17 | 10MHz |  | 16QAM | 5 | 7 |
| 18 | 10MHz |  | 16QAM | 24 | 4 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 and power class 6 UE |  |  |  |  |  |

Table 6.6.2.2EC.4.1-4: Test Configuration Table (network signalled value "NS_07")

TBD

Table 6.6.2.2EC.4.1-5: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 (Note 4) | 5MHz |  | QPSK | 1 | 0 |
| 2 (Note 5) | 5MHz |  | QPSK | 3 | 0 |
| 3 | 5MHz |  | QPSK | 24 | 0 |
| 4 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 5 | 5MHz |  | 16QAM | 3 | 0 |
| 6 (Note 5) | 5MHz |  | 16QAM | 5 | 0 |
| 7 | 5MHz |  | 16QAM | 24 | 0 |
| 8 | 10MHz |  | QPSK | 4 | 0 |
| 9 (Note 5) | 10MHz |  | QPSK | 5 | 0 |
| 10 | 10MHz |  | QPSK | 24 | 0 |
| 11 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 12 | 10MHz |  | 16QAM | 5 | 0 |
| 13 | 10MHz |  | 16QAM | 24 | 0 |
| 14 | 20MHz |  | QPSK | 18 | 0 |
| 15 | 20MHz |  | QPSK | 24 | 0 |
| 16 | 20MHz |  | 16QAM | 18 | 0 |
| 17 | 20MHz |  | 16QAM | 24 | 0 |
| High range |  |  |  |  |  |
| 1 (Note 4) | 5MHz |  | QPSK | 1 | 3 |
| 2 (Note 5) | 5MHz |  | QPSK | 3 | 3 |
| 3 | 5MHz |  | QPSK | 24 | 0 |
| 4 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 5 | 5MHz |  | 16QAM | 3 | 3 |
| 6 (Note 5) | 5MHz |  | 16QAM | 5 | 3 |
| 7 | 5MHz |  | 16QAM | 24 | 0 |
| 8 | 10MHz |  | QPSK | 4 | 7 |
| 9 (Note 5) | 10MHz |  | QPSK | 5 | 7 |
| 10 | 10MHz |  | QPSK | 24 | 4 |
| 11 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 12 | 10MHz |  | 16QAM | 5 | 7 |
| 13 | 10MHz |  | 16QAM | 24 | 4 |
| 14 | 20MHz |  | QPSK | 18 | 13 |
| 15 | 20MHz |  | QPSK | 24 | 12 |
| 16 | 20MHz |  | 16QAM | 18 | 13 |
| 17 | 20MHz |  | 16QAM | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 and power class 6 UE |  |  |  |  |  |

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Tables 6.6.2.2EC.4.1-1 to 6.6.2.2EC.4.1-5.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.2EC.4.3.

6.6.2.2EC.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to applicable table from Table 6.6.2.2EC.4.1-1 to 6.6.2.2EC.4.1-5. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.4EA.5-1 to 6.2.4EA.5-16 as appropriate. The period of the measurement shall be at least one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.2.2EC.5.1-1, 6.6.2.2EC.5.2-1, 6.6.2.2EC.5.3-1 and, 6.6.2.2EC.5.4-1 as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and with the same exceptions as in clauses 6.2.4.4.3.1 to 6.2.4.4.3.30 for each network signalled value.

Message contents are same as in clause 6.2.4EA.4.3.

##### 6.6.2.2EC.5	Test requirements

6.6.2.2EC.5.1	Test requirements (network signalled value "NS_03”)

When "NS_03” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EC.5-1 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EC.5.1-1, as applicable.

Table 6.6.2.2EC.5.1-1: Additional requirements (network signalled value "NS_03”)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4E Table 6.2.4E-2. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2EC.5.2	Test requirements (network signalled value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EC.5-2 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EC.5.2-1, as applicable.

Table 6.6.2.2EC.5.2-1: Additional requirements (network signalled value "NS_04")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -23.5 | -23.5 | -23.5 | -23.5 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4E Table 6.2.4E-2. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2EC.5.3	Test requirements (network signalled value "NS_06" or "NS_07")

When "NS_06" or "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EC.5-4A, 6.2.4EC.5-4B and 6.2.4EC.5-5 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EC.5.3-1, as applicable.

Table 6.6.2.2EC.5.3-1: Additional requirements (network signalled value "NS_06" or "NS_07")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4E Table 6.2.4E-2. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2EC.5.4	Test requirements (network signalled value "NS_35")

When "NS_35" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EC.5-16 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EC.5.4-1, as applicable.

Table 6.6.2.2EC.5.4-1: Additional requirements (network signalled value "NS_35")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| 0-0.1 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 0.1-6 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 6-10 | -23.55 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 10-15 |  | -23.55 | -11.5 | -11.5 | 100 kHz |
| 15-20 |  |  | -23.55 | -11.5 | 100 kHz |
| 20-25 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_35 as defined in TS 36.101 [2] subclause 6.2.4E Table 6.2.4E-2. Note 5: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

<< Clauses skipped here>>

#### 6.6.2.3EC	Adjacent Channel Leakage power Ratio for UE category M2

##### 6.6.2.3EC.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

##### 6.6.2.3EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.6.2.3EC.3	Minimum conformance requirements

ACLR requirements are specified for two scenarios for an adjacent E -UTRAACLR and UTRAACLR1/2 as shown in Figure 6.6.2.3EC.3-1.

Figure 6.6.2.3EC.3-1: Adjacent Channel Leakage Power Ratio requirements

6.6.2.3EC.3.1	Minimum conformance requirements for E-UTRA

E-UTRA ACLR (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidth specified in Table 6.6.2.3EC.3.1-1.

If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the valued specified in Table 6.6.2.3EC.3.1-1.

Table 6.6.2.3EC.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.1.

6.6.2.3EC.3.2	 Minimum conformance requirements for UTRA

UTRA ACLR (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent UTRA channel frequency.

UTRA ACLR is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2).The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor = 0.22. The assigned E-UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3EC.3.2-1.

If the measured UTRA channel power is greater than –50dBm then the UTRAACLR1, and UTRAACLR2 shall be higher than the valued specified in Table 6.6.2.3EC.3.2-1.

Table 6.6.2.3EC.3.2-1: General requirements for UTRAACLR1/2

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 subclause 6.6.2.3.2.

##### 6.6.2.3EC.4	Test description

6.6.2.3EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3EC.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 0 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 0 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 0 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 0 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 0 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 0 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 0 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 0 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 0 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 0 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 0 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 0 |
| 24 | 15MHz |  | QPSK | 16 | 16 | 0 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 0 |
| 26 | 15MHz |  | 16QAM | 16 | 16 | 0 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 0 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 0 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 0 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 0 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 0 |
| High range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 1 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 3 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 3 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 3 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 3 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 3 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 7 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 7 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 4 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 7 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 7 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 4 |
| 24 | 15MHz |  | QPSK | 15 | 15 | 9 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 8 |
| 26 | 15MHz |  | 16QAM | 15 | 15 | 9 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 8 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 13 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 12 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 13 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 and power class 6 UE |  |  |  |  |  |  |

Table 6.6.2.3EC.4.1-2: Test Configuration Table, subPRB allocation

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, 15 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 5MHz |  | π/2-BPSK | ¼ | 0 |  |
| 2 | 5MHz |  | QPSK | ¼ | 0 |  |
| 3 | 5MHz |  | QPSK | ½ | 0 |  |
| 4 | 10MHz |  | π/2-BPSK | ¼ | 0 |  |
| 5 | 10MHz |  | π/2-BPSK | ¼ | 4 |  |
| 6 | 10MHz |  | QPSK | ¼ | 0 |  |
| 7 | 10MHz |  | QPSK | ¼ | 4 |  |
| 8 | 10MHz |  | QPSK | ½ | 0 |  |
| 9 | 10MHz |  | QPSK | ½ | 4 |  |
| 10 | 15MHz |  | π/2-BPSK | ¼ | 0 |  |
| 11 | 15MHz |  | π/2-BPSK | ¼ | 8 |  |
| 12 | 15MHz |  | QPSK | ¼ | 0 |  |
| 13 | 15MHz |  | QPSK | ¼ | 8 |  |
| 14 | 15MHz |  | QPSK | ½ | 0 |  |
| 15 | 15MHz |  | QPSK | ½ | 8 |  |
| High range |  |  |  |  |  |  |
| 16 | 5MHz |  | π/2-BPSK | ¼ | 0 |  |
| 17 | 5MHz |  | QPSK | ¼ | 0 |  |
| 18 | 5MHz |  | QPSK | ½ | 0 |  |
| 19 | 10MHz |  | π/2-BPSK | ¼ | 0 |  |
| 20 | 10MHz |  | π/2-BPSK | ¼ | 4 |  |
| 21 | 10MHz |  | QPSK | ¼ | 0 |  |
| 22 | 10MHz |  | QPSK | ½ | 4 |  |
| 23 | 10MHz |  | QPSK | ¼ | 0 |  |
| 24 | 10MHz |  | QPSK | ½ | 4 |  |
| 25 | 15MHz |  | π/2-BPSK | ¼ | 0 |  |
| 26 | 15MHz |  | π/2-BPSK | ¼ | 8 |  |
| 27 | 15MHz |  | QPSK | ¼ | 0 |  |
| 28 | 15MHz |  | QPSK | ¼ | 8 |  |
| 29 | 15MHz |  | QPSK | ½ | 0 |  |
| 30 | 15MHz |  | QPSK | ½ | 8 |  |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |  |

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3EC.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.3EC.4.3.

6.6.2.3EC.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.2.3EC.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3EC.5-1 to 6.2.3EC.5-3. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel, respectively.

6.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the E-UTRA channel, respectively.

7.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

8.	Calculated the ratios of the power between the values measured in step 4 over step 6 for lower and upper UTRAACLR1, UTRAACLR2, respectively.

9.	For UE supporting subPRB allocation, repeat step 1-8 with UL RMC according to Table 6.6.2.3EC.4.1-2.

6.6.2.3EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.6.2.3EC.5	Test requirement

6.6.2.3EC.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3EC.5-1, 6.2.3EC.5-2 and 6.2.3EC.5-3 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3EC.5.1-1.

Table 6.6.2.3EC.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3EC.5.2	Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3EC.5.2-1.

Table 6.6.2.3EC.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

<< End of changes>>