3GPP TSG-RAN5 Meeting #91-e	R5-212923

# Electronic, 17th – 28th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1903 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of V2X test frequencies of band n47 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | V2X operation is specified on band n47. The test frequencies are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding V2X test frequencies on band n47. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | V2X testing on band n47 can’t be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

4.3.1.7	Test frequencies for Non-3GPP Access

4.3.1.7.1	WLAN Test frequencies

The same WLAN test frequencies as in TS 36.508 [2] clause 4.3.1.6 applies.

4.3.1.8	Test frequencies for NR Sidelink operating bands

4.3.1.8.1	Test frequencies for NR Sidelink operating bands in FR1

4.3.1.8.1.1	Reference test frequencies for NR Sidelink operating band n38

FFS

4.3.1.8.1.2	Reference test frequencies for NR Sidelink operating band n47

Table 4.3.1.8.1.2-1: Test frequencies for NR operating band n47 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 52 | Low | 5860.005 | 790667 | 5855.325 | 790355 | 0 | 790421 | 790673 | 790913 |
|  |  | Mid | 5889.99 | 792666 | 5794.59 | 786306 | 504 | 792420 | 792672 | 792912 |
|  |  | High | 5919.99 | 794666 | 5914.23 | 794282 | 6 | 794420 | 794672 | 794912 |
| 20 | 106 | Low | 5865 | 791000 | 5855.46 | 790364 | 0 | 790430 | 791006 | 791570 |
|  |  | Mid | 5889.99 | 792666 | 5789.73 | 785982 | 504 | 792096 | 792672 | 793236 |
|  |  | High | 5914.995 | 794333 | 5904.375 | 793625 | 6 | 793763 | 794339 | 794903 |
| 30 | 160 | Low | 5870.01 | 791334 | 5855.61 | 790374 | 0 | 790440 | 791340 | 792228 |
|  |  | Mid | 5889.99 | 792666 | 5784.87 | 785658 | 504 | 791772 | 792672 | 793560 |
|  |  | High | 5910 | 794000 | 5894.52 | 792968 | 6 | 793106 | 794006 | 794894 |
| 40 | 216 | Low | 5875.005 | 791667 | 5855.565 | 790371 | 0 | 790437 | 791673 | 792897 |
|  |  | Mid | 5889.99 | 792666 | 5779.83 | 785322 | 504 | 791436 | 792672 | 793896 |
|  |  | High | 5904.99 | 793666 | 5884.47 | 792298 | 6 | 792436 | 793672 | 794896 |

Table 4.3.1.8.1.2-2: Test frequencies for NR operating band n47 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 24 | Low | 5860.005 | 790667 | 5855.685 | 790379 | 0 | 790511 | 790679 | 790823 |
|  |  | Mid | 5889.99 | 792666 | 5704.23 | 780282 | 504 | 792510 | 792678 | 792822 |
|  |  | High | 5919.99 | 794666 | 5913.51 | 794234 | 6 | 794510 | 794678 | 794822 |
| 20 | 51 | Low | 5865 | 791000 | 5855.82 | 790388 | 0 | 790520 | 791000 | 791480 |
|  |  | Mid | 5889.99 | 792666 | 5699.37 | 779958 | 504 | 792186 | 792666 | 793146 |
|  |  | High | 5914.995 | 794333 | 5903.655 | 793577 | 6 | 793853 | 794333 | 794813 |
| 30 | 78 | Low | 5870.01 | 791334 | 5855.97 | 790398 | 0 | 790530 | 791346 | 792138 |
|  |  | Mid | 5889.9 | 792666 | 5694.51 | 779634 | 504 | 791862 | 792678 | 793470 |
|  |  | High | 5910 | 794000 | 5893.8 | 792920 | 6 | 793196 | 794012 | 794804 |
| 40 | 106 | Low | 5875.005 | 791667 | 5855.925 | 790395 | 0 | 790527 | 791679 | 792807 |
|  |  | Mid | 5889.99 | 792666 | 5689.47 | 779298 | 504 | 791526 | 792678 | 793806 |
|  |  | High | 5904.99 | 793666 | 5883.75 | 792250 | 6 | 792526 | 793678 | 794806 |

Table 4.3.1.8.1.2-3: Test frequencies for NR operating band n47 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 11 | Low | 5860.005 | 790667 | 5856.045 | 790403 | 0 | 790667 | 790667 | 790667 |
|  |  | Mid | 5889.99 | 792666 | 5523.15 | 768210 | 504 | 792666 | 792666 | 792666 |
|  |  | High | 5919.99 | 794666 | 5911.71 | 794114 | 6 | 794666 | 794666 | 794666 |
| 20 | 24 | Low | 5865 | 791000 | 5856.36 | 790424 | 0 | 790688 | 791024 | 791312 |
|  |  | Mid | 5889.99 | 792666 | 5518.47 | 767898 | 504 | 792354 | 792690 | 792978 |
|  |  | High | 5914.995 | 794333 | 5902.035 | 793469 | 6 | 794021 | 794357 | 794645 |
| 30 | 38 | Low | 5870.01 | 791334 | 5856.33 | 790422 | 0 | 790686 | 791358 | 791982 |
|  |  | Mid | 5889.99 | 792666 | 5513.43 | 767562 | 504 | 792018 | 792690 | 793314 |
|  |  | High | 5910 | 794000 | 5892 | 792800 | 6 | 793352 | 794024 | 794648 |
| 40 | 51 | Low | 5875.005 | 791667 | 5856.645 | 790443 | 0 | 790707 | 791667 | 792627 |
|  |  | Mid | 5889.99 | 792666 | 5508.75 | 767250 | 504 | 791706 | 792666 | 793626 |
|  |  | High | 5904.99 | 793666 | 5882.31 | 792154 | 6 | 792706 | 793666 | 794626 |

4.3.2	Radio conditions

4.3.2.1	FR1, normal propagation condition for connected

<End of Changes >