3GPP TSG-RAN5 Meeting #91-e	R5-213510

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2171 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 11.4.6-Handling of non-allowed tracking areas |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - There are two step 6, so the step number needs to be corrected. - Since T3540 will be started after the mobility registration procedure, the test case shall end with the switch off procedure with T3540, therefore, a new step is added to specify this condition. - System information combination NR-1 should be NR-2 since there is Cell reselection in the test procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the step number from step 6. - Added step 12 to refer to the generic switch off procedure with T3540. - Change System information combination NR-1 to NR-2. - According to the TP1, the UE shall enter 5GMM-REGISTERED.NON-ALLOWED-SERVICE and 5GMM-IDLE mode, in current implementation, the UE is in normal 5GMM-REGISTERED state before step 2, instead of 5GMM-REGISTERED.NON-ALLOWED-SERVICE. Only when the UE camped on a cell whose TAI is in the list of "non-allowed tracking areas", the UE will enter the state 5GMM-REGISTERED.NON-ALLOWED-SERVICE. Therefore, the UE shall perform mobility registration to NR Cell 11 before step 2. Step 2 is changed to perform mobility registration - According to following definition in TS 23.501, when the UE is in non-allowed area, the UE is allowed to initiate registrtion procedure without the Uplink data status IE, so it’s meaningless to prove that the UE camped on NR Cell 11 could perform mobility registration to NR Cell 2. TS 23.501: Non-Allowed Area: Area where the UE is allowed to initiate Registration procedure but no other communication as specified in clause 5.3.2.3. Therefore, the original step 9 and 10 have been removed, and the information for NR Cell 2 has also been removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-212436r2. |  |  |  |  |  |  |  |  |

<Start of modified section >

11.4.6	5GMM-REGISTERED.NON-ALLOWED-SERVICE / Emergency call establishment and release / Handling of non-allowed tracking areas

11.4.6.1	Test Purpose (TP)

(1)

with { UE in 5GMM-REGISTERED.NON-ALLOWED-SERVICE state and 5GMM-IDLE mode }

ensure that {

when { UE is requested to make an Emergency call }

then { UE establishes the Emergency call }

}

(2)

with { UE in 5GMM-REGISTERED.NON-ALLOWED-SERVICE state and 5GMM-CONNECTED mode having established an Emergency call }

ensure that {

when { UE is requested to release the Emergency call }

then { UE releases the Emergency call, and, the UE considers the current cell as belonging to non-allowed tracking areas }

}

11.4.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501 [22], subclause 5.3.5, TS 22.101 [42], subclause 10.1.1. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, subclause 5.3.5]

When the UE receives a Service area list IE with a non-allowed area indication during a registration procedure or a generic UE configuration update procedure, the UE shall delete the old list of "non-allowed tracking areas" and store the tracking areas in the non-allowed area as the list of "non-allowed tracking areas". If the UE has a stored list of "allowed tracking areas", the UE shall delete that list.

...

If the UE is successfully registered to a PLMN and has a stored list of "non-allowed tracking areas":

...

b)	while camped on a cell whose TAI is in the list of "non-allowed tracking areas", the UE shall enter the state 5GMM-REGISTERED.NON-ALLOWED-SERVICE, and:

1)	if the UE is in 5GMM-IDLE mode over 3GPP access, the UE:

...

ii)	shall not initiate a service request procedure except for emergency services, high priority access, responding to paging or notification or indicating a change of 3GPP PS data off UE status; and

[TS 22.101, subclause 10.1.1]

The ME shall identify an emergency number dialled by the end user as a valid emergency number and initiate emergency call establishment if it occurs under one or more of the following conditions. If it occurs outside of the following conditions, the ME should not initiate emergency call establishment but normal call establishment. Emergency number identification takes place before and takes precedence over any other (e.g. supplementary service related) number analysis.

a)	112 and 911 shall always be available. These numbers shall be stored on the ME.

b)	Any emergency call number stored on a SIM/USIM when the SIM/USIM is present.

c)	000, 08, 110, 999, 118 and 119 when a SIM/USIM is not present. These numbers shall be stored on the ME.

d)	Additional emergency call numbers that may have been downloaded by the serving network when the SIM/USIM is present.

11.4.6.3	Test description

11.4.6.3.1	Pre-test conditions

System Simulator:

-	3 NR Cells

-	NR Cell 1 and NR Cell 11 as defined in TS 38.508-1 [4] Table 4.4.2-3.

-	Maximum of 2 cells are active at any point of time.

-	On all cells when active: System information combination NR-2 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport.

UE:

None.

Preamble:

-	Cells power level configuration in accordance with TS 38.508-1 [4], Table 6.2.2.1-3:

-	NR Cell 1 "Serving cell"

-	NR Cell 11 "Non-Suitable "Off" cell"

-	The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1

-	During the initial registration:

-	The list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1 whereas the TAI of NR Cell 11 is indicated in the Service area list IE with non-allowed areas as specified in Table 11.4.6.3.3-2.

11.4.6.3.2	Test procedure sequence

Table 11.4.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NR Cell 11 as "Serving cell" - NR Cell 1 as "Non-Suitable "Off" cell" | - | - | - | - |
| - | The following messages are to be observed on NR Cell 11 unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE performs a registration for mobility procedure and the RRC connection is released according to generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1, which indicates that the UE is camped on NR Cell 11. | - | - | - | - |
| 3 | Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911. (NOTE 1) | - | - | - | - |
| 4 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11? | - | - | 1 | P |
| 5 | Make the UE release the emergency call. (NOTE 1) | - | - | - | - |
| 6 | The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 7 | SS releases the RRC connection | <-- | NR RRC: RRCRelease | - | - |
| 8 | Make the UE attempt an IMS none-emergency call. (NOTE 1) |  |  |  |  |
| 9 | Check: Does the UE transmit in the next 10 sec an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 2 | F |
| NOTE 1: This could be done by e.g. MMI or AT command. |  |  |  |  |  |

11.4.6.3.3	Specific message contents

Table 11.4.6.3.3-2: REGISTRATION ACCEPT (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Service area list |  |  |  |
| Allowed type (octet 1) | '1'B | TAIs in the list are in the non-allowed area |  |
| Type of list (octet 1) | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| Number of elements (octet 1) | '00000'B | 1 element |  |
| MCC | As defined for NR Cell 11 in TS 38.508-1 [4], Table 4.4.2-3. |  |  |
| MNC | As defined for NR Cell 11 in TS 38.508-1 [4], Table 4.4.2-3. |  |  |
| TAC 1 | As defined for NR Cell 11 in TS 38.508-1 [4], Table 4.4.2-3. |  |  |

<End of modified section >