3GPP TSG-RAN5 Meeting #91-e                      	R5-213930

# Electronic Meeting, 17th May – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1211 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.8 including TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test cases 5.6.2.8 is incomplete. TT is missing. Test parameters in 38.133 has been corrected. |  |  |  |  |  |  |  |  |
| R4- |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor’s notes removed - changed a4-Threshold value to -105dBm - Added Dedicated CORESET RMC configuration - Corrected other cell specific values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revised from: R5-213226 Corrected gap configuration for test 1 and 2 in message content |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of change

#### 5.6.2.8	EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection

5.6.2.8.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event in DRX within inter-frequency cell search requirements with SSB time index detection.

5.6.2.8.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC FR2 and long DRX cycle.

5.6.2.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.5.6.2.8.5.6.2.8.4	Test description

5.6.2.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.6.2.8.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 5.6.2.8.4.1-2. Test environment parameters are given in Table 5.6.2.8.4.1-3.

Table 5.6.2.8.4.1-1: EN-DC FR1-FR2 event triggered reporting tests in DRX with SSB time index detection supported test configurations

| Test Case ID | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 5.6.2.8-1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 5.6.2.8-2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 5.6.2.8-3 | LTE FDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| 5.6.2.8-4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |  |
| 5.6.2.8-5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 5.6.2.8-6 | LTE TDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The target NR cell3 has the same SCS, BW and duplex mode as NR serving cell2 |  |  |

Table A.5.6.2.8.4.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  |  |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 |  | 13 |  | As specified in TS 38.133 clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 39 |  | 39 |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1,4 | SSB.1 FR1 |  |  |  | As specified in clause A.3 |
|  |  | Config 2,5 | SSB.1 FR1 |  |  |  | As specified in clause A.3 |
|  |  | Config 3,6 | SSB.2 FR1 |  |  |  | As specified in clause A.3 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3,4,5,6 | SSB.3 FR2 |  |  |  | As specified in clause A.3 |
| offsetMO | dB | Config 1,2,3,4,5,6 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3,4,5,6 | -105 |  |  |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.5 |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 2,3,5,6 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 5.6.2.8.4-3: Test Environment test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table TBD and TS 38.508-1 [14] clause TBD |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 5.6.2.8.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR2 cells in different frequencies. Cell 2 is the PSCell and Cell 3 is the target cell. The power levels and settings for Cell 2 and Cell 3 are set according to Annex TBD.

3.	If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

4. The UE Rx beam peak direction for Cell 3 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

5.6.2.8.4.2	Test procedure

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR2 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 5.6.2.8.4.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table 5.6.2.8.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 5.6.2.8.4-2 The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.. T1 starts,

3. The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4. The UE shall transmit RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 5.6.2.8.4.1-2 .

6. UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delay measured from the beginning of time period T2 is less than 10082 ms for UE supporting power class 1, or 6242 ms for UE supporting other power class for Test 1 and Test 3 and 107522 ms for UE supporting power class 1, or 66562 ms for UE supporting other power class  for Test 2 and Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.  Set Cell 3 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On  according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 5.6.2.8.4.1-2 as appropriate.

TBD5.6.2.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause TBD with the following exceptions:

Table 5.6.2.8.4.3-1: Common Exception messages for Additional EN-DC FR1-FR2 event triggered reporting tests with SSB time index detection in DRX test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with condition SSB Index and A4-threshold = -105 Table H.3.1-6 with conditions gapUE and Pattern #0 for Test 1 and Test 2 Table H.3.1-6 with conditions gapFR2 and Pattern #13 for Test 3 and Test 4 Table H.3.4-4 with condition gapUE for Test 1 and Test 2 Table H.3.4-5 with condition Pattern #0 for Test 1 and Test 2 Table H.3.1-7 with condition SSB Index and INTER-FREQ Table H.3.7-1 with condition DRX.1 for Test 1 and Test 3 Table H.3.7-1 with condition DRX.2 for Test 2 and Test 4 Table H.3.4-1 Table H.3.4-2 Table H.3.4-3 |

Table 5.6.2.5.8.3-2: MeasObjectNR-DEFAULT: EN-DC FR1-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | 6 dB |  |  |
| } |  |  |  |

5.6.2.8.5	Test requirement

Table 5.6.2.8.5-1 defines the primary level settings including test tolerances for all tests.

Table 5.6.2.8.5-1: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection in DRX

| Parameter | Unit |  | Test configuration |  | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |  |
| AoA setup |  |  | Config 1,2,3,4,5,6 |  | NA |  |  |  | Setup 1 as specified in clause A.9 |  |  |  |
| Assumption for UE beamsNote 7 |  |  | Config 1,2,3,4,5,6 |  | N/A |  |  |  | Rough |  |  |  |
| NR RF Channel Number |  |  | Config 1,2,3,4,5,6 |  | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1,4 |  | FDD |  |  |  | TDD |  |  |  |
|  |  |  | Config 2,3,5,6 |  | TDD |  |  |  | TDD |  |  |  |
| BWchannel | MHz |  | Config 1,4 |  | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |  | 100: NRB,c = 66 |  |  |  |
| BWP BW | MHz |  | Config 1,4 |  | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |  | 100: NRB,c = 66 |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3,4,5,6 |  | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 |  | SR.1.1 FDD |  |  |  | - |  |  |  |
|  |  |  | Config 2,5 |  | SR.1.1 TDD |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  | SR2.1 TDD |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 |  | CR.1.1 FDD |  |  |  | - |  |  |  |
|  |  |  | Config 2,5 |  | CR.1.1 TDD |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  | CR2.1 TDD |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1,4 |  | CCR.1.1 FDD |  |  |  | CCR.3.1 TDD |  |  |  |
|  |  |  | Config 2,5 |  | CCR.1.1 TDD |  |  |  | CCR.3.1 TDD |  |  |  |
|  |  |  | Config 3,6 |  | CCR.2.1 TDD |  |  |  | CCR.3.1 TDD |  |  |  |
| TDD configuration |  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  | TDDConf.3.1 |  |  |  |
|  |  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  | TDDConf.3.1 |  |  |  |
| Initial DL BWP |  |  | Config 1,2,3,4,5,6 |  | DLBWP.0.1 |  |  |  | NA |  |  |  |
| Initial UL BWP |  |  | Config 1,2,3,4,5,6 |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
| Dedicated DL BWP |  |  | Config 1,2,3,4,5,6 |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
| Dedicated UL BWP |  |  | Config 1,2,3,4,5,6 |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,4 |  | SMTC.2 |  |  |  | SMTC.2 |  |  |  |
|  |  |  | Config 2,3,5,6 |  | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz |  | Config 1,2,4,5 |  | 15 |  |  |  | 120 |  |  |  |
|  |  |  | Config 3,6 |  | 30 |  |  |  | 120 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4,5,6 |  | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15kHz Note5 |  |  |  | NA Link only, see clause A.3.7A |  |  |  | -104.7 |  |  |  |
| Note2 | dBm/SCS Note4 |  | Config 1,2,4,5 |  |  |  |  |  | -95.7 |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  | -95.7 |  |  |  |
| SSB_RP Note 3 | dBm/SCS Note5 |  | Config 1,2,4,5 |  |  |  |  |  | -Infinity |  | -86.7 |  |
|  |  |  | Config 3,6 |  |  |  |  |  | -Infinity |  | -86.7 |  |
|  | dB |  | Config 1,2,3,4,5,6 |  |  |  |  |  | -Infinity |  | 9 |  |
|  | dB |  | Config 1,2,3,4,5,6 |  |  |  |  |  | -Infinity |  | 9 |  |
| IoNote3 | dBm/9.36MHz |  | Config 1,2,4,5 |  |  |  |  |  | - |  | - |  |
|  | dBm/38.16MHz |  | Config 3,6 |  |  |  |  |  | - |  | - |  |
|  | dBm/95.04 MHz Note5 |  | Config 1,2,3,4,5,6 |  |  |  |  |  | -66.7 |  | -57.2 |  |
| Propagation Condition |  |  | Config 1,2,3,4,5,6 |  |  |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap and in test 3 with per-FR gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

10080 for UE supporting power class 1, or

6240 for UE supporting other power class.

In test 2 with per-UE gap and in test 4 with per-FR gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

107520 for UE supporting power class 1, or

66560 for UE supporting other power class.

In test 1, 2, 3 and 4 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 10082 ms for power class 1 UE and 6242 ms for other power classes in test 1 and test 3 and 107522 for power class 1 UE and 66562 ms for other power classes in test 2 and test 4.

///End of change