3GPP TSG-RAN5 Meeting #91-e	R5-212066r1

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5014 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE TC 8.2.4.31.1 Conditional handover Success |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of LTE TC 8.2.4.31.1-Conditional handover/Success in accordance with work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new subclause 8.2.4.31 Add conditional handover / success test case in 8.2.4.31.1 Conditional handover / Success / A3 / A5 / A3+A5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.31.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR1339 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-212066r1 1. Update coversheet 2. Correction of title format 3. Delete referring TS36.300 for the test requirement. 4. Correction of test step to check RRC message for the TP. 5. Correction of message content ReportConfigEUTRA-A3-CHO to refer the new changes in 36.508 |  |  |  |  |  |  |  |  |

<Start of change>

8.2.4.31 RRC connection reconfiguration / Handover / Conditional Handover

8.2.4.31.1 Conditional handover / Success / A3 / A5 / A3+A5

8.2.4.31.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting conditional handover and receiving RRCConnectionReconfiguration message includes the conditionalReconfiguration }

ensure that {

when { any CHO execution condition is not satisfied }
    then { UE maintains connection with source eNB and starts evaluating the CHO execution conditions for the candidate cell(s) }

}

(2)

with { UE in RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE receives RRCConnectionReconfiguration message includes the conditionalReconfiguration and ony event A3 is configured as conditional handover trigger event }
    then { UE starts evaluating the CHO execution condition A3 and perform conditional handover to the neighbor cell triggered in conditional configuration execution }

}

(3)

with {  UE in RRC_CONNECTED state and supporting conditional handover }

ensure that {

when {  UE receives RRCConnectionReconfiguration message includes the conditionalReconfiguration and ony event A5 is configured as conditional handover trigger event }
    then { UE starts evaluating the CHO execution condition A5 and perform conditional handover to the neighbor cell triggered in conditional configuration execution }

}

(4)

with { UE in RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE receives RRCConnectionReconfiguration message includes the conditionalReconfiguration and both event A3 and event A5 are configured as conditional handover trigger events }
    then { UE starts evaluating the CHO execution condition A3 and A5 and perform conditional handover to the neighbor cell triggered in conditional configuration execution }

}

(5)

with { UE in RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE perform conditional handover procedure successfully }
    then { UE releases the stored CHO configurations }

}

8.2.4.31.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.3.5.9.1, 5.3.5.9.4, 5.3.5.9.5 and 5.3.5.3. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.3.5.9.1]

The network configures the UE with conditional reconfiguration (i.e. conditional handover) including per candidate target cell an RRCConnectionReconfiguration to be stored and to be applied upon the fulfilment of an associated execution condition.

The UE shall:

1>	if the received conditionalReconfiguration includes the condReconfigurationToRemoveList:

2>	perform the conditional reconfiguration removal procedure as specified in 5.3.5.9.2;

1>	if the received conditionalReconfiguration includes the condReconfigurationToAddModList:

2> perform the conditional reconfiguration addition/modification procedure as specified in 5.3.5.9.3;

[TS 36.331, clause 5.3.5.9.4]

If AS security has been activated successfully, the UE shall:

1>	if VarConditionalReconfiguration includes at least one condReconfigurationId:

2>	perform conditional reconfiguration evaluation;

1>	for each condReconfigurationId within the VarConditionalReconfiguration:

2>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon within condReconfigurationToApply to be an applicable cell;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the triggerCondition associated to condReconfigurationId:

3>	if the entry condition(s) applicable for this event associated with the condReconfigurationId, i.e. the event corresponding with the condEventId of the corresponding condReconfigurationTriggerEUTRA within VarConditionalReconfiguration, is fulfilled for the applicable cell for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfiguration:

4> consider the entry condition for the associated measId within triggerCondition as fulfilled;

3> if the leaving condition(s) applicable for this event associated with the condReconfigurationId, i.e. the event corresponding with the condEventId(s) of the corresponding condReconfigurationTriggerEUTRA within VarConditionalReconfiguration, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfiguration:

4> consider the event associated to that measId to be not fulfilled;

2>	if trigger conditions for all associated measId(s) within triggerCondition are fulfilled:

3> consider the target cell candidate within the stored condReconfigurationToApply, associated to that condReconfigurationId, as a triggered cell;

3> initiate the conditional reconfiguration execution, as specified in 5.3.5.9.5;

[TS 36.331, clause 5.3.5.9.5]

The UE shall:

1>	if more than one triggered cell exists:

2>	select one of the triggered cells as the selected cell for conditional reconfiguration;

1>	for the selected cell of conditional reconfiguration:

2>	apply the stored condReconfigurationToApply associated to that condReconfigurationId and perform the actions as specified in 5.3.5.4;

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…….

1>	if the RRCConnectionReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.9;

NOTE 6:	In case of conditional reconfiguration the text "if the received RRCConnectionReconfiguration. . ." corresponds to applying the stored RRCConnectionReconfiguration message (according to 5.3.5.9.5).

8.2.4.31.1.3	Test description

8.2.4.31.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell4.

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

8.2.4.31.1.3.2	Test procedure sequence

Table 8.2.4.31.1.3.2-1 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1", "T2" and "T3", are to be applied in the test procedure in Table 8.2.4.31.1.3.2-2.

Table 8.2.4.31.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -82 | -91 | -91 | Power levels are such that UE registered on Cell 1 and entry condition for event A3 is not satisfied |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -79 | -91 | Power levels are such that entry condition for event A3 is satisfied for Cell 2 |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -91 | -91 | -79 | Power levels are such that entry condition for event A5 is satisfied for Cell 4 |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -79 | -91 | -91 | Power levels are such that entry condition for event A3 and event A5 are satisfied for Cell 1 |

Table 8.2.4.31.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration including measConfig and conditionalReconfiguration to set Cell 2 and Cell 4 as target candidate cells and configure event A3 as trigger event in Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message in Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE attempt to perform the handover using MAC Random Access Preamble on Cell 2 within the next 5s? | --> | (PRACH Preamble) | 1 | F |
| 4 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 5 | Check: Does the UE ransmits RRCConnectionReconfigurationComplete message in Cell 2? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 6 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state in Cell 2? | - | - | 2 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration including measConfig and conditionalReconfiguration to set Cell 1 and Cell 4 as target candidate cells and configure event A5 as trigger event in Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message in Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | SS adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 10 | Check: Does the UE ransmits RRCConnectionReconfigurationComplete in Cell 4. | --> | RRCConnectionReconfigurationComplete | 3 | P |
| 11 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state in Cell 4? | - | - | 3 | P |
| 12 | The SS transmits an RRCConnectionReconfiguration including measConfig and conditionalReconfiguration for intra-frequency event A3 and A5 in Cell 4. | <-- | RRCConnectionReconfiguration | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message in Cell 4. | --> | RRCConnectionReconfigurationComplete | - | - |
| 14 | SS adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 15 | Check: Does the UE ransmits RRCConnectionReconfigurationComplete message in Cell 1. | --> | RRCConnectionReconfigurationComplete | 4 | P |
| 16 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state in Cell 1? | - | - | 4 | P |
| 17 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 18 | Check: Does the UE ransmit RRCConnectionReconfigurationComplete message in Cell 2 within 10s? | --> | RRCConnectionReconfigurationComplete | 5 | F |

8.2.4.31.1.3.3	Specific message contents

Table 8.2.4.31.1.3.3-1: RRCConnectionReconfiguration (Step 1, step 7 and step 12, Table 8.2.4.31.1.3.2-3)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig-DEFAULT | Table 8.2.4.31.1.3.3-2 |  |
| mobilityControlInfo |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| conditionalReconfiguration-r16 |  | ConditionalReconfiguration | Table 8.2.4.31.1.3.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.4.31.1.3.3-2: MeasConfig-DEFAULT (Table 8.2.4.31.1.3.3-1)

| Derivation Path: TS 36.508 Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| MeasObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC | Table 8.2.4.31.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigEUTRA | ReportConfigEUTRA-A3-CHO | Table 8.2.4.31.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigEUTRA | ReportConfigEUTRA-A5-CHO(-80,-86) with condition Not RSRQ |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1.. maxMeasId)) OF MeasIdToAddMod { | 1 entry |  | Step 1, Step 7 |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  | Step 1 |
|  | 2 |  | Step 7 |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1.. maxMeasId)) OF MeasIdToAddMod { | 2 entries |  | Step 12 |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-3: MeasObjectEUTRA-GENERIC (Table 8.2.4.31.1.3.3-2)

| Derivation Path: TS 36.508 Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC::= SEQUENCE { |  |  |  |
| carrierFreq | ARFCN-ValueEUTRA for Cell 1 |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-4: ReportConfigEUTRA-A3-CHO (Table 8.2.4.31.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.6-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| condReconfigurationTriggerEUTRA-r16 ::= SEQUENCE { |  |  |  |
| condEventId-r16 CHOICE { |  |  |  |
| condEventA3-r16 SEQUENCE { |  |  |  |
| a3-Offset-r16 | 2 | 1 dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-5: ConditionalReconfiguration (Table 8.2.4.31.1.3.3-1)

| Derivation Path: TS 36.331 clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigurationToAddModList-r16 | CondReconfigurationToAddModList-r16 (Step 1 and step 9) | Table 8.2.4.31.1.3.3-6 | Step 1, Step 7 |
|  | CondReconfigurationToAddModList-r16 (Step 16) | Table 8.2.4.31.1.3.3-9 | Step 12 |
| condReconfigurationToRemoveList-r16 | Not present |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-6: CondReconfigToAddModList-r16 (Step 1 and step 7, Table 8.2.4.31.1.3.3-5)

| Derivation Path: TS 36.331 clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigurationToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxCondConfig-r16)) OF CondReconfigurationAddMod-r16 { | 2 entries |  |  |
| CondReconfigurationAddMod-r16 [1] ::= SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO and HO Cell 2 | Table 8.2.4.31.1.3.3-7 | Step 1 |
|  | RRCConnectionReconfiguration with condition HO and HO Cell 1 | Table 8.2.4.31.1.3.3-7 | Step 7 |
| } |  |  |  |
| CondReconfigurationAddMod-r16 [2] ::= SEQUENCE { |  | entry 2 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO and HO Cell 4 | Table 8.2.4.31.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-7: RRCConnectionReconfiguration (Table 8.2.4.31.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo | Table 8.2.4.31.1.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-8: MobilityControlInfo (Table 8.2.4.31.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  | HO Cell 2 |
|  | PhysicalCellIdentity of Cell 1 |  | HO Cell 1 |
|  | PhysicalCellIdentity of Cell 4 |  | HO Cell 4 |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 2 |  | HO Cell 2 |
|  | Same downlink EARFCN as used for Cell 1 |  | HO Cell 1 |
|  | Same downlink EARFCN as used for Cell 4 |  | HO Cell 4 |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.31.1.3.3-9: CondReconfigurationToAddModList-r16 (Step 16, Table 8.2.4.31.1.3.3-6)

| Derivation Path: TS 36.331 clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigurationToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxCondConfig-r16)) OF CondReconfigurationAddMod-r16{ | 2 entries |  |  |
| CondReconfigurationAddMod-r16 [1] ::= SEQUENCE { |  | entry 1 |  |
| condReconfigurationId-r16 | 1 |  |  |
| triggerCondition-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| MeasId [2] | 2 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO and HO Cell 1 | Table 8.2.4.31.1.3.3-7 |  |
| } |  |  |  |
| CondReconfigurationAddMod-r16 [2] ::= SEQUENCE { |  | entry 2 |  |
| condReconfigurationId-r16 | 2 |  |  |
| triggerCondition-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condReconfigurationToApply-r16 | RRCConnectionReconfiguration with condition HO and HO Cell 2 | Table 8.1.4.4.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

<End of change>