3GPP TSG-RAN5 Meeting #91-e	draft_R5-212121r2

# Electronic Meeting, 17th May - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0282 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for 5GS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Apple Inc, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Miscellaneous corrections to test case applicabilities are needed: - TC 6.4 should list IMS de-registration instead of SM over IP receiver. - TC 7.8 needs to add session initiation capability due to TC change in TS 34.229-5 CR 0126 - A number of test cases miss precondition support in their applicability (note that support of preconditions does not necessarily imply actual usage of preconditions) - A number of test cases have UE’s use AMR-WB and EVS codecs in their SDP offers or responses but do not reflect that in the applicability statements of those test cases - Supporting preconditions and supporting the disabling of preconditons are different capabilities that are not sufficiently differentiated. - TC 7.29 exercises optional behavior of the UE and that should be flagged as such. - TC 7.33 forces the UE to use re-INVITE for session refresh. Some UEs may always want to use UPDATE for that purpose. - TC 8.32 does not list C23 conditions in full |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replaced SM over IP receiver by IMS de-registration capability. - Added session initiation capability to the test case. - Added precondition support where needed. - Added AMR-WB and EVS where needed. - Added the differentiation between support of preconditions and support of disabling preconditions where necessary - Added and used a PICS for session refreshes for own benefit for UE - Added and used PICS for usage of re-INVITE for session refresh. - Added missing conditions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Faulty resp incomplete applicability statements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, A.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: added Apple as co-source. Added changes 5 to 7. R2: added MediaTek as co-source. Added change 8. |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## 4.2	Applicabilities for test cases specified in TS 34.229-5

Table 4.2: Applicability of TS 34.229-5 tests

| Clause | Title |  | Release | Applicability | Comments |
| --- | --- | --- | --- | --- | --- |
| Registration |  |  |  |  |  |
| 6.1 | Initial Registration / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR (see Note 1 below) |
| 6.2 | Initial Registration Failures / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.3 | Re-Registration Scenarios / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.4 | De-Registration Scenarios / 5GS |  | Rel-15 | C13 | UE supports IMS security and NR and IMS de-registration |
| 6.5 | Refresh for ISIM parameters / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.6 | Re-Registration after capability update / 5GS |  | Rel-15 | C15 | UE supports IMS security and NR and Enabling/Disabling SMS over IMS |
| 6.7 | Authentication / MAC Parameter Invalid / Only two consecutive challenges / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.8 | Authentication / Security Server missing / SQN out of range / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.9 | Subscription / 503 Service Unavailable / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| Call Control |  |  |  |  |  |
| 7.1 | MTSI MO Voice Call / 503 Service Unavailable / 5GS | Rel-15 |  | C02 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions |
| 7.2 | MTSI MO Voice Call / 504 Server Time-out / 5GS | Rel-15 |  | C03 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session |
| 7.3 | Void |  |  |  |  |
| 7.4 | MTSI MO Voice call with preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C14 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.5 | MTSI MO Voice call without preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C27 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and disabling preconditions |
| 7.6 | MTSI MT Voice Call with preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C04 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions |
| 7.7 | MTSI MT Voice Call without preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C28 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and disabling preconditions |
| 7.8 | MTSI MT Voice Call without preconditions at originating UE and with preconditions at terminating UE / 5GS | Rel-15 |  | C02 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions |
| 7.9 | MTSI MT Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS | Rel-15 |  | C28 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and disabling preconditions |
| 7.10 | MTSI MT Voice call without preconditions and without SDP offer in MT INVITE / 5GS | Rel-15 |  | C28 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and disabling preconditions |
| 7.11 | MTSI MT Voice call without preconditions at terminating UE and originating UE requiring them / 5GS | Rel-15 |  | C05 | UE supports IMS security and NR and MTSI and MTSI speech and (not preconditions or (preconditions and disabling preconditions)) |
| 7.12 | MTSI MO Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS | Rel-15 |  | C02 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions |
| 7.14 | MTSI MO Video Call with preconditions at both originating and terminating UE / 5GS | Rel-15 |  | C16 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.264 CBP Level 1.2 and initiating a session and preconditions |
| 7.25 | MTSI MT Voice call without SDP offer in INVITE / 5GS | Rel-15 |  | C04 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions |
| 7.26 | Mobile Originating CAT / Forking Model / 5GS | Rel-15 |  | C17 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and early media and preconditions |
| 7.27 | Session Timer / MO Call / UE is able to refresh the session / 5GS | Rel-15 |  | C18 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer |
| 7.28 | Session Timer / MO Call / Remote end is refresher / 5GS | Rel-15 |  | C18 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer |
| 7.29 | Session Timer / MO Call / Remote end does not support Session Timer / 5GS | Rel-15 |  | C29 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer and Session Timer for own benefit |
| 7.30 | Session Timer / MO Call / Remote end supports but does not use Session Timer / 5GS | Rel-15 |  | C18 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer |
| 7.31 | Session Timer / MT Call / Remote end supports but does not send Session-Expires / 5GS | Rel-15 |  | C19 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer |
| 7.32 | Session Timer / MT Call / Remote end sends Session-Expires but does not choose refresher / 5GS | Rel-15 |  | C19 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer |
| 7.33 | Session Timer / MT Call / Remote end chooses UE as refresher / 5GS | Rel-15 |  | C30 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer and re-INVITE for session refresh |
| 7.34 | Session Timer / MT Call / Remote end does not support Session Timer / 5GS | Rel-15 |  | C19 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer |
| Supplementary Services |  |  |  |  |  |
| 8.1 | Originating Identification Presentation / Configuration / 5GS |  | Rel-15 | C06 | UE supports IMS security and NR and MTSI and MTSI Originating Identification Presentation and GBA for XCAP authentication |
| 8.18 | Barring of All Incoming Calls / except for a specific user / 5GS |  | Rel-15 | C07 | UE supports IMS security and NR and MTSI and MTSI Communication Barring and GBA for XCAP authentication |
| 8.26 | MO Call Hold without announcement / 5GS |  | Rel-15 | C20 | UE supports IMS security and NR and MTSI and MTSI speech and MTSI Communication Hold and initiating a session and preconditions |
| 8.28 | MT Call Hold without announcement / 5GS |  | Rel-15 | C21 | UE supports IMS security and NR and MTSI and MTSI speech and MTSI Communication Hold and preconditions |
| 8.30 | Subscription to the MWI event package / 5GS |  | Rel-15 | C22 | UE supports IMS security and NR and MTSI Message Waiting Indication |
| 8.31 | Creating and leaving a conference / 5GS |  | Rel-15 | C23 | UE supports IMS security and NR and MTSI and MTSI speech and MTSI Conference and preconditions |
| 8.32 | Inviting user to conference by sending a REFER request to the conference focus / 5GS |  | Rel-15 | C23 | UE supports IMS security and NR and MTSI and MTSI speech and MTSI Conference and preconditions |
| 8.34 | Three way session creation / 5GS |  | Rel-15 | C24 | UE supports IMS security and NR and MTSI and MTSI Conference and MTSI three way session and preconditions |
| 8.36 | MO Explicit Communication Transfer / Consultative Call Transfer / 5GS |  | Rel-15 | C25 | UE supports IMS security and NR and MTSI and MTSI speech and MTSI Explicit Communication Transfer - consultative transfer and preconditions |
| 8.38 | Communication Waiting and cancelling the call / 5GS |  | Rel-15 | C26 | UE supports IMS security and NR and MTSI and MTSI speech and MTSI Communication Waiting and preconditions |
| SMS |  |  |  |  |  |
| 9.1 | Mobile Originating SMS / 5GS |  | Rel-15 | C08 | UE supports IMS security and NR and SM-over-IP sender and UE configured to use SMS over IP |
| 9.2 | Mobile Terminating SMS / 5GS |  | Rel-15 | C09 | UE supports IMS security and NR and SM-over-IP receiver and UE configured to use SMS over IP |
| 9.3 | Mobile Originating Concatenated SMS / 5GS |  | Rel-15 | C10 | UE supports IMS security and NR and concatenated SM-over-IP sender and UE configured to use SMS over IP |
| 9.4 | Mobile Terminating Concatenated SMS / 5GS |  | Rel-15 | C11 | UE supports IMS security and NR and concatenated SM-over-IP receiver and UE configured to use SMS over IP |
| 9.5 | Mobile Originating SMS / RP-ERROR / 5GS |  | Rel-15 | C08 | UE supports IMS security and NR and SM-over-IP sender and UE configured to use SMS over IP |
| Emergency Calls |  |  |  |  |  |
| 10.1 | Emergency Call with emergency registration / Success / Location information available / 5GS |  | Rel-15 | C12 | UE supports IMS security and NR and IMS emergency services and is capable of obtaining location information |

NOTE 1:	If any one of the other Registration test cases 6.2 until 6.9 is executed, this test case is optional.

| Conditions/Options |  |  |
| --- | --- | --- |
| C01 | IF A.6a/2 AND A.18/5 THEN R ELSE N/A | IMS security and NR |
| C02 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions |
| C03 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session |
| C04 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions |
| C05 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND ( NOT A.4/16 OR (A.4/16 AND A.12/59)) THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and (not preconditions or (preconditions and disabling preconditions)) |
| C06 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.16/1 AND A.6a/3 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI Originating Identification Presentation and GBA for XCAP authentication |
| C07 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.16/7 AND A.6a/3 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI Communication Barring and GBA for XCAP authentication |
| C08 | IF A.6a/2 AND A.18/5 AND A.3A/61 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and SM-over-IP sender and UE configured to use SMS over IP |
| C09 | IF A.6a/2 AND A.18/5 AND A.3A/62 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and SM-over-IP receiver and UE configured to use SMS over IP |
| C10 | IF A.6a/2 AND A.18/5 AND A.3A/64 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and concatenated SM-over-IP sender and UE configured to use SMS over IP |
| C11 | IF A.6a/2 AND A.18/5 AND A.3A/65 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and concatenated SM-over-IP receiver and UE configured to use SMS over IP |
| C12 | IF A.6a/2 AND A.18/5 AND A.12/26 AND A.12/27 THEN R ELSE N/A | IMS security and NR and IMS emergency services and is capable of obtaining location information |
| C13 | IF A.6a/2 AND A.18/5 AND A.12/39 THEN R ELSE N/A | IMS security and NR and IMS de-registration |
| C14 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 AND A.21/1 AND A.22/1 AND A.22/2 AND A.22/3 AND A.22/4 AND A.22/5 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| C15 | IF A.6a/2 AND A.18/5 AND A.12/58 THEN R ELSE N/A | IMS security and NR and Enabling/Disabling SMS over IMS |
| C16 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.15/3 AND A.15/9 AND A.4/2B AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.264 CBP Level 1.2 and initiating a session and preconditions |
| C17 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.12/45 AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and early media and preconditions |
| C18 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 AND A.12/57 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer |
| C19 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/16 AND A.12/57 THEN R ELSE N/A | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer |
| C20 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.16/6 AND A.4/2B AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and MTSI Communication Hold and initiating a session and preconditions |
| C21 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.16/6 AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and MTSI Communication Hold and preconditions |
| C22 | IF A.6a/2 AND A.18/5 AND A.16/8 THEN R ELSE N/A | IMS security and NR and MTSI Message Waiting Indication |
| C23 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.16/9 AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and MTSI Conference and preconditions |
| C24 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.16/9 AND A.16/14 AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI Conference and MTSI three way session and preconditions |
| C25 | IF A.6a/2 AND A.18/5 A.3A/50 AND A.15/1 AND A.16/11 AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and MTSI Explicit Communication Transfer - consultative transfer and preconditions |
| C26 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.16/13 AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and MTSI Communication Waiting and preconditions |
| C27 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 AND A.12/59 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and disabling preconditions |
| C28 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/16 AND A.12/59 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and disabling preconditions |
| C29 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 AND A.12/57 AND A.12/60 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer and Session Timer for own benefit |
| C30 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/16 AND A.12/57 AND A.12/61 THEN R ELSE N/A | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer and re-INVITE for session refresh |

# *** Next Change ***

### A.4.5	Additional information

Table A.12: Additional information

| Item | Additional information | Reference | Status | Release | Mnemonic | Support |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Void |  |  |  |  |  |
| 2 | Void |  |  |  |  |  |
| 3 | Void |  |  |  |  |  |
| 4 | UE capable of being configured to initiate Dedicated PDP Context | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiatesDedicatedPDPContext |  |
| 5 | UE capable of being configured to initiate P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaPCO |  |
| 6 | Void |  |  |  |  |  |
| 7 | UE capable of being configured to initiate P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaDHCPv6 |  |
| 8 | UE supports P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaPCO |  |
| 9 | Void |  |  |  |  |  |
| 10 | UE supports P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaDHCPv6 |  |
| 11 | Void |  |  |  |  |  |
| 12 | UE capable of initiating a bidirectional voice session over IMS | 24.229 [10], 5.1.6 | o | Rel-5 | pc_BidirecVoiceOverIMS |  |
| 13 | Void |  |  |  |  |  |
| 14 | Void |  |  |  |  |  |
| 15 | Void |  |  |  |  |  |
| 16 | UE Supports " IPv6 address with embedded IPv4 address" in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 17 | UE Supports IPv4 address in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 18 | Void |  |  |  |  |  |
| 19 | UE supports UI capable of showing user notification for Message Waiting Indication | 24.173 [55], Annex F | o | Rel-7 |  |  |
| 20 | Void |  |  |  |  |  |
| 21 | Void |  |  |  |  |  |
| 22 | Void |  |  |  |  |  |
| 23 | Void |  |  |  |  |  |
| 24 | UE supports no reply timer setting | 24.604 [68], 4.9.1.4 | o | Rel-8 |  |  |
|  |  | IR.92 [83], 2.3.8 | m |  |  |  |
| 25 | UE supports sending DTMF events over RTP | 26.114 [56], Annex G | o | Rel-7 |  |  |
|  |  | IR.92 [83], 3.3 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 26 | UE supports IMS emergency services | 24.229 [10], 5.1.6 | o | Rel-9 | pc_IMS_EmergencyCall |  |
| 27 | UE is capable of obtain ing location information | 24.229 [10] 4.7, 36.509 [79] 4.1, 5.5.2, 34.109 [78] 5.4.2 | o | Rel-9 | pc_IMS_Geolocation |  |
| 28 | Void |  |  |  |  |  |
| 29 | UE supports emergency speech call over 1xRTT | C.S0005-E [74] | o |  | pc_CS_Em_Call_in_1xRTT |  |
| 30 | Void |  |  |  |  |  |
| 31 | UE supports end-to-access-edge media security using SDES | 24.229 [10], 4.2.B.2 | o | Rel-9 |  |  |
| 32 | UE supports video media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | o | Rel-13 | pc_IMS_Video_FeatureTag |  |
|  |  | IR.94 [75], 2.2 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 33 | UE supports Communication Hold during emergency call | 23.167 [76], 7.1.1 | o | Rel-9 | pc_CommunicationHold_DuringEmergencyCall |  |
| 34 | UE indicates g.3gpp.srvcc-alerting media feature tag in INVITE request or 180 (Ringing) response | 24.237 [77], 12.2.3B.1 | o | Rel-10 | pc_IMS_SRVCCAlert |  |
| 35 | Void |  |  |  |  |  |
| 36 | UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request | 24.237 [77] 6A.2.2.2 | o | Rel-12 | pc_BSRVCC |  |
| 37 | UE indicates OMA-TS-XDM_MO-V1_1-20080627-A.doc, section 5.2.8 "Node: /<X>/ AAUTHNAME" is configured | 24.623 [81], Annex B.2 | o | Rel-8 | pc_XCAP_UsernameIsConfiguredInUE |  |
| 38 | The UE use the default public user identity received in P-Associated -URI header in 200 OK for REGISTER as XCAP User Identity (XUI) | IR.92 [83] | m |  | pc_XCAP_XUIisDefaultPublicUserId |  |
|  |  | 24.623 [81] | o | Rel-8 |  |  |
| 39 | UE has the method that support IMS deregistration | 24.229 [10], 5.1.1.6 | o | Rel-5 | pc_IMS_Deregistration |  |
| 40 | UE supports Cs to PS SRVCC | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCC |  |
| 41 | UE supports Cs to PS SRVCC in alerting state | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCAlert |  |
| 42 | UE supports Cs to PS SRVCC and the MSC server assisted mid-call feature | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCMidCall |  |
| 43 | Void |  |  |  |  |  |
| 44 | Void |  |  |  |  |  |
| 45 | UE supports early media | 24.229 [10], 5.1.3 | o | Rel-8 | pc_EarlyMedia |  |
| 46 | UE indicates g.3gpp.accesstype media feature tag in REGISTER | 24.237 [77], 6.2.2 | c1 | Rel-11 |  |  |
| 47 | Void |  |  |  |  |  |
| 48 | Void |  |  |  |  |  |
| 49 | The UE uses XCAP without PDN connection in WLAN | IR.51 [84], 4.6 | o | Rel-11 | pc_WLAN_XCAP_without_PDN | If PICS is true, Wi-Fi access without PDN connection applies. Otherwise, EPC-integrated WLAN applies |
| 50 | The UE uses XCAP in fixed broadband access |  | o | Rel-12 | pc_fixed_broadband_XCAP | In the context of the present specification, a UE supporting fixed broadband, A.18/3, shall set this PICS to true. |
| 51 | The UE uses same public user Identity for From/To as in initial REGISTER | 24.229 [10] | o | Rel-8 | pc_Same_IMPU |  |
|  |  | IR.92 [83], 2.2.1 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 52 | UE reattempts IMS emergency service without registration after rejection of emergency registration. (For Rel-14, this is applicable only when the rejection indicates an anonymous emergency call support) | 23.167 [76], 4.1 | o | Rel-9 | pc_IMS_after_EmReg_rejection |  |
|  |  |  | m | Rel-14 |  |  |
| 53 | Void |  |  |  |  |  |
| 54 | UE supports IMS eCall type of emergency services | 24.229 [10], 5.1.6.11 | o | Rel-14 | pc_IMS_eCall_Capable |  |
| 55 | UE supports IMS eCall Only type of emergency services | 24.229 [10], 5.1.6.11 | o | Rel-14 | pc_IMS_eCall_Only |  |
| 56 | UE supports audio media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | m | Rel-13 | pc_IMS_Audio_FeatureTag |  |
|  |  | IR.92 [83], 2.2.1, 2.2.4 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 57 | UE supports Session Timer | 24.229 [10], 5.1.2A, 5.2.7.2, 5.2.7.3, 5.4.5.3 | o | Rel-6 | pc_IMS_Session_Timer |  |
|  |  | IR.92 [83], 2.2.8 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 58 | UE supports enabling/disabling SMS over IMS | 27.007 [97], 8.72 | o | Rel-15 | pc_Enable_Disable_SMSoverIMS |  |
| 59 | UE supports disabling preconditions | 24.229 [10], 5.1.5A | o | Rel-15 | pc_IMS_Disable_Preconditions |  |
| 60 |  |  | UE uses Session Timer for its benefit alone |  |  | 24.229 [10] |  |  | o |  |  | Rel-15 |  |  | pc_IMS_Session_Timer_own_benefit |  |  |  |  |  |
|  |  | NG.114 [96] | o |  |  |  |
|  |  |  |  |  |  | RFC 4028 [86] cl 7.2 |  |  | o |  |  |  |  |  |  |  |  |  |  |  |
| 61 |  |  | UE supports usage of re-INVITE for session refresh |  |  | 24.229 [10] |  |  | o |  |  | Rel-15 |  |  | pc_IMS_Session_Timer_reINVITE |  |  |  |  |  |
|  |  |  |  |  |  | NG.114 [96] |  |  | o |  |  |  |  |  |  |  |  |  |  |  |
|  |  | RFC 4028 [86] cl 7.2 | o |  |  |  |
| Conditions/Options |  |  |  |  |  |  |
| c1: IF [73] A.4.4-1/32 THEN m ELSE o - - SC UE indicates accesstype. |  |  |  |  |  |  |