3GPP TSG-RAN5 Meeting #91-e	R5-212253r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0307 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to minimum test time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - With the current description, it is not clear how the difference of SCS is understood in this table. - The minimum test time for 8.3.2.2.1 is missing in Annex G.3.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Redefine MNAS and MNS with slot unit rather than subframe unit. MNAS is multiplied by (SCS/15kHz) for the case SCS is not 15kHz so that the total test time[sec] becomes same as SCS=15kHz case(hence UE experiences same amount of fading variations). - Higher doppler frequency (35Hz compared to 5Hz in FR1) can lead to less minimum test time(in sec) due to fast variation of fading periodicity(roughly 1/7 of 5Hz case) . We propose to use the exisiting MNAS value from 5Hz case(with correct conversion depending on SCS) with conservative assumption. Further enhancement to reduce test time may be possible in case some analysis is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case cannot be executed with proper test time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 : - Redefine MNAS and MNS with slot unit rather than subframe unit. |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

G.3.4	Number of samples for throughput ratios

Table G.3.4-1: Test time for testing throughput ratios

| Demodulation scenario (doppler speed) | Minimum number of active slots (MNAS) | Scheduling pattern | MNAS to MNS Scaling factor (Note 2) | Minimum Number of Slots (MNS) after rounding up to nearest 2μ thousand MNS=1000*2μ*MNS2μ⋅1000 |
| --- | --- | --- | --- | --- |
| 5Hz | 100000 | FDD(15kHz SCS) | 1.0526 | 106000 |
| 5Hz | 200000 | FDD(30kHz SCS) | 1.0526 | 212000 |
| 5Hz | 200000 | TDD FR1.30-1 | 1.2903 | 260000 |
| 35Hz | 800000 | TDD FR2.120-1 | 1.2598 | 1008000 |
| 35Hz | 800000 | TDD FR2.120-2 | 1.3445 | 1080000 |
| Note 1: MNAS determined by theoretical estimations inherited from LTE based on R5-106393. Note 2: MNS/MNAS ratio decided by scheduling pattern (how much time is required to collect required number of active DL slots) Note 3: MNS apply for both denominator and numerator measurement Note 4: μ as defined in TS 38.211 Table 4.2-1 |  |  |  |  |

<<End of change>>