3GPP TSG-RAN WG5 Meeting #91-e	R5-212287r2

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0127 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default message content MCPTT-Info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to TS 24.379 the alert-ind is to be set to “false” while establishing an emergency call (CO and CT) - Emergency alert is a special case and this condition shall be handled in specific message contents, was removed in general conditions - Conditions for mcptt-called-party-id in UL will be handled in specific message content - not present per default - Condition BROADCAST-GROUPCALL will be handled via specific message content in test cases, was removed in general conditions - UE capability if emergency call always includes alert - Table 5.5.3.2.2-1: Encryption is missing for sensitive application data |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction of IE alert-ind default values - Condition EMERGENCY-ALERT removed - Conditions for mcptt-called-party-id removed In Table 5.5.3.2.1-1 mcptt-called-party-id (if present) is still needed for INVITE response (according to 24.379 Table H.2-1, Table H.3-1) - Condition BROADCAST-GROUPCALL removed - pc_MCX_EmergencyIndWithAlertInd added as UE capability for alert-ind - Table 5.5.3.2.2-1: Encryption added for sensitive application data; new table Table 5.5.3.2.2-1A for Encrypted MCPTT info parameters sent by the SS - Change of section title for 5.5.3.2.2 to “MCS Info Lists from the SS” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading message contents may result in incorrect MCPTT test case implementation, test cases cannot be validated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0144 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Extended change 3. Revision 2: Section title change of 5.5.3.2.2 |  |  |  |  |  |  |  |  |

#### 5.5.3.2	MCS Info Lists

##### 5.5.3.2.1	MCS Info Lists from the UE

##### -	MCPTT

Table 5.5.3.2.1-1: MCPTT-Info from the UE

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-access-token> with mcpttString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCPTT client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG, GROUPCONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | “chat” |  |  | CHAT-GROUP-CALL AND INVITE_REFER |
| mcptt-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_A_ID | The URI of the group |  | (GROUP-CALL OR CHAT-GROUP-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_User_B_ID | The URI of the invited MCPTT Client |  | PRIVATE-CALL AND INVITE_REFER |
|  | encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_A |  |  | POC-SETTINGS-EVENT |
| mcptt-calling-user-id | not present or encrypted (NOTE 2) <mcptt-calling-user-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  |  |
|  | not present |  |  | CONFIG, GROUPCONFIG, POC-SETTINGS-EVENT |
| mcptt-called-party-id | not present |  |  |  |
|  | not present or encrypted (NOTE 2) <mcptt-called-party-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  | INVITE-RSP |
| mcptt-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "true" |  |  | EMERGENCY-CALL AND INVITE_REFER |
| alert-ind | not present or encrypted (NOTE 2) <alert-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <alert-ind> with mcpttBoolean set to pc_MCX_EmergencyIndWithAlertInd |  |  | EMERGENCY-CALL AND INVITE_REFER |
| imminentperil-ind | not present or encrypted (NOTE 2) <imminentperil-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "true" |  |  | IMMPERIL-CALL AND INVITE_REFER |
| broadcast-ind | not present or “false” |  |  |  |
| mc-org" | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | px_MCPTT_Group_A_ID if mcptt-request-uri contains a temporary group identity; otherwise, not present | if the <mcptt-request-uri> element contains a group identity then this element can include an MCPTT group ID associated with the group identity in the <mcptt-request-uri> element. E.g. if the <mcptt-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCPTT group ID | TS 24.379 [9] clause F.1.3 | GROUP-CALL |
|  | not present |  |  | PRIVATE-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcptt-client-id | not present |  |  |  |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | The UUID URN of the MCPTT Client | RFC 4122 [106] TS 24.379 [9] clause 4.10 | ( GROUP-CALL OR CHAT-GROUP-CALL OR EMERGENCY-CALL OR IMMPERIL-CALL ) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) if present | in general mcptt-client-id is not mandatory (e.g. for SIP SUBSCRIBE) | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG, GROUPCONFIG |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in the SIP REGISTER or SIP PUBLISH for service authorisation according to TS 24.379 [9] clauses 7.2.1 and 7.2.2 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG AND REGISTER_PUBLISH |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in SIP PUBLISH for MCPTT service settings only, according to TS 24.379 [9] clause 7.2.3 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | POC-SETTINGS-EVENT |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present or any allowed value |  | TS 24.379 [9], clause F.1.3 |  |
| NOTE 1: The SS shall check the mcptt-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-1A |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| REGISTER_PUBLISH | MCPTT-Info in SIP REGISTER or SIP PUBLISH request for service authorisation |
| INVITE_REFER | MCPTT-Info in SIP INVITE or SIP REFER request for call establishment |
| INVITE-RSP | MCPTT-Info in SIP response to a SIP INVITE NOTE: INVITE-RSP is inherited from the SIP response, i.e. it shall be considered as true whenever set for the SIP response |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-1A: Encrypted MCPTT info parameter sent by the UE

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcptt parameter |  |  |  |

##### -	MCVideo

Table 5.5.3.2.1-2: MCVideo-Info from the UE

| Derivation Path: TS 24.281 [86] Clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
|  | "eyJhbGciOiJSUzI1NiJ9.eyJtY3B0dF9pZCI6ImFsaWNlQG9yZy5jb20iLCJleHAiOjE0NTM1MDYxMjEsInNjb3BlIjpbIm9wZW5pZCIsIjNncHA6bWNwdHQ6cHR0X3NlcnZlciJdLCJjbGllbnRfaWQiOiJtY3B0dF9jbGllbnQifQ.XYIqai4YKSZCKRNMLipGC_5nV4BE79IJpvjexWjIqqcqiEx6AmHHIRo0mhcxeCESrXei9krom9e8Goxr_hgF3szvgbwl8JRbFuv97XgepDLjEq4jL3Cbu41Q9b0WdXAdFmeEbiB8wo_xggiGwv6IDR1b3TgAAsdjkRxSK4ctIKPaOJSRmM7MKMcKhIug3BEkSC9-aXBTSIv5fAGN-ShDbPvHycBpjzKWXBvMIR5PaCg-9fwjELXZXdRwz8C6JbRM8aqzhdt4CVhQ3-Arip-S9CKd0tu-qhHfF2rvJDRlg8ZBiihdPH8mJs-qpTFep_1-kON3mL0_g54xVmlMwN0XQA" | The access token is opaque to the MCVideo client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | “chat” |  |  | CHAT-GROUP-CALL AND INVITE_REFER |
| mcvideo-request-uri | px_MCVideo_Group_A_ID | The URI of the group |  | GROUP-CALL |
|  | not present or px_MCVideo_User_B_ID | The URI of the invited MCVideo Client |  | PRIVATE-CALL |
| mcvideo-calling-user-id | not present or px_MCVideo_ID_User_A |  |  |  |
| mcvideo-called-party-id | not present |  |  |  |
| mcvideo-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | EMERGENCY-CALL |
| alert-ind | not present or if present then="false" |  |  |  |
|  | "false" |  |  | EMERGENCY-CALL |
| imminentperil-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | IMMPERIL-CALL |
| broadcast-ind | not present or “false” |  |  |  |
| mc-org" | not present |  |  |  |
| transmission-state | not present |  |  |  |
| associated-group-id | px_MCVideo_Group_A_ID if mcvideo-request-uri contains a temporary group identity; otherwise, not present | if the <mcvideo-request-uri> element contains a group identity then this element can include an MCVideo group ID associated with the group identity in the <mcvideo-request-uri> element. E.g. if the <mcvideo-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCVideo group ID | TS 24.281 [86] clause F.1.3 | GROUP-CALL |
|  | not present |  |  | PRIVATE-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcvideo-client-id | not present |  |  |  |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | The UUID URN of the MCVIDEO Client | RFC 4122 [106] TS 24.281 [86] clause 4.9 | (GROUP-CALL OR CHAT-GROUP-CALLOR EMERGENCY-CALL OR IMMPERIL-CALL ) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) if present | in general mcvideo-client-id is not mandatory (e.g. for SIP SUBSCRIBE) | RFC 4122 [106] TS 24.281 [86] clause 4.9 | CONFIG, GROUPCONFIG |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | mcvideo-client-id is mandatory in the SIP REGISTER or SIP PUBLISH for service authorisation according to TS 24.379 [9] clauses 7.2.1 and 7.2.2 | RFC 4122 [106] TS 24.281 [86] clause 4.9 | CONFIG AND REGISTER_PUBLISH |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | mcvideo-client-id is mandatory in SIP PUBLISH for MCPTT service settings only, according to TS 24.379 [9] clause 7.2.3 | RFC 4122 [106] TS 24.281 [86] clause 4.9 | POC-SETTINGS-EVENT |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present or any allowed value |  | TS 24.281 [86] clause F.1.3 |  |

##### -	MCData

Table 5.5.3.2.1-3: MCData-Info from the UE

| Derivation Path: TS 24.282 [87], Clause D.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-access-token | not present |  |  |  |
| request-type | "one-to-one-sds" |  |  | MCD_1to1 |
| request-type | “group-sds” |  |  | MCD_grp |
| mcdata-request-uri | px_MCData_Group_A |  |  | MCD_grp |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-called-party-id | not present |  |  |  |
| mcdata-calling-group-id | not present |  |  |  |
| alert-ind | not present |  |  |  |
| originated-by | not present |  |  |  |
| mcdata-client-id | px_MCData_Client_A_ID |  |  | MCD_grp |
| mcdata-controller-psi | not present |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A goup MCData call |
| For further conditions see table 5.5.1-1 |  |

##### 5.5.3.2.2	MCS Info Lists from the SS

##### -	MCPTT

Table 5.5.3.2.2-1: MCPTT-Info from the SS

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-access-token | not present |  |  |  |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL |
|  | "private" |  |  | PRIVATE-CALL |
|  | “chat” |  |  | CHAT-GROUP-CALL |
| mcptt-request-uri | Encrypted (NOTE 1) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_A | The URI of the called user |  |  |
| mcptt-calling-user-id | Encrypted (NOTE 1) <mcptt-calling-user-id> with mcpttURI set to px_MCPTT_ID_User_B | The URI of the calling user |  |  |
| mcptt-called-party-id | not present |  |  |  |
| mcptt-calling-group-id | not present |  |  |  |
|  | Encrypted (NOTE 1) <mcptt-calling-group-id> with mcpttURI set to px_MCPTT_Group_A_ID | The URI of the group |  | GROUP-CALLOR CHAT-GROUP-CALL |
| required | not present |  |  |  |
| emergency-ind | not present |  |  |  |
|  | Encrypted (NOTE 1) <emergency-ind> with mcpttBoolean set to "true" |  |  | EMERGENCY-CALL |
| alert-ind | not present |  |  |  |
|  | Encrypted (NOTE 1) <alert-ind> with mcpttBoolean set to "false" |  |  | EMERGENCY-CALL |
| imminentperil-ind | not present |  |  |  |
|  | Encrypted (NOTE 1) <imminentperil-ind> with mcpttBoolean set to "true" |  |  | IMMPERIL-CALL |
| broadcast-ind | not present |  |  |  |
| mc-org" | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | not present |  |  |  |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcptt-client-id | not present |  |  |  |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present |  | TS 24.379 [9], clause F.1.3 |  |
| NOTE 1: Encrypted element as described in Table 5.5.3.2.2-1A |  |  |  |  |

Table 5.5.3.2.2-1A: Encrypted MCPTT info parameter sent by the SS

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-2 containing encrypted element content of the mcptt parameter |  |  |  |

##### -	MCVideo

Table 5.5.3.2.2-2: MCVideo-Info from the SS

| Derivation Path: TS 24.281 [86] Clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
|  | "eyJhbGciOiJSUzI1NiJ9.eyJtY3B0dF9pZCI6ImFsaWNlQG9yZy5jb20iLCJleHAiOjE0NTM1MDYxMjEsInNjb3BlIjpbIm9wZW5pZCIsIjNncHA6bWNwdHQ6cHR0X3NlcnZlciJdLCJjbGllbnRfaWQiOiJtY3B0dF9jbGllbnQifQ.XYIqai4YKSZCKRNMLipGC_5nV4BE79IJpvjexWjIqqcqiEx6AmHHIRo0mhcxeCESrXei9krom9e8Goxr_hgF3szvgbwl8JRbFuv97XgepDLjEq4jL3Cbu41Q9b0WdXAdFmeEbiB8wo_xggiGwv6IDR1b3TgAAsdjkRxSK4ctIKPaOJSRmM7MKMcKhIug3BEkSC9-aXBTSIv5fAGN-ShDbPvHycBpjzKWXBvMIR5PaCg-9fwjELXZXdRwz8C6JbRM8aqzhdt4CVhQ3-Arip-S9CKd0tu-qhHfF2rvJDRlg8ZBiihdPH8mJs-qpTFep_1-kON3mL0_g54xVmlMwN0XQA" | The access token is opaque to the MCVideo client | TS 33.180 [94] clause B.4 RFC 6749 [77] | CONFIG |
| session-type | "prearranged" |  |  | GROUP-CALL |
|  | "private" |  |  | PRIVATE-CALL |
| mcvideo-request-uri | px_MCVideo_Group_A_ID | The URI of the group |  | GROUP-CALL |
|  | px_MCVideo_Client_B_ID | The URI of the invited MCVideo Client |  | PRIVATE-CALL |
| mcvideo-calling-user-id | not present or px_MCVideo_ID_User_A |  |  |  |
| mcvideo-called-party-id | not present or px_MCVideo_ID_User_B |  |  |  |
| mcvideo-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | EMERGENCY-CALL |
| alert-ind | not present or if present then="false" |  |  |  |
|  | "false" |  |  | EMERGENCY-CALL |
| imminentperil-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | IMMPERIL-CALL |
| broadcast-ind | not present |  |  |  |
| mc-org" | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | px_MCVideo_Group_A_ID if mcvideo-request-uri contains a temporary group identity; otherwise, not present | if the <mcvideo-request-uri> element contains a group identity then this element can include an MCVideo group ID associated with the group identity in the <mcvideo-request-uri> element. E.g. if the <mcvideo-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCVideo group ID | TS 24.281 [86] clause F.1.3 | GROUP-CALL |
|  | not present |  |  | PRIVATE-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcvideo-client-id | px_MCVideo_Client_A_ID | The URI of the MCVideo Client |  | PRIVATE-CALL GROUP-CALL EMERGENCY-CALL IMMPERIL-CALL |
|  | "eyJhbGciOiJSUzI1NiJ9.eyJzdWIiOiIxMjM0NTY3ODkwIiwiYXVkIjoibWNwdHRfY2xpZW50IiwiaXNzIjoiSWRNUy5zZXJ2ZXIuY29tOjkwMzEiLCJpYXQiOjE0NTM0OTgxNTgsImV4cCI6MTQ1MzQ5ODQ1OCwibWNwdHRfaWQiOiJhbGljZUBvcmcuY29tIn0.Dpn7AhIMaqMEgg12NYUUfJGSFJMPG8M2li9FLtPotDlHvwU2emBws8z5JLw81SXQnoLqZ8ZF8tIhZ1W7uuMbufF4Wsr7PAadZixz3CnV2wxFV9qR_VA1-0ccDTPukUsRHsic0SgZ3aIbcYKd6VsehFe_GDwfqysYzD7yPwCfPZo" | The MCVideo client may validate the user with the ID token and configure itself for the user | TS 33.180 [94] clause B.4 RFC 6749 [77] | CONFIG |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present or any allowed value |  | TS 24.281 [86] clause F.1.3 |  |

##### -	MCData

Table 5.5.3.2.2-3: MCData-Info from the SS

| Derivation Path: TS 24.282 [87], Clause D.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-access-token | not present |  |  |  |
| request-type | "one-to-one-sds" |  |  | MCD_1to1 |
| request-type | “group-sds” |  |  | MCD_grp |
| mcdata-request-uri | px_MCData_Group_A |  |  | MCD_grp |
| mcdata-calling-user-id | px_MCData_ID_User_B |  |  |  |
| mcdata-called-party-id | px_MCData_ID_User_A |  |  |  |
| mcdata-calling-group-id | not present |  |  |  |
| alert-ind | not present |  |  |  |
| originated-by | not present |  |  |  |
| mcdata-client-id | px_MCData_Client_B_ID |  |  |  |
| mcdata-controller-psi | not present |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A goup MCData call |
| For further conditions see table 5.5.1-1 |  |