3GPP TSG-RAN5 Meeting #91-e	R5-212624r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0511 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of power control in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Correction to power control test cases is proposed by Anritsu in MU discussion during RAN5#91e. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For power control test cases (absolute power tolerance, relative power tolerance, aggregate power tolerance) - Correct editor’s note, test condition, test procedure, message contents, test points, test requirements, test tolerance in 6.3.4 - Define MTSU and TT for absolute power tolerance in Table 6.3.4.2.5-4, Table 6.3.4.2.5-5, Table F.1.2-1, and Table F.3.2-1 - Correct RB allocation from Outer_Full to Inner_Full in Table 6.3.4.2.4.1-1 and Table 6.3.4.4.4.1-2 - Correct SCS from 60kHz to Highest in Table 6.3.4.3.4.1-1 - Delete Lowest SCS from Table 6.3.4.4.4.1-1 and Table 6.3.4.4.4.1-2 - Correct the typo and wrong reference |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Power control test cases cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | To be updated as per the outcome of the MU discussion during RAN5#91e: R5-212621, R5-212622, R5-212623 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Undo the changes according to the MU discussion result - Delete Lowest SCS from test configurations according to R5-212623r1 |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

6.3.4	Power control

6.3.4.1	General

The requirements on power control accuracy apply under normal conditions and are defined as a directional requirement. The requirements are verified in beam locked mode on beam peak direction.

6.3.4.2	Absolute power tolerance

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerances are FFS.

- Testing of extreme conditions for FR2 is FFS.

- UE transmitted power for PC 1, 2 and 4 are FFS

6.3.4.2.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

To verify the ability of the UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e. transmission gap is larger than 20 ms.

6.3.4.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.3.4.2.3	Minimum conformance requirements

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error RSRP estimate.

The minimum requirements specified in Table 6.3.4.2.3-1 apply in the power range bounded by the minimum output power as specified in sub-clause 6.3.1 (Pmin) and the maximum output power as specified in sub-clause 6.2.1.1 as minimum peak EIRP (‘Pmax’). The intermediate power point 'Pint' is defined in table 6.3.4.2.3-2.

Table 6.3.4.2.3-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

Table 6.3.4.2.3-2: Intermediate power point

| Power Parameter | Value |
| --- | --- |
| Pint | Pmax – 12.0 dB |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.3.4.2.

6.3.4.2.4	Test description

6.3.4.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3.4.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.4.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1. |  | Highest |  |
| Test Parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | N/A for absolute power tolerance test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3.4.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.4.2.4.3.

6.3.4.2.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 0_1 with TPC command 0dB for C_RNTI to schedule the UL RMC according to Table 6.3.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Configure the UE transmitted output power to test point 1 in section 6.3.4.2.4.3. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP of the first subframe in the Tx beam peak direction in the measurement bandwidth specified in Table 6.3.1.5-1 and Table 6.3.1.5-2 for the specific channel bandwidth under test. EIRP test procedure is defined in Annex K. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi. For TDD slots with transient periods are not under test.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

7.	Repeat test steps 1~6 for measurement of test point 2~3. The timing of the execution between the two test points shall be larger than 20ms.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.3.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with following exceptions:

Table 6.3.4.2.4.3-1: PUSCH-ConfigCommon (Test point 1) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -116 |  | 50MHz |
|  | -120 |  | 100MHz |
|  | -122 |  | 200MHz |
|  | -126 |  | 400MHz |
| } |  |  |  |

Table 6.3.4.2.4.3-2: PUSCH-ConfigCommon (Test point 2) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -112 |  | 50MHz |
|  | -116 |  | 100MHz |
|  | -118 |  | 200MHz |
|  | -122 |  | 400MHz |
| } |  |  |  |

Table 6.3.4.2.4.3-3: PUSCH-PowerControl (Test point 3) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -102 |  | 50MHz |
|  | -106 |  | 100MHz |
|  | -108 |  | 200MHz |
|  | -112 |  | 400MHz |
| } |  |  |  |
| } |  |  |  |

Table 6.3.4.2.4.3-4: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 4 |  | SCS_120kHz |
|  | 7 |  | SCS_240kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_120kHz | SCS=120kHz for SS/PBCH block |
| SCS_240kHz | SCS=240kHz for SS/PBCH block |

Table 6.3.4.2.4.3-5: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| tpc-Accumulation | disabled |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.3.4.2.5	Test requirement

The measured EIRP in step 5 and 7 shall not to exceed the values specified in Table 6.3.4.2.5-1 to 6.3.4.2.5-3.

Table 6.3.4.2.5-1: Absolute power tolerance: test point 1 for power class 3

|  | SCS | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | 60kHz | 8.1 | 7.1 | 8.1 | N/A |
| power | 120kHz | 8.1 | 7.1 | 8.1 | 7.1 |
| Power tolerance |  | ± (14+TT)dB |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3, and the higher power limit shall not exceed the Max EIRP defined in sub-clause 6.2.1.3. |  |  |  |  |  |

Table 6.3.4.2.5-2: Absolute power tolerance: test point 2 for power class 3

|  | SCS | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | 60kHz | 12.1 | 11.1 | 12.1 | N/A |
| power | 120kHz | 12.1 | 11.1 | 12.1 | 11.1 |
| Power tolerance |  | ± (12+TT)dB |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3, and the higher power limit shall not exceed the Max EIRP defined in sub-clause 6.2.1.3. |  |  |  |  |  |

Table 6.3.4.2.5-3: Absolute power tolerance: test point 3 for power class 3

|  | SCS | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | 60kHz | 22.1 | 21.1 | 22.1 | N/A |
| power | 120kHz | 22.1 | 21.1 | 22.1 | 21.1 |
| Power tolerance |  | ± (12+TT)dB |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3, and the higher power limit shall not exceed the Max EIRP defined in sub-clause 6.2.1.3. |  |  |  |  |  |

Table 6.3.4.2.5-4: Test Tolerance for power class 1, 2, 4

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | Same as Table 6.3.1.5-3 | Same as Table 6.3.1.5-3 |

Table 6.3.4.2.5-5: Test Tolerance for power class 3

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | Same as Table 6.3.1.5-4 | Same as Table 6.3.1.5-4 |

6.3.4.3	Relative power tolerance

Editor’s note: This clause is incomplete. The following items are either missing or not yet determined:

- MU and TT not defined

- Starting power at ramp up/ramp down/alternating sub-test is TBD (6.3.4.3 MU dependent)

- Testability of test points needs further analysis, based on MU outcome

6.3.4.3.1	Test purpose

To verify the ability of the UE transmitter to set its output power in a target sub-frame relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is less than or equal to 20 ms.

6.3.4.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.3.4.3.3	Minimum conformance requirements

The minimum requirements specified in Table 6.3.4.3.3-1 apply when the power of the target and reference sub-frames are within the power range bounded by the minimum output power as defined in sub-clause 6.3.1 and Pint as defined in sub-clause 6.3.4.3-2. The minimum requirements specified in Table 6.3.4.3-2 apply when the power of the target and reference sub-frames are within the power range bounded by Pint as defined in sub-clause 6.3.4.2 and the measured PUMAX as defined in sub-clause 6.2.4.

For a test pattern that is either a monotonically increasing or monotonically decreasing power sweep over the range specified for Tables 6.3.4.3.3-1and 6.3.4.3.3-2, 3 exceptions are allowed for each of the test patterns. For these exceptions, the power tolerance limit is a maximum of ±11.0 dB.

Table 6.3.4.3.3-1: Relative power tolerance, Pint ≥ P ≥ Pmin

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | ±5.0 |
| 2 ≤ ΔP < 3 | ±6.0 |
| 3 ≤ ΔP < 4 | ±7.0 |
| 4 ≤ ΔP < 10 | ±8.0 |
| 10 ≤ ΔP < 15 | ±10.0 |
| 15 ≤ ΔP | ±11.0 |
| NOTE: The requirements apply with ue-BeamLockFunction enabled. |  |

Table 6.3.4.3.3-2: Relative power tolerance, PUMAX ≥ P > Pint

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | ±3.0 |
| 2 ≤ ΔP < 3 | ±4.0 |
| 3 ≤ ΔP < 4 | ±5.0 |
| 4 ≤ ΔP < 10 | ±6.0 |
| 10 ≤ ΔP < 15 | ±8.0 |
| 15 ≤ ΔP | ±9.0 |
| NOTE 1: The requirements apply with ue-BeamLockFunction enabled. NOTE 2: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, guard periods: for a power step ΔP = 1 dB, the relative power tolerance for transmission is ± 1.0 dB. |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.3.4.3.

6.3.4.3.4	Test description

6.3.4.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in table 6.3.4.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.4.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Low Range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | 100MHz |  |
| Test SCS as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Highest |  |
| Test Parameters |  |  |  |  |
| Ch BW | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB Allocation | Modulation | RB allocation (NOTE 1) |
| 100MHz | N/A for Relative power tolerance test case |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-1 See Table 6.3.4.3.5-2 See Table 6.3.4.3.5-3 |
| Note 1: The starting resource block shall be RB# 44. |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.2 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.3.4.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.4.3.4.3

6.3.4.3.4.2	Test procedure

The procedure is separated in various subtests to verify different aspects of relative power control. The power patterns of the subtests are described in figure 6.3.4.3.4.2-1 thru figure 6.3.4.3.4.2-3. The power patterns and corresponding sub frame numberings are derived from Table A.2.3-1.

Figure 6.3.4.3.4.2-1: TDD ramping up test power patterns, SCS 60kHz

Figure 6.3.4.3.4.2-2: TDD ramping down test power patterns, SCS 60kHz

Figure 6.3.4.3.4.2-3: Alternating Test Power patterns, SCS 60kHz

1.	Sub test: ramping up pattern

1.1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.4.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.2	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

1.3	Send the appropriate TPC commands in the uplink scheduling information to UE until the UE EIRP measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Pmin, where:

-	Pmin is the minimum output power according to Table 6.3.1.3-1.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + [5dB] (UE power step tolerance)  + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-2 [3], clause 6.3.4.3 and is [5dB] for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

1.4	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

1.5	Schedule the UE's PUSCH data transmission as described in Figure 6.3.4.3.4.2-1 (TDD) pattern A: Uplink RB allocation as defined in table 6.3.4.3.5-1. On the PDCCH format 0_1 for the scheduling of the PUSCH the SS will transmit +1dB TPC commands over a sequence of 75 (note1) active uplink sub-frames to ensure that the UE reaches maximum power threshold. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

1.6	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, to verify the UE relative power control meet test requirements in 6.3.4.3.5. EIRP test procedure is defined in Annex K.1.3. EIRP is calculated considering both polarizations, theta and phi. Measurement of the power is not required in sub-frame after the mean power has exceeded the maximum power threshold. For power transients between sub-frames, transient periods of 40us between sub-frames are excluded. For ON/OFF or OFF/ON transients, transient periods of 20 us at the beginning of the sub-frames are excluded.

1.7	Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3.4.3.5-1 to force different UE power steps at various points in the power range.

1.8	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

2.	Sub test: ramping down pattern

2.1 SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.4.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.2 Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

2.3 Send the appropriate TPC commands in the uplink scheduling information to UE until the UE EIRP measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level PUMAX, where:

-	PUMAX is the maximum output power according to Table 6.2.1.1.3.3-2.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + [1dB] (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-2 [3], Table 6.3.4.3-2 and is [1dB] for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

2.4	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

2.5. Schedule the UE's PUSCH data transmission as described in Figure 6.3.4.3.4.2-2 (TDD) pattern A: Uplink RB allocation as defined in table 6.3.4.3.5-1. On the PDCCH format 0_1 for the scheduling of the PUSCH the SS will transmit -1dB TPC commands over a sequence of 75 (note1) active uplink sub-frames to ensure that the UE reaches minimum power threshold. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

2.6. Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, to verify the UE relative power control meet test requirements in 6.3.4.3.5. EIRP test procedure is defined in Annex K.1.3. EIRP is calculated considering both polarizations, theta and phi. Measurement of the power is not required in sub-frame after the mean power has exceeded the maximum power threshold. For power transients between sub-frame, transient periods of 40us between sub-frame are excluded. For ON/OFF or OFF/ON transients, transient periods of 20 us at the beginning of the sub-frame are excluded.

2.7. Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3.4.3.5-2/6.3.4.3.5-4/ 6.3.4.3.5-6 to force different UE power steps at various points in the power range.

2.8 SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

3.	Sub test: alternating pattern

3.1 SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.4.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The initial uplink RB allocation is defined as the smaller uplink RB allocation value specified in table 6.3.4.3.4.1-1. The power level and RB allocation are reset for each sub-test.

3.2 Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.3 Send the appropriate TPC commands in the uplink scheduling information to UE until the UE EIRP measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level 0 dBm, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size is same as defined in step 1.3.

Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.4	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

3.5. Schedule the UE's PUSCH data transmission as described in Figure 6.3.5.2.4.2-3 for 5 frames with an uplink RB allocation alternating pattern as defined in table 6.3.4.3.5-3 while transmitting 0dB TPC command for PUSCH via the PDCCH.

3.6. Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, to verify the UE relative power control meet test requirements in 6.3.4.3.5. EIRP test procedure is defined in Annex K.1.3. EIRP is calculated considering both polarizations, theta and phi. Measurement of the power is not required in sub-frame after the mean power has exceeded the maximum power threshold. For power transients between sub-frames, transient periods of 40us between sub-frames are excluded. For ON/OFF or OFF/ON transients, transient periods of 20 us at the beginning of the sub-frame are excluded.

3.7 SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

NOTE 2:	These numbers of TPC commands are given as examples. The actual number of TPC commands transmitted in these steps shall be enough to ensure that the UE reaches the relevant maximum or minimum power threshold in each step, as shown in figure 6.3.4.3.4.2-1 thru 6.3.4.3.4.2-3.

6.3.4.3.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.3.4.3.5	Test requirement

Each UE power step measured in the test procedure 6.3.4.3.4.2 should satisfy the test requirements specified in Table 6.3.4.3.5-1 thru 6.3.4.3.5-3.

For a test pattern that is either a monotonically increasing or monotonically decreasing power sweep over the range specified for Tables 6.3.4.3.3-1and 6.3.4.3.3-2, 3 exceptions are allowed for each of the test patterns. For these exceptions, the power tolerance limit is a maximum of ± (11.0 + TT) dB. If there is an exception in the power step caused by the RB change for all test patterns (A, B, C) then fail the UE.

Table 6.3.4.3.5-1: Test Requirements Relative Power Tolerance for Transmission, channel BW 100MHz, SCS 60kHz, ramp up sub-test

| Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  | Sub-frames before RB change | 105RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 1 | RB change | 105RBs to 128 RBs | TPC=+1dB | 1.86 | ΔP < 2dB | 1.86 +/- (5.0 + TT) (NOTE 1) 1.86 +/- (3.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 128 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 90RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 2 | RB change | 90RBs to 128 RBs | TPC=+1dB | 2.53 | 2dB ≤ ΔP < 3dB | 2.53 +/- (6.0 + TT) (NOTE 1) 2.53 +/- (4.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 128 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 79RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 3 | RB change | 79RBs to 128 RBs | TPC=+1dB | 3.10 | 3dB ≤ ΔP < 4dB | 3,10 +/- (7.0 + TT) (NOTE 1) 3,10 +/- (5.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 128RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 32RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 4 | RB change | 32RBs to 128 RBs | TPC=+1dB | 7.02 | 4dB ≤ ΔP < 10dB | 7.02 +/- (8.0 + TT) (NOTE 1) 7.02 +/- (6.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 128 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 7RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 5 | RB change | 7RBs to 128 RBs | TPC=+1dB | 13.62 | 10dB ≤ ΔP < 15dB | 13.62 +/- (10.0 + TT) (NOTE 1) 13.62 +/- (8.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 128RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 6 | RB change | 1RB to 128 RBs | TPC=+1dB | 22.07 | 15dB < ΔP | 22.07 +/- (11.0 + TT) (NOTE 1) 22.07 +/- (9.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 128 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| NOTE 1: Applicable if Pint ≥ P ≥ Pmin. NOTE 2: Applicable if PUMAX ≥ P > Pint. NOTE 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |

Table 6.3.4.3.5-2: Test Requirements Relative Power Tolerance for Transmission, channel BW 100MHz, SCS 60kHz, ramp down sub-test

| Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  | Sub-frames before RB change | 128RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 1 | RB change | 128RBs to 105 RBs | TPC=-1dB | 1.86 | ΔP < 2dB | 1.86 +/- (5.0 + TT) (NOTE 1) 1.86 +/- (3.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 105 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 128RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 2 | RB change | 128RBs to 90 RBs | TPC=-1dB | 2.53 | 2dB ≤ ΔP < 3dB | 2.53 +/- (6.0 + TT) (NOTE 1) 2.53 +/- (4.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 90 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 128RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 3 | RB change | 128RBs to 79 RBs | TPC=-1dB | 3.10 | 3dB ≤ ΔP < 4dB | 3,10 +/- (7.0 + TT) (NOTE 1) 3,10 +/- (5.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 79RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 128RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 4 | RB change | 128RBs to 32 RBs | TPC=-1dB | 7.02 | 4dB ≤ ΔP < 10dB | 7.02 +/- (8.0 + TT) (NOTE 1) 7.02 +/- (6.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 32 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 128RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 5 | RB change | 128RBs to 7 RBs | TPC=-1dB | 13.62 | 10dB ≤ ΔP < 15dB | 13.62 +/- (10.0 + TT) (NOTE 1) 13.62 +/- (8.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 7RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
|  | Sub-frames before RB change | 128RB | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| 6 | RB change | 128RB to 1 RBs | TPC=-1dB | 22.07 | 15dB < ΔP | 22.07 +/- (11.0 + TT) (NOTE 1) 22.07 +/- (9.0 + TT) (NOTE 2) |
|  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (1.0 + TT) |
| NOTE 1: Applicable if Pint ≥ P ≥ Pmin. NOTE 2: Applicable if PUMAX ≥ P > Pint. NOTE 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |

Table 6.3.4.3.5-3: Test Requirements Relative Power Tolerance for Transmission, channel BW 100MHz, SCS 60kHz, alternating sub-test

| Sub-test ID | Uplink RB allocation | TPC command | Expected power step size (Up/Down) | Power step size range (Up/Down) | PUSCH |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
| 1 | Alternating 105 and 128 | TPC=0dB | 0.86 | ΔP < 2dB | 0.86 +/- (5.0 + TT) (NOTE 1) 0.86 +/- (3.0 + TT) (NOTE 2) |
| 2 | Alternating 79 and 128 | TPC=0dB | 2.10 | 2dB ≤ ΔP < 3dB | 2.10 +/- (6.0 + TT) (NOTE 1) 2.10 +/- (4.0 + TT) (NOTE 2) |
| 3 | Alternating 64 and 128 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (7.0 + TT) (NOTE 1) 3.01 +/- (5.0 + TT) (NOTE 2) |
| 4 | Alternating 32 and 128 | TPC=0dB | 6.02 | 4dB ≤ ΔP < 10dB | 6.02 +/- (8.0 + TT) (NOTE 1) 6.02 +/- (6.0 + TT) (NOTE 2) |
| 5 | Alternating 7 and 128 | TPC=0dB | 12.62 | 10dB ≤ ΔP < 15dB | 12.62 +/- (10.0 + TT) (NOTE 1) 12.62 +/- (8.0 + TT) (NOTE 2) |
| 6 | Alternating 1 and 128 | TPC=0dB | 21.07 | 15dB < ΔP | 21.07 +/- (11.0 + TT) (NOTE 1) 21.07 +/- (9.0 + TT) (NOTE 2) |
| NOTE 1: Applicable if Pint ≥ P ≥ Pmin. NOTE 2: Applicable if PUMAX ≥ P > Pint. NOTE 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |

6.3.4.4	Aggregate power tolerance

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerances are FFS.

- UE transmitted power for power class 1, 2 and 4 is FFS.

- Power window is FFS.

6.3.4.4.1	Test purpose

To verify the ability of the UE transmitter to maintain its power during non-contiguous transmissions within 21ms in response to 0 dB commands with respect to the first UE transmission and all other power control parameters as specified in 38.213 kept constant.

6.3.4.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.3.4.4.3	Minimum conformance requirements

The aggregate power control tolerance is the ability of the UE transmitter to maintain its power in a sub-frame (1 ms) non-contiguous transmissions within 21ms in response to 0 dB TPC commands with respect to the first UE transmission and all other power control parameters as specified in 38.213 kept constant.

The minimum requirements specified in Table 6.3.4.4.3-1 apply when the power of the target and reference sub-frames are within the power range bounded by the minimum output power as defined in sub-clause 6.3.1 and Pint as defined in sub-clause 6.3.4.2. The minimum requirements specified in Table 6.3.4.4.3-2 apply when the power of the target and reference sub-frames are within the power range bounded by Pint as defined in sub-clause 6.3.4.2 and the maximum output power as specified in sub-clause 6.2.1.

Table 6.3.4.4.3-1: Aggregate power tolerance, Pint ≥ P ≥ Pmin

| TPC command | UL channel | Aggregate power tolerance within 21ms |
| --- | --- | --- |
| 0 dB | PUCCH | ± 5.5 dB |
| 0 dB | PUSCH | ± 5.5 dB |

Table 6.3.4.4.3-2: Aggregate power tolerance, Pmax ≥ P ≥ Pint

| TPC command | UL channel | Aggregate power tolerance within 21ms |
| --- | --- | --- |
| 0 dB | PUCCH | ± 3.5 dB |
| 0 dB | PUSCH | ± 3.5 dB |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.3.4.4

6.3.4.4.4	Test description

6.3.4.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3.4.4.4.1-1 and table 6.3.4.4.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.4.4.4.1-1: Test Configuration Table: PUCCH subtest

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid range |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid and Highest |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |
| Test Parameters for Channel Bandwidths |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |
|  | N/A for aggregate power tolerance testcase | PUCCH format = Format 1 Length in OFDM symbols = 14 |
| 1 |  |  |

Table 6.3.4.4.4.1-2: Test Configuration Table: PUSCH subtest

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid and Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for aggregate power tolerance testcase | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3.4.4.4.1-1 (for PUCCH subtest) and Table 6.3.4.2.4.1-2 (for PUSCH subtest).

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.4.2.4.3.

6.3.4.4.4.2	Test procedure

The procedure is separated in two subtests to verify PUCCH and PUSCH aggregate power control tolerance respectively. The uplink transmission patterns are described in figure 6.3.4.4.4.2-1.

Figure 6.3.4.4.4.2-1: Test uplink transmission

1. PUCCH subtest:

1.1. Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1.

1.2. SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

1.3. The SS transmits PDSCH via PDCCH DCI format 0_1 for C_RNTI to transmit the DL RMC according to Table 6.3.4.4.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH.  Send uplink power control commands for PUCCH to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within PW of the target power level specified in Table 6.3.4.4.4.2-1 according to the power class with power ID = 1. PW is the power window according to Table 6.3.4.4.4.2-2 for the carrier frequency f and the channel bandwidth BW.

1.4. Every 10 sub-frames (10ms) transmit to the UE downlink PDSCH MAC padding bits as well as 0 dB TPC command for PUCCH via the PDCCH to make the UE transmit ACK/NACK on the PUCCH for 1 sub-frame (1ms). The downlink transmission is scheduled in the appropriate slots to make the UE transmit PUCCH as described in figure 6.3.4.4.4.2-1.

1.5. Measure the UE EIRP of 3 consecutive PUCCH transmissions in the Tx beam peak direction of in the measurement bandwidth specified in Table 6.3.1.5-1 and Table 6.3.1.5-2 to verify the UE transmitted PUCCH power is maintained within 21ms. EIRP test procedure is defined in Annex K. EIRP is calculated considering both polarizations, theta and phi. For TDD slots with transient periods are not under test.

1.6. SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

1.7. Repeat test steps 1.2 to 1.6 for measurement for power ID = 2 in Table 6.3.4.4.4.2-1.

2. PUSCH subtest:

2.1. Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1.

2.2. SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

2.3. The SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the PUSCH. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.  Send uplink power control commands for PUSCH to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within PW of the target power level specified in Table 6.3.4.4.4.2-1 according to the power class with power ID = 1. PW is the power window according to Table 6.3.4.4.4.2-2 for the carrier frequency f and the channel bandwidth BW.

2.4. Every 10 sub-frames (10ms) schedule the UE's PUSCH data transmission for 1 sub-frame (1ms)and transmit 0 dB TPC command for PUSCH via the PDCCH to make the UE transmit PUSCH. The uplink transmission patterns are described in figure 6.3.4.4.4.2-1.

2.5. Measure the UE EIRP of 3 consecutive PUSCH transmissions in the Tx beam peak direction of in the measurement bandwidth specified in Table 6.3.1.5-1 and Table 6.3.1.5-2 to verify the UE transmitted PUSCH power is maintained within 21ms. EIRP test procedure is defined in Annex K. EIRP is calculated considering both polarizations, theta and phi. For TDD slots with transient periods are not under test.

2.6. SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

2.7. Repeat test steps 2.2 to 2.6 for measurement for power ID = 2 in Table 6.3.4.4.4.2-1.

Table 6.3.4.4.4.2-1: Parameters for Aggregate power tolerance

| Power ID | Unit | power class 1 | power class 2 | power class 3 | power class 4 |
| --- | --- | --- | --- | --- | --- |
| 1 | dBm | TBD | TBD | 0 | TBD |
| 2 | dBm | TBD | TBD | 15 | TBD |

Table 6.3.4.4.4.2-2: Power Window (dB) for Aggregate Power tolerance for PUSCH and PUCCH

FFS

6.3.4.4.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.3.4.4.5	Test requirement

The requirement for the power measurements made in step (1.5) and (2.5) of the test procedure shall not exceed the values specified in Table 6.3.4.4.5-1 and Table 6.3.4.4.5-2. The power measurement period shall be 1 sub-frame (1ms).

Table 6.3.4.4.5-1: Power control tolerance (Pint ≥ P ≥ Pmin)

| TPC command | UL channel | Test requirement measured power |
| --- | --- | --- |
| 0 dB | PUCCH | Given 3 power measurements in the pattern, the 2nd, and later measurements shall be within ± 5.5+TTdB of the 1st measurement. |
| 0 dB | PUSCH | Given 3 power measurements in the pattern, the 2nd, and later measurements shall be within ± 5.5+TTdB of the 1st measurement. |
| Note 1: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3.4.4.5-3. |  |  |

Table 6.3.4.4.5-2: Power control tolerance (Pmax ≥ P > Pint)

| TPC command | UL channel | Test requirement measured power |
| --- | --- | --- |
| 0 dB | PUCCH | Given 3 power measurements in the pattern, the 2nd, and later measurements shall be within ± 3.5+TTdB of the 1st measurement. |
| 0 dB | PUSCH | Given 3 power measurements in the pattern, the 2nd, and later measurements shall be within ± 3.5+TTdB of the 1st measurement. |
| Note 1: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3.4.4.5-4. |  |  |

Table 6.3.4.4.5-3: Test Tolerance (Pint ≥ P ≥ Pmin)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | FFS | FFS |

Table 6.3.4.4.5-4: Test Tolerance (Pmax ≥ P > Pint)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | FFS | FFS |

<<Unchaged sections skipped>>

F.1	Acceptable uncertainty of Test System (normative)

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. Care should be taken to ensure that each conformance test implementation including the OTA chamber aspects meets the specified measurement uncertainty for each test case by requiring the test laboratory to maintain a detailed measurement uncertainty test report showing compliance to all the measurement uncertainty requirements. The detailed measurement uncertainty report would contain the justification for each measurement uncertainty component and its value and distribution. The derivation of these values is based on the minimum conformance requirements plus relaxation, i.e., test tolerance is not to be considered. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

The downlink signal uncertainties apply at the defined quiet zone with the UE properly positioned in the quiet zone. The uplink signal uncertainties apply at the measurement equipment with the UE positioned properly in the quiet zone.

F.1.1	Measurement of test environments

TBD

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2-2 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Quiet Zone size ≤ 30 cm ±4.42 dB (FR2a) ±4.62 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-2 in TR 38.903) |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC3 Quiet Zone size ≤ 30 cm ±4.60 dB (FR2a) ±5.20 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-4 in TR 38.903) |
| 6.2.2 UE maximum output power for modulation / channel bandwidth | PC3 Quiet Zone size ≤ 30 cm ±4.92 dB (FR2a) ±5.10 dB (FR2b) | MTSU = 1.00 x MU (from Table B.4.4.2 in TR 38.903) |
| 6.2.3 UE maximum output power with additional requirements | TBD |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD | MTSU = 1.00 x MU (from Table B.4.2-2 in TR 38.903) |
| 6.3.1 Minimum output power | PC1 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm FFS (FR2a) FFS dB (FR2b) PC3 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm ±6.15 dB (FR2a & FR2b) |  |
| 6.3.2 Transmit OFF power | PC3: Quiet Zone size ≤ 30 cm ±5.49 dB (FR2a) | MTSU = 1.00 x MU (from Table B.8-2-2 in TR 38.903) |
| 6.3.3.2 General ON/OFF time mask | PC3: ON power: TBD OFF power: Quiet Zone size ≤ 30 cm ±6.15 dB (FR2a & FR2b) |  |
| 6.3.3.4 PRACH time mask | PC3: PRACH power: TBD OFF power: Quiet Zone size ≤ 30 cm ±6.15 dB (FR2a & FR2b) |  |
| 6.3.3.6 SRS time mask | TBD |  |
| 6.3.4.2 Absolute power tolerance | Same as 6.3.1 |  |
| 6.3.4.3 Relative power tolerance | TBD |  |
| 6.3.4.4 Aggregate power tolerance | TBD |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) | TBD |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) | TBD |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) | TBD |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) | TBD |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) | TBD |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) | TBD |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) | TBD |  |
| 6.3A.2.1 Transmit OFF power for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.2 Transmit OFF power for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.3 Transmit OFF power for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO | PC3: OFF Power Quiet Zone size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |
| 6.3D.3.4 SRS time mask for UL MIMO | PC3: OFF Power Quiet Zone size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4.1 Frequency error | ± 0.01 ppm | MTSU = 1.00 x MU (from B.10.1 and B.10.2 in TR 38.903) |
| 6.4.2.1 Error vector magnitude | TBD |  |
| 6.4.2.2 Carrier leakage | TBD |  |
| 6.4.2.3 In-band emissions | TBD |  |
| 6.4.2.4 EVM equalizer spectrum flatness | TBD |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation | TBD |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) | TBD |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) | TBD |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) | TBD |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) | TBD |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) | TBD |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) | TBD |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) | TBD |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) | TBD |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) | TBD |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) | TBD |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) | TBD |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) | TBD |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) | TBD |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) | TBD |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) | TBD |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) | TBD |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) | TBD |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) | TBD |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) | TBD |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) | TBD |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) | TBD |  |
| 6.5.1 Occupied bandwidth | Quiet Zone size ≤ 30cm PC3: FR2a: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.2 [%CBW] (BW 200MHz) ±1.2 [%CBW] (BW 400MHz) FR2b: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.3 [%CBW] (BW 200MHz) ±1.3 [%CBW] (BW 400MHz) FR2c: TBD |  |
| 6.5.2.1 Spectrum Emission Mask | Quiet Zone size ≤ 30 cm ±4.94 dB (FR2a) ±5.32 dB (FR2b) | MTSU = 1.00 x MU (from Table B.16.2-2, B.16.2-3 in TR 38.903) |
| 6.5.2.3 Adjacent Channel Leakage Ratio | Quiet Zone size ≤ 30cm FR2a: ±5.63 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) FR2b: ±6.09 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) | MTSU = 1.00 x MU (from Table B.17-1 in TR 38.903) |
| 6.5.3.1 Transmitter Spurious emissions | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz) ±5.11 dB (12.75GHz < f ≤ 23.45GHz) ±5.41 dB (23.45GHz < f < 40.8GHz) ±7.42 dB (40.8GHz ≤ f ≤ 66GHz) ±7.72 dB (66GHz ≤ f ≤ 80GHz) | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |
| 6.5.3.2 Spurious emission band UE co-existence | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz Protected band n260, n261, n257: ±6.00 dB Protected frequency 57 GHz ≤ f ≤ 66GHz: ±8.01 dB Protected frequency 36 GHz ≤ f ≤ 37GHz: ±6.00 dB | MTSU = 1.00 x MU (from Table B.18-1a in TR 38.903) |
| 6.5.3.3 Additional Spurious emission | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz), NS_202 ±5.70 dB (12.75GHz < f ≤ 23.45GHz), NS_202 ±6.00 dB (23.45GHz < f < 40.8GHz), NS_202, NS_203 ±8.01 dB (40.8GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band), NS_202 | MTSU = 1.00 x MU (from Table B.18-1b in TR 38.903) |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Quiet Zone size ≤ 30cm PC3: FR2a: TBD FR2b: TBD FR2c: TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) | TBD |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) | TBD |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) | TBD |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) | TBD |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD | MTSU = 1.00 x MU (from Table B.17.2-2 and Table B.17.2-4 in TR 38.309) |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) | TBD |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) | TBD |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) | TBD |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) | TBD |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.4 Transmitter Spurious emissions for CA (5UL CA) | TBD |  |
| 6.5A.3.1.5 Transmitter Spurious emissions for CA (6UL CA) | TBD |  |
| 6.5A.3.1.6 Transmitter Spurious emissions for CA (7UL CA) | TBD |  |
| 6.5A.3.1.7 Transmitter Spurious emissions for CA (8UL CA) | TBD |  |
| 6.5A.3.2.1 Spurious emission band UE co-existence for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.2.2 Spurious emission band UE co-existence for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.2.3 Spurious emission band UE co-existence for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.2.4 Spurious emission band UE co-existence for CA (5UL CA) | TBD |  |
| 6.5A.3.2.5 Spurious emission band UE co-existence for CA (6UL CA) | TBD |  |
| 6.5A.3.2.6 Spurious emission band UE co-existence for CA (7UL CA) | TBD |  |
| 6.5A.3.2.7 Spurious emission band UE co-existence for CA (8UL CA) | TBD |  |
| 6.5A.3.3.1 Additional spurious emissions for CA (2UL CA) | TBD |  |
| 6.5A.3.3.2 Additional spurious emissions for CA (3UL CA) | TBD |  |
| 6.5A.3.3.3 Additional spurious emissions for CA (4UL CA) | TBD |  |
| 6.5A.3.3.4 Additional spurious emissions for CA (5UL CA) | TBD |  |
| 6.5A.3.3.5 Additional spurious emissions for CA (6UL CA) | TBD |  |
| 6.5A.3.3.6 Additional spurious emissions for CA (7UL CA) | TBD |  |
| 6.5A.3.3.7 Additional spurious emissions for CA (8UL CA) | TBD |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause |  | Maximum Test System Uncertainty |  | Derivation of MTSU |  |
| --- | --- | --- | --- | --- | --- |
| 7.3.2 Reference sensitivity power level |  | PC3 ±5.19 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b, NTC testing) ±5.45 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.19-2-2 in TR 38.903) |  |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3.4 EIS spherical coverage |  | PC3 ±4.90 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b) |  | MTSU = 1.00 x MU (from Table B.19-2-2 in TR 38.903) |  |
| 7.4 Maximum input level |  | TBD |  |  |  |
| 7.5 Adjacent channel selectivity |  | TBD |  |  |  |
| 7.6.2 In-band blocking |  | TBD |  |  |  |
| 7.9 Spurious emissions |  | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz For Band n257: ±5.50dB (6GHz ≤ f ≤ 12.75GHz) ±5.46dB (12.75GHz < f ≤ 23.45GHz) ±6.11dB (23.45GHz < f < 40.8GHz) ±7.65dB (40.8GHz ≤ f ≤ 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) |  | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |  |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.1.2-1. |  |  |  |  |  |

F.2	Interpretation of measurement results (normative)

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause F.1 of the present document.

If the Test System using one of the permitted test methods defined in TR38.903 [20] for a test is known to have a measurement uncertainty greater than that specified in clause F.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows:

Any additional uncertainty in the Test System over and above that specified in clause F.1 shall be used to tighten the Test Requirement, making the test harder to pass. For some tests, for example receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with clause F.1does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause F.1 had been used.

F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

F.3.1	Measurement of test environments

TBD

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) |  | PC3 Minimum peak EIRP IFF (Quiet Zone size ≤ 30 cm) 2.87 dB (FR2a) 2.87 dB (FR2b) Max EIRP 0 dB |  | Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) |  |
| 6.2.1.1 UE maximum output power (TRP) |  | PC3 Max TRP IFF (Quiet Zone size ≤ 30 cm) 2.65 dB (FR2a) 2.77 dB (FR2b) |  | Max TRP TT = 0.60 x MTSUIFF |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) |  | PC1 TBD PC2 TBD PC3 IFF (Quiet Zone size ≤ 30 cm) 2.58 dB (FR2a) 2.58 dB (FR2b) PC4 TBD |  | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) |  |
| 6.2.2 UE maximum output power for modulation / channel bandwidth |  | PC3 Minimum peak EIRP IFF (Quiet Zone size ≤ 30 cm) 3.11 dB (FR2a) 3.11 dB (FR2b) |  | Minimum peak EIRP TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) |  |
| 6.2.3 UE maximum output power with additional requirements |  | TBD |  |  |  |
| 6.2.4 Configured transmitted power |  | TBD |  |  |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) |  | Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3.1 Minimum output power |  | PC3 Minimum EIRP IFF (Quiet Zone size ≤ 30 cm) 3.35 dB (FR2a) 3.35 dB (FR2b) |  | Minimum EIRP TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) |  |
| 6.3.2 Transmit OFF power |  | 0 dB |  |  |  |
| 6.3.3.2 General ON/OFF time mask |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD |  | ON Power TBD |  |
| 6.3.3.4 PRACH time mask |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD |  | ON Power TBD |  |
| 6.3.4.2 Absolute power tolerance |  | Same as 6.3.1 |  |  |  |
| 6.3.4.3 Relative power tolerance |  | TBD |  |  |  |
| 6.3.4.4 Aggregate power tolerance |  | TBD |  |  |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) |  | TBD |  |  |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) |  | TBD |  |  |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) |  | TBD |  |  |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) |  | TBD |  |  |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) |  | TBD |  |  |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) |  | TBD |  |  |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) |  | TBD |  |  |  |
| 6.3A.2.1 Transmit OFF power for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.2.2 Transmit OFF power for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.2.3 Transmit OFF power for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3D.3.4 SRS time mask for UL MIMO |  | PC3: OFF Power Quiet Zone size ≤ 30cm 0 dB ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4.1 Frequency error |  | 0.005 ppm |  | TT = 0.5 x MTSU |  |
| 6.4.2.1 Error vector magnitude |  | 0%, up to 64QAM |  | Minimum requirement + TT |  |
| 6.4.2.2 Carrier leakage |  | TBD |  |  |  |
| 6.4.2.3 In-band emissions |  | TBD |  |  |  |
| 6.4.2.4 EVM equalizer spectrum flatness |  | TBD |  |  |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation |  | TBD |  |  |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) |  | TBD |  |  |  |
| 6.5.1 Occupied bandwidth |  | 0 kHz |  | Minimum requirement + TT |  |
| 6.5.2.1 Spectrum Emission Mask |  | IFF (Quiet Zone size ≤ 30 cm) 3.21 dB (FR2a) 3.46 dB (FR2b) |  | TT = 0.65 x MTSUIFF |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio |  | Absolute requirement 0 dB Relative requirement IFF (Quiet Zone size ≤ 30 cm) FR2a: ±4.66 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) FR2b: ±4.96 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5.3.1 Transmitter Spurious emissions |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.2 Spurious emission band UE co-existence |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.3 Additional spurious emission |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |  |  |  |

F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 7.3.2 Reference sensitivity power level |  | PC3 IFF (Quiet Zone size ≤ 30 cm, FR2a, FR2b) 2.34 dB |  | TT = 0.45 x MTSUIFF |  |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3.4 EIS spherical coverage |  | IFF (Quiet Zone size ≤ 30 cm, FR2a, FR2b) 2.21 dB |  | PC3 TT = 0.45 x MTSUIFF |  |
| 7.4 Maximum input level |  | TBD |  |  |  |
| 7.5 Adjacent channel selectivity |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6.2 In-band blocking |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.9 Spurious emissions |  | 0 dB |  | Minimum requirement + TT T-put limit unchanged |  |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.3.2-1. |  |  |  |  |  |

<<End of change>>