3GPP TSG RAN Meeting #91-e	 	R5-213320r2

Electronic Meeting, 17th May – 28th May 2021

Source:		Rohde & Schwarz

Title:	Correction of testability analysis for FR2b ACLR test cases

Agenda Item:		5.3.2.17

Document for:	Discussion and Endorsement

# Introduction

At RAN5 #90-e the relaxation of ACLR test requirements has been corrected for FR2a [1]. In this contribution, we discuss the testability of ACLR requirements for FR2b.

# Discussion

Applying the same analysis as in [1] for FR2b, PC3, IFF, and 30cm Quiet Zone, results in the SNR values for the adjacent channel power summarized in Table 1. Cells with red shading result in a SNR values above 1.00 dB.

Table 1: ACP SNR for PC3, FR2b IFF 30cm Quiet Zone

| MPR | T(MPR) | MBR | Carrier SNR | SNR incl. MBR, MPR,T(MPR) | ACP SNR | ACP SNR | ACP SNR | ACP SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz | CHBW 400 MHz |
|  |  |  | (Note 1) | (Note 2) | (Note 3) | (Note 3) | (Note 3) | (Note 3) |
| 0 | 0 | 0.75 | 26.1 | 25.35 | 18.38 | 15.37 | 12.36 | 9.35 |
| 0.5 | 1.5 | 0.75 | 26.1 | 23.35 | 16.38 | 13.37 | 10.36 | 7.35 |
| 1 | 1.5 | 0.75 | 26.1 | 22.85 | 15.88 | 12.87 | 9.86 | 6.85 |
| 1.5 | 1.5 | 0.75 | 26.1 | 22.35 | 15.38 | 12.37 | 9.36 | 6.35 |
| 2 | 1.5 | 0.75 | 26.1 | 21.85 | 14.88 | 11.87 | 8.86 | 5.85 |
| 2.5 | 2 | 0.75 | 26.1 | 20.85 | 13.88 | 10.87 | 7.86 | 4.85 |
| 3 | 2 | 0.75 | 26.1 | 20.35 | 13.38 | 10.37 | 7.36 | 4.35 |
| 3.5 | 3 | 0.75 | 26.1 | 18.85 | 11.88 | 8.87 | 5.86 | 2.85 |
| 4 | 3 | 0.75 | 26.1 | 18.35 | 11.38 | 8.37 | 5.36 | 2.35 |
| 4.5 | 4 | 0.75 | 26.1 | 16.85 | 9.88 | 6.87 | 3.86 | 0.85 |
| 5 | 4 | 0.75 | 26.1 | 16.35 | 9.38 | 6.37 | 3.36 | 0.35 |
| 5.5 | 5 | 0.75 | 26.1 | 14.85 | 7.88 | 4.87 | 1.86 | -1.15 |
| 6 | 5 | 0.75 | 26.1 | 14.35 | 7.38 | 4.37 | 1.36 | -1.65 |
| 6.5 | 5 | 0.75 | 26.1 | 13.85 | 6.88 | 3.87 | 0.86 | -2.15 |
| 7 | 5 | 0.75 | 26.1 | 13.35 | 6.38 | 3.37 | 0.36 | -2.65 |
| 7.5 | 5 | 0.75 | 26.1 | 12.85 | 5.88 | 2.87 | -0.14 | -3.15 |
| 8 | 5 | 0.75 | 26.1 | 12.35 | 5.38 | 2.37 | -0.64 | -3.65 |
| 8.5 | 5 | 0.75 | 26.1 | 11.85 | 4.88 | 1.87 | -1.14 | -4.15 |
| 9 | 5 | 0.75 | 26.1 | 11.35 | 4.38 | 1.37 | -1.64 | -4.65 |
| Note 1: Based on R5-200868 [2]. Note 2: Carrier SNR - MBR - MPR - T(MPR) Note 3: SNR incl. MBR, MPR, T(MPR) - 16dB (ACLR) |  |  |  |  |  |  |  |  |

Table 2 summarizes the maximum testable SNR per channel bandwidth. 400 MHz channel bandwidth is for information only since in a previous meeting RAN5 restricted ACLR test for FR2b to channel bandwidths below 400 MHz.

Table 2: Maximum testable MPR for ACLR, FR2b, PC3, IFF 30cm Quiet Zone

| CHBW | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| MPR limit (dB) | 7.5 | 5.0 | 3.5 | 2.0 |

Since RAN5 has previously agreed to skip points with SNR above 1 dB TPs with MPR values above the limits shown in Table 2 shall not be tested.

The test point selection in TS 38.521-2 is already aligned to the values in Table 2. In contrast to the maximum testable MPR values of CHBWs above 50 MHz, the value for 50 MHz CHBW is not documented in TR 38.903 [5].

Proposal 1: Document the maximum testable MPR for 50 MHz for FR2b, PC3, IFF 30cm Quiet Zone in TR 38.903.

Accompanying CR [3] provides a documentation of the assumptions and limits in TR 38.903 [5].

# Conclusion

In summary, testability of ACLR test points has been discussed for FR2b, PC3, IFF 30 cm Quiet Zone size.

Proposal 1: Document the maximum testable MPR for 50 MHz for FR2b, PC3, IFF 30cm Quiet Zone in TR 38.903.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | FR2b |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #4 | Max EIRP | EIRP MAX= +43 dBm (rms) |
| #5 | Temperature range of the test equipment | 20°C – 35°C |

# References

- R5-204746, Noise impact for FR2 test case where MPR applies, Anritsu, 3GPP TSG RAN WG5 Meeting #88-e, August 2020

- R5-200868, On the SNR for FR2 TRx test cases, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #86-e, February 2020

- R5-213322, Update of ACLR testability, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

- TR 38.903, V16.7.0, 3GPP TSG RAN WG5, March 2021

- TS 38.521-2, V16.7.0, 3GPP TSG RAN WG5, March 2021