3GPP TSG-RAN5 Meeting #91-e	R5-213383r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1364 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NB-IoT Common test environment parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some NB-IoT Common test environment parameters are missing from 38.508 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the default configuration and parameters for NB-IoT Common test environment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB_IOTenh2 may not implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Complete the frequency domain locations in table 8.1.3.3.3-2 |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

8	NB-IoT test environment

8.1	NB-IoT Common test environment

8.1.1	NB-IoT Environmental conditions

Same environmental conditions as the ones described in section 4.1 apply to NB-IoT UEs.

8.1.2	NB-IoT Common requirements of test equipment

Same common requirements of test equipment as the ones described in section 4.2 apply to NB-IoT with the following exceptions:

-	Only FDD Mode is required for Rel-13 and Rel-14 NB-IoT

-	Both FDD and TDD Mode are required for Rel-15 and forward NB-IoT

-	Supported physical channels for NB-IoT are:

| Physical channel | Minimum number | Comments |
| --- | --- | --- |
| NPBCH | 1 | Narrowband Physical Broadcast Channel |
| NPDCCH | 1 | The Narrowband Physical Downlink Control channel carries control information. |
| NPDSCH | 1 | Narrowband Physical Downlink Shared Channel |
| NPUSCH | 1 | Narrowband Physical Uplink Shared Channel |
| NPRACH | 1 | Narrowband Physical Random Access Channel |

-	Supported physical signals for NB-IoT are:

| Physical signal | Minimum number | Comments |
| --- | --- | --- |
| Narrowband Reference Signal | NA | DL |
| Narrowband Primary Synchronization Signal | NA | DL |
| Narrowband Secondary Synchronization Signal | NA | DL |
| Demodulation Reference Signal | NA | UL |

8.1.3	NB-IoT Reference test conditions

This clause contains the reference test conditions, which apply to all NB-IoT test cases unless otherwise specified.

8.1.3.1	NB-IoT Test frequencies

NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 25, 26, 28, 31, 41, 66, 70, 71, 72, 73, 74, 85, 87 and 88 which are defined in Table 5.5-1 in [21]. NB-IoT system operates in HD-FDD and TDD duplex modes.

The test frequencies are based on the E-UTRA frequency bands defined in the core specifications.

The raster spacing is 100 KHz.

E-UTRA/FDD is designed to operate in paired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 8.1.3.1.1.

E-UTRA/TDD is designed to operate in unpaired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 8.1.3.1.2.

<Unchanged Text Skipped>

8.1.3.3.3	NB-IoT Mapping of downlink physical channels and signals to physical resources

Parameters for mapping of NB-IoT downlink physical channels and signals are specified as follows:

Normal Cyclic Prefix

, Physical layer cell identity	= 0 is used as the default physical layer cell identity

For RF testing, the mapping of DL physical channels to resource element is defined TS 36.521-1 [21] Annex C.1.

Table 8.1.3.3.3-1: NB-IoT Mapping of DL Physical Channels to Resource Elements (FDD)

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| NPBCH | - NPBCH is transmitted in subframe 0 in every radio frame - NPBCH consists of 8 independently decodable blocks of 80 ms duration - The time interval where MIB remains unchanged is 640 ms - NPBCH does not use the first 3 symbols in a subframe in in-band operation. - For stand-alone and guard-band, the first 3 symbols (of the subframe transmitting NPBCH) contain no NPBCH. | Occupies any of the 12 subcarriers not reserved for transmission of reference signals | Mapping rule is specified in TS36.211 [35] sub clause 10.2.4.4 |
| NPSS | - NPSS is transmitted in subframe 5 - NPSS uses the last 11 OFDM symbols of subframes in which NB-PSS occurs for normal CP - NB-IoT PSS/SSS do not use: - the LTE PDCCH control region - REs used by LTE CRS - The periodicity of NPSS transmission is 10ms. | NPSS is mapped to sub-carriers #0-10 of the NB-IoT carrier | Mapping rule is specified in TS36.211 [35] sub clause 10.2.7.1.2 |
| NSSS | - NSSS is transmitted in subframe 9 - NSSS uses the last 11 OFDM symbols of subframes in which NB-SSS occurs for normal CP - NB-IoT PSS/SSS do not use: - the LTE PDCCH control region - REs used by LTE CRS - NSSS periodicity is 20ms | The number of subcarriers for NSSS is 12 | Mapping rule is specified in TS36.211 [35] sub clause 10.2.7.2.2 |
| NPDCCH | - NPDCCH on a given NB-IoT carrier are not mapped to the subframes containing NPSS/NSSS/PBCH on that carrier - NPDCCH are not be mapped onto resources elements used for NRS - NPDCCH are not overlapped with PBCH, PSS, SSS, or CRS - One or two NPDCCHs can be transmitted in a subframe. - In in-band, first three OFDM symbols are not used for NPDCCH - In stand-alone and guard-band, all OFDM symbols are available for NPDCCH - NPDCCH and NPDSCH are multiplexed only based on TDM at subframe level - It means that only cross subframe scheduling is supported - The start of an NPDCCH search space is >=4ms after the end of the last NPDCCH search space | - NPDCCH is transmitted on an aggregation of one or two consecutive narrowband control channel elements (NCCEs), where a narrowband control channel element corresponds to 6 consecutive subcarriers in a subframe where NCCE 0 occupies subcarriers 0 through 5 and NCCE 1 occupies subcarriers 6 through 11. - REG is not defined for NPDCCH | Mapping rule is specified in TS36.211 [35] sub clause 10.2.5.5 |
| NPDSCH | - The start of NB-PDSCH transmission is >=4ms later than the end of its associated DL assignment - NPDSCH on a given NB-IoT carrier are not mapped to the subframes containing NPSS/NSSS/PBCH on that carrier - NPDSCH resources elements should be different from the ones used for NRS - NPDSCH resources elements should be different from the ones used for CRS | Occupies any of the 12 subcarriers not reserved for transmission of reference signals | Mapping rule is specified in TS36.211 [35] sub clause 10.2.3.4 |
| NRS | - Narrowband reference signals are transmitted in all NB-IoT downlink subframes in a cell supporting NPDSCH transmission. - NRS is not transmitted in subframes that are not NB-IoT downlink subframes, except if these subframes contain NPBCH or NPDSCH carrying SystemInformationBlockType1-NB where NRS shall be transmitted. - Narrowband reference signals are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS [when no NB-IoT downlink subframes configuration has not been established]. - The narrowband reference signals shall not be mapped to subframes containing NPSS or NSSS. - NRS are transmitted on one or two antenna ports 0 to 1 (refer to Figure 10.2.6.2-1 in TS 36.211 [35]). | 2 subcarriers per antenna port in any OFDM symbols where NRS is transmitted |  |

Table 8.1.3.3.3-2: NB-IoT Mapping of DL Physical Channels to Resource Elements (TDD)

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| NPBCH | Same as in Table 8.1.3.3.3-1, except for - NPBCH is transmitted in subframe 9 in every radio frame | Same as in Table 8.1.3.3.3-1 |  |
| NPSS | Same as in Table 8.1.3.3.3-1 | Same as in Table 8.1.3.3.3-1 |  |
| NSSS | Same as in Table 8.1.3.3.3-1, except for - NSSS is transmitted in subframe 0 | Same as in Table 8.1.3.3.3-1 |  |
| NPDCCH | Same as in Table 8.1.3.3.3-1, except for - Except in a special subframe when NPDCCH is transmitted in more than one subframe, - In a special subframe where the NPDCCH is transmitted in one subframe, NPDCCH are mapped into DwPTS - In a special subframe where the NPDCCH is transmitted in more than one subframe, NPDCCH are not mapped to NRS locations when the subframe is not a special subframe. | Same as in Table 8.1.3.3.3-1 |  |
| NPDSCH | Same as in Table 8.1.3.3.3-1, except for - Except in a special subframe when MrepNPDSCH >1, NPDSCH resources elements should be different from the ones used for NRS - NPDSCH resources elements should be different from the ones used for CRS - In a special subframe, if MrepNPDSCH=1, NPDSCH are mapped into DwPTS - In a special subframe, if MrepNPDSCH>1, NPDSCH are not mapped to NRS locations in subframes which are not special subframes. | Same as in Table 8.1.3.3.3-1 |  |
| NRS | Same as in Table 8.1.3.3.3-1, except for - Narrowband reference signals are transmitted in subframes #9 and in subframes #0 not containing NSSS [when no ddNB-IoT downlink subframes configuration has not been established]. | Same as in Table 8.1.3.3.3-1 |  |

8.1.3.3.4	NB-IoT Uplink physical channels and physical signals

The NB-IoT uplink setting in table 8.1.3.3.4-1 is used unless otherwise specified in a test case.

Table 8.1.3.3.4-1: NB-IoT Default Uplink setting

|  | Unit | Channel bandwidth |
| --- | --- | --- |
|  |  | 200 kHz |
| Number of Tones |  | 1 |
| Sub-carrier spacing | kHz | 15 |

<Unchanged Text Skipped>

8.1.4.3.2	NB-IoT Common contents of system information messages

-	MasterInformationBlock-NB

The MasterInformationBlock-NB includes the system information transmitted on BCH.

Table 8.1.4.3.2-1: MasterInformationBlock-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| systemFrameNumber-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| hyperSFN-LSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-r13 | 2 | TBS = 208 bits, 16 repetitions |  |
| systemInfoValueTag-r13 | 0 |  |  |
| ab-Enabled-r13 | FALSE |  |  |
| operationModeInfo-r13 CHOICE { |  |  |  |
| inband-SamePCI-r13 SEQUENCE { |  |  | Inband-Same |
| eutra-CRS-SequenceInfo-r13 | 23 | As specified in TS 36.213 [29] Clause 16.8 | PRB30 |
|  | 24 | As specified in TS 36.213 [29] Clause 16.8 | PRB35 |
|  | 7 | As specified in TS 36.213 [29] Clause 16.8 | PRB17 |
|  | 8 | As specified in TS 36.213 [29] Clause 16.8 | PRB22 |
| } |  |  |  |
| inband-DifferentPCI-r13 SEQUENCE { |  |  | Inband_Different |
| eutra-NumCRS-Ports-r13 | same | same number of ports as NRS |  |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| spare | ’00’B |  |  |
| } |  |  |  |
| guardband-r13 SEQUENCE { |  |  | Guardband |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| spare | ‘000’B |  |  |
| } |  |  |  |
| standalone-r13 SEQUENCE { |  |  | Standalone |
| spare | ‘0000 0’B |  |  |
| } |  |  |  |
| } |  |  |  |
| additionalTransmissionSIB1-r15 | FALSE | A Rel-13/Rel-14 UE will interpret FALSE as spare ‘0’ |  |
| spare | ‘0000 0000 00’B |  |  |
| } |  |  |  |

| Condition |  | Explanation |  |
| --- | --- | --- | --- |
| Inband-Same |  | In-band with same PCI test environment |  |
| Inband-Different |  | In-band with different PCI test environment |  |
| PRB30 |  | For 10 MHz LTE Cell with In-Band NB-IoT in PRB 30 |  |
| PRB35 |  | For 10 MHz LTE Cell with In-Band NB-IoT in PRB 35 |  |
| PRB17 |  | For 5 MHz LTE Cell with In-Band NB-IoT in PRB 17 |  |
| PRB22 |  | For 5 MHz LTE Cell with In-Band NB-IoT in PRB 22 |  |
| Standalone |  | Standalone test environment |  |
| Guardband |  | Guard-band test environment |  |

<Unchanged Text Skipped>

8.3.2.3.1	NB-IoT Test frequencies for signalling test

Test frequencies for signalling test are specified in table 8.3.2.3.1-1 for FDD in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.

Table 8.3.2.3.1-1: Test frequencies for NB-IoT FDD (200 kHz)

| NB-IoT Operating Band |  | Bandwidth [MHz] |  | f1, f5 |  |  |  | f2 |  |  |  | f3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NUL |  | NDL |  | NUL |  | NDL |  | NUL |  | NDL |  |
| 1 |  | 60 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 2 |  | 60 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 3 |  | 75 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 4 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 5 |  | 25 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  | 35 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  | 20 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 12 |  | 17 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 13 |  | 10 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 14 |  | 10 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  | 12 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 18 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 19 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 20 |  | 30 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 21 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  | 65 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 26 |  | 35 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 66 |  | 70+901 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 70 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 71 |  | 35 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 73 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 85 |  | 18 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 87 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 88 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| NOTE 1: Asymmetric operating band (UL + DL). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Test frequencies for signalling test of MFBI are specified in Table 8.3.2.3.1-1a for FDD in terms of Low, Mid and High which are referred to as Low Range, Mid Range and High range in clause 8.1.3.1.

Table 8.3.2.3.1-1a: Test frequencies for NB-IoT FDD MFBI (200 kHz)

| NB-IoT Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL |
| 2 | 25 | 60 | Mid | Mid | High | High | Low | Low |
| 5 | 18 | 15 | Mid | Mid | 23999 | 5999 | Low | Low |
| 5 | 19 | 15 | Mid | Mid | 20609 | 2609 | Low | Low |
| 5 | 26 | 25 | Mid | Mid | High | High | Low | Low |
| 12 | 17 | 12 | Mid | Mid | High | High | Low | Low |
| 17 | 12 | 12 | Mid | Mid | High | High | Low | Low |
| 19 | 5 | 15 | Mid | Mid | High | High | Low | Low |
| 19 | 26 | 15 | Mid | Mid | High | High | Low | Low |
| 25 | 2 | 60 | Mid | Mid | 26639 | 8639 | Low | Low |
| 26 | 5 | 25 | Mid | Mid | High | High | Low | Low |
| 26 | 18 | 15 | Mid | Mid | High | High | Low | Low |
| 26 | 19 | 15 | Mid | Mid | High | High | Low | Low |

Test frequencies for signalling test are specified in table 8.3.2.3.1-2 for TDD in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.

Table 8.3.2.3.1-2: Test frequencies for NB-IoT TDD (200 kHz)

| NB-IoT Operating Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 60 | Mid | Mid | High | High | Low | Low |