3GPP TSG-RAN WG5 Meeting #92-e	R5-214646

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0175 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.4.3 - HTTP POST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to 24.481 clause 6.3.14.2 the GC shall set the Request-URI of the HTTP POST request to an XCAP URI in users tree where the XUI is set to a group creation XUI configuration parameter and with the document selector identifying the temporary MCS group to be created; XCAP root URI shall be absolute URI. - According to 24.484 clause 6.2.5.1 the GMS shall handle a HTTP request as specified for the HTTP server in annex A of 24.482; according to 24.482 A.2.3 the HTTP server (i.e. the GMS) expects Authorization header field with the "Bearer" authentication scheme and a bearer access token. - The content type for GMOP is "application/vnd.3gpp.GMOP+xml" - Condition TEMPGROUP shall be moved to Table 5.5.4-1 as it is needed for Table 5.5.4.6-1 too |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Request URI corrected according to 24.481 clause 6.3.14.2 - Condition TEMPGROUP added to Authorization in Table 5.5.4.3-1 - Content type corrected - Condition table removed from clause 5.5.4.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Client may send invalid HTTP POST without failing the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.579-1 CR 0174 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See also documents R5w210209 and R5w210211 discussed at TTCN Workshop #54 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.4.3	POST

Table 5.5.4.3-1: HTTP POST

| Derivation Path: RFC 2616 [26] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| Status-Line |  |  |  |  |  |  |  |  |  |
| Method |  | "POST" |  |  |  |  |  |  |  |
| Request-URI |  |  |  |  |  |  |  |  |  |
| uri |  | tsc_MCX_IdMS_auth_UriPath |  | points to the Authorisation endpoint of the IdM Server |  | TS 33.180 [94] |  | AUTH, USERAUTH |  |
|  |  | tsc_MCX_IdMS_userauth_UriPath |  | points to the endpoint verifying the user authentication; same URI as provided to the UE in the action attribute of the HTML login form |  | TS 33.180 [94] HTML 4.01 Specification [105] |  | USERAUTH |  |
|  |  | tsc_MCX_IdMS_token_UriPath |  | points to the Token endpoint of the IdM Server |  | TS 33.180 [94] |  | TOKEN |  |
|  |  | tsc_MCX_KMS_init_UriPath |  | "KMS Initialize" request according to TS 33.180 [94] D.2.3 |  | TS 33.180 [94] |  | KMSINIT |  |
|  |  | tsc_MCX_KMS_keyprov_UriPath |  | "KMS KeyProvision" request according to TS 33.180 [94] D.2.4 |  | TS 33.180 [94] |  | KMSKEY |  |
|  |  | tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/users/" & px_MCPTT_GroupCreationXUI & "/" & px_MCPTT_Group_T_ID |  | Points to the temporary group configuration document to be created |  | TS 24.481[11] clause 6.3.14.2 |  | TEMPGROUP |  |
| HTTP-Version |  | "HTTP/1.1" |  |  |  |  |  |  |  |
| Cache-Control |  |  |  |  |  | RFC 2616 [26] |  |  |  |
| cache-directive |  | "no-cache" |  |  |  |  |  |  |  |
| Authorization |  |  |  |  |  | RFC 2617 [72] |  | KMSINIT, KMSKEY, TEMPGROUP |  |
| authentication-scheme |  | “Bearer” |  |  |  | RFC 6750 [104] |  |  |  |
| b64token |  | Access token as assigned to the UE by Token Response |  |  |  | RFC 6750 [104] |  |  |  |
| Content-Type |  |  |  |  |  |  |  | AUTH, USERAUTH, TOKEN |  |
| media-type |  | "application/x-www-form-urlencoded" |  |  |  |  |  |  |  |
| Content-Type |  |  |  | present in case of KMS request security |  |  |  | (KMSINIT OR KMSKEY) AND pc_MCX_KMS_RequestSecurity |  |
| media-type |  | "application/xml" |  |  |  | RFC 7303 [112] |  |  |  |
| Content-Type |  |  |  |  |  |  |  | TEMPGROUP |  |
| media-type |  | "application/vnd.3gpp.GMOP+xml" |  |  |  |  |  |  |  |
| Message-body |  |  |  |  |  |  |  | AUTH |  |
| Authentication Request |  | As described in Table 5.5.4.10.1-1 |  |  |  |  |  |  |  |
| Message-body |  |  |  |  |  | HTML 4.01 Specification [105] |  | USERAUTH |  |
| user |  | px_MCX_User_A_username |  |  |  |  |  |  |  |
| password |  | px_MCX_User_A_password |  |  |  |  |  |  |  |
| Message-body |  |  |  |  |  |  |  | TOKEN |  |
| Token request |  | As described in Table 5.5.4.10.3-1 |  |  |  |  |  |  |  |
| Message-body |  |  |  | present in case of KMS request security |  |  |  | (KMSINIT OR KMSKEY) AND pc_MCX_KMS_RequestSecurity |  |
| Signed KMS Request |  | As described in Table 5.5.4.10.9-1 |  |  |  |  |  |  |  |
| Message-body |  |  |  |  |  |  |  | TEMPGROUP |  |
| Temporary Group Creation Document" |  | As described in Table 5.5.7.1-3 |  |  |  |  |  |  |  |