3GPP TSG-RAN5 Meeting #92-e	R5-215407

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2440 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of System information combination for NR-DC PDCP test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR-1 system information combination would be inline with real-deployment rather than NR-4 in case of FR1+FR2 NR-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applied NR-1 for FR1+FR2 NR-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start o f Change>

### 7.1.3	PDCP

#### 7.1.3.0	Default Pre-Test Conditions for all PDCP test cases

The following pre-test conditions shall be applied in all PDCP test cases until the test case explicitly over writes these conditions

System Simulator:

-	The SS configures the test environment in accordance to the execution conditions in Table 7.1.3.0-1.

UE:

-	None

Preamble:

-	The SS performs the generic procedure in [4] to get UE in state RRC_CONNECTED in accordance to the execution conditions in Table 7.1.3.0-2 and using the message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU.

Table 7.1.3.0-1: Test environment

| Execution Condition | Cell configuration | System Information Combination |
| --- | --- | --- |
| IF pc_NG_RAN_NR and Connectivity(NR-DC) | NR Cell 1 is PCell NR Cell 10 is PSCell | NR System information Combination NR-1 |
| IF pc_NG_RAN_NR | NR Cell 1 | NR System information Combination NR-1 |
| ELSE IF pc_EN_DC | E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |
| ELSE IF pc_NGEN_DC | NG-RAN E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |

Table 7.1.3.0-2: Preamble parameters

| Execution Condition | Multi-PDN / Multi-PDU Sessions Condition | Generic Procedure Parameters | Primary DRB used for Data testing |
| --- | --- | --- | --- |
| IF pc_NG_RAN_NR | FALSE | Connectivity(NR), Test loop function(On) One DRB | Default DRB of the first PDU session on NR Cell |
|  |  | Connectivity(NR-DC), DC bearer(One MN Terminated MCG bearer and One SN Terminated SCG bearer), Test loop function(On) | SN terminated SCG DRB |
|  | TRUE | Connectivity(NR), Test loop function(On) N DRBs (N ≥ 2) | Default DRB of the first PDU session on NR Cell |
|  |  | Connectivity(NR-DC), DC bearer(N MN Terminated MCG bearers and One SN Terminated SCG bearer), N DRBs (N ≥ 2) Test loop function(On) | SN terminated SCG DRB |
| ELSE IF pc_EN_DC | FALSE | Connectivity(EN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |
| ELSE IF pc_NGEN_DC | FALSE | Connectivity(NGEN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |

Table 7.1.3.0-3: Message conditions

| Execution Condition | Message condition exceptions |
| --- | --- |
| IF pc_NG_RAN_NR | Message with condition AM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF pc_EN_DC | Message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF pc_NGEN_DC | Message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |

<End of Change>