3GPP TSG-RAN WG5 Meeting #93-e	R5-216753

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0060 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.579-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - MCData needs further external function for AEAD Decryption. - According to TS 24.379 [9] clause 13.2.2 and clause 13.3.4.2 the UE shall use for location reporting the URI of the participating function which has done the location reporting configuration. This URI does not necessarily need to be the same as the common MCPTT server URI. - Several PIXITs are not service specific but common for MCX. - Several PIXITs for MCVideo and MCData are missing. - Several PIXITs are not needed anymore. - For MCPTT px_MCPTT_PublicServiceId_A has been replaced by tsc_MCPTT_PublicServiceId_A; similar changes are required for MCVideo and MCData. - For release 16 location procedures (clause 17) have been added to 24.282. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New external function fx_AEAD_Decrypt added. - New PIXIT tsc_MCPTT_PublicServiceId_PF_A to be used in 36.579-1. - MCPTT independent PIXITs renamed for MCX. - Missing PIXITS for MCVideo and MCData added. - Unneeded PIXITs removed. - px_MCVideo_PublicServiceId_A replaced by tsc_MCVideo_PublicServiceId_A; px_MCData_PublicServiceId_A replaced by tsc_MCData_PublicServiceId_A. - XML schema for mcdatalocationinfo added for completeness. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid or incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2, 9, D.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

# 9	IXIT Proforma

## 9.1	General

This partial IXIT proforma contained in the present document is provided for completion, when the related Abstract Test Suite is to be used against the Implementation Under Test (IUT).

Text in italics is a comment for guidance for the production of an IXIT, and is not to be included in the actual IXIT.

The completed partial IXIT will normally be used in conjunction with the completed ICS, as it adds precision to the information provided by the ICS.

## 9.2	MCX

### 9.2.1	MCX Client PIXIT

Table 9.2.1-1: MCX Client Common PIXIT

| Parameter Name |  | Parameter Type |  | Default Value |  | Supported Values |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Client relevant IXIT |  |  |  |  |  |  |  |  |  |
| px_MCX_Client_B_ID |  | charstring |  | "urn:uuid:cdcd34fb-c2f1-4488-a03a-6b5a5d360c47" |  |  |  | MCX client ID for a remote user: As defined in TS 24.379 [9] clause 4.10 it is a UUID URN according to RFC 4122 [62], i.e. has the format "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" |  |
| Users relevant IXIT |  |  |  |  |  |  |  |  |  |
| px_MCPTT_ID_User_A |  | charstring |  | "mcptt-user-A-id@ mcptt-op.gov" |  | NOTE 1 |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |
| px_MCVideo_ID_User_A |  | charstring |  |  |  | NOTE 1 |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |
| px_MCData_ID_User_A |  | charstring |  |  |  | NOTE 1 |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |
| px_MCPTT_User_A_Alias |  | charstring |  | "mcptt-user-A-alias" |  |  |  | Alphanumeric alias of MCPTT user. Ref. TS 24.483 [13]. |  |
| px_MCVideo_User_A_Alias |  | charstring |  | "mcvideo-user-A-alias" |  |  |  | Alphanumeric alias of MCVideo user. Ref. TS 24.483 [13]. |  |
| px_MCData_User_A_Alias |  | charstring |  | "mcdata-user-A-alias" |  |  |  | Alphanumeric alias of MCData user. Ref. TS 24.483 [13]. |  |
| px_MCPTT_ID_FA_A |  | charstring |  | "mcptt-FA-A-id@ mcptt-op.gov" |  |  |  | The value is a "uri" attribute. TS 24.483 [13] clause 5.2.48W6 |  |
| px_MCX_User_A_ParticipantType |  | charstring |  | "first responder" |  |  |  | Participant type of the MCPTT user. Ref. TS 24.483 [13]. |  |
| px_MCX_DomainName_Organization_A |  | charstring |  | "mcptt-op.gov" |  |  |  | Domain name of the organization the user belongs to. |  |
| px_MCX_User_A_username |  | charstring |  | "MCPTT#U01" |  |  |  | UE's User username used for user authentication |  |
| px_MCX_User_A_password |  | charstring |  | "psw@MCPTT&7" |  |  |  | UE's User password used for user authentication |  |
| px_MCX_TokenRsaPrivateKey |  | octetstring |  |  |  |  |  | Private key being used to create signature for ID and Access Token |  |
| px_MCX_TrK |  | bitstring |  |  |  |  |  | Transport key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |
| px_MCX_TrK_ID |  | B32_Type |  |  |  |  |  | Transport key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |
| px_MCX_InK |  | bitstring |  |  |  |  |  | Integrity protection key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |
| px_MCX_InK_ID |  | B32_Type |  |  |  |  |  | Integrity protection key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |
| px_MCPTT_ID_User_B |  | charstring |  | "mcptt-user-B-id@ mcptt-op.gov" |  |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |
| px_MCVideo_ID_User_B |  | charstring |  | "mcvideo-user-B-id@ mcptt-op.gov" |  |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |
| px_MCData_ID_User_B |  | charstring |  | "mcdata-user-B-id@ mcptt-op.gov" |  |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |
| px_MCX_User_B_ParticipantType |  | charstring |  | "first responder" |  |  |  | Participant type of the MCX user. Ref. TS 24.483 [13]. |  |
| px_MCPTT_ID_User_C |  | charstring |  | "mcptt-user-C-id@ mcptt-op.gov" |  |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |
| px_MCVideo_ID_User_C |  | charstring |  | "mcvideo-user-C-id@ mcptt-op.gov" |  |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |
| px_MCData_ID_User_C |  | charstring |  | "mcdata-user-C-id@ mcptt-op.gov" |  |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |
| px_MCX_User_C_ParticipantType |  | charstring |  | "first responder" |  |  |  | Participant type of the MCX user. Ref. TS 24.483 [13]. |  |
| px_MCPTT_ID_User_D |  | charstring |  | "mcptt-user-D-id@ mcptt-op.gov" |  |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |
| px_MCX_SIP_PublicUserId_A_1 |  | charstring |  |  |  |  |  | (First) public user id of the SIP subscriber (user A) as stored in the HSS and on the ISIM, if the UE has an ISIM; the parameter shall be a SIP URI |  |
| px_MCX_SIP_PrivateUserId_A |  | charstring |  |  |  |  |  | Private user id of the SIP subscriber (user A) as stored in the HSS and on the ISIM, if the UE has an ISIM |  |
| px_MCX_SIP_HomeDomain_A |  | charstring |  |  |  |  |  | Home domain name of the SIP subscriber (user A) as stored in the HSS and on the ISIM, if the UE has an ISIM |  |
| px_MCX_SIP_RegistrationWithTemporaryIdentities |  | boolean |  |  |  |  |  | If true the UE shall derive the public user id, private user id and home domain name from the IMSI for SIP registration |  |
| Groups relevant IXIT |  |  |  |  |  |  |  |  |  |
| px_MCPTT_Group_A_ID |  | charstring |  | "mcptt-group-A@mcptt-op.gov" |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |
| px_MCVideo_Group_A_ID |  | charstring |  | "mcvideo-group-A@mcptt-op.gov" |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |
| px_MCData_Group_A_ID |  | charstring |  | "mcdata-group-A@mcptt-op.gov" |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |
| px_MCPTT_Group_A_Name |  | charstring |  | "mcptt-group-A-name" |  |  |  | A human readable Group name for the group |  |
| px_MCVideo_Group_A_Name |  | charstring |  | "mcvideo-group-A-name" |  |  |  | A human readable Group name for the group |  |
| px_MCData_Group_A_Name |  | charstring |  | "mcdata-group-A-name" |  |  |  | A human readable Group name for the group |  |
| px_MCX_Group_A_Owner_Organization |  | charstring |  | "mcx-op.gov" |  |  |  | Indicates the group's owner organization the group belongs to. Ref. TS 24.483 [13]. |  |
| px_MCPTT_Group_A_preferred_VCodec |  | charstring |  | "AMR-WB" |  |  |  | Preferred voice codec for the group (a RTP payload). MCPTT clients shall support the AMR-WB codec. RFC 4566 [25] TS 26.171 [26] |  |
| px_MCPTT_Group_B_ID |  | charstring |  | "mcptt-group-B@mcptt-op.gov" |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |
| px_MCPTT_Group_B_Name |  | charstring |  | "mcptt-group-B-name" |  |  |  | A human readable Group name for the group |  |
| px_MCPTT_Group_T_ID |  | charstring |  | "mcptt-group-T@mcptt-op.gov" |  |  |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |
| Miscellaneous IXIT |  |  |  |  |  |  |  |  |  |
| px_MCPTT_IP_ConnectionAddressAll |  | charstring |  | "0.0.0.0" |  |  |  | The unicast IP address |  |
| px_MCX_APN |  | charstring |  | "mcx-apn" |  |  |  | A single APN which the UE shall use to access each and all MCX relevant services. The APN is provided in the initial UE configuration as specified in TS 36.579-1 [2] Table 5.5.8.1-1. |  |
| px_MCX_InitialRegistration_TypeOfPDN1 |  | MCX_Registration_PDN_Type |  | mcx |  | ims, internet, mcx |  | First PDN registered during initial registration (either ‘ims’ or ‘internet’ or ‘mcx’; ‘none’ is not applicable as first PDN) |  |
| px_MCX_InitialRegistration_TypeOfPDN2 |  | MCX_Registration_PDN_Type |  | none |  | ims, internet, mcx, none |  | Second PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no second PDN connectivity requested by the UE during initial registration |  |
| px_MCX_InitialRegistration_TypeOfPDN3 |  | MCX_Registration_PDN_Type |  | none |  | ims, internet, mcx, none |  | Third PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no third PDN connectivity requested by the UE during initial registration |  |
| px_MCX_CoordinateLatitude_Client_B |  | float |  | none |  |  |  | Simulated latitude value sent to the UE to inform about Client_B location |  |
| px_MCX_CoordinateLongitude_Client_B |  | float |  | none |  |  |  | Simulated longitude value sent to the UE to inform about Client_B location |  |
| NOTE 1: According to TS 23.280 [54] clause 8.1.2 a MC service ID shall be a URI; nevertheless in context of this specification only URIs consisting of userinfo, host and path elements (see RFC 3986 [53]) are supported. Editor's note: A similar note may need to be added to other PIXITs too |  |  |  |  |  |  |  |  |  |

### 9.2.2	MCX Server PIXIT

Table 9.2.2-1: MCX Server Common PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_MCX_GroupCreationXUI | charstring | "mcx-gms@mcptt-op.gov" |  | Indicates the group creation XUI information for creation of groups. Ref. TS 23.003 [21]. |
| px_MCX_IdMS_auth_IPAddress | charstring |  |  | IPv4/IPv6 address of the IdMS authorization endpoint |
| px_MCX_IdMS_auth_Port | integer |  |  | Port number of the IdMS authorization endpoint |
| px_MCX_IdMS_auth_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the IdMS authorization endpoint |
| px_MCX_IdMS_token_IPAddress | charstring |  |  | IPv4/IPv6 address of the IdMS token endpoint |
| px_MCX_IdMS_token_Port | integer |  |  | Port number of the IdMS token endpoint |
| px_MCX_IdMS_token_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the IdMS token endpoint |
| px_MCX_HTTP_Proxy_IPAddress | charstring |  |  | IPv4/IPv6 address of the HTTP proxy |
| px_MCX_HTTP_Proxy_Port | integer |  |  | Port number of the HTTP proxy |
| px_MCX_HTTP_Proxy_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the HTTP proxy |
| px_MCX_InitialConfigServer_IPAddress | charstring |  |  | IPv4/IPv6 address of the server optionally being used to download the Initial UE Configuration document |
| px_MCX_InitialConfigServer_Port | integer |  |  | Port number of the server optionally being used to download the Initial UE Configuration document |
| px_MCX_InitialConfigServer_UriPath | charstring | "/cms/initial-ue-config" |  | URI Path component: Absolute path used for HTTP requests addressing a server to download the UE initial configuration document |
| px_MCX_TLS_CipherSuite | TLS_CIPHER_Type | TLS_RSA_WITH_NULL_SHA |  | Cipher suite to be used for TLS connections |
| px_MCX_OAuth_ClientId_A | charstring |  |  | Client ID of the UE's MCX application as used in OAuth signalling with the IdMS |
| px_MCX_OAuth_RedirectURI_A | charstring |  |  | Redirect URI used by the UE implementation (user agent, MCX client and OS) to redirect the OAuth authentication response to the MCX client application |

### 9.2.3	MCX Constant PIXIT Definitions

Several parameters for MCX conformance testing can be defined as constants as they are neither preconfigured at the UE nor at the SS. Table 9.2.3-1 lists these constants.

Table 9.2.3-1: MCX Constant PIXIT Definitions

| Constant Name | Constant Type | Value | Description |
| --- | --- | --- | --- |
| tsc_MCX_KMS_Hostname | charstring | "kms." & px_MCX_DomainName_Organization_A | FQDN of the KMS; used in initial UE configuration as domain name for the 'kms' URI in the App-Server-Info. |
| tsc_MCX_CMS_Hostname | charstring | "cms." & px_MCX_DomainName_Organization_A | FQDN of the CMS; used in initial UE configuration as domain name for the 'cms' URI in the App-Server-Info. |
| tsc_MCX_GMS_Hostname | charstring | "gms." & px_MCX_DomainName_Organization_A | FQDN of the GMS; used in initial UE configuration as domain name for the 'gms' URI in the App-Server-Info. |
| tsc_MCX_KMS_CertUri | charstring | "certificate1." & tsc_MCX_KMS_Hostname | Name of the KMS certificate sent to the UE during MCX user authentication |
| tsc_MCX_IdMS_auth_UriPath | charstring | "/idms/auth" | URI path to address the IdMS authorization endpoint |
| tsc_MCX_IdMS_token_UriPath | charstring | "/idms/token" | URI path to address the IdMS token endpoint |
| tsc_MCX_IdMS_userauth_UriPath | charstring | "/idms/userauth" | relative URI at IdMS used for user authentication |
| tsc_MCX_KMS_init_UriPath | charstring | "/keymanagement/identity/v1/init" | Request-URI for "KMS Initialize" request according to TS 33.180 [43] D.2.3 |
| tsc_MCX_KMS_keyprov_UriPath | charstring | "/keymanagement/identity/v1/keyprov" | Request-URI for "KMS KeyProvision" request according to TS 33.180 [43] D.2.4 |
| tsc_MCX_KMS_ClientReqUrl_init | charstring | "https://" & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_init_UriPath | used as <ClientReqUrl> entry in the KMS Key Set |
| tsc_MCX_KMS_ClientReqUrl | charstring | "https://" & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_keyprov_UriPath | used as <ClientReqUrl> entry in the KMS Key Set |
| tsc_MCX_GMSURI | charstring | "subscription-proxy." & tsc_MCX_GMS_Hostname | used for <GMS-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9; sub-domain of the GMS's hostname |
| tsc_MCX_CMSXCAPRootURI | charstring | "http://xcap." & tsc_MCX_CMS_Hostname | used for <CMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9C |
| tsc_MCX_GMSXCAPRootURI | charstring | "http://xcap." & tsc_MCX_GMS_Hostname | used for <GMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9B |
| tsc_MCX_MC_ID_User_A | charstring | px_MCX_User_A_username | According to TS 24.482 [12] clause 6.3.1 the MC ID set to the user name |
| tsc_MCPTT_PublicServiceId_A | charstring | "sip:" & "mcptt-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCPTT Server which is simulated by the SS |
| tsc_MCVideo_PublicServiceId_A | charstring | "sip:" & "mcvideo-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCVideo Server which is simulated by the SS |
| tsc_MCData_PublicServiceId_A | charstring | "sip:" & "mcdata-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCData Server which is simulated by the SS |
| tsc_MCPTT_PublicServiceId_B | charstring | "sip:" & "mcptt-server-B@" & px_MCX_DomainName_Organization_A | The URI of a second MCPTT Server which is implemented in the DUT used in MCPTT Server testing. |
| tsc_MCPTT_PublicServiceId_PF_A | charstring | "sip:" & "participating-mcptt-function-A@" & px_MCX_DomainName_Organization_A | The URI of the participating MCPTT function which configures the location reporting at the UE according to TS 24.379 [9] clause 13.2.2. |
| tsc_MCX_SessionID_A | charstring | "sip:" & "sessionA@cf." & px_MCX_DomainName_Organization_A" | The URI of the MCPTT session A identity. Ref. TS 24.483 [13]. SIP URI according to TS 24.379 [9] clause 4.5. |
| tsc_MCX_SessionID_B | charstring | "sip:" & "sessionB@cf." & px_MCX_DomainName_Organization_A | The URI of the MCPTT session B identity. Ref. TS 24.483 [13]. SIP URI according to TS 24.379 [9] clause 4.5. |
| tsc_MCPTT_Group_A_ProSeLayer2GroupID | octetstring | '00000A'O | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |
| tsc_MCPTT_Group_D_ProSeLayer2GroupID | octetstring | '00000D'O | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |

## 9.3	MCX IPCAN test model

Table 9.3-1: MCX IPCAN test model specific PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_MCX_IPCAN_EpsBearerId_IMS | MCX_IPCAN_EpsBearerId_Type | 5 | 5..13 | Default EPS bearer for IMS PDN (if any) |
| px_MCX_IPCAN_EpsBearerId_INTERNET | MCX_IPCAN_EpsBearerId_Type | 6 | 5..13 | Default EPS bearer for Internet PDN (if any) |
| px_MCX_IPCAN_EpsBearerId_MCX | MCX_IPCAN_EpsBearerId_Type | 7 | 5..13 | Default EPS bearer for MCX PDN |
| px_MCX_IPCAN_EpsBearerId_MCX_Audio | MCX_IPCAN_EpsBearerId_Type | 8 | 5..13 | Dedicated EPS bearer for MCX audio |
| px_MCX_IPCAN_EpsBearerId_MCX_Video | MCX_IPCAN_EpsBearerId_Type | 9 | 5..13 | Dedicated EPS bearer for MCX video |

<End of modification>

<Start of modification>

## 8.2	External function definitions

The external functions specified in TS 36.523-3 [27] clause 8.2 apply to the present document.

In addition there are the following MCX specific external functions:

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SAKKE_GeneratePublicKey |  |
| Description | Generate KMS public key (Z_T) for SAKKE (RFC 6508 [45] clause 2.2): Z_T := [z_T]P |  |
| Parameters | p_MasterSecret | master secret z_T (octetstring) |
|  | p_ParameterSet | parameter set to be used; 1 per default, indicating use of parameter set 1 as defined in appendix A of RFC 6509 [46] |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SAKKE_GenerateRSK |  |
| Description | Generate receiver secret key (RSK) for SAKKE (RFC 6508 [45] clause 2.2): RSK := [(a + z_T)^-1]P with 'a' being the identifier (UID) corresponding to the receiver's URI |  |
| Parameters | p_MasterSecret | master secret z_T (octetstring) |
|  | p_Identifier | UID for a given URI (octetstring) |
|  | p_ParameterSet | parameter set to be used; 1 per default, indicating use of parameter set 1 as defined in appendix A of RFC 6509 [46] |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SAKKE_EncapsulateKey |  |
| Description | Generate encapsulated data for SAKKE exchange according to RFC 6508 [45] |  |
| Parameters | p_SSV | Shared secret value (octetstring): Key to be exchanged; according to 33.180 [43] E.1.1: The GMK, PCK, CSK and MuSiK shall be 16 octets in length |
|  | p_SakkePublicKey | SAKKE public key (octetstring) generated with fx_SAKKE_GeneratePublicKey |
|  | p_UID | UID (octetstring) generated for the receiving entity's URI (in general the same URI as in IDRr payload of the MIKEY message carrying the encapsulated data) |
|  | p_ParameterSet | parameter set to be used; 1 per default, indicating use of parameter set 1 as defined in appendix A of RFC 6509 [46] |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SAKKE_ExtractKey |  |
| Description | Extract and validate 16 octet key from the encapsulated date for SAKKE exchange according to RFC 6508 [45] clause 6.2.2: if validation fails (TEST does not equal R_(b,S)) then omit shall be returned |  |
| Parameters | p_EncapsulatedData | encapsulated data (octetstring) as received in the SAKKE payload of a MIKEY message |
|  | p_SakkeRSK | receiver secret key (RSK) for SAKKE (octetstring) |
|  | p_SakkePublicKey | SAKKE public key (octetstring) generated with fx_SAKKE_GeneratePublicKey |
|  | p_UID | UID (octetstring) generated for the receiving entity's URI (should be the same URI as in IDRr payload of the MIKEY message carrying the encapsulated data |
|  | p_ParameterSet | parameter set to be used; 1 per default, indicating use of parameter set 1 as defined in appendix A of RFC 6509 [46] |
| Return Value | template (omit) octetstring (16 octets if the key valid, omit otherwise) |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_ECCSI_GenerateKPAK |  |
| Description | Generate KMS Public Authentication Key (KPAK) for ECCSI (RFC 6507 [44] clause 4.2): KPAK := [KSAK]G The P-256 elliptic curve, base point and SHA-256 function as according to RFC 6509 [46] clause 2.1.1 shall be used. |  |
| Parameters | p_KSAK | KMS Secret Authentication Key (KSAK): random secret non-zero integer modulo q (octetstring) |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_ECCSI_GenerateSskPvtPair |  |
| Description | Generate (SSK,PVT) Pair according to clause 5.1.1 of RFC 6507 [44]; if either the SSK or HS is zero modulo q (step 5 of the algorithm), the function shall return omit The P-256 elliptic curve, base point and SHA-256 function as according to RFC 6509 [46] clause 2.1.1 shall be used. |  |
| Parameters | p_UID | User ID (octetstring) |
|  | p_KSAK | KMS Secret Authentication Key (KSAK) (octetstring) |
|  | p_KPAK | KMS Public Authentication Key (KPAK) (octetstring) |
|  | p_EphemeralValue | random (ephemeral) non-zero integer value v according to RFC 6507 [44] clause 5.1.1 |
| Return Value | template (omit) type record ECCSI_SskPvtPair_Type { octetstring SSK, octetstring PVT } |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_ECCSI_SignMessage |  |
| Description | Sign a message according to RFC 6507 [44] clause 5.2.1: return signature of the message or omit if HE + r * SSK is non-zero modulo q (step 4 of the algorithm) The P-256 elliptic curve, base point and SHA-256 function as according to RFC 6509 [46] clause 2.1.1 shall be used. |  |
| Parameters | p_Message | Message to be signed (octetstring) |
|  | p_KPAK | KMS Public Authentication Key (KPAK) (octetstring) |
|  | p_UID | Signer's User ID (octetstring) |
|  | p_SSK | Secret Signing Key (octetstring) |
|  | p_PVT | Public Validation Token (octetstring) |
|  | p_EphemeralValue | random (ephemeral) non-zero integer value j according to RFC 6507 [44] clause 5.2.1 |
| Return Value | template (omit) octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_ECCSI_VerifySignature |  |
| Description | Verify a signature according to RFC 6507 [44] clause 5.2.2: return true, when the signature is valid, false otherwise The P-256 elliptic curve, base point and SHA-256 function as according to RFC 6509 [46] clause 2.1.1 shall be used. |  |
| Parameters | p_Message | Message (octetstring) |
|  | p_Signature | Message's signature (octetstring) |
|  | p_KPAK | KMS Public Authentication Key (KPAK) (octetstring) |
|  | p_UID | Signer's User ID (octetstring) |
| Return Value | boolean |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_Encrypt |  |
| Description | Encrypt data NOTE: the function is defined similar to openssl_encrypt and in principle it is independent from XML; nevertheless it is used for XML encrytion in context of MCX |  |
| Parameters | p_Data | (octetstring) |
|  | p_Method | type enumerated XML_EncryptionMethod_Type { AES_128_GCM, // AES-GCM according to clause 5.2.4 of // https://www.w3.org/TR/xmlenc-core1/ [41] // with 96 bit Initialization Vector and //128 bit Authentication Tag AES_256_KEY_WRAP // according to RFC 3394 [47] } |
|  | p_Key | key for encryption (bitstring) |
|  | p_IV | initial vector (octetstring) |
| Return Value | octetstring | In case of an authenticated encryption mechanism (e.g. AES_128_GCM) the Authentication Tag is added at the end of the encrypted data. |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_Decrypt |  |
| Description | Decrypt data |  |
| Parameters | p_Data | In case of an authenticated encryption mechanism (e.g. AES_128_GCM) the input data is the concatination of the encrypted data and Authentication Tag (octetstring) |
|  | p_Method | type enumerated XML_EncryptionMethod_Type { AES_128_GCM, // AES-GCM according to clause 5.2.4 of // https://www.w3.org/TR/xmlenc-core1/ [41] // with 96 bit Initialization Vector and //128 bit Authentication Tag AES_256_KEY_WRAP // according to RFC 3394 [47] } |
|  | p_Key | key for encryption (bitstring) |
|  | p_IV | initial vector (octetstring) |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_MCX_XML_AddEnvelopedSignature |  |
| Description | Add XML signature to the given XML document and return resulting XML document; according to W3C: "XML Signature Syntax and Processing (Second Edition)" [48]: 1. The given document has a Signature element with the name of the key to be used to sign the Signature's SignedInfo 2. Elements(s) to be signed: For elements which are addressed by a reference URI in the Signature's SignedInfo the DigestValue shall be generated and added to the corresponding Reference element of the Signature's SignedInfo 3. The SignedInfo shall be signed by generating the hash for the Signature's SignedInfo using the given key; this hash value shall be added to the Signature's SignatureValue. |  |
| Parameters | p_XmlDocument | XML document (charstring) to be signed; the document contains all information to get signed with the given key: - id(s) for the object(s) to be signed (e.g. KMS response) - SignedInfo with reference(s) to objects to be signed within the document ("same-document" reference(s) according to RFC 3986 [53] clause 4.4), DigestAlgorithm, empty DigestValue |
|  | p_Key | Key corresponding to the KeyName in the Signature's KeyInfo element (bitstring) |
| Return Value | universal charstring containing the document with all DigestValues and the signature of the SignedInfo |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_MCX_XML_CreateDetachedSignature |  |
| Description | Create XML signature for the given XML document and return resulting (encoded) XML signature (detached signature); according to TS 33.180 [43] clause 9.3.5, TS 24.379 [9] clause 6.6.3.1 and W3C: "XML Signature Syntax and Processing (Second Edition)" [48]: p_SignatureInfo contains the parameters needed to create the signature and to sign the document; the whole document gets signed irrespective of the Reference URI |  |
| Parameters | p_XmlDocument | XML document to be signed |
|  | p_XmldsigDocument | XML xmldsig document with signature element containing all information to sign the document (p_XmlDocument) |
|  | p_Key | Key corresponding to the KeyName in the Signature's KeyInfo element |
| Return Value | universal charstring | XML xmldsig document with DigestValue and SignatureValue added to p_XmldsigDocument |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_GetElementContent |  |
| Description | Return content of the node in an XML tree or XML branch as pointed at by the xpath expression |  |
| Parameters | p_XmlBranch | XML tree (including the root element) or branch ot an XML tree |
|  | p_XpathExpression | XCAP expression according to https://www.w3.org/TR/1999/REC-xpath-19991116/ |
| Return Value | universal charstring | string containing the content of the element or empty string if p_XpathExpression does not address an existing element of the given XML branch NOTE: if the element may be empty, it may be necessary to check existence of the element with fx_XML_GetElement first |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_SetElementContent |  |
| Description | Return XML tree or XML branch with replaced content of the node pointed at by the xpath expression |  |
| Parameters | p_XmlBranch | XML tree (including the root element) or branch ot an XML tree |
|  | p_XpathExpression | XCAP expression according to https://www.w3.org/TR/1999/REC-xpath-19991116/ |
|  | p_NewContent | new content for the given node |
| Return Value | universal charstring | modified XML tree or XML branch |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SHA_2 |  |
| Description | Generic SHA-2 function |  |
| Parameters | p_Function | type enumerated HASH_Function_Type { SHA_256 // may be extended e.g SHA_224 etc. } |
|  | p_Data | data (octetstring) |
| Return Value | octetstring (representing 256 bits for SHA-256, 224 bits for SHA-224, ...) |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_JWK_Signature |  |
| Description | Generate JWK signature according to RFC 7515 [49] |  |
| Parameters | p_String | string for which the signature shall be generated (charstring) |
|  | p_Algorithm | algorithm to generate the hash: type enumerated JWK_HashAlgorithm_Type { // RFC 7515 [49] HS256, // HMAC SHA-256 RS256 // RSASSA-PKCS1-v1_5 SHA-256 } |
|  | p_Key | key (octetstring) |
| Return Value | charstring (base64url encoded signature according to RFC 7515 [49]) |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_Log |  |
| Description | Log function for (unencrypted) XML documents |  |
| Parameters | p_FileName | TTCN module from where the function is called |
|  | p_LineNo | Line number from where the function is called |
|  | p_UplinkDownlink | XML document has been received (UPLINK) or will be sent (DOWNLINK) |
|  | p_XmlDocument | XML document |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_AEAD_Encrypt |  |
| Description | Encrypt data using AEAD algorithms according to RFC 5116 |  |
| Parameters | p_Data | (octetstring) |
|  | p_AssociatedData | (octetstring) |
|  | p_Method | type enumerated AEAD_EncryptionMethod_Type { AEAD_AES_128_GCM, // RFC 5116 clause 5.1 AEAD_AES_256_GCM // RFC 5116 clause 5.2 } |
|  | p_Key | key for encryption (bitstring) |
|  | p_IV | initial vector (octetstring) |
| Return Value | octetstring | Encrypted data appended by authentication tag |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_AEAD_Decrypt |  |
| Description | Decrypt data using AEAD algorithms according to RFC 5116 |  |
| Parameters | p_Data | (octetstring) |
|  | p_AssociatedData | (octetstring) |
|  | p_Method | type enumerated AEAD_EncryptionMethod_Type { AEAD_AES_128_GCM, // RFC 5116 clause 5.1 AEAD_AES_256_GCM // RFC 5116 clause 5.2 } |
|  | p_Key | key for decryption (bitstring) |
|  | p_IV | initial vector (octetstring) |
| Return Value | octetstring | Decrypted data |

<End of modification>

<Start of modification>

# D.1	XML Schema Definitions (XSD)

Common XML schema definitions according to TS 34.229-3 [28] Table G.0.1-1 are used. In addition there are the MCX specific XML schema definitions as according to table D.1-1.

Table D.1-1: MCX specific definitions

| XML Schema (XSD) | Source | Name space |
| --- | --- | --- |
| RFC4354-poc-settings | RFC 4354 [52] clause 6.1 | urn:oma:params:xml:ns:poc:poc-settings |
| RFC4661-SimpleFilter | RFC 4661 [34] clause 7 | urn:ietf:params:xml:ns:simple-filter |
| RFC5261-patch-ops (NOTE 2) | RFC 5261 [50] clause 8 | (NOTE 1) |
| RFC5874-xcap-diff | RFC 5874 [51] clause 4 | urn:ietf:params:xml:ns:xcap-diff |
| IANA-resource-lists | RFC 4826 [34] clause 3.2 https://www.iana.org/assignments/xml-registry/schema/resource-lists.xsd | urn:ietf:params:xml:ns:resource-lists |
| poc_listService-v1_0 | OMA [37] | urn:oma:xml:poc:list-service |
| xdm_extensions-v1_0 | OMA [39] | urn:oma:xml:xdm:extensions |
| xdm_rsrclst_uriusage-v1_0 | OMA [40] | urn:oma:xml:xdm:resource-list:oma-uriusage |
| xenc-schema | W3C [41] | http://www.w3.org/2001/04/xmlenc#' |
| xmldsig-core-schema | W3C [42] | http://www.w3.org/2000/09/xmldsig# |
| TS24281_mcvideoinfo (NOTE 3) | TS 24.281 [55] Annex F.1.2 | urn:3gpp:ns:mcvideoInfo:1.0 |
| TS24281_mcvideolocationinfo | TS 24.281 [55] Annex F.3.2 | urn:3gpp:ns:mcvideoLocationInfo:1.0 |
| TS24281_mcvideoPresInfo | TS 24.281 [55] Table 8.3.1.2-1 | urn:3gpp:ns:mcvideoPresInfo:1.0 |
| TS24282_mcdatainfo.xsd | TS 24.282 [57] Annex D.1.2 | "urn:3gpp:ns:mcdataInfo:1.0" |
| TS24282_mcdatalocationinfo | TS 24.282 [57] Annex D.4.2 | urn:3gpp:ns:mcdataLocationInfo:1.0 |
| TS24282_mcdataPresInfo | TS 24.282 [57] Table 8.4.1.2-1 | "urn:3gpp:ns:mcdataPresInfo:1.0" |
| TS24379_mcpttaff (NOTE 4) | TS 24.379 [9] Annex F.4.2 TS 24.281 [55] Annex F.4.2 TS 24.282 [57] Annex D.3.2 | urn:3gpp:ns:affiliationCommand:1.0 |
| TS24379_mcpttinfo | TS 24.379 [9] Annex F.1.2 | urn:3gpp:ns:mcpttInfo:1.0 |
| TS24379_mcpttlocationinfo | TS 24.379 [9] Annex F.3.2 | urn:3gpp:ns:mcpttLocationInfo:1.0 |
| TS24379_mcpttsigneddoc | TS 24.379 [9] Annex F.6.2 | urn:3gpp:ns:mcpttSignedDoc:1.0 |
| TS24379_mcpttPresInfo | TS 24.379 [9] Table 9.3.1.2-1 | urn:3gpp:ns:mcpttPresInfo:1.0 |
| TS24379_mcpttPresInfoFA | TS 24.379 [9] Table 9A.3.1.2-1 | urn:3gpp:ns:mcpttPresInfoFA:1.0 |
| TS24379_poc-settings (NOTE 5) | TS 24.379 [9] Table 7.4.1.2.2-2 TS 24.281 [55] Table 7.4.1.2.2-2 | urn:3gpp:mcsSettings:1.0 |
| TS24481-mcptt-group | TS 24.481 [11] clause 7.2.4.2 | urn:3gpp:ns:mcpttGroupInfo:1.0 |
| TS24481-mcpttGMOP | TS 24.481 [11] clause 7.3.3 | urn:3gpp:ns:mcpttGMOP:1.0 |
| TS24481-mcpttGKTP | TS 24.481 [11] clause 7.7.4.2 | urn:3gpp:ns:mcpttGKTP:1.0 |
| TS24484-ue-init-config | TS 24.484 [14] clause 7.2.2.3 | urn:3gpp:mcptt:mcpttUEinitConfig:1.0 |
| TS24484- mcptt-ue-config | TS 24.484 [14] clause 8.2.2.3 | urn:3gpp:mcptt:mcpttUEConfig:1.0 |
| TS24484-mcvideo-ue-config | TS 24.484 [14] clause 9.2.2.3 | urn:3gpp:mcvideo:mcvideoUEConfig:1.0 |
| TS24484-mcdata-ue-config | TS 24.484 [14] clause 10.2.2.3 | urn:3gpp:mcdata:mcdataUEConfig:1.0 |
| TS24484-mcptt-user-profile | TS 24.484 [14] clause 8.3.2.3 | urn:3gpp:mcptt:user-profile:1.0 |
| TS24484-mcvideo-user-profile | TS 24.484 [14] clause 9.3.2.3 | urn:3gpp:ns:mcvideo:user-profile:1.0 |
| TS24484-mcdata-user-profile | TS 24.484 [14] clause 10.3.2.3 | urn:3gpp:ns:mcdata:user-profile:1.0 |
| TS24484-mcptt-service-config | TS 24.484 [14] clause 8.4.2.3 | urn:3gpp:ns:mcpttServiceConfig:1.0 |
| TS24484-mcvideo-service-config | TS 24.484 [14] clause 9.4.2.3 | urn:3gpp:ns:mcvideoServiceConfig:1.0 |
| TS24484-mcdata-service-config | TS 24.484 [14] clause 10.4.2.3 | urn:3gpp:ns:mcdataServiceConfig:1.0 |
| TS33180-mcsecKMSInterface | TS 33.180 [43] Annex D.3.5.1 | urn:3gpp:ns:mcsecKMSInterface:1.0 |
| TS33180-mcsecKMSKRR | TS 33.180 [43] Annex D.4.4 | urn:3gpp:ns:mcsecKMSKRR:1.0 |
| NOTE 1: The schema does not define any target namespace but is referred to as "urn:ietf:params:xml:schema:patch-ops" e.g. in the original include statement in urn:ietf:params:xml:ns:xcap-diff. NOTE 2: The XML schema in RFC 5261 [50] clause 8 is specified as DTD with ENTITY declarations being referred by the pattern of the schema. These references are resolved in RFC5261-patch-ops in order to get schema definitions which can be processed by TTCN-3 compilers. NOTE 3: The schema specifies import of non-existing namespace urn:3gpp:ns:mcvideoGKTP:1.0. Assumption: urn:3gpp:ns:mcpttGKTP:1.0 to be imported instead NOTE 4: The schemata in TS 24.379 [9] F.4.2, TS 24.281 [55] F.4.2 and TS 24.282 [57] Annex D.1.2 are effectively the same apart from the namespace prefix ("mcpttaff" vs. "mcvideoaff" vs. "mcdataaff"). Assumption: The namespace prefix does not matter in XML documents, in general any prefix can be used. TS24379_mcpttaff is used for MCPTT, MCVideo and MCData. NOTE 5: The schemata in TS 24.379 [9] Table 7.4.1.2.2-2 and TS 24.281 [55] Table 7.4.1.2.2-2 are the same. TS24379_poc-settings is used for MCPTT and MCVideo. |  |  |

In order to avoid ambiguities and unexpected side effects due to tool specific behaviour the schemaLocation in xs:import statements of XSD files are modified to use local references rather than references to internet locations.

The schemaLocations of imported namespaces are shown in table D.1-2. The schemaLocations of included namespaces are shown in table D.1-3.

Table D.1-2: schemaLocation of imported namespaces

| XML Schema |  | Imported name space |  | Original schemaLocation |  | Modified schemaLocation |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RFC4354-poc-settings |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| RFC4661-SimpleFilter |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| IANA-resource-lists |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| poc_listService-v1_0 |  | urn:ietf:params:xml:ns:common-policy |  | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd |  | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |  |
| poc_listService-v1_0 |  | urn:ietf:params:xml:ns:resource-lists |  | http://www.iana.org/assignments/xml-registry/schema/resource-lists.xsd |  | IANA-resource-lists.xsd (NOTE 2) |  |
| xdm_extensions-v1_0 |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/03/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| xenc-schema |  | http://www.w3.org/2000/09/xmldsig# |  | xmldsig-core-schema.xsd |  | (no change needed) |  |
| TS24282_mcdatainfo.xsd |  | http://www.w3.org/2001/04/xmlenc# |  | http://www.w3.org/TR/xmlenc-core/xenc-schema.xsd |  | xenc-schema.xsd |  |
| TS24379_mcpttsigneddoc |  | http://www.w3.org/2000/09/xmldsig# |  | http://www.w3.org/TR/xmldsig-core/xmldsig-core-schema.xsd |  | xmldsig-core-schema.xsd |  |
| TS24484-ue-init-config |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| TS24484- mcptt-ue-config |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| TS24484-mcvideo-ue-config |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| TS24484-mcdata-ue-config |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| TS24484-mcptt-user-profile |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| TS24484-mcptt-user-profile |  | urn:ietf:params:xml:ns:common-policy |  | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd |  | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |  |
| TS24484-mcvideo-user-profile |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| TS24484-mcvideo-user-profile |  | urn:ietf:params:xml:ns:common-policy |  | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd |  | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |  |
| TS24484-mcdata-user-profile |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| TS24484-mcdata-user-profile |  | urn:ietf:params:xml:ns:common-policy |  | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd |  | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |  |
| TS24484-mcptt-service-config |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| TS24484-mcdata-service-config |  | http://www.w3.org/XML/1998/namespace |  | http://www.w3.org/2001/xml.xsd |  | ../../Common/IMS_XSD/xml.xsd |  |
| NOTE 1: The namespace urn:ietf:params:xml:ns:common-policy is specified in RFC 4745 and at http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd with difference in the minOccurs for identityType and validityType. Nevertheless for use in TTCN there shall be only one schema as part 9 of the TTCN-3 language [31] does not support different variants of one and the same namespace. In addition according to DIFF1 for RFC 4745 it seems that the RFC's variant is newer than the IANA's variant. NOTE 2: The namespace urn:ietf:params:xml:ns:resource-lists is specified in RFC 4826 and at http://www.iana.org/assignments/xml-registry/schema/resource-lists.xsd with the difference of attribute anchor not being mandatory in the RFC. |  |  |  |  |  |  |  |

Table D.1-3: schemaLocation of included namespaces

| XML Schema | Included schema | Original schemaLocation | Modified schemaLocation |
| --- | --- | --- | --- |
| RFC5874-xcap-diff | urn:ietf:params:xml:schema:patch-ops | urn:ietf:params:xml:schema:patch-ops | RFC5261-patch-ops.xsd |

<End of modification>