3GPP TSG-RAN5 Meeting #93-e	R5-217740

# Electronic Meeting, 8- 19 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1242 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to DC_1A_n79A spurious emission test case 6.5B.3.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test points and test requirement was added for DC_1A_n79A at RAN5#92-e by CRs in R5-214281 (38.521-3) and R5-214246 (38.905) The minimum requirements for DC_1A_n79A was removed by mistake at RAN5#92-e by the CR in R5-216131. There is a need to add the minimum requirements back to Table 6.5B.3.3.2.3-1 and Table 6.5B.3.3.2.3-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added minimum requirement for DC_1A_n79A to Table 6.5B.3.3.2.3-1 and Table 6.5B.3.3.2.3-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tets points and test requirements specified but no minimum requirements specified for DC_1A_n79A. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.5B.3.3.2	Spurious emission band UE co-existence for Inter-band within FR1

Editor's note:	The default and additional test configuration is analysed based on the assumption that only intermodulation products need to be tested.

6.5B.3.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions for band UE co-existence for inter-band EN-DC.

6.5B.3.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward supporting inter-band EN-DC.

6.5B.3.3.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.2.3-1 and Table 6.5B.3.3.2.3-2 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band EN_DC with the uplink assigned to one LTE band and one NR band, the requirements in Table 6.5B.3.3.2.3-1 and Table 6.5B.3.3.2.3-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.2.3-1: Spurious emission band UE co-existence limits Rel-15

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band n3, n34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n78 DC_1_n84_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_2_n5 | E-UTRA Bands 4, 5, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 41, 43, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n66 | E-UTRA Bands 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Bands 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band n71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_2_n78 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n7 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3_n28 | E-UTRA Band 1, 42, 43, 50, 51, 65, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_3_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 DC_3_n80_ULSUP-TDM_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n66 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_5_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2,7 |
| DC_7_n28 | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 42, 43, 50, 65, 66, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_8_n77 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_8_n78 DC_8_n81_ULSUP-TDM_n78 | E-UTRA Band 1, 20, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_11_n77 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n79 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_12_n66 | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 4, 48, 50, 51, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_19_n77 | E-UTRA Band 1, 3, 11, 21, 28, 34, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_20_n28 DC_20_n83 | E-UTRA Band 3, 7, 8, 31, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n78 DC_20_n82_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | FDL_low |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_21_n77 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n79 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_25_n41 | E-UTRA band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 48, 66, 70,71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA/NR Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_26_n41 | E-UTRA/NR Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26_n77 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n78 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 |  | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n79 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n77 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_30_n5 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_38_n78 | N/A |  |  |  |  |  |  |
| DC_39_n79 | E-UTRA Band 1, 8, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |
| DC_40_n77 | N/A |  |  |  |  |  |  |
| DC_41_n77 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 40, 44, 45, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n78 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 40, 42, 44, 45, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_42_n77 | N/A |  |  |  |  |  |  |
| DC_42_n78 | N/A |  |  |  |  |  |  |
| DC_42_n79 | N/A |  |  |  |  |  |  |
| DC_66_n5 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n71 | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7,22, 25, 41, 42, 48, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_66_n78, DC_66_n86_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 of TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. |  |  |  |  |  |  |  |

Table 6.5B.3.3.2.3-2: Spurious emission band UE co-existence limits Rel-16

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_1_n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,17 |  |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 17 |  |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 17 |  |
| DC_1_n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_1_n77 DC_1_n84_ULSUP-TDM_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |
| DC_1_n78 DC_1_n84_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |
| DC_1_n79 DC_1_n84_ULSUP-TDM_n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |
| DC_2_n5 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |
|  | E-UTRA Band 2, 25, 48 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 41, 43, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n41 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 43 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n66 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 42, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25, 41, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band n71 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_2_n78 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_3_n1 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,17 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 17 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 17 |  |
| DC_3_n7 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_3_n28 | E-UTRA Band 1, 42, 43, 50, 51, 65, 74, 75, 76 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |  |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_3_n41, DC_3_n80_ULSUP-TDM_n41 | E-UTRA Band 1, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n77 DC_3_n80_ULSUP-TDM_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n79 DC_3_n80_ULSUP-TDM_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_5_n2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | NR Band n2 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |
|  | E-UTRA Band 41, 43, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_5_n66 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |
|  | E-UTRA Band 41, 42, 48, 52 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_5_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2,7 |  |
| DC_7_n1 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,16 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7,16 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7,16 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n28 | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 4, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_8_n1 | E-UTRA Band 20, 28, 31, 32, 38, 40, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_8_n3 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  | E-UTRA Band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
| DC_8_n40 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_8_n41, DC_8_n81_ULSUP-TDM_n41 | E-UTRA Band 1, 11, 21, 28, 34, 39, 40, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_8_n77 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
| DC_8_n78 DC_8_n81_ULSUP-TDM_n78, DC_8_n81_ULSUP-FDM_n78 | E-UTRA Band 1, 20, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
| DC_11_n77 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_11_n78 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_11_n79 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_12_n66 | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 4, 48, 50, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_12_n78 | E-UTRA Band 2, 5, 7, 13, 17, 25, 26, 41, 71 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 4, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA band 12 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_13_n2 | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2,14, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_13_n66 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 30, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  | Frequency range | 799 | - | 803 | -35 | 0.00625 | 5 |  |
| DC_14_n2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 2, 25 | FDL_low | - | FDL_high | -50 | 1 |  | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 |  | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  | 5 |
| DC_14_n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 48 | FDL_low | - | FDL_high | -50 | 1 |  | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 |  | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  | 5 |
| DC_19_n77 | E-UTRA Band 1, 3, 11, 21, 28, 34, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_19_n78 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_19_n79 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_20_n1 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 34, 40, 43, 50, 51, 65, 67, 68, 72, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 38, 42, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |  |
| DC_20_n3 | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 20 E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 38, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |  |
| DC_20_n28 DC_20_n83 | E-UTRA Band 3, 7, 8, 31, 34 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_20_n28 DC_20_n83 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 34, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_20_n78 DC_20_n82_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_21_n77 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_21_n78 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_21_n79 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_25_n41 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 48, 66, 70,71 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25, | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_26_n41 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
| DC_26_n77 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_26_n78 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_26_n79 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_28_n3 | E-UTRA Band 1, 22, 42, 43, 50, 51, 65, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_28_n77 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_28_n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_30_n5 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_38_n78 | N/A |  |  |  |  |  |  |  |
| DC_39_n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 5 |  |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 5, 7, 19 |  |
| DC_39_n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |  |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |  |
| DC_40_n1 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_40_n41 | Bands 1, 3, 5, 8, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_40_n77 | N/A |  |  |  |  |  |  |  |
| DC_40_n78 | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n77 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 40, 44, 45, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n78 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 40, 42, 44, 45, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_42_n77 | N/A |  |  |  |  |  |  |  |
| DC_42_n78 | N/A |  |  |  |  |  |  |  |
| DC_42_n79 | N/A |  |  |  |  |  |  |  |
| DC_48_n5 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_48_n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_66_n2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRANR Band n2 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42, 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n5 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41, 42, 48, 52 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n41 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 43, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n71 | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 7, 22, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_66_n78, DC_66_n86_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. NOTE 20: Void. NOTE 21: Void. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.3.2, Table 6.5B.3.3.2-1.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.5B.3.3.2.4	Test description

6.5B.3.3.2.4.1	Initial conditions

Same initial conditions as described in clause 6.5B.3.1.2.4.1 with the following exceptions:

-	Instead of Table 6.5B.3.1.2.4.1-1 --> use Table 6.5B.3.3.2.4.1-1.

Table 6.5B.3.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to "Range" columns. |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [6] clause 4.3.1 and TS 38.508-1 clause 4.3.1 |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |
| ID | DC Configuration / NRB_agg |  |  |  |  |  | DL Allocation |  | UL Allocation (Note 1,2) |  |  |
|  | DC Configuration |  |  |  | E-UTRA Ch BW/NRB | NR Ch BW/NRB | CC MOD E-UTRA/NR | E-UTRA & NR RB allocation | CC MOD E-UTRA/NR | E-UTRA & NR allocations (LCRB @ RBstart) |  |
|  | E-UTRA |  | NR |  |  |  |  |  |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low | Y | Low | Highest Ch BW /Highest NRB | Highest Ch BW /Highest NRB | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | X | High | Y | High | Highest Ch BW /Highest NRB | Highest Ch BW /Highest NRB | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@RBmax | 1@RBmax |
| Test Setting for DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 3 | Low | 10/50 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 1 | High | 3 | Mid | 10/50 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@0 |
| Test Setting for DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | High | 77 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@0 |
| 2 | 1 | Low | 77 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 3 | 1 | Low | 77 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 4 | 1 | Low | 77 | NOTE 18 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 5 | 1 | Low | 77 | NOTE 18 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 6 | 1 | Low | 77 | NOTE 18 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 7 | 1 | Low | 77 | NOTE 18 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 8 | 1 | Low | 77 | NOTE 18 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_1A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 28 | Low | 10/50 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 1 | Low | 28 | High | 10/50 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 1 | High | 28 | High | 10/50 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 10/50 | 20/106 |
| Test Setting for DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | High | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 2 | 1 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 3 | 1 | Low | 78 | Note 8 | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_1A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 79 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@172 |
| 2 | 1 | High | 79 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 3 | 1 | High | 79 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 4 | 1 | Low | 79 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@201 |
| 5 | 1 | High | 79 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| Test Setting for DC_2A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 2 | Low | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@30 |
| 3 | 2 | Low | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@62 |
| 4 | 2 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@209 |
| 5 | 2 | High | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 6 | 2 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | n71 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | 66 | Low | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 2 | High | 66 | High | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@215 |
| 3 | 2 | High | 66 | Low | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 4 | 2 | High | 66 | Low | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 5 | 2 | High | 66 | High | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@215 |
| Test Setting for DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 2 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 3 | 2 | High | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 4 | 2 | Low | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| Test Settings for DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | 1 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 3 | Low | 1 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@105 |
| Test Setting for DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | 7 | High | 20/100 | 10/52 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@47 | 1@51 |
| 2 | 3 | Mid | 7 | High | 20/100 | 10/52 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@51 |
| 3 | 3 | Low | 7 | Low | 20/100 | 10/52 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@51 |
| 4 | 3 | Mid | 7 | Low | 20/100 | 10/52 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 5 | 3 | Low | 7 | High | 20/100 | 10/52 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@51 |
| Test Settings for DC_3A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | 28 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 3 | Low | 28 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@105 |
| 3 | 3 | High | 28 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@105 |
| Test Settings for DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 3 | High | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 3 | 3 | Mid | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 4 | 3 | Note 4 | 41 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@272 |
| Test Setting for DC_3A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Low | 77 | NOTE 19 | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 3 | Low | 77 | NOTE 19 | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 3 | Low | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 4 | 3 | Low | 77 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@75 |
| 5 | 3 | Low | 77 | NOTE 19 | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 6 | 3 | Low | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 7 | 3 | High | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| Test Setting for DC_3A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | High | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 2 | 3 | High | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 3 | 3 | Low | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| Test Settings for DC_3A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | High | 79 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@105 |  |
| 2 | 3 | High | 79 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |  |
| 3 | 3 | Low | 79 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |  |
| 4 | 3 | High | 79 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@134 |  |
| 5 | 3 | Low | 79 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@66 |  |
| Test Settings for DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 | High | 66 | Mid | 10/50 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@0 |
| 2 | 5 | High | 66 | High | 10/50 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@215 |
| 3 | 5 | Low | 66 | Low | 10/50 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@215 |
| 4 | 5 | High | 66 | Low | 10/50 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@0 |
| Test Setting for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 | Low | 78 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 5 | Low | n78 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 3 | 5 | Low | n78 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 4 | 5 | High | n78 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@29 |
| 5 | 5 | Low | n78 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| Test Setting for DC_7A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Low | 1 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 7 | Low | 1 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@105 |
| 3 | 7 | Mid | 1 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@22 | 1@105 |
| Test Setting for DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | High | 3 | Low | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@77 |
| 2 | 7 | Low | 3 | Low | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@59 |
| 3 | 7 | High | 3 | High | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@86 |
| 4 | 7 | High | 3 | Low | 10/50 | 5/25 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@0 |
| 5 | 7 | High | 3 | Low | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 6 | 7 | Low | 3 | Mid | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@66 |
| Test Setting for DC_7A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | High | 28 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@105 |
| Test Setting for DC_7A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Low | 66 | High | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@215 |
| 2 | 7 | Low | 66 | High | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 7 | Mid | 66 | High | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@50 | 1@215 |
| 4 | 7 | Low | 66 | High | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@215 |
| 5 | 7 | Mid | 66 | Low | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 6 | 7 | High | 66 | Low | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| Test Setting for DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 7 | Mid | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 3 | 7 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@27 | 1@272 |
| 4 | 7 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 5 | 7 | High | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@77 | 1@0 |
| 6 | 7 | Mid | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 7 | 7 | High | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 8 | 7 | High | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 9 | 7 | High | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 10 | 7 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@165 |
| Test Settings for DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | Low | 1 | Low | 10/50 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Settings for DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | High | 3 | Low | 10/50 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@0 |
| 2 | 8 | Low | 3 | Low | 10/50 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@159 |
| 3 | 8 | High | 3 | High | 10/50 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@159 |
| Test Settings for DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | Low | 41 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 8 | Low | 41 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 3 | 8 | Low | 41 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 4 | 8 | LHighow | 41 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@272 |
| Test Settings for DC_8A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | Low | 77 | Note 9 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 8 | High | 77 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@37 |
| 3 | 8 | High | 77 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@97 |
| 4 | 8 | Low | 77 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@211 |
| 5 | 8 | Low | 77 | Note 9 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 6 | 8 | High | 77 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@272 |
| 7 | 8 | High | 77 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@0 |
| 8 | 8 | Low | 77 | Note 9 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 9 | 8 | High | 77 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@85 |
| Test Settings for DC_8A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | High | 78 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@272 |
| 2 | 8 | High | 78 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 8 | Low | 78 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@272 |
| 4 | 8 | High | 78 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 5 | 8 | High | 78 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@0 |
| 6 | 8 | Mid | 78 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@25 | 1@0 |
| Test Settings for DC_11A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 11 | Low | 77 | Note 10 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 11 | Low | 77 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 11 | Low | 77 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@63 |
| 4 | 11 | Low | 77 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@104 |
| 5 | 11 | Low | 77 | Note 10 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 6 | 11 | Low | 77 | Note 10 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 7 | 11 | Low | 77 | Note 10 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 8 | 11 | Low | 77 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@75 |
| 9 | 11 | Low | 77 | Note 10 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 10 | 11 | Low | 77 | Note 10 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Settings for DC_11A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 11 | Low | 78 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@184 |
| 2 | 11 | Low | 78 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 11 | Low | 78 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@63 |
| 4 | 11 | Low | 78 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@104 |
| 5 | 11 | Low | 78 | Note 11 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 6 | 11 | Low | 78 | Note 11 | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 7 | 11 | Low | 78 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@75 |
| Test Settings for DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 11 | Low | 79 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 11 | Low | 79 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@132 |
| 3 | 11 | High | 79 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@30 |
| 4 | 11 | Low | 79 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| Test Setting for DC_12A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 | Low | 66 | Low | 10/50 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 12 | High | 66 | High | 10/50 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@215 |
| 3 | 12 | Low | 66 | High | 10/50 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@215 |
| Test Setting for DC_12A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 | Low | 78 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 12 | High | 78 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@272 |
| 3 | 12 | Low | 78 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@136 |
| Test Setting for DC_13A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 13 | Low | 2 | Low | 10/50 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_14A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 | Low | n2 | Low | 10/50 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_14A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 | Low | n66 | Low | 10/50 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | Low | 79 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_19A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | High | 78 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@272 |
| 2 | 19 | Low | 78 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 19 | High | 78 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@0 |
| 4 | 19 | Low | 78 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@165 |
| 5 | 19 | Low | 78 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 6 | 19 | Low | 78 | Note 17 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_20A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Low | 1 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 20 | High | 1 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@105 |
| Test Setting for DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Low | 3 | Low | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 20 | High | 3 | High | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@159 |
| 3 | 20 | Low | 3 | High | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@159 |
| Test Setting for DC_20A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Low | 28 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@105 |
| Test Setting for DC_21A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Low | 77 | Note 21 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 21 | Low | 77 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 21 | Low | 77 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@174 |
| 4 | 21 | Low | 77 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@215 |
| 5 | 21 | High | 77 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@0 |
| 6 | 21 | Low | 77 | Note 21 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 7 | 21 | Low | 77 | Note 21 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 8 | 21 | Low | 77 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@130 |
| 9 | 21 | Low | 77 | Note 21 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 10 | 21 | High | 77 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@0 |
| Test Setting for DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Low | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 20 | Low | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 20 | Low | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@205 |
| 4 | 20 | High | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@104 |
| 5 | 20 | Low | 78 | Note 24 | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_21A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Low | 78 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 21 | Low | 78 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 21 | High | 78 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@272 |
| 4 | 21 | Low | 78 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@215 |
| 5 | 21 | High | 78 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@0 |
| 6 | 21 | Low | 78 | Note 20 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 7 | 21 | Low | 78 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@130 |
| Test Setting for DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Low | 79 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 21 | Low | 79 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 21 | Low | 79 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@38 |
| 4 | 21 | Low | 79 | Note 22 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_25A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 25 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 25 | Low | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@30 |
| 3 | 25 | Low | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@62 |
| 4 | 25 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@209 |
| 5 | 25 | High | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 6 | 25 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 | High | 41 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@272 |
| 2 | 26 | Low | 41 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@12 |
| 3 | 26 | Low | 41 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@41 |
| 4 | 26 | High | 41 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@0 |
| 5 | 26 | Low | 41 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@232 |
| Test Setting for DC_26A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 | Low | 77 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 26 | Low | 77 | NOTE 25 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 26 | High | 77 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@34 |
| 4 | 26 | Low | 77 | NOTE 25 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 5 | 26 | Low | 77 | NOTE 25 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 6 | 26 | Low | 77 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@201 |
| 7 | 26 | Low | 77 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 8 | 26 | Low | 77 | NOTE 25 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 | Low | 78 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 26 | Low | 78 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 26 | High | 78 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@74 | 1@34 |
| 4 | 26 | Low | 78 | Mid | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@78 |
| 5 | 26 | Low | 78 | Low | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@201 |
| 6 | 26 | Low | 78 | Note 13 | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| N/A (Note 14) |  |  |  |  |  |  |  |  |  |  |  |
| Test Setting for DC_28A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | Low | 3 | Low | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 28 | High | 3 | High | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@159 |
| 3 | 28 | High | 3 | Low | 20/100 | 30/160 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| Test Setting for DC_28A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | High | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 2 | 28 | Low | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@158 |
| 3 | 28 | High | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@130 |
| 4 | 28 | Low | 77 | Note 23 | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 5 | 28 | Low | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 6 | 28 | Low | 77 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 7 | 28 | Low | 77 | Note 23 | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_28A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | Low | n78 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |  |
| 2 | 28 | High | n78 | High | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@272 |  |
| 3 | 28 | Mid | n78 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@272 |  |
| 4 | 28 | High | n78 | Low | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@190 |  |
| 5 | 28 | High | n78 | Mid | 10/50 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |  |
| Test Setting for DC_28A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | High | n79 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| Test Setting for DC_30A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 30 | High | 5 | High | 10/50 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@49 | 1@105 |
| Test Settings for DC_39A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 39 | High | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| Test Settings for DC_39A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 39 | Low | 79 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@146 |
| 2 | 39 | Low | 79 | Note 5 | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Settings for DC_40A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 40 | Low | 1 | LOW | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 40 | High | 1 | LOW | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 40 | High | 1 | LOW | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@81 |
| 4 | 40 | Mid | 1 | LOW | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Settings for DC_40A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 40 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 40 | High | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 3 | 40 | Mid | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@50 | 1@272 |
| 4 | 40 | Low | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| Test Settings for DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 40 | Low | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@79 |
| 2 | 40 | High | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@110 |
| 3 | 40 | Low | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 4 | 40 | Low | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@234 |
| 5 | 40 | High | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@29 |
| 6 | 40 | Low | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@50 |
| 7 | 40 | Low | 78 | High | 15/75 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 8 | 40 | High | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 9 | 40 | Low | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@110 |
| 10 | 40 | Low | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@155 |
| 11 | 40 | High | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@183 |
| 12 | 40 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| Test Settings for DC_41A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | Mid | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 41 | Low | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@185 |
| 3 | 41 | Low | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@57 |
| 4 | 41 | Low | 77 | Note 15 | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 5 | 41 | Low | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 6 | 41 | High | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@201 |
| 7 | 41 | Mid | 77 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@46 |
| 8 | 41 | Low | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@214 |
| 9 | 41 | Low | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@29 |
| 10 | 41 | High | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 11 | 41 | Mid | 77 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 12 | 41 | Low | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 13 | 41 | Low | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 14 | 41 | Mid | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 15 | 41 | High | 77 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@161 |
| 16 | 41 | High | 77 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@122 |
| 17 | 41 | Mid | 77 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@27 |
| Test Settings for DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | High | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 2 | 41 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@185 |
| 3 | 41 | High | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 4 | 41 | Low | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 5 | 41 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@214 |
| 6 | 41 | Low | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@69 |
| 7 | 41 | Low | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 8 | 41 | High | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@161 |
| 9 | 41 | Mid | 78 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 10 | 41 | Low | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@98 |
| Test Settings for DC_41A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | Low | 79 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 41 | High | 79 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 3 | 41 | High | 79 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@188 |
| 4 | 41 | High | 79 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@93 |
| 5 | 41 | Mid | 79 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 6 | 41 | High | 79 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@151 |
| 7 | 41 | High | 79 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 8 | 41 | Low | 79 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Setting for DC_42A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| N/A (Note 14) |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_48A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 48 | High | 5 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@105 |
| Test Settings for DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 48 | High | 66 | Low | 20/100 | 40/216 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| Test Settings for DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low | 2 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 66 | High | 2 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@105 |
| 3 | 66 | Low | 2 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@105 |
| 4 | 66 | Low | 2 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@105 |
| 5 | 66 | Low | 2 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@46 |
| Test Settings for DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low | 5 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 2 | 66 | High | 5 | High | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@105 |
| 3 | 66 | Low | 5 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 4 | 66 | High | 5 | Low | 20/100 | 20/106 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| Test Settings for DC_66A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| 2 | 66 | Low | 41 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@0 |
| 3 | 66 | High | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@59 |
| 4 | 66 | Low | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@248 |
| 5 | 66 | High | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@89 |
| 6 | 66 | Low | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@218 |
| 7 | 66 | High | 41 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| 8 | 66 | Low | 41 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@186 |
| 9 | 66 | Low | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@203 |
| 10 | 66 | High | 41 | Low | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@0 |
| Test Settings for DC_66A_n71A Configuration – N/A |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@143 |
| 2 | 66 | High | 78 | High | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@99 | 1@272 |
| 3 | 66 | Low | 78 | Mid | 20/100 | 100/273 | QPSK/CP-OFDM QPSK | NA | QPSK / CP-OFDM QPSK | 1@0 | 1@272 |
| Note 1: Use DC Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 2: X, Y correspond to the different bands in the DC Configuration. E.g. for DC_1A_n3A, X=1, Y=3. Note 3: Void Note 4: Test Point ID 4 for DC_3A_n41A have the centre carrier frequency of 1773 MHz in Band 3 (EARFCN=19830). Note 5: Test Point ID 4 for DC_39A_n79A have the centre carrier frequency of 4649.96 MHz in Band 79 (NR ARFCN=709998). Note 6: Void. Note 7: Void. Note 8: Test Point ID 3 for DC_1A_n78A have the centre carrier frequency of 3473.43 MHz in Band 78 (NR ARFCN=631562). Note 9: Test Point ID 1 for DC_8A_n77A have the centre carrier frequency of 3950.13 MHz in Band n77 (NR ARFCN=663342). Test Point ID 5 for have the centre carrier frequency of 3524.55 MHz in Band n77 (NR ARFCN=634970). Test Point ID 8 for have the centre carrier frequency of 3584.55 MHz in Band n77 (NR ARFCN=638970). Note 10: Test Point ID 1 for DC_11A_n77A have the centre carrier frequency of 3617.43 MHz in Band n77 (NR ARFCN=641162). Test Point ID 5 for have the centre carrier frequency of 3686.43 MHz in Band n77 (NR ARFCN=645762). Test Point ID 6 for have the centre carrier frequency of 3494.94 MHz in Band n77 (NR ARFCN=632996). Test Point ID 7 for have the centre carrier frequency of 3858.42 MHz in Band n77 (NR ARFCN=657228). Test Point ID 9 for have the centre carrier frequency of 4037.43 MHz in Band n77 (NR ARFCN=669162). Test Point ID 10 for have the centre carrier frequency of 4097.43 MHz in Band n77 (NR ARFCN=673162). Note 11: Test Point ID 5 for have the centre carrier frequency of 3686.43 MHz in Band n78 (NR ARFCN=645762). Test Point ID 6 for have the centre carrier frequency of 3494.94 MHz in Band n78 (NR ARFCN=632996). Note 12: Test Point ID 2 for have the centre carrier frequency of 3521.13 MHz in Band n77 (NR ARFCN=634742). Test Point ID 4 for have the centre carrier frequency of 3578.73 MHz in Band n77 (NR ARFCN=638582). Test Point ID 5 for have the centre carrier frequency of 4028.64 MHz in Band n77 (NR ARFCN=668576). Test Point ID 8 for have the centre carrier frequency of 3483.78 MHz in Band n77 (NR ARFCN=632252). Note 13: Test Point ID 6 for have the centre carrier frequency of 3483.78 MHz in Band n78 (NR ARFCN=632252). Note 14: Test case not applicable for the EN-DC configuration as no IM products occurs in the protected bands. Note 15: Test Point ID 4 for have the centre carrier frequency of 3488.55 MHz in Band n77 (NR ARFCN=632570). Note 16: Void Note 17: Test Point ID 6 for have the centre carrier frequency of 3499,8 MHz in Band n78 (NR ARFCN=633320). Note 18: Test Point ID 4 for have the centre carrier frequency of 3472,95 MHz in Band n77 (NR ARFCN=631530). Test Point ID 5 for have the centre carrier frequency of 3987,03 MHz in Band n77 (NR ARFCN=665802). Test Point ID 6 for have the centre carrier frequency of 3857,04 MHz in Band n77 (NR ARFCN=657136). Test Point ID 7 for have the centre carrier frequency of 3874,53 MHz in Band n77 (NR ARFCN=658302). Test Point ID 8 for have the centre carrier frequency of 3887,04 MHz in Band n77 (NR ARFCN=659136). Note 19: Test Point ID 1 for DC_3A_n77A have the centre carrier frequency of 3900,03 MHz in Band n77 (NR ARFCN=660002). Test Point ID 2 for have the centre carrier frequency of 3602,55 MHz in Band n77 (NR ARFCN=640170). Test Point ID 5 for have the centre carrier frequency of 3660,15 MHz in Band n77 (NR ARFCN=644010). Note 20: Test Point ID 6 for have the centre carrier frequency of 3515,19 MHz in Band n78 (NR ARFCN=634346). Note 21: Test Point ID 1 for DC_21A_n77A have the centre carrier frequency of 3637,68 MHz in Band n77 (NR ARFCN=642512). Test Point ID 6 for have the centre carrier frequency of 3515,19 MHz in Band n77 (NR ARFCN=634346). Test Point ID 7 for have the centre carrier frequency of 3898,92 MHz in Band n77 (NR ARFCN=659928). Test Point ID 9 for have the centre carrier frequency of 4057,68 MHz in Band n77 (NR ARFCN=670512). Note 22: Test Point ID 4 for have the centre carrier frequency of 4846,53 MHz in Band n79 (NR ARFCN=723102).Note 23: Test Point ID 4 for have the centre carrier frequency of 3474,63 MHz in Band n77 (NR ARFCN=631642). Test Point ID 7 for have the centre carrier frequency of 3597,12 MHz in Band n77 (NR ARFCN=639808). Note 24: Test Point ID 5 for have the centre carrier frequency of 3477.03 MHz in Band n78 (NR ARFCN=631802). Note 25: Test Point ID 2 for have the centre carrier frequency of 3521.13 MHz in Band n77 (NR ARFCN=634742). Test Point ID 4 for have the centre carrier frequency of 3578.73 MHz in Band n77 (NR ARFCN=638582). Test Point ID 5 for have the centre carrier frequency of 4028.64 MHz in Band n77 (NR ARFCN=668576). Test Point ID 8 for have the centre carrier frequency of 3483.78 MHz in Band n77 (NR ARFCN=632252). |  |  |  |  |  |  |  |  |  |  |  |

Additional step 8 when both bands are TDD:

8.	For both E-UTRA and NR UL uplink carriers active when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.3.3.2.4.2	Test Procedure

Same test procedure as described in clause 6.5B.3.1.2.4.2 with the following exceptions:

Instead of Table 6.5B.3.1.2.3-1 --> use Table 6.5B.3.3.2.5-1 and 6.5B.3.3.2.5-2.

In addition to test configurations above, EN-DC only capable UEs and UEs not supporting SA mode in the tested band needs to be tested according to LTE anchor agnostic approach below.

Same test description as in clause 6.5.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 6.5.3.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.3.3.2.4.3	Message Contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.3.3.2.4.3-1: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | Sa2 |  |  |
| specialSubframePatterns | Ssp7 |  |  |
| } |  |  |  |

Table 6.5B3.3.2.4.3-1a: Void

Table 6.5B.3.3.2.4.3-2: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Table 6.5B.3.3.2.4.3-3: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

6.5B.3.3.2.5	Test Requirement

The test requirements are in Table 6.5B.3.3.2.5-1 and Table 6.5B.3.3.2.5-2. For EN-DC only capable devices, in addition to Table 6.5B.3.3.2.5-1 and Table 6.5B.3.3.2.5-2, the test requirements as in clause 6.5.3.2.5 in TS 38.521-1 [8] are also needed.

For inter-band EN_DC with the uplink assigned to one LTE band and one NR band, the test requirements in Table 6.5B.3.3.2.5-1 and Table 6.5B.3.3.2.5-2 can be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.2.5-1: Requirements for inter-band within FR1 for Rel-15

| EN-DC Configuration |  | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |
| DC_1_n28 |  | E-UTRA Band 7, 31, 41, 72, 73 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 42, 52 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |  |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |  |
| DC_1_n77 |  | E-UTRA Band 1, 3, 11, 21, 34, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |  |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |  |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | 1.6 |  | 5 |  | 5, 7, 8 |  |
| DC_1_n78 |  | E-UTRA Band 3, 11, 21, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |  |
| DC_1_n79 |  | E-UTRA Band 5, 7, 8, 18, 19, 26, 28, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| EDC_2_n5 |  | E-UTRA Band 42, 48 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 41, 43 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_2_n66 |  | E-UTRA Band 4, 10, 24, 50, 66, 70, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |  |
|  |  | E-UTRA Band 42, 48 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_2_n71 |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_2_n78 |  | E-UTRA Band 24, 50, 51, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_3_n7 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_3_n28 |  | NR band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | E-UTRA Band 7, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_3_n41 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 44 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 18, 19 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 14, 20 |  |
|  |  | E-UTRA Band 42, NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_3_n77 |  | E-UTRA Band 1, 3, 28, 34, 39, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_3_n78 |  | E-UTRA Band 3, 34, 39 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_3_n79 |  | E-UTRA Band 5, 8, 11, 18, 19, 21, 41, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_5_n66 |  | E-UTRA Band 5, 6, 7, 8, 38 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 26 |  | 859 |  | - |  | 869 |  | -27 |  | 1 |  |  |  |
|  |  | E-UTRA Band 41, 42, 52 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | NR Band n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5_n78 |  | E-UTRA Band 1, 2, 3, 4, 7, 25, 34, 38, 65, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 7 |  |
| DC_7_n28 |  | E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | NR band n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_7_n78 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 50, 51, 67, 68, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_8_n77 |  | E-UTRA Band 1, 32, 33, 34, 38, 39, 40,50, 65, 69, 74, 75 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA band 3, 7, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 12 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |  |
| DC_8_n78 |  | E-UTRA band 34, 39, 40, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 3, 7, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 12 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |  |
| DC_11_n77 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| DC_11_n78 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_11_n79 |  | E-UTRA Band 1, 3, 34, 42, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_12_n66 |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_20_n78 |  | E-UTRA Band 1, 3, 7, 33, 34, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 38, 69 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_21_n77 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| DC_19_n78 |  | E-UTRA Band 1, 3, 34, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| DC_19_n79 |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_20_n28 |  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_21_n78 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_21_n79 |  | E-UTRA Band 3, 21, 34, 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_25_n41 |  | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_26_n41 |  | E-UTRA Band 3, 5, 18, 19, 26, 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
| DC_26_n77 |  | E-UTRA Band 1, 3, 34, 39, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| DC_26_n78 |  | E-UTRA Band 1, 3, 34, 39, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  | 2 |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| DC_28_n77 |  | E-UTRA Band 3, 7, 34, 39, 40, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 1, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |  |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 |  | E-UTRA Band 3, 7, 34, 39, 40, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 1, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_28_n79 |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_30_n5 |  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_39_n41 |  | E-UTRA Band 42, 44 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_39_n79 |  | E-UTRA Band 8, 41, 44 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_40_n41 |  | Bands 1, 34, 39, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 19 |  |
| DC_41_n77 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_41_n78 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_41_n79 |  | E-UTRA Band 1, 3, 5, 8, 18, 19, 26, 28, 34, 40, 44, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_66_n5 |  | E-UTRA Band 5, 6, 7, 8, 26, 38 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | E-UTRA Band 41, 42, 52 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_66_n78 |  | E-UTRA Band 3, 34, 39 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 of TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 – 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink. NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB: B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5B.3.3.2.5-2: Requirements for inter-band within FR1 for Rel-16

| EN-DC Configuration |  | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  |  |  |  |  |  |  |  |  |  | Maximum Level (dBm) |  |  |  | MBW (MHz) |  |  |  | NOTE |  |  |  |
| DC_1_n3 |  | E-UTRA Band 1, 11, 21, 32, 43, 50, 65, 74, 75 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band 77, 78 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_1_n28 |  | E-UTRA Band 7, 31, 41, 72, 73 NR Band n79 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 42, 52 NR Band n77, n78 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range |  | 470 |  |  |  | - |  |  |  | 694 |  |  |  | -42 |  |  |  | 8 |  |  |  | 5, 17 |  |  |  |
|  |  | Frequency range |  | 470 |  |  |  | - |  |  |  | 710 |  |  |  | -26.2 |  |  |  | 6 |  |  |  | 14 |  |  |  |
| DC_1_n77 |  | E-UTRA Band 1, 3, 11, 21, 34, 65, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  | 1880 |  |  |  | - |  |  |  | 1895 |  |  |  | -40 |  |  |  | 1 |  |  |  | 5, 8 |  |  |  |
|  |  | Frequency range |  | 1895 |  |  |  | - |  |  |  | 1915 |  |  |  | -15.5 |  |  |  | 5 |  |  |  | 5, 7, 8 |  |  |  |
|  |  | Frequency range |  | 1915 |  |  |  | - |  |  |  | 1920 |  |  |  | 1.6 |  |  |  | 5 |  |  |  | 5, 7, 8 |  |  |  |
| DC_1_n78 |  | E-UTRA Band 3, 11, 21, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  | 1880 |  |  |  | - |  |  |  | 1895 |  |  |  | -40 |  |  |  | 1 |  |  |  | 5, 8 |  |  |  |
| DC_1_n79 |  | E-UTRA Band 5, 7, 8, 18, 19, 26, 28, 41 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_2_n5 |  | E-UTRA Band 42, 48 NR band n77 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41, 43 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_2_n41 |  | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Bands 2 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 5 |  |  |  |
| DC_2_n66 |  | E-UTRA Band 4, 10, 24, 50, 66, 70, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 5 |  |  |  |
|  |  | E-UTRA Band 42, 48 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_2_n78 |  | E-UTRA Band 24, 50, 51, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_3_n1 |  | E-UTRA Band 1, 11, 21, 32, 50, 65, 74, 75 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_3_n7 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 42 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_3_n41 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 44 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 18, 19 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 14, 20 |  |  |  |
|  |  | E-UTRA Band 42, NR Band n77, n78, n79 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_3_n77 |  | E-UTRA Band 1, 3, 28, 34, 39, 40, 65, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_3_n78 |  | E-UTRA Band 3, 34, 39 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_3_n79 |  | E-UTRA Band 5, 8, 11, 18, 19, 21, 41, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_5_n2 |  | E-UTRA Band 42 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41, 43 NR Band n77 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_5_n66 |  | E-UTRA Band 5, 6, 7, 8, 38 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 26 |  | 859 |  |  |  | - |  |  |  | 869 |  |  |  | -27 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41, 42, 52 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | NR Band n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5_n78 |  | E-UTRA Band 1, 2, 3, 4, 7, 25, 34, 38, 65, 66, 70 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2, 7 |  |  |  |
| DC_7_n1 |  | E-UTRA Band 32, 50, 51, 75, 76 or NR Band n79 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_7_n3 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 67, 68 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA band 42, 52 NR Band n77, n78 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_7_n28 |  | E-UTRA Band 3 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR band n78 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_7_n78 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 50, 51, 67, 68, 74, 75, 76 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_8_n1 |  | E-UTRA Band 43 or NR Band n77, n78, n79 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_8_n3 |  | E-UTRA Band 20, 28, 38, 44 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 7, 22, 41, 42, 43 or NR Band n77, n78, n79 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range |  | 860 |  |  |  | - |  |  |  | 890 |  |  |  | -40 |  |  |  | 1 |  |  |  | 5, 12 |  |  |  |
| DC_8_n41 |  | E-UTRA Band 28 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA band 3, 42, 52 NR Band n77, n78, n79 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 8 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 5 |  |  |  |
|  |  | Frequency range |  | 860 |  |  |  | - |  |  |  | 890 |  |  |  | -40 |  |  |  | 1 |  |  |  | 5, 12 |  |  |  |
| DC_8_n77 |  | E-UTRA Band 1, 32, 33, 34, 38, 39, 40,50, 65, 69, 74, 75 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA band 3, 7, 41 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 12 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3, 12 |  |  |  |
| DC_8_n78 |  | E-UTRA band 34, 39, 40, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 3, 7, 41 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 12 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3, 12 |  |  |  |
| DC_7_n66 |  | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28, 29, 85 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA band 42 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_11_n77 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
|  |  | Frequency range |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_11_n78 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_11_n79 |  | E-UTRA Band 1, 3, 34, 42, 65 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_12_n66 |  | E-UTRA Band 41, 53 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR band n77 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_12_n78 |  | E-UTRA Band 2, 7, 25, 41 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 4, 66 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_13_n2 |  | E-UTRA Band 41 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_13_n66 |  | E-UTRA Band 41, 53 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n77 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_14_n2 |  | E-UTRA Band 41 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_14_n66 |  | E-UTRA Band 41 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_19_n78 |  | E-UTRA Band 1, 3, 34, 65 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
|  |  | Frequency range |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_19_n79 |  | E-UTRA Band 42 |  |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |
| DC_20_n1 |  | E-UTRA Band 22, 42, 43 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band 77, 78 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_20_n3 |  | E-UTRA Band 7, 8 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 38, 42, 52 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_20_n28 |  | E-UTRA Band 8 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_21_n77 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |
|  |  | Frequency range |  |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |
|  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |
|  |  | Frequency range |  |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |
|  |  | Frequency range |  |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |
| DC_20_n78 |  | E-UTRA Band 1, 3, 7, 33, 34, 65 |  |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |
|  |  | E-UTRA Band 38, 69 |  |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |
| DC_21_n78 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |
|  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |
| DC_21_n79 |  | E-UTRA Band 3, 21, 34, 42 |  |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |
|  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |
| DC_25_n41 |  | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n77 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_26_n41 |  | E-UTRA Band 3, 5, 18, 19, 26, 42 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  |  | 799 |  |  |  | - |  |  |  | 803 |  |  |  | -40 |  |  |  | 1 |  |  |  | 5 |  |  |  |
|  |  | Frequency range |  |  | 945 |  |  |  | - |  |  |  | 960 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_26_n77 |  | E-UTRA Band 1, 3, 34, 39, 40, 65 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
|  |  | Frequency range |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_26_n78 |  | E-UTRA Band 1, 3, 34, 39, 65 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
|  |  | Frequency range |  |  | 2545 |  |  |  | - |  |  |  | 2575 |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range |  |  | 2595 |  |  |  | - |  |  |  | 2645 |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_28_n3 |  | NR Band n77 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 7, 41 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_28A_n77 |  | E-UTRA Band 3, 7, 34, 39, 40, 41 |  |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |
|  |  | E-UTRA Band 1, 65 |  |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |
|  |  | E-UTRA Band 1 |  |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 9, 11 |  |  |
|  |  | Frequency range |  |  |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3, 9 |  |  |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 |  | E-UTRA Band 3, 7, 34, 39, 40, 41 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 1, 65 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 1 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 9, 11 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_28_n79 |  | E-UTRA Band 42 |  |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_30_n5 |  | E-UTRA Band 71 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR band n77 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_39_n41 |  | E-UTRA Band 42, 44 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n77, n78, n79 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_39_n79 |  | E-UTRA Band 8, 28, 41, 44 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_40_n1 |  | E-UTRA Band 7, 31, 32, 41, 45, 50, 72, 73, 74, 75 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_40_n41 |  | Bands 1, 34, 39, 65 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n79 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3, 19 |  |  |  |
| DC_41_n77 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  |  |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_41_n78 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_40_n78 |  | E-UTRA Band 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 44, 45, 50, 51, 74, 75, 76 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n79 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_41_n79 |  | E-UTRA Band 1, 3, 5, 8, 9, 18, 19, 26, 28, 34, 40, 44, 65 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_48_n5 |  | E-UTRA Band 25, 70 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | - |  |  |  | 1915.7 |  |  |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_48_n66 |  | E-UTRA Band 2, 25 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_66_n2 |  | E-UTRA Band 4, 10, 22, 24, 50, 66, 70, 74 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 5 |  |  |  |
|  |  | E-UTRA Band 42, 43 NR Band n77 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_66_n5 |  | E-UTRA Band 5, 6, 7, 8, 26, 38 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41, 42, 52 NR Band n77 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_66_n41 |  | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28, 29, 43, 85 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 42, 48, NR band n77 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_66_n78 |  | E-UTRA Band 3, 34, 39 |  | FDL_low |  |  |  | - |  |  |  | FDL_high |  |  |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. NOTE 20: Void. NOTE 21: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |