3GPP TSG-RAN5 Meeting #93-e	R5-216423r2

# Electronic Meeting, 8th Nov 2021 – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1377 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update RRCConnectionReconfiguration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add connectivity NE-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The RRCConnectionReconfiguration has been updated - NE-DC is added - Table for SCG-Configuration-r12-NE-DC has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The connectivity NE-DC will not be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision R5-216423r1. The SCG-Configuration-r12-NE-DC is updated. 2nd revision R5-216423r2. The SCG-Configuration-r12-NE-DC is updated. |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	RRCConnectionReconfiguration

Table 4.6.1-8: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  |  | RRC-TransactionIdentifier-DL |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |  |  |  |
| measConfig |  |  | Not present |  |  |  |  |
|  |  |  | MeasConfig-DEFAULT |  |  |  | MEAS |
| mobilityControlInfo |  |  | Not present |  |  |  |  |
|  |  |  | MobilityControlInfo-HO |  |  |  | HO, HO-TO-EUTRA, HO-TO-EN-DC |
| dedicatedInfoNASList |  |  | Not present |  |  |  |  |
|  |  |  | Set according to specific message content |  |  |  | SRB2-DRB(n, m) DRB(n, m) SRB1-SRB2-DRB(n,m) AM-DRB-ADD(bid) UM-DRB-ADD(bid) DRB-REL(bid) DRB-Mod MCG_and_split EN-DC_SRB2-DRB |
| radioResourceConfigDedicated |  |  | Not present |  |  |  |  |
|  |  |  | RadioResourceConfigDedicated-SRB2-DRB(n, m) |  |  |  | SRB2-DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-DRB(n, m) |  |  |  | DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-AM-DRB-ADD(bid) |  |  |  | AM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-UM-DRB-ADD(bid) |  |  |  | UM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-DRB-REL(bid) |  |  |  | DRB-REL(bid) |
|  |  |  | RadioResourceConfigDedicated-HO |  |  |  | HO |
|  |  |  | RadioResourceConfigDedicated-HO-TO-EUTRA(n, m) |  |  |  | HO-TO-EUTRA(n,m), HO-TO-EN-DC(n,m) |
|  |  |  | RadioResourceConfigDedicated-DRB-Mod |  |  |  | DRB-Mod |
|  |  |  | RadioResourceConfigDedicated-SCell_AddMod |  |  |  | SCell_AddMod |
|  |  |  | RadioResourceConfigDedicated-PCell-PATTERN |  |  |  | PCell-PATTERN |
|  |  |  | RadioResourceConfigDedicated-EN-DC-Split |  |  |  | MCG_and_split |
|  |  |  | RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid) |  |  |  | EN-DC_Split_Rel(bid) |
|  |  |  | RadioResourceConfigDedicated-SRB4 |  |  |  | SRB4 |
|  |  |  | RadioResourceConfigDedicated-EN-DC-SRB2-DRB |  |  |  | EN-DC_SRB2-DRB |
| securityConfigHO |  |  | Not present |  |  |  |  |
|  |  |  | SecurityConfigHO-DEFAULT |  |  |  | HO, HO-TO-EUTRA, HO-TO-EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v890-IEs |  |  |
| lateNonCriticalExtension |  |  | Not present |  | RRCConnectionReconfiguration-v8m0-IEs |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v8m0-IEs |  | 4Rx_PCC |  |
| lateNonCriticalExtension | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v10i0-IEs |  |  |  |
| antennaInfoDedicatedPCell-v10i0 SEQUENCE { |  |  |  |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension | Not present |  | RRCConnectionReconfiguration-v10l0-IEs |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v920-IEs |  |  |  |
| otherConfig-r9 | Not present |  |  |  |  |  |
|  | OtherConfig-r9 |  |  |  | OtherConfig |  |
| fullConfig-r9 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1020-IEs |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |  |  |  |
| sCellToAddModList-r10 | Not present |  |  |  |  |  |
| sCellToAddModList-r10 SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellToAddMod-r10 { | The same number of entries as the configured SCell(s) |  | n denotes the index of the entry |  | SCell_AddMod |  |
| SCellToAddMod-r10[n] | SCellToAddMod-r10-DEFAULT |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1130-IEs |  |  |  |
| systemInfomationBlockType1Dedicated-r11 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1250-IEs |  |  |  |
| wlan-OffloadInfo-r12 | Not present |  |  |  |  |  |
| wlan-OffloadInfo-r12 CHOICE { |  |  |  |  | WLAN-OffloadSetup, WLAN-OffloadRelease |  |
| Release | NULL |  |  |  | WLAN-OffloadRelease |  |
| setup SEQUENCE { |  |  |  |  | WLAN-OffloadSetup |  |
| wlan-OffloadConfigDedicated-r12 | WLAN-OffloadConfig-DEFAULT |  |  |  |  |  |
| t350-r12 | min10 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| scg-Configuration-r12 | Not present |  |  |  |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12-DEFAULT |  |  |  | PSCell_Release, PSCell_Mod, PSCell_Add_Split_DRB, PSCell_Add_SCG_DRB |  |
| scg-Configuration-r12 | SCG-Configuration-r12-NE-DC |  |  |  | NE-DC |  |
| sl-SyncTxControl-r12 | Not present |  |  |  |  |  |
| sl-DiscConfig-r12 | Not present |  |  |  |  |  |
| sl-CommConfig-r12 | Not present |  |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1310-IEs |  | LWA |  |
| lwa-configuration-r13 | Set as specified in test case |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1310-IEs |  | LWIP |  |
| lwip-configuration-r13 | Set as specified in test case |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1310-IEs |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), MCG_NR_PDCP, EN-DC_PSCell_Rel, SRB_NR_PDCP, EN-DC_EmbedNR_RRCRecon, EN-DC_Embed_RBConfig, EN-DC_PSCell_HO, CHO |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1430-IEs |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1510-IEs |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |
| release |  |  |  |  | EN-DC_PSCell_Rel |  |
| setup SEQUENCE { |  |  |  |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), MCG_NR_PDCP, EN-DC_EmbedNR_RRCRecon, EN-DC_Embed_RBConfig, EN-DC_PSCell_HO |  |
| endc-ReleaseAndAdd-r15 | false |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC. |  |  |  | MCG_and_SCG，HO-TO-EN-DC(n,m) |  |
|  | OCTET STRING including the RRCReconfiguration message, the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC and the IE PDCP-Config according TS 38.508-1 [67], table 4.6.3-99 with condition Split. |  |  |  | MCG_and_split |  |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.8.1-2. |  |  |  | SCG-DRB(n,m) |  |
|  | OCTET STRING including the test case specific RRCReconfiguration |  |  |  | EN-DC_EmbedNR_RRCRecon |  |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC_HO |  |  |  | EN-DC_PSCell_HO |  |
| p-MaxEUTRA-r15 | Not present |  |  |  |  |  |
|  | 23 |  |  |  | SIG AND EN-DC AND pc_dynamicPowerSharing |  |
|  | 20 |  |  |  | SIG AND EN-DC AND (NOT pc_dynamicPowerSharing) |  |
|  | 10 |  |  |  | RF AND EN-DC_PC3 |  |
|  | 13 |  |  |  | RF AND EN-DC_PC2 |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| sk-Counter-r15 | 0 |  |  |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), HO-TO-EN-DC(n,m) |  |
|  | Not Present |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with condition EN-DC_DRB. |  |  |  | MCG_and_SCG, MCG_and_split，HO-TO-EN-DC(n,m) |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.8.1-3. |  |  |  | SCG-DRB(n,m) |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with condition MCG_NR_PDCP |  |  |  | MCG_NR_PDCP |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with condition SRB_NR_PDCP |  |  |  | SRB_NR_PDCP |  |
|  | OCTET STRING including the test case specific RadioBearerConfig |  |  |  | EN-DC_Embed_RBConfig |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB and Re-establish_PDCP |  |  |  | EN-DC_PSCell_HO AND RBConfig_KeyChange |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB and Recover_PDCP |  |  |  | EN-DC_PSCell_HO AND RBConfig_NoKeyChange |  |
| nr-RadioBearerConfig2-r15 | Not present |  |  |  |  |  |
| tdm-PatternConfig-r15 | Not present |  |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1530-IEs |  | CHO |  |
| securityConfigHO-v1530 | Not present |  |  |  |  |  |
| sCellGroupToReleaseList-r15 | Not present |  |  |  |  |  |
| sCellGroupToAddModList-r15 | Not present |  |  |  |  |  |
| dedicatedInfoNASList-r15 | Not present |  |  |  |  |  |
| p-MaxUE-FR1-r15 | Not present |  |  |  |  |  |
| smtc-r15 | Not present |  |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1610-IEs |  | CHO |  |
| conditionalReconfiguration-r16 | Not present |  |  |  |  |  |
|  | ConditionalReconfiguration-r16-HO |  |  |  | CHO |  |
| daps-SourceRelease-r16 | Not present |  |  |  |  |  |
| tdm-PatternConfig2-r16 | Not present |  |  |  |  |  |
| sl-ConfigDedicatedForNR-r16 | Not present |  |  |  |  |  |
| sl-SSB-PriorityEUTRA-r16 | Not present |  |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB2-DRB(n,m) | Establishment of a SRB and DRB combination with n x AM DRB and m x UM DRB (including establishment of SRB2) |
| DRB(n,m) | Establishment of additional n x AM DRB:s and m x UM DRB:s (SRB2 should already be established) |
| AM-DRB-ADD(bid) | Establishment of a single additional AM DRB with bearer identity bid (SRB2 should already be established) |
| UM-DRB-ADD(bid) | Establishment of a single additional UM DRB with bearer identity bid (SRB2 should already be established) |
| DRB-REL(bid) | Release of the DRB with bearer identity bid |
| HO | Intra LTE handover |
| MEAS | A measurement is configured |
| HO-TO-EUTRA(n,m) | Inter-RAT handover to E-UTRA including the establishment of a SRB1, SRB2 and n x AM DRB plus m x UM DRB |
| nonFullConfig | The field is not present in case of handover within E-UTRA when the fullConfig is included or in case of handover of handover to E-UTRA; otherwise it is optional present, need ON. |
| DRB-Mod | Modification of already established DRB ID 2 and is used for sending Modify EPS Bearer Context Request message (SRB2 should already be established) |
| SCell_AddMod | Addition or modification of SCell(s) |
| PCell-PATTERN | eICIC Serving Cell Pattern |
| OtherConfig | Other Configuration is configured |
| WLAN-OffloadSetup | When Dedicated WLAN Offload parameters need to be configured in UE |
| WLAN-OffloadRelease | When Dedicated WLAN Offload parameters need to be released in UE |
| PSCell_Mod | Modification of PSCell(s) |
| PSCell_Release | Release of PSCell |
| PSCell_Add_Split_DRB | Add PSCell(s) and setup of Split DRB |
| PSCell_Add_SCG_DRB | Add PSCell(s) and setup of SCG DRB |
| 4Rx_PCC | For 4Rx testcases with more than 2 Layers on PCC |
| MCG_and_SCG | EN-DC with MCG and SCG. |
| MCG_and_split | EN-DC with MCG and split. |
| SCG-DRB(n,m) | EN-DC establishment of additional n x AM DRB:s and m x UM DRB:s in SCG. |
| EN-DC_PSCell_Rel | Release of EN-DC PSCell |
| EN-DC_Split_Rel(bid) | Release of E-UTRA leg of EN-DC Split DRB with bearer identity bid |
| MCG_NR_PDCP | EN-DC configuration of MCG DRB with NR PDCP. |
| SRB_NR_PDCP | EN-DC configuration of SRB1 and SRB2 with NR PDCP. |
| EN-DC_SRB2-DRB | Establishment of SRB2 and DRB combination for EN-DC |
| EN-DC_EmbedNR_RRCRecon | To modify NR Cell Configuration in EN-DC by including RRCReconfiguration |
| EN-DC_EmbedNR_RBConfig | To modify NR RB Configuration in EN-DC by including RadioBearerConfig |
| SRB4 | Establishment of SRB4 |
| LWA | LWA is being configured |
| LWIP | LWIP is being configured |
| HO-TO-EN-DC(n,m) | Inter-RAT handover to EN-DC including the establishment of a SRB1, SRB2 and n x AM DRB plus m x UM DRB |
| EN-DC_PSCell_HO | RRCReconfiguration to perform PSCell handover (SCG change) in EN-DC |
| RBConfig_KeyChange | RadioBearerConfig to perform PSCell handover (SCG change) with security key change in EN-DC |
| RBConfig _NoKeyChange | RadioBearerConfig to perform PSCell handover (SCG change) without security key change in EN-DC |
| EN-DC_NR_CA | Addition, modification or release of NR SCell |
| EN-DC_PC3 | Power Class 3 UE testing Power Class 3 requirements for EN-DC RF test cases except Rx test cases that need to configure PCMAX – XdB for EUTRA CC |
| EN-DC_PC2 | Power Class 2 UE testing Power Class 2 requirements for EN-DC RF test cases except Rx test cases that need to configure PCMAX – XdB for EUTRA CC |
| CHO | Conditional handover |
| NE-DC | NR E-UTRA Dual Connectivity is configured |

<End of modified section>

<Start of next modified section>

#### –	SCG-Configuration-r12-NE-DC

Table 4.6.3-19G: SCG-Configuration-r12-NE-DC

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark |  | Comment | Condition |  |
| scg-Configuration-r12 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| scg-ConfigPartMCG-r12 | Not present |  |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicatedSCG-r12 SEQUENCE { |  |  |  |  |
| drb-ToAddModListSCG-r12 SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  |  |  |
| DRB-ToAddModSCG-r12[1] SEQUENCE { |  |  |  |  |
| drb-Identity-r12 | DRB ID used for NE-DC SCG |  |  |  |
| drb-Type-r12 CHOICE { |  |  |  |  |
| scg-r12 SEQUENCE { |  |  |  |  |
| eps-BearerIdentity-r12 | Not present |  |  |  |
| pdcp-Config-r12 | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| rlc-ConfigSCG-r12 | RLC-Config-DRB-AM |  |  |  |
| rlc-Config-v1250 | Not present |  |  |  |
| logicalChannelIdentitySCG-r12 | 2 |  |  |  |
| logicalChannelConfigSCG-r12 | LogicalChannelConfig-DRB using condition LO |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| mac-MainConfigSCG-r12 CHOICE { |  |  |  |  |
| explicitValue | MAC-MainConfig-RBC using condition DRX_L |  |  |  |
| } |  |  |  |  |
| rlf-TimersAndConstantsSCG-r12 | Not present |  |  |  |
| } |  |  |  |  |
| sCellToReleaseListSCG-r12 | Not Present |  |  |  |
| pSCellToAddMod-r12 SEQUENCE { |  |  |  |  |
| sCellIndex-r12 | 1 |  |  |  |
| cellIdentification-r12 SEQUENCE { |  |  |  |  |
| physCellId-r12 | Set according to specific message content |  |  |  |
| dl-CarrierFreq-r12 | Set according to specific message content |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioResourceConfigCommonPSCell-r12 SEQUENCE { |  |  |  |  |
| basicFields-r12 | RadioResourceConfigCommonSCell-r10-DEFAULT |  |  |  |
| pucch-ConfigCommon-r12 | PUCCH-ConfigCommon-DEFAULT |  |  |  |
| rach-ConfigCommon-r12 | RACH-ConfigCommon-DEFAULT |  |  |  |
| uplinkPowerControlCommonPSCell-r12 SEQUENCE { |  |  |  |  |
| deltaF-PUCCH-Format3-r12 | deltaF0 | Same value as in UplinkPowerControlCommon-v1020-DEFAULT used for the PCell |  |  |
| deltaF-PUCCH-Format1bCS-r12 | deltaF1 | Same value as in UplinkPowerControlCommon-v1020-DEFAULT used for the PCell |  |  |
| p0-NominalPUCCH-r12 | -117 (-117 dBm) | Same value as in UplinkPowerControlCommon--DEFAULT used for the PCell |  |  |
| deltaFList-PUCCH-r12 SEQUENCE { |  |  |  |  |
| deltaF-PUCCH-Format1 | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| deltaF-PUCCH-Format1b | deltaF3 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| deltaF-PUCCH-Format2 | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| deltaF-PUCCH-Format2a | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| deltaF-PUCCH-Format2b | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioResourceConfigDedicatedPSCell-r12 SEQUENCE { |  |  |  |  |
| physicalConfigDedicatedPSCell-r12 | PhysicalConfigDedicated-DEFAULT using condition PSCell_Add_SCG_DRB | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |  |
| sps-Config-r12 | Not present |  |  |  |
| naics-Info-r12 | Not present |  |  |  |
| } |  |  |  |  |
| antennaInfoDedicatedPSCell-v1280 | Not present |  |  |  |
| } |  |  |  |  |
| sCellToAddModListSCG-r12 | Not Present |  |  |  |
| mobilityControlInfoSCG-r12 |  |  |  |  |
| t307-r12 | ms2000 | ENUMERATED {ms50, ms100, ms150, ms200, ms500,ms1000, ms2000} |  |  |
| ue-IdentitySCG-r12 | Any allowed value |  |  |  |
| rach-ConfigDedicated-r12 | Rach-ConfigDedicated-DEFAULT |  |  |  |
| cipheringAlgorithmSCG-r12 | Set according to PIXIT parameter for default ciphering algorithm |  |  | For SIG |
|  | eea0 |  |  | For RF |
| } |  |  |  |  |
| } |  |  |  |  |

<End of modified section>