3GPP TSG-RAN5 Meeting #93-e	R5-216434r1

# Electronic meeting, 8th – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5358 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional spurious emissions for UL 64QAM test configuration for NS_56 (B24) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing Additional spurious emissions for UL 64QAM test configuration for NS_56 (B24) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove editor’s notes / FFS from clause 6.6.3.3_1 - Add Additional spurious emissions for UL 64QAM test configuration details into Table 6.6.3.3_1.4.1-23 for NS_56 (B24) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional spurious emissions test requirements for NS_56 not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.3_1, 6.6.3.3_1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Correct FUL in Table 6.6.3.3_1.4.1-23 |  |  |  |  |  |  |  |  |

-------- Start of changes --------

#### 6.6.3.3_1	Additional spurious emissions for UL 64QAM

-------- Skipped unchanged clauses --------

##### 6.6.3.3_1.4	Test description

6.6.3.3_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3.3_1.4.1-1 through Table 6.6.3.3_1.4.1-21. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

-------- Skipped unchanged clauses --------

Table 6.6.3.3_1.4.1-22: Test Configuration Table (network signalled value "NS_36")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 700.5 MHz (NUL = low range) - 705.5 MHz (NUL =132747) 10 MHz CH BW: - 703 MHz (NUL = low range) - 708 MHz (NUL = 132772) 15 MHz CH BW: - 705.5 MHz (NUL = low range) - 710.5 MHz (NUL =132797 ) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz |  |  |
| Test Parameters for NS_36 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 700.5 | 5MHz | N/A for A-MPR testing |  | 64QAM | 25 | 0 |
| 2 | 705.5 | 5MHz |  |  | 64QAM | 25 | 0 |
| 3 | 703 | 10MHz |  |  | 64QAM | 50 | 0 |
| 4 | 708 | 10MHz |  |  | 64QAM | 50 | 0 |
| 5 | 705.5 | 15MHz |  |  | 64QAM | 75 | 0 |
| 6 | 710.5 | 15MHz |  |  | 64QAM | 75 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Void. Note 3: Void. Note 4: The 64QAM configurations shall not apply to UEs for which TC 6.6.3.3_2 is executed with network signalled value NS_36. |  |  |  |  |  |  |  |

Table 6.6.3.3_1.4.1-23: Test Configuration Table (network signalled value "NS_56")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: 1630 MHz (low range) 10 MHz CH BW: 1632.5 MHz (low range) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz |  |  |
| Test Parameters for NS_56 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 1630 | 5MHz | N/A for A-MPR testing |  | 64QAM | 1 | 0 |
| 2 | 1630 | 5MHz |  |  | 64QAM | 25 | 0 |
| 3 | 1632.5 | 10MHz |  |  | 64QAM | 1 | 0 |
| 4 | 1632.5 | 10MHz |  |  | 64QAM | 50 | 0 |
| Note 1: The 64QAM configurations shall not apply to UEs for which TC 6.6.3.3_2 is executed with network signalled value NS_56. |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of section 6.6.3.3_1.4.1. As new tables are added to this section, these lines should always follow the tables.

1.	Connect the SS to the UE to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.7.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.3.3_1.4.1-1 to Table 6.6.3.3_1.4.1-22 depending on network signal value.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.3.3_1.4.3.

-------- End of changes --------