3GPP TSG-RAN5 Meeting #93-e	R5-216532

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1398 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to additional spurious emission test case for n24 and n99 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adds configurations and requirements for additional spurious emission (ASE) test cases for NS_56 for n24 and n99 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of tables related to ASE test configurations for NS_56; Addition of table related to ASE test requirements for NS_56 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ASE test cases for n24 and n99 will be not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.3, 6.5.3.3.4.1, 6.5.3.3.5, 6.5C.3.3, 6.5C.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: i) Removed overlap with R5-217237 by removing changes to 6.5.3.3.4.2; ii) updated test configuration table with specific frequency values with “low” |  |  |  |  |  |  |  |  |

#### 6.5.3.3	Additional spurious emissions

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined

Initial conditions for NS_37, NS_38, NS_39, NS_40, NS_41, NS_42 and NS_45 are incomplete.

The requirements of this test case for NS_44, NS_46, NS_47, NS_48, and NS_49 apply to all types of NR UE release 16 forward, and release 15 if the corresponding channel bandwidths are supported.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

6.5.3.3.4	Test description

6.5.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1.  All these configurations shall be tested with applicable test parameters for each channel bandwidth and sub-carrier spacing, are shown in Tables 6.5.3.3.4.1-1 through Table 6.5.3.3.4.1-26 for different NS values. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<Unchanged Text Skipped>>

Table 6.5.3.3.4.1-26: Test Configuration Table (network signalling value "NS_56")

Same test configuration as listed in Table 6.2.3.4.1-30 shall be used.

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.3.4.1-1 through Table 6.5.3.3.4.1-26.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5.  Message contents are defined in clause 6.5.3.3.4.3.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

6.5.3.3.5	Test requirement

This clause specifies the requirements for the specified NR band for an additional spectrum emission requirement with protected bands as indicated from Table 6.5.3.3.5.1 to Table 6.5.3.3.5.25 for different NS_values.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<<Unchanged Text Skipped>>

6.5.3.3.5.25	Test requirement (network signalling value “NS_56”)

When "NS 56" is indicated in the cell,

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-36.

The power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.5.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.5.25-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.5C.3.3	Additional spurious emissions for SUL

6.5C.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5C.3.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.3.3	Minimum conformance requirements

The additional spurious emission requirements specified in 6.5.3.3.3 apply to the UE operating on SUL bands.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.3.3

6.5C.3.3.4	Test description

Same test description as specified in clause 6.5.3.3.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	For NS_05, instead of table 6.5.3.3.4.4-1 use Table 6.5C.3.3.4-1

-	For NS_43, instead of table 6.5.3.3.4.1-5 use Table 6.5C.3.3.4-2

-	For NS_18, instead of table 6.5.3.3.4.1-3 use Table 6.2C.3.3.4-3

-	For NS_48, instead of table 6.5.3.3.4.1-5 use Table 6.2C.3.3.4-4

-	For NS_49, instead of table 6.5.3.3.4.1-23 use Table 6.2C.3.3.4-5

-	For NS_56, instead of table 6.5.3.3.4.1-26 use Table 6.2C.3.3.4-6

<<Unchanged Text Skipped>>

Table 6.5C.3.3.4-6: Test Configuration Table (network signalled value “NS_56”)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | 15 kHz for all carriers |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz |
| 1 | Low | 5 | N/A for A-MPR testing | N/A | 14 | DFT-s-OFDM | PI/2 BPSK |  | Edge_1RB_Left |
| 2 | Low | 5 |  |  | 6 |  | PI/2 BPSK |  | Outer_Full |
| 3 | Low | 5 |  |  | 4 |  | PI/2 BPSK |  | 20@4 |
| 4 | Low | 5 |  |  | 4 |  | PI/2 BPSK |  | 1@4 |
| 5 | Low | 5 |  |  | 14 |  | QPSK |  | Edge_1RB_Left |
| 6 | Low | 5 |  |  | 6 |  | QPSK |  | Outer_Full |
| 7 | Low | 5 |  |  | 4 |  | QPSK |  | 20@4 |
| 8 | Low | 5 |  |  | 4 |  | QPSK |  | 1@4 |
| 9 | Low | 5 |  |  | 14 |  | 16 QAM |  | Edge_1RB_Left |
| 10 | Low | 5 |  |  | 6 |  | 16 QAM |  | Outer_Full |
| 11 | Low | 5 |  |  | 4 |  | 16 QAM |  | 20@4 |
| 12 | Low | 5 |  |  | 4 |  | 16 QAM |  | 1@4 |
| 13 | Low | 5 |  |  | 14 |  | 64 QAM |  | Edge_1RB_Left |
| 14 | Low | 5 |  |  | 6 |  | 64 QAM |  | Outer_Full |
| 15 | Low | 5 |  |  | 4 |  | 64 QAM |  | 20@4 |
| 16 | Low | 5 |  |  | 4 |  | 64 QAM |  | 1@4 |
| 17 | Low | 5 |  |  | 14 |  | 256 QAM |  | Edge_1RB_Left |
| 18 | Low | 5 |  |  | 6 |  | 256 QAM |  | Outer_Full |
| 19 | Low | 5 |  |  | 4 |  | 256 QAM |  | 20@4 |
| 20 | Low | 5 |  |  | 4 |  | 256 QAM |  | 1@4 |
| 21 | Low | 10 |  |  | 12 |  | PI/2 BPSK |  | Edge_1RB_Left |
| 22 | Low | 10 |  |  | 8 |  | PI/2 BPSK |  | Outer_Full |
| 23 | Low | 10 |  |  | 6 |  | PI/2 BPSK |  | 1@3 |
| 24 | Low | 10 |  |  | 6 |  | PI/2 BPSK |  | 40@9 |
| 25 | Low | 10 |  |  | 4 |  | PI/2 BPSK |  | 1@35 |
| 26 | Low | 10 |  |  | 2 |  | PI/2 BPSK |  | 16@35 |
| 27 | Low | 10 |  |  | 5 |  | PI/2 BPSK |  | Edge_1RB_Right |
| 28 | Low | 10 |  |  | 5 |  | PI/2 BPSK |  | 1@40 |
| 29 | Low | 10 |  |  | 3 |  | PI/2 BPSK |  | 8@44 |
| 30 | Low | 10 |  |  | 12 |  | QPSK |  | Edge_1RB_Left |
| 31 | Low | 10 |  |  | 8 |  | QPSK |  | Outer_Full |
| 32 | Low | 10 |  |  | 6 |  | QPSK |  | 1@3 |
| 33 | Low | 10 |  |  | 6 |  | QPSK |  | 40@9 |
| 34 | Low | 10 |  |  | 4 |  | QPSK |  | 1@35 |
| 35 | Low | 10 |  |  | 2 |  | QPSK |  | 16@35 |
| 36 | Low | 10 |  |  | 5 |  | QPSK |  | Edge_1RB_Right |
| 37 | Low | 10 |  |  | 5 |  | QPSK |  | 1@40 |
| 38 | Low | 10 |  |  | 3 |  | QPSK |  | 8@44 |
| 39 | Low | 10 |  |  | 12 |  | 16 QAM |  | Edge_1RB_Left |
| 40 | Low | 10 |  |  | 8 |  | 16 QAM |  | Outer_Full |
| 41 | Low | 10 |  |  | 6 |  | 16 QAM |  | 1@3 |
| 42 | Low | 10 |  |  | 6 |  | 16 QAM |  | 40@9 |
| 43 | Low | 10 |  |  | 4 |  | 16 QAM |  | 1@35 |
| 44 | Low | 10 |  |  | 2 |  | 16 QAM |  | 16@35 |
| 45 | Low | 10 |  |  | 5 |  | 16 QAM |  | Edge_1RB_Right |
| 46 | Low | 10 |  |  | 5 |  | 16 QAM |  | 1@40 |
| 47 | Low | 10 |  |  | 3 |  | 16 QAM |  | 8@44 |
| 48 | Low | 10 |  |  | 12 |  | 64 QAM |  | Edge_1RB_Left Outer_Full |
| 49 | Low | 10 |  |  | 8 |  | 64 QAM |  |  |
| 50 | Low | 10 |  |  | 6 |  | 64 QAM |  | 1@3 40@9 |
| 51 | Low | 10 |  |  | 6 |  | 64 QAM |  |  |
| 52 | Low | 10 |  |  | 4 |  | 64 QAM |  | 1@35 |
| 53 | Low | 10 |  |  | 2 |  | 64 QAM |  | 16@35 |
| 54 | Low | 10 |  |  | 5 |  | 64 QAM |  | Edge_1RB_Right |
| 55 | Low | 10 |  |  | 5 |  | 64 QAM |  | 1@40 |
| 56 | Low | 10 |  |  | 3 |  | 64 QAM |  | 8@44 |
| 57 | Low | 10 |  |  | 12 |  | 256 QAM |  | Edge_1RB_Left |
| 58 | Low | 10 |  |  | 8 |  | 256 QAM |  | Outer_Full |
| 59 | Low | 10 |  |  | 6 |  | 256 QAM |  | 1@3 |
| 60 | Low | 10 |  |  | 6 |  | 256 QAM |  | 40@9 |
| 61 | Low | 10 |  |  | 4 |  | 256 QAM |  | 1@35 |
| 62 | Low | 10 |  |  | 2 |  | 256 QAM |  | 16@35 |
| 63 | Low | 10 |  |  | 5 |  | 256 QAM |  | Edge_1RB_Right |
| 64 | Low | 10 |  |  | 5 |  | 256 QAM |  | 1@40 |
| 65 | Low | 10 |  |  | 3 |  | 256 QAM |  | 8@44 |
| 66 | Low | 5 | N/A for A-MPR testing | N/A | 14 | CP-OFDM | QPSK |  | Edge_1RB_Left |
| 67 | Low | 5 |  |  | 6 |  | QPSK |  | Outer_Full |
| 68 | Low | 5 |  |  | 4 |  | QPSK |  | 21@4 |
| 69 | Low | 5 |  |  | 4 |  | QPSK |  | 1@4 |
| 70 | Low | 5 |  |  | 14 |  | 16 QAM |  | Edge_1RB_Left |
| 71 | Low | 5 |  |  | 6 |  | 16 QAM |  | Outer_Full |
| 72 | Low | 5 |  |  | 4 |  | 16 QAM |  | 21@4 |
| 73 | Low | 5 |  |  | 4 |  | 16 QAM |  | 1@4 |
| 74 | Low | 5 |  |  | 14 |  | 64 QAM |  | Edge_1RB_Left |
| 75 | Low | 5 |  |  | 6 |  | 64 QAM |  | Outer_Full |
| 76 | Low | 5 |  |  | 4 |  | 64 QAM |  | 21@4 |
| 77 | Low | 5 |  |  | 4 |  | 64 QAM |  | 1@4 |
| 78 | Low | 5 |  |  | 14 |  | 256 QAM |  | Edge_1RB_Left |
| 79 | Low | 5 |  |  | 6 |  | 256 QAM |  | Outer_Full |
| 80 | Low | 5 |  |  | 4 |  | 256 QAM |  | 21@4 |
| 81 | Low | 5 |  |  | 4 |  | 256 QAM |  | 1@4 |
| 82 | Low | 10 |  |  | 12 |  | QPSK |  | Edge_1RB_Left |
| 83 | Low | 10 |  |  | 8 |  | QPSK |  | Outer_Full |
| 84 | Low | 10 |  |  | 6 |  | QPSK |  | 1@3 |
| 85 | Low | 10 |  |  | 6 |  | QPSK |  | 43@9 |
| 86 | Low | 10 |  |  | 4 |  | QPSK |  | 1@35 |
| 87 | Low | 10 |  |  | 2 |  | QPSK |  | 17@35 |
| 88 | Low | 10 |  |  | 5 |  | QPSK |  | Edge_1RB_Right |
| 89 | Low | 10 |  |  | 5 |  | QPSK |  | 1@40 |
| 90 | Low | 10 |  |  | 3 |  | QPSK |  | 8@44 |
| 91 | Low | 10 |  |  | 12 |  | 16 QAM |  | Edge_1RB_Left |
| 92 | Low | 10 |  |  | 8 |  | 16 QAM |  | Outer_Full |
| 93 | Low | 10 |  |  | 6 |  | 16 QAM |  | 1@3 |
| 94 | Low | 10 |  |  | 6 |  | 16 QAM |  | 43@9 |
| 95 | Low | 10 |  |  | 4 |  | 16 QAM |  | 1@35 |
| 96 | Low | 10 |  |  | 2 |  | 16 QAM |  | 17@35 |
| 97 | Low | 10 |  |  | 5 |  | 16 QAM |  | Edge_1RB_Right |
| 98 | Low | 10 |  |  | 5 |  | 16 QAM |  | 1@40 |
| 99 | Low | 10 |  |  | 3 |  | 16 QAM |  | 8@44 |
| 100 | Low | 10 |  |  | 12 |  | 64 QAM |  | Edge_1RB_Left |
| 101 | Low | 10 |  |  | 8 |  | 64 QAM |  | Outer_Full |
| 102 | Low | 10 |  |  | 6 |  | 64 QAM |  | 1@3 |
| 103 | Low | 10 |  |  | 6 |  | 64 QAM |  | 43@9 |
| 104 | Low | 10 |  |  | 4 |  | 64 QAM |  | 1@35 |
| 105 | Low | 10 |  |  | 2 |  | 64 QAM |  | 17@35 |
| 106 | Low | 10 |  |  | 5 |  | 64 QAM |  | Edge_1RB_Right |
| 107 | Low | 10 |  |  | 5 |  | 64 QAM |  | 1@40 |
| 108 | Low | 10 |  |  | 3 |  | 64 QAM |  | 8@44 |
| 109 | Low | 10 |  |  | 12 |  | 256 QAM |  | Edge_1RB_Left |
| 110 | Low | 10 |  |  | 8 |  | 256 QAM |  | Outer_Full |
| 111 | Low | 10 |  |  | 6 |  | 256 QAM |  | 1@3 |
| 112 | Low | 10 |  |  | 6 |  | 256 QAM |  | 43@9 |
| 113 | Low | 10 |  |  | 4 |  | 256 QAM |  | 1@35 |
| 114 | Low | 10 |  |  | 2 |  | 256 QAM |  | 17@35 |
| 115 | Low | 10 |  |  | 5 |  | 256 QAM |  | Edge_1RB_Right |
| 116 | Low | 10 |  |  | 5 |  | 256 QAM |  | 1@40 |
| 117 | Low | 10 |  |  | 3 |  | 256 QAM |  | 8@44 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------