3GPP TSG-RAN5 Meeting #93-e	R5-216561r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2497 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MDT test case 8.1.6.1.2.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Cell power levels for FR2 are FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated cell power levels for FR2 in table 8.1.6.1.2.10.3.2-2 and corrected typos in main procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Corrected editorial mistakes in step 4 and step 17. |  |  |  |  |  |  |  |  |

8.1.6.1.2.10	Logged MDT / Maintaining logged measurement configuration / UE mobility

8.1.6.1.2.10.1	Test Purpose (TP)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE reselected to a cell belong to non MDT PLMN}

then { UE does not indicate availability of Logged MDT measurements}

}

(2)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport}

ensure that {

when { UE reselects to a cell belong to non MDT PLMN}

then { UE suspends Logged MDT measurements}

}

(3)

with { UE received LoggedMeasurementConfiguration message and has reselected to a cell belong to non MDT PLMN }

ensure that {

when { UE returns to a cell belong to MDT PLMN}

then { UE indicate availability of Logged MDT measurements }

}

(4)

with { UE received LoggedMeasurementConfiguration message and has reselected to a cell belong to non MDT PLMN }

ensure that {

when { UE returns to a cell belong to MDT PLMN}

then { UE resumes Logged MDT measurements }

}

8.1.6.1.2.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3 and 5.7.10.3. [TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

- set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.2.10.3	Test description

8.1.6.1.2.10.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 3, NR Cell 11

-	System information combination NR-3 as defined in TS 38.508-1[4] clause 4.4.3.1.2 is used in NR cells.

-	When NR Cell 3 is used its PLMN will be set to PLMN2

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.6.1.2.10.3.2	Test procedure sequence

Table 8.1.6.1.2.10.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.10.3.2-1: Time instances of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | NR Cell 3 |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | Off | Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | Off | -85 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | -85 | Off |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. |  |  |  |  |  |

Table 8.1.6.1.2.10.3.2-2: Time instances of cell power level for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | NR Cell 3 |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Off | Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | Off | -82 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | -82 | Off |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. |  |  |  |  |  |

Table 8.1.6.1.2.10.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 10 seconds for UE performing the logging at regular time intervals | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 3 level according to the row "T1" in Table 8.1.6.1.2.10.3.2-1/2. | - | - | - | - |
| 5-10 | Steps 1 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 3. | - | - | - | - |
| 11 | Wait 10 seconds for UE to perform the logging at regular time intervals. | - | - | - | - |
| 12 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 13 | The UE transmits an RRCRequest message. | --> | NR RRC: RRCRequest | - | - |
| 14 | The SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 15 | Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? | --> | NR RRC: RRCSetupComplete | 1 | P |
| 16 | The SS transmits a RRCRelease message to release RRC connection and move to RRC_IDLE on Cell 3. | <-- | NR RRC: RRCRelease | - | - |
| 17 | The SS changes Cell 1, Cell 11 and Cell 3 level according to the row "T2" in Table 8.1.6.1.2.10.3.2-1/2. | - | - | - | - |
| 18-23 | Steps 1 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 11 | - | - | - | - |
| 24 | Wait 10 seconds for UE performing the logging at regular time intervals |  |  |  |  |
| 25 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 26 | The UE transmits an RRCRequest message. | --> | NR RRC: RRCRequest | - | - |
| 27 | The SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 28 | Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable set as true? | --> | NR RRC: RRCSetupComplete | 3 | P |
| 29-31 | Steps 6 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 32 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 33 | Check: Does the UE send an UEInformationResponse message including at least one logMeasReport with serving cell measurements for Cell 1 and Cell 11 and without serving cell measurements for Cell 3? | --> | NR RRC: UEInformationResponse | 2, 4, | P |

8.1.6.1.2.10.3.3	Specific message contents

Table 8.1.6.1.2.10.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.10.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-5AA LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| LoggedMeasurementConfiguration SEQUENCE { |  |  |  |  |
| reportType CHOICE { |  |  |  |  |
| periodical SEQUENCE { |  |  |  |  |
| loggingInterval-r16 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.2.10.3.3-2: RRCSetupComplete (step 28, Table 8.1.6.1.2.10.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-23 RRCSetupComplete |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension { |  |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |  |
| logMeasAvailable-r16 |  | True |  |  |
| logMeasAvailableBT-r16 |  | Not Present |  |  |
| logMeasAvailableWLAN-r16 |  | Not Present |  |  |
| connEstFailInfoAvailable-r16 |  | Not Present |  |  |
| rlf-InfoAvailable-r16 |  | Not Present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.2.10.3.3-3: UEInformationRequest (step 32, Table 8.1.6.1.2.10.3.2-3)

Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-32A UEInformationRequest with condition LOG

Table 8.1.6.1.2.10.3.3-4: UEInformationResponse (step 33, Table 8.1.6.1.2.10.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-32B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions{ |  |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF SEQUENCE { |  | At least 2 entries where at least one entry complies to entry with index ‘x’ below. At least 1 entry with “servCellIdentity-r16” set to Cell 1 and at least 1 entry with “servCellIdentity-r16” set to Cell 11. |  |  |
| locationInfo-r16[x] |  | Not checked |  |  |
| relativeTimeStamp-r16 [x] |  | SS record the value |  |  |
| servCellIdentity-r16 [x] |  | Same as Cell 1 or Cell 11 |  |  |
| measResultServCell-16 [x] SEQUENCE { |  |  |  |  |
| resultsSSB-Cell-r16 |  | MeasQuantityResults of Cell 1 or Cell 11 |  |  |
| resultsSSB SEQUENCE { |  |  |  |  |
| best-ssb-Index |  | Not checked |  |  |
| best-ssb-Results |  | Not checked |  |  |
| numberOfGoodSSB |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells-r16 [x] SEQUENCE { |  |  |  |  |
| MeasResultLogging2NR-r16 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |  |
| carrierFreq-r16[x] |  | Same as Cell 11 or Cell 1, different with servCellIdentity-r16 in the entry. |  |  |
| MeasResultListLoggingNR -r16[x] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |
| physCellId |  | Same as Cell 11 or Cell1, different with servCellIdentity-r16 in the entry. |  |  |
| resultsSSB-Cell-r16 |  | MeasQuantityResults of Cell 11 or Cell 1 |  |  |
| numberOfGoodSSB-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListEUTRA-r16 |  | Not present |  |  |
| } |  |  |  |  |
| anyCellSelectionDetected-r16[x] |  | Not present |  |  |
| } |  |  |  |  |
| logMeasAvailable-r16 |  | Not present |  |  |
| logMeasAvailableBT-r16 |  | Not present |  |  |
| logMeasAvailableWLAN-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |