3GPP TSG-RAN5 Meeting #93-e	R5-216565r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0233 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new 5GS IMS test case 8.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New 5GS IMS test case 8.2 need to be added according to WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 8.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1 : Added comments "the "active" attribute may not be present but if present it is set to "true"" in Table 8.2.3.3-1. Corrected editorial mistakes in main behavior table. Added reference number of TS38.508-1. Added initial Simservs document for SS in step 4. |  |  |  |  |  |  |  |  |

8.2	Originating Identification Restriction / Configuration / 5GS

8.2.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to activate OIR }

then { UE authenticates itself using GBA or Digest }

}

(2)

with { UE having started authentication }

ensure that {

when { UE receives 200 OK concluding the authentication }

then { UE sends HTTP request to activate OIR }

}

(3)

with { UE having concluded activation of OIR }

ensure that {

when { UE is made to de-activate OIR }

then { UE sends HTTP request to de-activate OIR }

}

8.2.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.607 clause 4.2.1]:

The OIR service is a service offered to the originating user. It restricts presentation of the originating user's identity information to the terminating user.

When the OIR service is applicable and activated, the originating network provides the destination network with the indication that the originating user's identity information is not allowed to be presented to the terminating user. In this case, no originating user's identity information shall be included in the requests sent to the terminating user. The presentation restriction function shall not influence the forwarding of the originating user's identity information within the network as part of the simulation service procedures.

[TS 24.607 clause 4.10.1]:

The OIR service can be activated/deactivated using the active attribute of the <originating-identity-presentation-restriction> service element. Activating the OIR service this way activates the temporary mode OIR service. When deactivated and not overruled by operator settings, basic communication procedures apply.

8.2.3	Test description

8.2.3.1	Pre-test conditions

System Simulator:

-	SS is configured with shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE.

-	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	At the SS, a HTTP Server is established at port 80 to simulate the XCAP server

-	1 NR Cell

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured with the name of the XCAP root directory on the XCAP server and the user's directory name.

-	UE has activated an IPCAN bearer with SS.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1[21]) and registered to IMS.

-	The UE has established a PDN connectivity for IMS XCAP signalling. The UE may either be configured to re-use the Internet APN for XCAP signalling or the UE uses a specific XCAP-only APN

-	During these procedures the UE may request a DNS server address via NAS signalling and as parallel behaviour the UE may resolve the IP address of the XCAP server via DNS.

8.2.3.2	Test procedure sequence

Table 8.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered for activation of supplementary service OIR | - | - | - | - |
| 2-5b | Check: Does the UE perform steps 2-5b of the generic test procedure for activation of Supplementary Services according to annex A.21 | - | - | 1 | - |
| 6 | Check: Does the Simservs document stored in the SS contain the information supplied by UE as according to table 8.2.3.3-1? | - | - | 2 | P |
| 7 | UE is triggered for deactivation of supplementary service OIR | - | - | - | - |
| 8-8b | Check: Does the UE perform steps 8-8b of the generic test procedure for activation of Supplementary Services according to annex A.21? | - | - | 3 | - |
| 9 | Check: Does the Simservs document stored in the SS contain the information supplied by UE as according to table 8.2.3.3-2? | - | - | 3 | P |

8.2.3.3	Specific message contents

Table 8.2.3.3-1: Simservs document (step 4)

| Derivation Path: TS 24.607 clause 4.10.2 |  |
| --- | --- |
| Content | Comment |
| <simservs> |  |
| <originating-identity-presentation-restriction active="false"> |  |
| </simservs> |  |

Table 8.2.3.3-2: Simservs document (step 6)

| Derivation Path: TS 24.607 clause 4.10.2 |  |
| --- | --- |
| Content | Comment |
| <simservs> |  |
| <originating-identity-presentation-restriction active="true"> | the "active" attribute may not be present but if present it is set to "true" |
| </simservs> |  |

Table 8.2.3.3-3: Simservs document (step 9)

| Derivation Path: TS 24.607 clause 4.10.2 |  |
| --- | --- |
| Content | Comment |
| <simservs> |  |
| <originating-identity-presentation-restriction active="false"> |  |
| </simservs> |  |