3GPP TSG-RAN5 Meeting #93-e	R5-216574r3

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0242 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new 5GS IMS test case 8.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New 5GS IMS test case 8.13 need to be added according to WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 8.13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: corrected table number of Simservs document in sepcific message contents. Revison 2: Updated comments in specific message contents. Revision 3: Corrected step number in specific message contents. |  |  |  |  |  |  |  |  |

8.13	Communication Forwarding on Subscriber Not Reachable / Configuration / 5GS

8.13.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to activate CFNRc }

then { UE authenticates itself using GBA or Digest }

}

(2)

with { UE having started authentication }

ensure that {

when { UE receives 200 OK concluding the authentication }

then { UE sends HTTP request to activate CFNRc }

}

(3)

with { UE having concluded activation of CFNRc }

ensure that {

when { UE is made to de-activate CFNRC }

then { UE sends HTTP request to de-activate CFNRc }

}

8.13.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

Communication Forwarding on Subscriber Not Reachable (CFNRc)

The CFNRc service enables an user to have the network redirect all incoming communications, when the user is not reachable (e.g. there is no IP connectivity to the user's terminal), to another user. The CFNRc service may operate on all communications, or just those associated with specified services. The user's ability to originate communications is unaffected by the CFNRc simulation service.

As a service provider option, a subscription option can be provided to enable the user to receive an indication that the CFNRc service has been activated. This indication may be provided when the user originates a communication if the CFNRc service has been activated for the user and for the service requested for the communication.

The maximum number of diversions permitted for each communication is a service provider option. The service provider shall define the upper limit of diversions. When counting the number of diversions, all types of diversion are included.

8.13.3	Test description

8.13.3.1	Pre-test conditions

System Simulator:

-	SS is configured with shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE.

-	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	At the SS, a HTTP Server is established at port 80 to simulate the XCAP server

-	1 NR Cell

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured with the name of the XCAP root directory on the XCAP server and the user's directory name.

-	UE has activated an IPCAN bearer with SS.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1) and registered to IMS.

-	The UE has established a PDN connectivity for IMS XCAP signalling. The UE may either be configured to re-use the Internet APN for XCAP signalling or the UE uses a specific XCAP-only APN

-	During these procedures the UE may request a DNS server address via NAS signalling and as parallel behaviour the UE may resolve the IP address of the XCAP server via DNS.

8.13.3.2	Test procedure sequence

Table 8.13.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered for activation of supplementary service CFNRC | - | - | - | - |
| 2-5b | Check: Does the UE perform steps 2-5b of the generic test procedure for activation of Supplementary Services according to annex A.21 | - | - | 1 | - |
| 6 | Check: Does the Simservs document stored in the SS contain the information supplied by UE as according to table 8.13.3.3-1? | - |  | 2 | P |
| 7 | UE is triggered for deactivation of supplementary service CFNRC | - | - | - | - |
| 8-8b | Check: Does the UE perform steps 8 – 8b of the generic test procedure for activation of Supplementary Services according to annex A.21 | - | - | 3 | - |
| 9 | Check: Does the Simservs document stored in the SS contain the information supplied by UE as according to table 8.13.3.3-2 or 8.13.3.3-3? | - | - | 3 | P |

8.13.3.3	Specific message contents

Table 8.13.3.3-1: Simservs document (step 5b)

| Derivation Path: TS 24.604 clause 4.9.2 |  |
| --- | --- |
| Content | Further restrictions/Comments |
| <simservs xmlns=http://uri.etsi.org/ngn/params/xml/simservs/xcap xmlns:cp="urn:ietf:params:xml:ns:common-policy" xmlns:ocp="urn:oma:xml:xdm:common-policy"> |  |
| <communication-diversion active="true"> | the "active" attribute may not be present but if present it is set to "true" |
| <cp:ruleset> |  |
| <cp:rule id=rule1> |  |
| <cp:conditions> |  |
| <not-reachable/> |  |
| <rule-deactivated/> | containing a <rule-deactivated> element |
| </cp:conditions> |  |
| <cp:actions> |  |
| <forward-to> |  |
| <target> px_XCAP_TargetUri</target> |  |
| <notify-caller>true</notify-caller> |  |
| </forward-to> |  |
| </cp:actions> |  |
| </cp:rule> |  |
| </cp:ruleset> |  |
| </communication-diversion> |  |
| </simservs> |  |

Table 8.13.3.3-2: Simservs document (step 6)

| Derivation Path: TS 24.604 clause 4.9.2 |  |
| --- | --- |
| Content | Further restrictions/Comments |
| <simservs xmlns=http://uri.etsi.org/ngn/params/xml/simservs/xcap xmlns:cp="urn:ietf:params:xml:ns:common-policy" xmlns:ocp="urn:oma:xml:xdm:common-policy"> |  |
| <communication-diversion active="true"> | the "active" attribute may not be present but if present it is set to "true" |
| <cp:ruleset> |  |
| <cp:rule id=rule1> |  |
| <cp:conditions> | not containing a <rule-deactivated> element |
| <not-reachable/> |  |
| </cp:conditions> |  |
| <cp:actions> |  |
| <forward-to> |  |
| <target> px_XCAP_TargetUri</target> |  |
| <notify-caller>true</notify-caller> |  |
| </forward-to> |  |
| </cp:actions> |  |
| </cp:rule> |  |
| </cp:ruleset> |  |
| </communication-diversion> |  |
| </simservs> |  |

Table 8.13.3.3-3: Simservs document (step 9)

| Derivation Path: TS 24.604 clause 4.9.2 |  |
| --- | --- |
| Content | Further restrictions/Comments |
| <simservs xmlns=http://uri.etsi.org/ngn/params/xml/simservs/xcap xmlns:cp="urn:ietf:params:xml:ns:common-policy" xmlns:ocp="urn:oma:xml:xdm:common-policy"> |  |
| <communication-diversion active="true"> | the "active" attribute may not be present but if present it is set to "true" |
| <cp:ruleset> |  |
| <cp:rule id=rule1> |  |
| <cp:conditions> |  |
| <not-reachable/> |  |
| <rule-deactivated/> | containing a <rule-deactivated> element |
| </cp:conditions> |  |
| <cp:actions> |  |
| <forward-to> |  |
| <target> px_XCAP_TargetUri</target> |  |
| <notify-caller>true</notify-caller> |  |
| </forward-to> |  |
| </cp:actions> |  |
| </cp:rule> |  |
| </cp:ruleset> |  |
| </communication-diversion> |  |
| </simservs> |  |