3GPP TSG-RAN5 Meeting #93-e	R5-216579r2

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0247 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new 5GS IMS test case 8.22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New 5GS IMS test case 8.22 need to be added according to WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 8.22 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.22(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Updated conformance requirements, test procedure and specific message contents. Revision 2: Corrected step number in specific message contents. |  |  |  |  |  |  |  |  |

8.22	Barring of Outgoing International Calls / Configuration / 5GS

8.22.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to activate barring of all Outgoing Calls (OCB intl) }

then { UE authenticates itself using GBA or Digest }

}

(2)

with { UE having started authentication }

ensure that {

when { UE receives 200 OK concluding the authentication }

then { UE sends HTTP request to activate  OCB intl }

}

(3)

with { UE having concluded activation of OCB intl }

ensure that {

when { UE is made to de-activate OCB intl }

then { UE sends HTTP request to de-activate OCB intl }

}

8.22.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.611, clause 4.2.1]:

The Outgoing Communication Barring (OCB) service makes it possible for a user to have barring of certain categories of outgoing communications according to a provisioned or user configured barring program and is valid for all outgoing communications. A barring program is expressed as a set of rules in which the rules have a conditional part and an action part. An example condition is whether the request uri matches a specific public user identity. The action part can specify for a rule that contains a matching condition that the specific outgoing communication it to be barred. The complete set of conditions and actions that apply to this service and their semantics is described in clause 4.9.

[TS 24.611, clause 4.5.2.4.1]:

The AS providing the OCB service shall operate as either an AS acting as a SIP proxy as specified in clause 5.7.4 of 3GPP TS 24.229 [2] or an AS providing 3rd party call control, and specifically as a routeing B2BUA, as specified in clause 5.7.5 of 3GPP TS 24.229 [2]. An AS providing the OCB service and rejecting the request shall operate as a terminating UA, as specified in clause 5.7.2 of 3GPP TS 24.229 [2].

NOTE:	For the case when the session is not subject to OCB according the requirements in this document, and is the only service being applied by the AS, then the AS only needs to act as a SIP proxy. If additional services are applied, then the AS might need to act as a routeing B2BUA.

The AS providing the OCB service shall reject outgoing communications when the evaluation of the served users outgoing communication barring rules according to the algorithm as specified in clause 4.9.1.2 evaluates to (allow="false"),.Outgoing communications towards emergency services are always allowed irrespective of what barring settings the user has defined. To allow emergency calls to go through, the operator creates an allow list, as described in clause 4.9.1.3, including emergency numbers in any useful format including emergency service URNs specified in RFC 5031[23]. For the purpose of OCB, the AS shall evaluate the "cp:identity" and "ocp:external-list" conditions against the called party identity taken from Request-URI or additionally taken from the To header field.

When the AS providing the OCB service rejects a communication, the AS shall send an indication to the calling user by sending a 603 (Decline) response Additionally, before terminating the communication the AS can provide an announcement to the originating user. The procedure of invoking an announcement is described within 3GPP TS 24.628 [10].

[TS 24.611, clause 4.9.1.4]:

international: This condition evaluates to true when the request URI of the outgoing SIP request:

-	corresponds to a telephone number, i.e. a SIP URI with a "user" URI parameter set to "phone" or a tel URI; and

-	does not point to a destination served by a network within the country where the originating user is located when initiating the communication.

8.22.3	Test description

8.22.3.1	Pre-test conditions

System Simulator:

-	SS is configured with shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE.

-	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	At the SS, a HTTP Server is established at port 80 to simulate the XCAP server

-	1 NR Cell

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured with the name of the XCAP root directory on the XCAP server and the user's directory name.

-	UE has activated an IPCAN bearer with SS.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1[21]) and registered to IMS.

-	The UE has established a PDN connectivity for IMS XCAP signalling. The UE may either be configured to re-use the Internet APN for XCAP signalling or the UE uses a specific XCAP-only APN

-	During these procedures the UE may request a DNS server address via NAS signalling and as parallel behaviour the UE may resolve the IP address of the XCAP server via DNS.

8.22.3.2	Test procedure sequence

Table 8.22.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered for activation of supplementary service OCB intl. | - | - | - | - |
| 2-5b | Check: Does the UE perform steps 2-5b of the generic test procedure for activation of Supplementary Services according to annex A.21? | - | - | 1 | - |
| 6 | Check: Does the Simservs document stored in the SS contain the information supplied by UE as according to table 8.22.3.3-1? | - | - | 2 | P |
| 7 | UE is triggered for deactivation of supplementary service OCB intl. | - | - | - | - |
| 8-8b | Check: Does the UE perform steps 8-8b of the generic test procedure for activation of Supplementary Services according to annex A.21? | - | - | 3 | - |
| 9 | Check: Does the Simservs document stored in the SS contain the information supplied by UE as according to table 8.22.3.3-2 or table 8.22.3.3-3? | - | - | 3 | P |

8.22.3.3	Specific message contents

Table 8.22.3.3-1: Simservs document (step 5b)

| Derivation Path: TS 24.611 clause 4.9.2 |  |
| --- | --- |
| Content | Further restrictions/Comments |
| <simservs xmlns=http://uri.etsi.org/ngn/params/xml/simservs/xcap xmlns:cp="urn:ietf:params:xml:ns:common-policy" xmlns:ocp="urn:oma:xml:xdm:common-policy"> |  |
| <outcoming-communication-barring active=true> | the "active" attribute may not be present but if present it is set to "true" |
| <cp:ruleset> |  |
| <cp:rule id=rule1> |  |
| <cp:conditions> |  |
| <international/> |  |
| <rule-deactivated/> | containing a <rule-deactivated> element |
| </cp:conditions> |  |
| <cp:actions> |  |
| <allow>false</allow> |  |
| </cp:actions> |  |
| </cp:rule> |  |
| </cp:ruleset> |  |
| </outcoming-communication-barring> |  |
| </simservs> |  |

Table 8.22.3.3-2: Simservs document (step 6)

| Derivation Path: TS 24.611 clause 4.9.2 |  |
| --- | --- |
| Content | Further restrictions/Comments |
| <simservs xmlns=http://uri.etsi.org/ngn/params/xml/simservs/xcap xmlns:cp="urn:ietf:params:xml:ns:common-policy" xmlns:ocp="urn:oma:xml:xdm:common-policy"> |  |
| < outcoming-communication-barring active="true" > | the "active" attribute may not be present but if present it is set to "true" |
| <cp:ruleset> |  |
| <cp:rule id=rule1> | list of conditions not containing a <rule-deactivated> element |
| <cp:conditions> |  |
| <international/> |  |
| </cp:conditions> |  |
| <cp:actions> |  |
| <allow>false</allow> |  |
| </cp:actions> |  |
| </cp:rule> |  |
| </cp:ruleset> |  |
| </outcoming-communication-barring> |  |
| </simservs> |  |

Table 8.22.3.3-3: Simservs document (step 9)

| Derivation Path: TS 24.611 clause 4.9.2 |  |
| --- | --- |
| Content | Further restrictions/Comments |
| <simservs xmlns=http://uri.etsi.org/ngn/params/xml/simservs/xcap xmlns:cp="urn:ietf:params:xml:ns:common-policy" xmlns:ocp="urn:oma:xml:xdm:common-policy"> |  |
| <outcoming-communication-barring active=true> | the "active" attribute may not be present but if present it is set to "true" |
| <cp:ruleset> |  |
| <cp:rule id=rule1> |  |
| <cp:conditions> |  |
| <international/> |  |
| <rule-deactivated/> | containing a <rule-deactivated> element |
| </cp:conditions> |  |
| <cp:actions> |  |
| <allow>false</allow> |  |
| </cp:actions> |  |
| </cp:rule> |  |
| </cp:ruleset> |  |
| </outcoming-communication-barring> |  |
| </simservs> |  |