3GPP TSG-RAN5 Meeting #93-e	R5-216590r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0006 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Test Case - 6.3.1 MCData Enhanced Status CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A test case for Enhanced Status – Client Originated did not exist and the feature needs to be tested |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Added clause 6.3 2) Added new test cases - 6.3.1 On-network / Enhanced Status (ES) / Client Originated (CO) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Enhanced Status feature may not work properly in commercial networks |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Corrected the release of the conformance requirements to Rel-15 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 6.3	Enhanced Status (ES)

### 6.3.1	On-network / Enhanced Status (ES) / Client Originated (CO)

6.3.1.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send an Enahnced Status with a disposition of only Delivery }

then { UE (MCDATA Client) sends an Enahnced Status with a disposition request of only Delivery via s SIP MESSAGE message }

}

(2)

with { UE (MCDATA Client) having sent an Enahnced Status with a disposition request of DELIVERY }

ensure that {

when { the UE (MCDATA Client) receives a disposition response via a SIP MESSAGE message from the SS (MCDATA Server) }

then { UE (MCData Client) responds to the SIP MESSAGE message by sending a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

6.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 14.2.1.1, 9.2.2.2.1, 6.2.2.1, 6.2.4.1, 12.2.1.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-15 requirements.

[TS 24.282, clause 14.2.1.1]

Upon receiving a request from the MCData user to send an enhanced status to an MCData group and the <mcdata-allow-enhanced-status> element under the <list-service> element as defined in 3GPP TS 24.481 [11] is set to "true", the MCData client:

1)	shall use the "id" attribute of the MCData user selected operation value from <mcdata-enhanced-status-operational-values> element under <list-service> element as defined in 3GPP TS 24.481 [11], to generate a group standalone SDS message by following the procedure described in clause 9.2.2.2.1.

[TS 24.282, clause 9.2.2.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

...

3)	if a group standalone SDS message is to be sent:

a)	if the "/<x>/<x>/Common/MCData/AllowedSDS" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request to send SDS and not continue with the rest of the steps in this subclause; and

b)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with:

i)	the <request-type> element set to a value of "group-sds";

ii)	the <mcdata-request-uri> element set to the MCData group identity; and

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client;

4)	shall generate a standalone SDS message as specified in subclause 6.2.2.1; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 6.2.2.1]

In order to generate an SDS message, the MCData client:

1)	shall generate an SDS SIGNALLING PAYLOAD message as specified in clause 15.1.2;

2)	shall generate a DATA PAYLOAD message as specified in clause 15.1.4;

3)	shall include in the SIP request, the SDS SIGNALLING PAYLOAD message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in clause E.1; and

4)	shall include in the SIP request, the DATA PAYLOAD message in an application/vnd.3gpp.mcdata-payload MIME body as specified in clause E.2.

When generating an SDS SIGNALLING PAYLOAD message as specified in clause 15.1.2, the MCData client:

1)	shall set the Date and time IE to the current time as specified in clause 15.2.8;

2)	if the SDS message starts a new conversation, shall set the Conversation ID IE to a newly generated Conversation ID value as specified in clause 15.2.9;

3)	if the SDS message continues an existing unfinished conversation, shall set the Conversation ID IE to the Conversation ID value of the existing conversation as specified in clause 15.2.9;

4)	shall set the Message ID IE to a newly generated Message ID value as specified in clause 15.2.10;

5)	if the SDS message is in reply to a previously received SDS message, shall include the InReplyTo message ID IE with the Message ID value in the previously received SDS message;

6)	if the SDS message is for user consumption, shall not include an Application ID IE as specified in clause 15.2.7and shall not include an Extended application ID IE as specified in clause 15.2.24;

7)	if the SDS message is intended for an application on the terminating MCData client, shall include:

a)	an Application ID IE with a Application ID value representing the intended application as specified in clause 15.2.7; or

b)	an Extended application ID IE with an Extended application ID value representing the intended application as specified in clause 15.2.24;

NOTE:	The value chosen for the Application ID value is decided by the mission critical organisation.

8)	if only a delivery disposition notification is required shall include a SDS disposition request type IE set to "DELIVERY" as specified in clause 15.2.3;

9)	if only a read disposition notification is required shall include a SDS disposition request type IE set to "READ" as specified in clause 15.2.3; and

10)	if both a delivery and read disposition notification is required shall include a SDS disposition request type IE set to "DELIVERY AND READ" as specified in clause 15.2.3.

When generating an DATA PAYLOAD message for SDS as specified in clause 15.1.4, the MCData client:

1)	shall set the Number of payloads IE to the number of Payload IEs that needs to be encoded, as specified in clause 15.2.12;

2)	if end-to-end security is required for a one-to-one communication, shall include the Security parameters and Payload IE with security parameters as described in 3GPP TS 33.180 [26]. Otherwise, if end-to-end security is not required for a one-to-one communication, shall include the Payload IE as specified in clause 15.1.4; and

3)	for each Payload IE included:

a)	if the payload is text, shall set the Payload content type as "TEXT" as specified in clause 15.2.13;

b)	if the payload is binary data, shall set the Payload content type as "BINARY" as specified in clause 15.2.13;

c)	if the payload is hyperlinks, shall set the Payload content type as "HYPERLINKS" as specified in clause 15.2.13;

d)	if the payload is location, shall set the Payload content type as "LOCATION" as specified in clause 15.2.13;

e)	if payload is enhanced status for a group, shall set the Payload content type as "ENHANCED STATUS" as specified in subclase 15.2.13; and

f)	shall include the data to be sent in the Payload data.

[TS 24.282, clause 6.2.4.1]

This subclause is referenced from other procedures.

In a SIP MESSAGE request, the MCData client:

1)	when sending SDS messages or SDS disposition notifications:

a)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

b)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8]; and

c)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

…

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [5]; and

4)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the MCData user.

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

6.3.1.3	Test description

6.3.1.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	The <max-payload-size-sds-cplane-bytes> element in the MCData service configuration document as specified in 3GPP TS 24.484 [12], shall be large enough to allow the sending of the standalone SDS message using the signalling plane.

Preamble:

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.3.1.3.2	Test procedure sequence

Table 6.3.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCDATA User request to send an enhanced status to Group A using Enhanced Status Id "1" with a disposition request type of "DELIVERY". NOTE: This is expected to be done via a suitable implementation dependent MMI. | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCDATA call establishment are described in TS 36.579-1 [2], subclause 5.4.3B 'Generic Test Procedure for MCData CO communication in E-UTRA'. The test sequence below shows only the MCDATA relevant messages exchanged. | - | - | - | - |
| 2 | Check: Does the UE (MCDATA Client) send an Enhanced Status with Enhanced Status Id "1" via a SIP MESSAGE message? | --> | SIP MESSAGE | 1 | P |
| 3 | SS (MCDATA Server) responds with a SIP 202 (Accepted) message | <-- | SIP 202 (Accepted) | - | - |
| 4 | SS (MCDATA Server) sends a disposition notification type of "DELIVERED" in response to the standalone one-to-one SDS message. | <-- | SIP MESSAGE | - | - |
| 5 | Check: Does the UE (MCDATA Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | 2 | P |
| - | EXCEPTION: SS (MCDATA Server) releases the E-UTRA connection. | - | - | - | - |
| 6 | Check: Does the UE (MCDATA Client) deliver the notification the MCDATA User? NOTE: This is expected to be done via a suitable implementation dependent MMI. | - | - | 2 | P |

6.3.1.3.3	Specific message contents

Table 6.3.1.3.3-1: SIP MESSAGE (step 2, Table 6.3.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "g.3gpp.mcdata.sds" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCDATA_ID_User_A | The public user identity of the MCData User |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.3.1.3.3-2 |  |  |  |
| MIME body part |  | DATA PAYLOAD |  |  |
| MIME-part-body | As described in Table 6.3.1.3.3-3 |  |  |  |

Table 6.3.1.3.3-2: MCDATA-Info (Table 6.3.1.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp

Table 6.3.1.3.3-3: DATA PAYLOAD (Table 6.3.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.9-1, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Payload |  |  |  |  |
| Payload content type | "00000110" | ENHANCED STATUS | TS 24.282 [31], Table 15.2.13-2 |  |
| Payload data | "1" | The id as defined in the MCData Group Configuration Document | TS 36.579-1 [2], Table 5.5.7.3-1 |  |

Table 6.3.1.3.3-4: SIP MESSAGE (step 4, Table 6.3.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA, PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | As described in Table 6.3.1.3.3-5 |  |  |  |
| MIME body part |  | SDS NOTIFICATION |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-signalling " |  |  |  |
| MIME-part-body | As described in Table 6.3.1.3.3-6 |  |  |  |
| MIME body part | not present | SDS SIGNALLING PAYLOAD |  |  |
| MIME body part | not present | DATA PAYLOAD |  |  |

Table 6.3.1.3.3-5: MCDATA-Info (Table 6.3.1.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | not present |  |  |  |
| mcdata-request-uri | px_MCDATA_ID_User_A |  |  |  |
| mcdata-calling-group-id | px_MCDATA_Group_A |  |  |  |
| mcdata-client-id | not present |  |  |  |

Table 6.3.1.3.3-6: SDS NOTIFICATION (Table 6.3.1.3.3-4)

| Derivation Path: TS 24.282 [31], Table 15.1.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SDS notification message identity | "00000101" | SDS Notification | TS 24.282 [31] clause 15.2.2 |  |
| SDS disposition notification type | "00000010" | DELIVERED | TS 24.282 [31] clause 15.2.5 |  |
| Date and time | The current date and time | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [31] clause 15.2.8 |  |
| Conversation ID | same value as received in MESSAGE | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [31] clause 15.2.9 |  |
| Message ID | same value as received in MESSAGE | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [31] clause 15.2.10 |  |
| Application ID | Not present |  | TS 24.282 [31] clause 15.2.7 |  |

Table 6.3.1.3.3-7: SIP 200 (OK) (step 5, Table 6.3.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Record-Route | not present |  |  |  |
| Content-Type | not present |  |  |  |
| Content-Length |  |  |  |  |
| value | "0" | No message body included |  |  |

<END OF CHANGES>