3GPP TSG-RAN5 Meeting #93-e	R5-216643r1

# Electronic Meeting, 8th Nov – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2165 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G V2X: Initial Test Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR sidelink test model needs to be introduced to be able to implement the test cases in TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced NR V2X test model aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X test cases cannot be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.6 (new clause), 7.3.8 (new clause), 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: corrected spec number in the coversheet. |  |  |  |  |  |  |  |  |

#### 5.2.1.5	NR dual connectivity

Figure 5.2.1.5-1: Test model for NR/5GC NR-DC Layer3: MCG and SCG

Figure 5.2.1.5-2: Test model for NR/5GC NR-DC Layer3 testing: MCG and split DRB

Figure 5.2.1.5-3: Test model for NR/5GC NR-DC Layer3 testing: Split SRB(s), DRBs removed for clarity

The NR/5GC Layer3 NR-DC test model builds on top of the NR/5GC Layer3 test model, with the differences specified hereafter:

-	SRB1 and SRB2 are configured only in the PCell,

-	SRB3 can be configured in the PSCell,

-	The NR PDCP can be configured in split for one or more SRBs or DRBs. In this case NR PDCP is configured in one cell and a proxy PDCP will be configured in the other cell. The SS shall route data to/from either cell, via the routing information provided.

#### 5.2.1.6	NR sidelink

Figure 5.2.1.6-1: Test model for Layer3 NR sidelink PC5 broadcast and groupcast

Figure 5.2.1.6-2: Test model for Layer3 NR sidelink PC5 unicast

The UE is configured in normal mode or in Test Loop Mode E. On the UE side on the Uu interface, ciphering and integrity (PDCP and NAS) are enabled and ROHC is not configured. On the UE side on NR-PC5 interface, the NULL integrity algorithm and no ciphering is applied and ROHC is not configured.

For testing NR sidelink, the system simulator (SS) shall implement one or several simulated NR cells as specified in clause 5.1.1.1 and one or several simulated UEs, called hereafter NR-SS-UE. The NR-SS-UE is used to send/receive data with the UE under test over the PC5 interface. A GNSS Simulator is configured when GNSS synchronisation source or a geographical position is required in the test. The requirements for the GNSS simulator (also referred as positioning simulator) are specified in TS 37.571-4 [xx].  This is depicted in the NR sidelink test model of Figure 5.2.1.6-1 for PC5 broadcast and groupcast and Figure 5.2.1.6-2 for PC5 unicast.

The NR-SS-UE is controlled by TTCN in the NR Sidelink PTC and is configured for NR sidelink by TTCN over sidelink system control NR_SL_SYS port. The NR-SS-UE can be configured in coverage of an NR cell, out of coverage, UTC synchronised or synchronised on the UE under test. The NR-SS-UE can also be configured for transmission/reception of an SLSS/PSBCH. There is no ciphering and NULL integrity algorithm is applied. L1, MAC, RLC, PDCP and SDAP are configured in normal way; they shall perform all of their functions. The NR_SL_DATA port for transmission and reception of STCH data is above SDAP. The STCH data is considered as raw data

Editor’s note: MAC, RLC, PDCP and SDAP transparent modes are FFS

< TEXT SKIPPED>

#### 7.3.7.2	Procedure delays in PUCCH synchronized state

The following SS and UE configurations apply:

By default in TS 38.508-1[5] DRX is not configured.

By default SS is configured to retransmit any DL MAC PDU max 4 times.

HARQ retransmissions in UL cannot be compensated, any HARQ error in UL shall result in an inconclusive verdict for the test case (otherwise a UE may get fail due to a HARQ error).

SS is configured to report  UL HARQ ACK/NACK received from UE to TTCN.

NOTE:	Due to L2 signalling (e.g. RLC STATUS PDUs) it is necessary to limit the reporting of UL HARQ ACK/NACK to the time between sending of the DL RRC message and receiving the HARQ ACK.

To avoid unexpected side effects the timeAlignmentTimer is set to infinity and TTCN configures the SS to not send any Timing Advance Command MAC CE during the test case body (since this may result in additional UL HARQ ACK/NACK).

SS is configured to report HARQ errors and in the case of an UL HARQ error, an inconclusive verdict is assigned.

The following timing applies:

Let Nsf be the maximum allowed RRC procedure delay value (in ms) as specified in the test case prose. Nsf is the maximum time allowed for UE from the end of reception of the DL message on the UE physical layer up to when the UE shall be ready for the reception of UL grant for the transmission of the response message.

NOTE:	A tolerance may have been added to the Nsf value in the test case prose on top of the RRC procedure delay specified in TS 38.331 [16] clause 12.

Let Nslot = (Nsf * Nslotsubframe,μ) + 1

TTCN schedules at time T1 the transmission of the DL message to the UE.

T2 is the time HARQ ACK is received.

TTCN schedules UL grants from T3: T3 = T1 + Nslot + ∆1

∆1 is expressed in slots. T1 and T3 timing are expressed in SFN/subframe/slots.

For FDD

∆1 = 0

The UL grant is repeated continuously in every slot

For FR1 TDD SCS=15kHz

∆1 = 5 - (T1 + Nslot) mod 5. This is the possible slot delay since not all slots can be used for UL grant.

The UL grant periodicity is set to 5 slots. It is repeated in subframe#0 and subframe#5, to cope with K2=4 and the UL subframe#4 and subframe#9.

For FR1 TDD SCS=30kHz

∆1 = (4 - (T1 + Nslot) mod 10) mod10. This is the possible slot delay since not all slots can be used for UL grant.

The UL grant periodicity is set to 10 slots. It is repeated in slot#0 in subframe#2 and subframe#7, to cope with K2=4 and the UL slots: slot#0 in subframe#4 and subframe#9.

For FR2 TDD SCS=120kHz

∆1 = (5- (T1 + Nslot) mod 5) mod 5. This is the possible slot delay since not all slots can be used for UL grant.

The UL grant periodicity is set to 5 slots. It is repeated for possible transmission in every UL slots.

The UL data is received by SS at T4.

The RRC procedure delay requirements are fulfilled when:  (T4-K2) - (T2 - K1) <= Nslot + ∆2

∆2 is expressed in slots. T4 and T2 timing are expressed in SFN/subframe/slots.

For FDD

∆2 =0

For FR1 TDD SCS=15kHz

∆2 = 5 - (T2-K1 + Nslot) mod 5

For FR1 TDD SCS=30kHz

∆2 = (4 - (T2-K1 + Nslot) mod 10) mod10

For FR2 TDD SCS=120kHz

∆2 = (5- (T2-K1 + Nslot) mod 5) mod 5

Figure 7.3.7.2-1 shows the RRC procedure delay check sequence that will be applied when the DL Message is retransmitted once when UE is in PUCCH synchronized state and can respond to UL grants.

Figure 7.3.7.2-1: Delays in PUCCH synchronized state

### 7.3.8	NR sidelink

#### 7.3.8.1	Introduction

Test cases testing sidelink require a special test method.

#### 7.3.8.2	Timing synchronisation

The SS shall provide one system time common across all RATs and NR-SS-UEs. The timing of each configured NR-SS-UE is specified as an offset to this common system time.

For timing synchronisation, the NR-SS-UE can be configured:

-	In coverage of an NR cell: the NR-SS-UE is associated with an NR cell, it selects the NR Cell as synchronisation reference source. NR-SS-UE applies the same timing start and timing offsets as the NR cell, the DFN corresponds to the SFN.

-	Out of coverage synchronising from the GNSS signal: the NR-SS-UE selects the GNSS signal as the synchronisation reference source. The DFN, subframe and slot numbers are derived from the UTC time of the GNSS signal as specified in TS 38.331 [16] clause 5.8.12.

-	Out of coverage synchronising from UE: the UE under test is the synchronisation source, i.e. the SyncRef UE. The NR-SS-UE shall use the received SLSS and MasterInformationBlock-SL-V2X  transmitted by UE as synchronisation source as specified in TS 38.331 [16], clause 5.8.6.

-	Out of coverage without synchronisation reference source: NR-SS-UE shall initiate autonomously the DirectFrameNumber, Subframe and slot numbers using respective offsets specified in test case prose or Table 7.3.8.2-1. The timing of each configured NR-SS-UE is specified as an offset to the common system time.

Table 7.3.8.2-1: Timing parameters of NR-SS-UEs

| NR cell Id | DFN-offset (note 1) | Tcell offset (note 2) | Tc-offset (note 3) |
| --- | --- | --- | --- |
| NR SS UE 1 | 0 | 0 | 0 |
| NR SS UE 2 | 5 | 1 | 216 |
| NR SS UE 3 | 253 | 4 | 1523 |
| NOTE 1: DFN-offset corresponds to the offset applied on system frame number (0..1023). NOTE 2: Tcell corresponds to the timing offset in Ts. Ts = 1/(15000 * 2048) as for NR. NOTE 3: Tc-offset corresponds to the timing offset in Tc. = Ts/Tc = 64 with Tc = 1/(480000 * 4096). See TS 38.211 [19] subclause 4.1 and TS 36.211 [23] subclause 4). |  |  |  |

No timing advance is applied to NR-SS-UE.

#### 7.3.8.3	SLSS/PSBCH transmission/reception

When the NR-SS-UE is configured to transmit S-SS/PSBCH, the following rules apply:

-	TTCN provides the MasterInformationBlockSidelink to the NR-SS-UE as an structured ASN.1 type using the sidelink system control port. The NR-SS-UE shall set the directFrameNumber and slotIndex in the MasterInformationBlockSidelink according to TS 38.331 [16] clause 5.8.9.4.3. A dummy value is provided by TTCN.

-	The NR-SS-UE shall encode the MasterInformationBlockSidelink ASN.1 message as specified in Table 8.1-1.

-	The NR-SS-UE shall transmit the encoded MasterInformationBlockSidelink message periodically as specified in TS 38.331 [16]. For each transmission, the SS shall update directFrameNumber and slotIndex value as specified above.

When the NR-SS-UE is configured to receive S-SS/PSBCH, the following rules apply:

-	NR-SS-UE can be configured in out of coverage synchronising from UE.

-	NR-SS-UE shall report SL-SSID and MasterInformationBlockSidelink over the sidelink system indication port when configured to do so.

#### 7.3.8.4	Sidelink data transmission and reception

##### 7.3.8.4.1	Broadcast and groupcast

UE and NR-SS-UE are configured with the same pool(s) of resources.

When configured in transmission, the NR-SS-UE is configured with a source and a destination layer 2 identity corresponding to the identity expected in the transmitted data.

When configured in reception, the NR-SS-UE is configured with a destination layer 2 identity; NR-SS-UE shall send to TTCN only the SDAP data packets received with the correct destination layer 2 identities in the MAC header. The source layer 2 identity, randomly and uniquely self-selected by UE, can be any value.

##### 7.3.8.4.2	Unicast

FFS

< TEXT SKIPPED>

## 9.2	E-UTRA and NR PIXIT

Table 9.2-1: EUTRA and NR PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_ENDC_BandCombination | ENDC_BandCombination_Type | DC_1A_n28A |  | Band combination for EN-DC test case |
| px_ENDC_CA_BandCombination | ENDC_CA_BandCombination_Type | DC_1A_n28A_n78A |  | EN-DC CA Band Combination |
| px_ENDC_SecondaryBandCombination | ENDC_BandCombination_Type | DC_1A_n77A |  | Secondary band combination for EN-DC test case |
| px_NR_CA_BandCombination | NR_CA_BandCombination_Type | CA_n3A_n77A |  | NR CA Band Combination |
| px_NR_DC_BandCombination | NR_DC_BandCombination_Type | DC_n78A_n257A |  | NR DC Band Combination |
| px_NR_DC_CA_BandCombination | NR_DC_CA_BandCombination_Type | DC_n78A_n257G |  | NR DC CA Band Combination |
| px_NR_CipheringAlgorithm | CipheringAlgorithm | nea2 |  | Ciphering Algorithm (see Note 1) |
| px_NR_IntegrityProtAlgorithm | IntegrityProtAlgorithm | nia2 |  | Integrity Algorithm (see Note 1) |
| px_NR_OverlappingNotSupportedBand_MFBI | integer | 1 |  | A not supported NR band that is overlapping with a supported band (px_NR_PrimaryBand). Applied to MFBI test case scenario. |
| px_NR_PrimaryBand | integer | 1 |  | NR primary band |
| px_NR_SecondaryBand | integer | 2 |  | NR secondary band. Applied to inter-band and SUL test cases. |
| px_NR_SidelinkBand | integer | 47 |  | NR sidelink band |
| NOTE 1: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |