3GPP TSG-RAN WG5 Meeting #93-e	R5-216661r1

# Online, , 8th November 2021 - 19th November 2021

Agenda Item:	6.3.1.20

Source:	MCC TF160

Title:	Streamlining PICS for IMS emergency capabilities

Document for:	Discussion & endorsement

# 1	Introduction

At RAN5#92-e the following RAN5 SIG action point was raised:

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#92.01 | SIG | Review and streamline appropriate PICS for IMS Emergency Call & eCall capability covering LTE, NR & IMS test specs | TF160, R&S, Qualcomm, MediaTek | R5-215727 R5-215717 R5-216330 | RAN5#93 | Pending |

The present contribution analyses the current situation with IMS emergency (non-eCall and eCall) related PICS and their usage in the applicability of various test cases, identifies issues and then proposes solutions to resolve those issues.

# 2	Current situation

## 2.1	IMS emergency

### 2.1.1	PICS definitions

TS 36.523-2-ga0 specifies the following IMS emergency related PICS:

TS 36.523-2 - Table A.4.2.1.1-1: Definition of Bearer Services

| Item | Definition of Bearer Services | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 4 | Support of IMS emergency call | 22.101 | Rel-9 | pc_IMS_emergency_call | For Rel-9 or later releases: mandatory for UEs which supports IMS speech. |

TS 22.101 "Service aspects; Service principles”.

TS 22.101 is an SA1 specification describing the high-level aspects of various services (features) provided by a PLMN, i.e. in principle encompassing all protocol layers.

=> Observation1: It is not crystal clear from its definition what the scope of the above PICS is.

TS 38.508-2-h20 specifies the following IMS emergency related PICS:

TS 38.508-2 - Table A.4.3.7-1: UE General Capabilities

| Item | UE General Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| […] |  |  |  |  |  |  |  |
| 14 | Support of emergency services in NR connected to 5GCN | 24.501 | Rel-15 | pc_NR_5GC_EmergencyServices | No |  |  |

TS 24.501 "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

The definition of this PICS specifically mentions about NR+5GCN level support of emergency services.

=> Observation2: It seems clear from its definition that the scope of the above PICS is NR+5GCN level support of emergency services.

TS 34.229-2-g00 specifies the following IMS emergency related PICS:

TS 34.229-2 - Table A.12: Additional information

| Item | Additional information | Reference | Status | Release | Mnemonic | Support |
| --- | --- | --- | --- | --- | --- | --- |
| […] |  |  |  |  |  |  |
| 26 | UE supports IMS emergency services | 24.229 [10], 5.1.6 | o | Rel-9 | pc_IMS_EmergencyCall |  |

TS 24.229 "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

=> Observation3: There is a small discrepancy between the PICS name (emergency call) and the PICS description (emergency services). It is assumed that the meaning and the underlying feature are the same (note that 24.229 clause 5.1.6 title is “Emergency service”).

=> Observation4: The scope seems to clearly reflect that the above PICS pertains to the IMS level support of emergency services. Furthermore, it can be inferred that the scope of this PICS is RAT-agnostic.

In summary we have the following PICS defined pertaining to IMS emergency call/services:

Table 2.1.1-1: IMS emergency related PICS

| Test spec | Mnemonics | Core spec | Scope | Reference |
| --- | --- | --- | --- | --- |
| 36.523-2 | pc_IMS_emergency_call | 22.101 | All layers | A.4.2.1.1-1/4 |
| 38.508-2 | pc_NR_5GC_EmergencyServices | 24.501 | 5GC NAS | A.4.3.7-1/14 |
| 34.229-2 | pc_IMS_EmergencyCall | 24.229 | IMS | A.12/26 |

### 2.1.2	PICS usage in 36.523 test cases applicability

TS 36.523-2 - Table 4-1a: Applicability of tests Conditions

C71 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 THEN R ELSE N/A

C71a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND A.4.4-1/2 AND A.4.4-1/49 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C71b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

[…]

C109 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND (A.4.4-1/35 OR A.4.4-1/36) THEN R ELSE N/A

C109a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND (A.4.4-1/35 OR A.4.4-1/36) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

[…]

C139 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/32 AND A.4.2.1.1-1/4 AND (A.4.5-1a/27 or A.4.5-1b/27) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

[…]

C172 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND A.4.4-1/37 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

[…]

C182 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND [8]A.2/2 AND NOT A.4.2.1.1-1/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

[…]

C231 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/32 AND A.4.2.1.1-1/4 AND (A.4.5-1a/9 or A.4.5-1b/9) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

[…]

C263 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/121 AND A.4.2.1.1-1/4 THEN R ELSE N/A

[…]

C316 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/54 AND [8]A.10/17 AND A.4.2.1.1-1/4 THEN R ELSE N/A

[…]

C366 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND [8] A. 20/90 THEN R ELSE N/A

Those test conditions are used by the following test cases:

TS 36.523-2 - Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release | Applicability |  | Additional Information |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |

| 6.2.2.5 | Cell selection / No USIM | Rel-8 | C182 | UEs supporting E-UTRA and UTRA and not supporting of IMS emergency call and NOT Category M1 | pc_eFDD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | pc_eTDD |  |  | Rel-9 UTRA TDD |

| 9.2.1.1.29 | Attach / Rejected / IMEI not accepted | Rel-9 | C366 | UEs supporting E-UTRA and IMS emergency call and no USIM test execution | pc_eFDD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | pc_eTDD |  |  |  |

| 9.2.4.1.3 | Attach & Normal tracking area Procedure / Success / Emergency Calls/ without Idle eDRX parameters / With Idle eDRX parameters | Rel-13 | C263 | UEs supporting E-UTRA and Extended DRX and IMS emergency call | pc_eFDD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | pc_eTDD |  |  |  |

| 11.2.1 | Emergency bearer services / Normal cell / NORMAL-SERVICE / Local Emergency Numbers List sent in the Attach / PDN connect new emergency EPS bearer context / Service request / Emergency PDN disconnect | Rel-9 | C71 | UEs supporting E-UTRA and IMS emergency call | pc_eFDD, pc_eTDD, pc_IPv4, pc_IPv6, pb_IPv4_DHCPv4_AAUP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11.2.2 | Emergency bearer services / Normal cell / LIMITED-SERVICE / Attach / PDN connect | Rel-9 | C71 | UEs supporting E-UTRA and IMS emergency call | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.3 | Emergency bearer services / CSG cell / LIMITED-SERVICE / Attach / Security mode control procedure without prior authentication / PDN connect / Service request / PDN disconnect / Detach upon UE switched off / Temporary storage of EMM information | Rel-9 | C71a | UEs supporting E-UTRA and IMS emergency call and allowed CSG list and manual CSG selection and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.4 | Emergency bearer services / Normal cell / NO-IMSI / Attach / No EPS security context / PDN connect / Service request / Timer T3412 expires | Rel-9 | C366 | UEs supporting E-UTRA and IMS emergency call and no USIM test execution | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.5 | Emergency bearer services / Normal cell / NORMAL-SERVICE / Local Emergency Numbers List NOT sent in the Attach / PDN connect new emergency EPS bearer context / Authentication SQN code failure - MME aborts authentication continues using current security context / Service request | Rel-9 | C71 | UEs supporting E-UTRA and IMS emergency call | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.6 | Handling of Local Emergency Numbers List provided during Attach and Normal tracking area update procedures | Rel-9 | C71 | UEs supporting E-UTRA and IMS emergency call | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.7 | UE has PDN connection for emergency bearer services / Normal tracking area update / Accepted / Local Emergency Numbers List is not sent by the network / Handling of the lists of forbidden tracking areas | Rel-9 | C71 | UEs supporting E-UTRA and IMS emergency call | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.8 | Attach for emergency bearer services / Rejected / No suitable cells in tracking area / Emergency call using the CS domain / UTRA or GERAN | Rel-9 | C109a | UEs supporting E-UTRA and IMS emergency call and establishing the emergency call using the CS domain in UTRA or GERAN and NOT Category M1 | pc_eFDD |  | 1 Execution (Note 2) Either TC 11.2.8 or TC 11.2.8a shall be executed | Rel-8 UTRA FDD or Rel-8 GERAN |
|  |  |  |  |  | pc_eTDD |  |  | Rel-9 UTRA TDD or Rel-8 GERAN |
| 11.2.8a | Attach for emergency bearer services / Rejected / No suitable cells in tracking area / Emergency call using the CS domain / CDMA2000 1xRTT | Rel-9 | C172 | UEs supporting E-UTRA and IMS emergency call and establishing the emergency call using the CS domain in 1xRTT and NOT Category M1 | pc_eFDD |  | Either TC 11.2.8 or TC 11.2.8a shall be executed |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.9 | Void |  |  |  |  |  |  |  |
| 11.2.10 | LIMITED-SERVICE / EPS does not support IMS Emergency / Emergency call using the CS domain | Rel-9 | C71b | UEs supporting E-UTRA and UTRA and IMS emergency call and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.11 | LIMITED-SERVICE / Inter-system mobility / E-UTRA to UTRA CS / SRVCC Emergency Call Handover to UTRAN | Rel-9 | C139 | UEs supporting E-UTRA and UTRA and SRVCC and IMS emergency call and FGI 27 and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.2.12 | LIMITED-SERVICE / Inter-system mobility / E-UTRA to GSM CS / SRVCC Emergency Call Handover to GERAN | Rel-9 | C231 | UEs supporting E-UTRA and GERAN and SRVCC and IMS emergency call and FGI 9 and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |

| 11.3.3 | eCall capable / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA or GERAN | Rel-14 | C316 | UEs supporting E-UTRA and UTRA or GERAN and IMS eCall and eCall Capable and Automatic type of eCall initiation and IMS emergency call | pc_eFDD |  | Note 7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | pc_eTDD |  |  |  |

| 13.5.3 | Emergency call / Success / SSAC / 0% access probability for MTSI MO speech call | Rel-9 | C71 | UEs supporting E-UTRA and IMS emergency call | pc_eFDD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 13.5.3a | Emergency call / Success / SSAC in Connected mode / 0% access probability for MTSI MO speech call | Rel-12 | C71 | UEs supporting E-UTRA and IMS emergency call | pc_eFDD |  | Note 7 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |

### 2.1.3	PICS usage in 38.523 test cases applicability

TS 38.523-2 - Table 4.2-1: Applicability of Protocol conformance test cases Conditions

| Condition | Test case Selection Expression | Comment |
| --- | --- | --- |
| […] |  |  |
| C47 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/11 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |
| […] |  |  |
| C85 | IF (A.4.1-5/1 AND A.4.4-2/8) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 THEN R ELSE N/A | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |
| C85A | IF (A.4.1-5/1 AND A.4.4-2/9) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 THEN R ELSE N/A | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |
| […] |  |  |
| C92 | IF A.4.1-5/1 AND A.4.3.7-1/14 THEN R ELSE N/A | UEs supporting 5G Core and emergency services in NR connected to 5GCN |

Those test conditions are used by the following test cases:

TS 38.523-2 - Table 4.1-5a: Applicability of Protocol conformance Multi-layer test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 11 | Multi-layer and Services |  |  |  |
| 11.1 | 5GS / EPS Fallback |  |  |  |
| […] |  |  |  |  |
| 11.1.7 | Emergency call setup from NR RRC_IDLE / Emergency Services Fallback to EPS with redirection / Single registration mode with N26 interface / Success | Rel-15 | C47 | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |
| […] |  |  |  |  |
| 11.3 | Unified Access Control (UAC) |  |  |  |
| […] |  |  |  |  |
| 11.3.7 | UAC / Access Identity 11..15 / High Priority Access / HPLMN/0% accessibility AC2/Emergency call | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| […] |  |  |  |  |
| 11.4 | Emergency Services |  |  |  |
| 11.4.1 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / Network failing the authentication check (5G AKA) | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.2 | 5GMM-DEREGISTERED.LIMITED-SERVICE / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services / Handling of forbidden PLMNs | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.3 | 5GMM-DEREGISTERED.NO-SUPI / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.4 | 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE T3346 running / Emergency call establishment / 5GMM-REGISTERED.NORMAL-SERVICE / Emergency call establishment before T3396 expiry | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.5 | 5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.6 | 5GMM-REGISTERED.NON-ALLOWED-SERVICE / Emergency call establishment and release / Handling of non-allowed tracking areas | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.7 | Handling of Local and Extended emergency numbers / Mobility | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.8 | Handling of Local and extended emergency numbers / Switch-off and maximum local numbers storage | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.9 | 5GMM-DEREGISTERED.LIMITED-SERVICE No suitable cells in tracking area / Emergency call establishment and release | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.10 | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / N1 mode to S1 mode transfer of an existing emergency PDU session | Rel-15 | C85 | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and emergency services in NR connected to 5GCN |
| 11.4.11 | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / S1 mode to N1 mode transfer of an existing emergency PDN connection | Rel-15 | C85A | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and emergency services in NR connected to 5GCN |

=> Observation5: The way the 36.523-2 PICS is used in the above tables, it has been assumed that pc_IMS_emergency_call pertains to the specific EPS (E-UTRA/EPC) level support of IMS emergency.

=> Observation6: The way the 36.523-2 & 38.508-2 PICS are used in the above tables, it has been assumed that it is possible for a UE to support IMS emergency only on one RAT (E-UTRA/EPC or NR/5GC).

### 2.1.4	PICS usage in 34.229 test cases applicability

For 34.229-1 test cases:

TS 34.229-2 - Table 4.1: Applicability of TS 34.229-1 tests Conditions

| Conditions/Options |  |  |
| --- | --- | --- |
| […] |  |  |
| C59 | IF A.12/26 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS emergency services and E-UTRA and not UE category M1 |
| […] |  |  |
| C62 | IF A.12/26 AND [34.123-2] A.2/2 AND ([73] A.4.1-1/6 OR [73] A.4.1-1/7) THEN R ELSE N/A | IMS emergency services and emergency speech call and (UTRAN or GERAN) |
| […] |  |  |
| C64 | IF A.12/26 AND A.18/1 AND NOT [73] A.4.3.2-2A/1THEN R ELSE N/A | IMS emergency services and E-UTRA and not UE category M1 |
| […] |  |  |
| C66 | IF A.12/26 AND A.12/12 AND A.15/1 AND ([34.123-2] A.2/2 AND ([73] A.4.1-1/6 OR [73] A.4.1-1/7)36.523-2THEN R ELSE N/A | IMS emergency services and initiating bidirectional voice session over IMS and MTSI speech and emergency call and (UTRAN or GERAN) |
| […] |  |  |
| C72 | IF A.12/26 AND A.3A/50 AND A.15/1 AND A.12/33 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS emergency services and MTSI and MTSI speech and MTSI Communication Hold during emergency call and preconditions and E-UTRA and not UE category M1 |
| C73 | IF A.12/26 AND A.3A/50 AND A.15/1 AND A.18/1 AND NOT [73] A.4.3.2-2A/1THEN R ELSE N/A | IMS emergency services and MTSI and MTSI speech and E-UTRA and not UE category M1 |
| C74 | IF A.12/26 AND [3] A.2/2 AND [73] A.4.1-1/4 THEN R ELSE N/A | IMS emergency services and emergency speech call and 1xRTT |
| […] |  |  |
| C76 | IF A.12/26 AND A.12/27 AND A.18/1 AND NOT [73] A.4.3.2-2A/1THEN R ELSE N/A | IMS emergency services and capable of obtaining location Information and E-UTRA and not UE category M1 |
| […] |  |  |
| C121 | IF A.12/26 AND A.18/4 THEN R ELSE N/A | IMS emergency services and WLAN |
| […] |  |  |
| C124 | IF A.6a/2 AND A.6a/1 AND A.12/26 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS security and GIBA and IMS emergency service and E-UTRA and not UE category M1 |
| C125 | IF A.12/26 AND A.12/52 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS emergency service and IMS emergency call without registration after rejection of emergency registration and E-UTRA and not UE category M1 |
| […] |  |  |
| C132 | IF A.12/26 AND A.12/27 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS emergency services and capable of obtaining location Information and UE category M1 |
| C133 | IF A.12/26 AND [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS emergency services and UE category M1 |
| C134 | IF A.6a/2 AND NOT A.6a/1 AND A.12/26 AND A.12/52 AND A.18/1 AND NOT [73] A.4.3.2-2A/1THEN R ELSE N/A | IMS security and NOT GIBA and IMS emergency service and IMS emergency call without registration after rejection of emergency registration. UE supports E-UTRA and not UE category M1 |
| […] |  |  |
| C162 | IF A.12/26 AND A.18/1 AND [3] A.20/90 AND NOT [73] A.4.3.2-2A/1 THEN R ELSE N/A | IMS emergency services and E-UTRA and no USIM test execution and not UE category M1 |

Those test conditions are used by the following 34.229-1 test cases:

TS 34.229-2 - Table 4.1: Applicability of TS 34.229-1 tests

| Clause | Title | Release | Applicability | Comments |
| --- | --- | --- | --- | --- |
| […] |  |  |  |  |
| Emergency Service |  |  |  |  |
| 19.1.1 | Emergency call with emergency registration / Success / Location information available | Rel-9 | C76 | UE supports IMS emergency services and is capable of obtaining location information and E-UTRA and not UE category M1 |
| 19.1.2 | Emergency call with emergency registration / Success / Location information not available | Rel-9 | C59 | UE supports IMS emergency services and E-UTRA and not UE category M1 |
| 19.1.3 | Emergency call with emergency registration / Abnormal case / IM CN sends a 380 / UE performs emergency call via CS domain / UTRAN or GERAN | Rel-9 | C62 | UE supports IMS emergency services and Emergency speech call and (UTRAN or GERAN) (NOTE 2, 6) |
| 19.1.3a | Emergency call with emergency registration / Abnormal case / IM CN sends a 380 / UE performs emergency call via CS domain / CDMA 2000 1xRTT | Rel-9 | C74 | UE supports IMS emergency services and Emergency speech call and 1XRTT (NOTE 2) |
| 19.1.3b | Void |  |  |  |
| 19.1.3c | Emergency call with emergency registration / Abnormal case / IM CN sends 503 Service Unavailable / UE performs emergency call via CS domain / UTRAN or GERAN | Rel-9 | C62 | UE supports IMS emergency services and Emergency speech call and (UTRAN or GERAN) (NOTE 6) |
| 19.1.4 | Void |  |  |  |
| 19.1.5 | Emergency call with emergency registration / Emergency SIP signalling and media in parallel with an other ongoing IM CN subsystem signalling and media | Rel-9 | C72 | UE supports MTSI and MTSI speech and IMS emergency services and Communication Hold during emergency call and preconditions and E-UTRA and not UE category M1 |
| 19.1.6 | Emergency call with emergency registration / Success / GIBA against a network with GIBA support only | Rel-9 | C124 | UE supports IMS security and GIBA and IMS emergency service and E-UTRA and not UE category M1 |
| 19.3.1 | Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not | Rel-9 | C73 | UE supports IMS emergency services and MTSI and MTSI speech and preconditions and E-UTRA and not UE category M1 |
| 19.3.2 | Non-UE detectable emergency call / IM CN sends 380 Alternative Service including emergency service URN and no emergency subservice type / Non-emergency IMS registration / UTRAN or GERAN | Rel-9 | C66 | UE supports MTSI and MTSI Speech and IMS emergency call and emergency call and (UTRAN or GERAN) (NOTE 2, 6) |
| […] |  |  |  |  |
| 19.3.2b | Non-UE detectable emergency call / IM CN sends a 380 with unavailable emergency service URN / UE performs normal call via CS domain / UTRAN or GERAN | Rel-9 | C66 | UE supports MTSI and MTSI Speech and IMS emergency services and emergency call and (UTRAN or GERAN) (NOTE 6) |
| 19.3.2c | Non-UE detectable emergency call / IM CN sends a 380 with available emergency service URN / UE performs CS Emergency call via CS domain / UTRAN or GERAN | Rel-9 | C66 | UE supports MTSI and MTSI Speech and IMS emergency services and emergency call and (UTRAN or GERAN) (NOTE 6) |
| 19.3.3 | Non-UE detectable emergency call / IM CN sends 380 Alternative Service / Emergency IMS registration | Rel-9 | C73 | UE supports IMS emergency services and MTSI and MTSI speech and E-UTRA and not UE category M1 |
| 19.3.4 | Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired | Rel-9 | C73 | UE supports IMS emergency services and MTSI and MTSI speech and E-UTRA and not UE category M1 |
| 19.4.1 | Emergency call without emergency registration / EPS / UE does not contain an ISIM or USIM | Rel-9 | C162 | UE supports IMS emergency services and E-UTRA and not UE category M1 and no USIM test execution |
| 19.4.2 | Emergency call without emergency registration / EPS / UE contains an ISIM or USIM / UE is in state EMM-REGISTERED.LIMITED-SERVICE | Rel-9 | C64 | UE supports IMS emergency services and E-UTRA and not UE category M1 |
| […] |  |  |  |  |
| 19.4.5 | Emergency call without emergency registration / UE credentials are not accepted | Rel-9 | C59 | UE supports IMS emergency services and E-UTRA and not UE category M1 |
| 19.4.6 | Emergency call without emergency registration / Failure of registration / Rejected by 403 (Forbidden) | Rel-9 | C125 | UE supports IMS emergency service and IMS emergency call without registration after rejection of emergency registration and E-UTRA and not UE category M1 |
| 19.4.7 | Emergency call without emergency registration / Failure of registration / against a network with GIBA support only | Rel-9 | C134 | UE supports IMS emergency service and IMS emergency call without registration after rejection of emergency registration that used IMS security. The UE does not support GIBA. UE supports IMS security and E-UTRA and not UE category M1 |
| 19.5.1 | New initial emergency registration / UE obtains from the serving IP-CAN an IP address different than the IP address used for the emergency registration | Rel-9 | C59 | UE supports IMS emergency services and E-UTRA and not UE category M1 |
| 19.5.6 | User-initiated emergency reregistration / UE has emergency related ongoing dialog | Rel-9 | C59 | UE supports IMS emergency services and E-UTRA and not UE category M1 |
| 19.5.7 | User-initiated emergency reregistration / The user initiates an emergency call | Rel-9 | C59 | UE supports IMS emergency services and E-UTRA and not UE category M1 |
| 19.5.8 | Void |  |  |  |
| 19.5.9 | In parallel emergency and non-emergency registrations | Rel-9 | C59 | UE supports IMS emergency services and E-UTRA and not UE category M1 |
| 19.5.10 | Deregistration upon emergency registration expiration | Rel-9 | C59 | UE supports IMS emergency services and E-UTRA and not UE category M1 |
| […] |  |  |  |  |
| Emergency Service over IMS / WLAN |  |  |  |  |
| G.19.1 | Emergency call with emergency registration / WLAN | Rel-11 | C121 | UE supports IMS emergency services and WLAN |
| […] |  |  |  |  |
| Emergency Service over IMS / UE category M1 |  |  |  |  |
| J.19.1 | Emergency call with emergency registration / Success / Location information available / UE category M1 | Rel-13 | C132 | UE supports IMS emergency services and is capable of obtaining location information and UE category M1 |
| J.19.2 | Emergency call with emergency registration / Success / Location information not available / UE category M1 | Rel-13 | C133 | UE supports IMS emergency services and UE category M1 |

For 34.229-5 test cases:

TS 34.229-2 - Table 4.2: Applicability of TS 34.229-5 tests Conditions

| Conditions/Options |  |  |
| --- | --- | --- |
| […] |  |  |
| C12 | IF A.6a/2 AND A.18/5 AND A.12/26 AND A.12/27 THEN R ELSE N/A | IMS security and NR and IMS emergency services and is capable of obtaining location information |
| […] |  |  |
| C39 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.12/26 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and IMS emergency services |
| C40 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.4/16 AND A.12/26 AND A.12/33 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and preconditions and IMS emergency services and Communication Hold during emergency call |
| C41 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.4/16 AND A.12/26 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and preconditions and IMS emergency services |

Those test conditions are used by the following 34.229-5 test cases:

TS 34.229-2 - Table 4.2: Applicability of TS 34.229-5 tests

| Clause | Title | Release | Applicability | Comments |
| --- | --- | --- | --- | --- |
| […] |  |  |  |  |
| Emergency Calls |  |  |  |  |
| 10.1 | Emergency Call with emergency registration / Success / Location information available / 5GS | Rel-15 | C12 | UE supports IMS security and NR and IMS emergency services and is capable of obtaining location information |
| 10.2 | Emergency Call with emergency registration / Success / Location information not available / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS emergency services |
| 10.3 | Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS | Rel-15 | C40 | UE supports IMS security and NR MTSI and MTSI speech and preconditions and IMS emergency services and Communication Hold during emergency call |
| 10.4 | Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not / 5GS | Rel-15 | C41 | UE supports IMS security and NR and MTSI and MTSI speech and preconditions and IMS emergency services |
| 10.6 | Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS emergency services |
| 10.9 | Emergency call without emergency registration / UE credentials are not accepted / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS emergency services |
| 10.11 | Emergency call without emergency registration / Failure of registration / Rejected by 403 (Forbidden) / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS emergency services |
| 10.12 | User-initiated emergency reregistration / UE has emergency related ongoing dialog / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS emergency services |
| 10.13 | User-initiated emergency reregistration / User initiates an emergency call / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS emergency services |
| 10.14 | In parallel emergency and non-emergency registrations / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS emergency services |
| 10.15 | Deregistration upon emergency registration expiration / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS emergency services |

=> Observation7: The way the 34.229-2 PICS is used in the above tables, it has been assumed that the setting of this PICS is RAT-agnostic. This is in line with Observation4 but in contradiction with Observation6.

## 2.2	IMS eCall

### 2.2.1	PICS definitions

TS 34.123-2-f20 specifies the following eCall related PICS.

TS 34.123-2 - Table A.10: Other UE Service Capabilities

| Item | Other UE Service Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| […] |  |  |  |  |  |
| 14 | eCall Only Support on the USIM | 24.008, 4.2.1.1 | Rel-8 | pc_eCallOnly | UEs that contain USIM with subscription for eCall only service are identified as eCall Only capable UE. |
| 15 | eCall Capable Support on the USIM | 24.008 | Rel-8 | pc_eCallCapable | UEs that contain USIM with subscription for eCall and other services are identified as eCall Capable UE. |
| 16 | Capability to Initiate Manual eCall | 24.008 | Rel-8 | pc_eCall_manual_Initiated | UE providing a means to trigger a manual call |
| 17 | Capability to Initiate Automatic eCall | 24.008 | Rel-8 | pc_eCall_automatic_Initiated | UE providing a means to trigger a automatic call |
| 18 | Capability to trigger a reconfiguration eCall | 24.008 | Rel-8 | pc_eCall_Reconfiguration_Call | UE providing a means to trigger a reconfiguration eCall |
| 19 | Capability to trigger a Test eCall | 24.008 | Rel-8 | pc_eCall_Test_Call | UE providing a means to trigger a Test eCall |

=> Observation8: In general the above 34.123-2 PICS seem to be RAT-agnostic.

TS 36.523-2-ga0 contains no IMS eCall related PICS.

TS 38.508-2-h20 contains no IMS eCall related PICS.

TS 34.229-2-g00 specifies the following IMS eCall related PICS.

TS 34.229-2 - Table A.12: Additional information

| Item | Additional information | Reference | Status | Release | Mnemonic | Support |
| --- | --- | --- | --- | --- | --- | --- |
| […] |  |  |  |  |  |  |
| 54 | UE supports IMS eCall type of emergency services | 24.229 [10], 5.1.6.11 | o | Rel-14 | pc_IMS_eCall_Capable |  |
| 55 | UE supports IMS eCall Only type of emergency services | 24.229 [10], 5.1.6.11 | o | Rel-14 | pc_IMS_eCall_Only |  |

TS 24.229 "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

=> Observation9: The scope seems to clearly reflect that the scope of the above PICS is the IMS level support of eCall. Furthermore, it can be inferred that the scope of this PICS is RAT-agnostic.

In summary we have the following PICS defined pertaining to IMS eCall:

Table 2.2.1-1: IMS eCall related PICS

| Test spec | Mnemonics | Core spec | Scope | Reference |
| --- | --- | --- | --- | --- |
| 34.229-2 | pc_IMS_eCall_Capable | 24.229 | IMS | A.12/54 |
| 34.229-2 | pc_IMS_eCall_Only | 24.229 | IMS | A.12/55 |

### 2.2.2	PICS usage in 36.523 test cases applicability

TS 36.523-2 - Table 4-1a: Applicability of tests Conditions

[…]

C314a IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/17 THEN R ELSE N/A

[…]

C314 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A

C315 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/16 AND [8]A.10/19 THEN R ELSE N/A

C316 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/54 AND [8]A.10/17 AND A.4.2.1.1-1/4 THEN R ELSE N/A

C317 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/55 AND [8]A.10/17 THEN R ELSE N/A

C318 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A

C319 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A

Those test conditions are used by the following test cases:

TS 36.523-2 - Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release | Applicability |  | Additional Information |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |

| 8.2.4.28 | eCall Only mode / RRC connection reconfiguration / Inter-frequency Handover / Success | Rel-14 | C314a | UEs supporting E-UTRA and IMS eCall and eCall only and Automatic type of eCall initiation | pc_eFDD |  | Note 7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | pc_eTDD |  |  |  |

| 11.3 | eCall over IMS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11.3.1 | eCall Only mode / T3444 / eCall inactivity procedure / Removal of eCall only restriction after an eCall over IMS | Rel-14 | C314 | UEs supporting E-UTRA and IMS eCall and eCall only and Manual type of eCall initiation | pc_eFDD |  | Note 7 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.2 | eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service | Rel-14 | C315 | UEs supporting E-UTRA and IMS eCall and eCall only and Manual type of eCall initiation and capable of triggering a Test eCall | pc_eFDD |  | Note 7 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.3 | eCall capable / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA or GERAN | Rel-14 | C316 | UEs supporting E-UTRA and UTRA or GERAN and IMS eCall and eCall Capable and Automatic type of eCall initiation and IMS emergency call | pc_eFDD |  | Note 7 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.4 | eCall Only mode / EPS supports IMS voice over PS session / EPS does not support emergency service / eCall over IMS is not supported / eCall using CS domain / eCall failure if CS domain is not available | Rel-14 | C317 | UEs supporting E-UTRA and UTRA or GERAN and IMS eCall and eCall only and Automatic type of eCall initiation | pc_eFDD |  | Note 7 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.5 | eCall Only mode / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is supported / RACH failure in EUTRA cell / eCall using the CS domain | Rel-14 | C317 | UEs supporting E-UTRA and UTRA or GERAN and IMS eCall and eCall only and Automatic type of eCall initiation | pc_eFDD |  | Note 7 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.6 | eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted | Rel-14 | C315 | UEs supporting E-UTRA and IMS eCall and eCall only and Manual type of eCall initiation and capable of triggering a Test eCall | pc_eFDD |  | Note 7 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.7 | eCall Only mode / SRVCC Handover to CS domain / UTRAN / MSD Update / Success | Rel-14 | C318 | UEs supporting E-UTRA and UTRA and IMS eCall and eCall only and Manual type of eCall initiation | pc_eFDD |  | Note 7 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.8 | eCall Only mode / SRVCC Handover to CS domain / GERAN / MSD Update / Success | Rel-14 | C319 | UEs supporting E-UTRA and GERAN and IMS eCall and eCall only and Manual type of eCall initiation | pc_eFDD |  | Note 7 |  |
|  |  |  |  |  | pc_eTDD |  |  |  |

### 2.2.3	PICS usage in 38.523 test cases applicability

None as the IMS eCall over NR Work Item is ongoing in RAN5 and no test case has yet been added into TS 38.523-series.

### 2.2.4	PICS usage in 34.229 test cases applicability

For 34.229-1 test cases:

TS 34.229-2 - Table 4.1: Applicability of TS 34.229-1 tests Conditions

| Conditions/Options |  |  |
| --- | --- | --- |
| […] |  |  |
| C147 | IF A.12/54 AND [3] A.10/16 THEN R ELSE N/A | IMS eCall type of emergency service and eCallCapable and manual type of eCall |
| C148 | IF A.12/54 AND [3] A.10/17 THEN R ELSE N/A | IMS eCall type of emergency service and eCallCapable and automatic type of eCall |
| C149 | IF A.12/55 AND [3] A.10/16 THEN R ELSE N/A | IMS eCall type of emergency service and eCallOnly and manual type of eCall |
| C150 | IF A.12/55 AND [3] A.10/17 THEN R ELSE N/A | IMS eCall type of emergency service and eCallOnly and automatic type of eCall |

Those test conditions are used by the following 34.229-1 test cases:

TS 34.229-2 - Table 4.1: Applicability of TS 34.229-1 tests

| Clause | Title | Release | Applicability | Comments |
| --- | --- | --- | --- | --- |
| eCall over IMS (eCall) |  |  |  |  |
| 21.1 | eCall over IMS / Manual initiation / Normal registration / Emergency registration / Success / 200 OK with ACK | Rel-14 | C147 | UE supports IMS eCall and eCallCapable and Manual type of eCall (NOTE 7) |
| 21.2 | eCall over IMS / Automatic initiation / Normal registration / Emergency registration / Success / 200 OK with ACK | Rel-14 | C148 | UE supports IMS eCall and eCallCapable and Automatic type of eCall (NOTE 7) |
| 21.3 | eCall over IMS / Manual initiation / MSD transfer Failure / UE performs eCall in CS domain after Timer expiry / UTRAN or GERAN | Rel-14 | C147 | UE supports IMS eCall and eCallCapable and manual type of eCall (NOTE 7) |
| 21.4 | eCall over IMS / Manual initiation / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / Success | Rel-14 | C147 | UE supports IMS eCall and eCallCapable and Manual type of eCall (NOTE 7) |
| 21.5 | eCall over IMS / Automatic initiation / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / Success | Rel-14 | C148 | UE supports IMS eCall and eCallCapable and Automatic type of eCall (NOTE 7) |
| 21.6 | eCall over IMS / Automatic initiation / MSD transfer and 200 OK with ACK / SIP INFO request for unsupported MSD / UE indicates unsuccessful in SIP INFO | Rel-14 | C148 | UE supports IMS eCall and eCallCapable and Automatic type of eCall (NOTE 7) |
| 21.13 | eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN | Rel-14 | C149 | UE supports IMS eCall and eCallOnly and manual type of call (NOTE 7) |
| 21.14 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN | Rel-14 | C150 | UE supports IMS eCall and eCallOnly and Automatic type of eCall (NOTE 7) |
| 21.15 | eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN | Rel-14 | C149 | UE supports IMS eCall and eCallOnly and manual type of call (NOTE 7) |
| 21.16 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN | Rel-14 | C150 | UE supports IMS eCall and eCallOnly and Automatic type of eCall (NOTE 7) |
| 21.17 | eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) / UE performs eCall in CS domain / UTRAN or GERAN | Rel-14 | C149 | UE supports IMS eCall and eCallOnly and manual type of call (NOTE 7) |
| 21.18 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) / UE performs eCall in CS domain / UTRAN or GERAN | Rel-14 | C150 | UE supports IMS eCall and eCallOnly and Automatic type of eCall (NOTE 7) |

For 34.229-5 test cases:

TS 34.229-2 - Table 4.2: Applicability of TS 34.229-5 tests Conditions

| Conditions/Options |  |  |
| --- | --- | --- |
| […] |  |  |
| C42 | IF A.12/54 AND A.18/5 AND [3] A.10/16 THEN R ELSE N/A | UE supports IMS eCall type of emergency services and NR and Manual type of eCall |

Those test conditions are used by the following 34.229-5 test cases:

TS 34.229-2 - Table 4.2: Applicability of TS 34.229-5 tests

| Clause | Title | Release | Applicability | Comments |
| --- | --- | --- | --- | --- |
| eCall over IMS |  |  |  |  |
| 11.1 | eCall over IMS / Manual initiation / Normal registration / Emergency registration / Success / 200 OK with ACK / 5GS | Rel-16 | C42 | UE supports IMS eCall type of emergency services and NR and Manual type of eCall |

=> Observation10: The way the 34.229-2 PICS is used in the above tables, it has been assumed that the setting of this PICS is RAT-agnostic. This is in line with Observation9.

# 3	Discussion

The handling of IMS eCall UE capabilities is so far consistent: IMS eCall capabilities are RAT-agnostic. Therefore, a UE supporting IMS eCall at the IMS level shall be capable of IMS eCall on all the RATs (E-UTRA/EPC, NR/5GC) it supports.

The handling of IMS emergency call/services is so far inconsistent in the various RAN5 test specifications: in some parts IMS emergency is considered RAT-agnostic while in other parts it has been rendered RAT-specific. This shall be harmonized across all RAN5 test specifications.

Furthermore, there seems to be no reason to take a different approach between IMS emergency call/services and IMS eCall: both should be either RAT-agnostic or RAT-specific.

## 3.1	RAT-agnostic

If RAN5 decides to follow a RAT-agnostic approach, then the following PICS shall be deleted:

Table 3.1-1: Obsolete PICS

| Test spec | Mnemonics | Core spec | Scope | Reference |
| --- | --- | --- | --- | --- |
| 36.523-2 | pc_IMS_emergency_call | 22.101 | All layers | A.4.2.1.1-1/4 |
| 38.508-2 | pc_NR_5GC_EmergencyServices | 24.501 | 5GC NAS | A.4.3.7-1/14 |

And test cases from 36.523-series + 38.523-series shall refer to 34.229-2 PICS pc_IMS_EmergencyCall.

## 3.2	RAT-specific

If RAN5 decides to follow a RAT-specific approach, then the definition & mnemonic of the 36.523-2 PICS shall be updated, and the 34.229-2 PICS shall be deleted.

Furthermore, new PICS for IMS eCall on specific RAT shall be introduced and used in the various IMS eCall test cases.

# 4	Proposal

Given that we are talking about emergency, which is a critical service to provide to the users, it is proposed that RAN5 goes forward with the RAT-agnostic approach.

Based on RAN5 decision, follow-on prose CRs will be required to be raised and agreed.

# 5	Revision1 - Way forward

## 5.1	High level endorsements

RAN5 SIG group prefers the specification of a RAT-specific approach for both IMS emergency and IMS eCall.

In addition, it was suggested that pc_NR_5GC_EmergencyServices is not needed because according to TS 38.306 clause 6:

| Features | Condition |
| --- | --- |
| IMS emergency call | It is mandatory to support IMS emergency call for UEs which are IMS voice capable in NR. |

=> It is sufficient to use the (new) PICS pc_voiceOverNR. (added in R5-217182).

Also, it was suggested to update the 36.523-2 PICS definition to clarify that it is EPS (LTE) specific.

Also, with the RAT-specific approach, the 34.229-2 PICS pc_IMS_EmergencyCall is not needed anymore and can be deleted.

Finally a new PICS is required for 34.229-1 TC G.19.1, for support of IMS emergency over WLAN.

## 5.2	Detailed changes for IMS emergency

### 5.2.1	In 36.523-2

TS 36.523-2 - Table A.4.2.1.1-1: Definition of Bearer Services

| Item | Definition of Bearer Services | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 4 | Support of IMS emergency call in EPS | 36.306, 7.2.1, 24.229, L.2.2.6 | Rel-9 | pc_EPS_IMS_EmergencyCall | For Rel-9 or later releases: mandatory for UEs which supports IMS speech in EPS. |

### 5.2.2	In 38.508-2

TS 38.508-2 - Table A.4.3.7-1: UE General Capabilities

| Item | UE General Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| […] |  |  |  |  |  |  |  |
| 14 | Void |  |  |  |  |  |  |

Note: From R5-217182 (with updates):

TS 38.508-2 - Table A.4.3.7-1: UE General Capabilities

| Item |  | UE General Capabilities |  | Ref. |  | Release |  | Mnemonic |  | M |  | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| […] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XX |  | Support IMS voice over NR |  | 38.306, 4.2.13 |  | Rel-15 |  | pc_voiceOverNR |  | No |  |  |  | It is mandatory to support IMS emergency call for UEs which are IMS voice capable in NR |  |

### 5.2.3	In 38.523-2

TS 38.523-2 - Table 4.2-1: Applicability of Protocol conformance test cases Conditions

| Condition | Test case Selection Expression | Comment |
| --- | --- | --- |
| […] |  |  |
| C47 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/11 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |
| […] |  |  |
| C85 | IF (A.4.1-5/1 AND A.4.4-2/8) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 THEN R ELSE N/A | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |
| C85A | IF (A.4.1-5/1 AND A.4.4-2/9) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 THEN R ELSE N/A | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |
| […] |  |  |
| C92 | IF A.4.1-5/1 AND A.4.3.7-1/14 THEN R ELSE N/A | UEs supporting 5G Core and IMS voice over NR |

TS 38.523-2 - Table 4.1-5a: Applicability of Protocol conformance Multi-layer test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 11 | Multi-layer and Services |  |  |  |
| 11.1 | 5GS / EPS Fallback |  |  |  |
| […] |  |  |  |  |
| 11.1.7 | Emergency call setup from NR RRC_IDLE / Emergency Services Fallback to EPS with redirection / Single registration mode with N26 interface / Success | Rel-15 | C47 | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |
| […] |  |  |  |  |
| 11.3 | Unified Access Control (UAC) |  |  |  |
| […] |  |  |  |  |
| 11.3.7 | UAC / Access Identity 11..15 / High Priority Access / HPLMN/0% accessibility AC2/Emergency call | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| […] |  |  |  |  |
| 11.4 | Emergency Services |  |  |  |
| 11.4.1 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / Network failing the authentication check (5G AKA) | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| 11.4.2 | 5GMM-DEREGISTERED.LIMITED-SERVICE / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services / Handling of forbidden PLMNs | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| 11.4.3 | 5GMM-DEREGISTERED.NO-SUPI / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| 11.4.4 | 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE T3346 running / Emergency call establishment / 5GMM-REGISTERED.NORMAL-SERVICE / Emergency call establishment before T3396 expiry | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| 11.4.5 | 5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| 11.4.6 | 5GMM-REGISTERED.NON-ALLOWED-SERVICE / Emergency call establishment and release / Handling of non-allowed tracking areas | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| 11.4.7 | Handling of Local and Extended emergency numbers / Mobility | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| 11.4.8 | Handling of Local and extended emergency numbers / Switch-off and maximum local numbers storage | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| 11.4.9 | 5GMM-DEREGISTERED.LIMITED-SERVICE No suitable cells in tracking area / Emergency call establishment and release | Rel-15 | C92 | UEs supporting 5G Core and IMS voice over NR |
| 11.4.10 | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / N1 mode to S1 mode transfer of an existing emergency PDU session | Rel-15 | C85 | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and IMS voice over NR |
| 11.4.11 | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / S1 mode to N1 mode transfer of an existing emergency PDN connection | Rel-15 | C85A | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and IMS voice over NR |

### 5.2.4	In 34.229-2

TS 34.229-2 - Table A.12: Additional information

| Item | Additional information | Reference | Status | Release | Mnemonic | Support |
| --- | --- | --- | --- | --- | --- | --- |
| […] |  |  |  |  |  |  |
| 26 | Void |  |  |  |  |  |
| […] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| YY | UE supports IMS emergency call over WLAN | 24.229 [10], 5.1.6, R.2.2.6 | o | Rel-13 | pc_WLAN_IMS_EmergencyCall |  |

TS 34.229-2 - Table 4.1: Applicability of TS 34.229-1 tests Conditions

| Conditions/Options |  |  |
| --- | --- | --- |
| […] |  |  |
| C59 | IF A.18/1 AND NOT [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | E-UTRA and not UE category M1 and EPS IMS emergency call |
| […] |  |  |
| C62 | IF [34.123-2] A.2/2 AND ([73] A.4.1-1/6 OR [73] A.4.1-1/7) AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | emergency speech call and (UTRAN or GERAN) and EPS IMS emergency call |
| […] |  |  |
| C64 | IF A.18/1 AND NOT [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | E-UTRA and not UE category M1 and EPS IMS emergency call |
| […] |  |  |
| C66 | IF A.12/12 AND A.15/1 AND ([34.123-2] A.2/2 AND ([73] A.4.1-1/6 OR [73] A.4.1-1/7) AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | initiating bidirectional voice session over IMS and MTSI speech and emergency call and (UTRAN or GERAN) and EPS IMS emergency call |
| […] |  |  |
| C72 | IF A.3A/50 AND A.15/1 AND A.12/33 AND A.4/16 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | MTSI and MTSI speech and MTSI Communication Hold during emergency call and preconditions and E-UTRA and not UE category M1 and EPS IMS emergency call |
| C73 | IF A.3A/50 AND A.15/1 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | MTSI and MTSI speech and E-UTRA and not UE category M1 and EPS IMS emergency call |
| C74 | IF [3] A.2/2 AND [73] A.4.1-1/4 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | emergency speech call and 1xRTT and EPS IMS emergency call |
| […] |  |  |
| C76 | IF A.12/27 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | capable of obtaining location Information and E-UTRA and not UE category M1 and EPS IMS emergency call |
| […] |  |  |
| C121 | IF A.18/4 AND A.12/YY THEN R ELSE N/A | WLAN and IMS emergency call over WLAN |
| […] |  |  |
| C124 | IF A.6a/2 AND A.6a/1 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | IMS security and GIBA and E-UTRA and not UE category M1 and EPS IMS emergency call |
| C125 | IF A.12/52 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | IMS emergency call without registration after rejection of emergency registration and E-UTRA and not UE category M1 and EPS IMS emergency call |
| […] |  |  |
| C132 | IF A.12/27 AND [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | capable of obtaining location Information and UE category M1 and EPS IMS emergency call |
| C133 | IF [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | UE category M1 and EPS IMS emergency call |
| C134 | IF A.6a/2 AND NOT A.6a/1 AND A.12/52 AND A.18/1 AND NOT [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | IMS security and NOT GIBA and IMS emergency call without registration after rejection of emergency registration. UE supports E-UTRA and not UE category M1 and EPS IMS emergency call |
| […] |  |  |
| C162 | IF A.18/1 AND [3] A.20/90 AND NOT [73] A.4.3.2-2A/1 AND [73] A.4.2.1.1-1/4 THEN R ELSE N/A | E-UTRA and no USIM test execution and not UE category M1 and EPS IMS emergency call |

TS 34.229-2 - Table 4.1: Applicability of TS 34.229-1 tests

| Clause | Title | Release | Applicability | Comments |
| --- | --- | --- | --- | --- |
| […] |  |  |  |  |
| Emergency Service |  |  |  |  |
| 19.1.1 | Emergency call with emergency registration / Success / Location information available | Rel-9 | C76 | UE supports is capable of obtaining location information and E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.1.2 | Emergency call with emergency registration / Success / Location information not available | Rel-9 | C59 | UE supports E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.1.3 | Emergency call with emergency registration / Abnormal case / IM CN sends a 380 / UE performs emergency call via CS domain / UTRAN or GERAN | Rel-9 | C62 | UE supports Emergency speech call and (UTRAN or GERAN) and EPS IMS emergency call (NOTE 2, 6) |
| 19.1.3a | Emergency call with emergency registration / Abnormal case / IM CN sends a 380 / UE performs emergency call via CS domain / CDMA 2000 1xRTT | Rel-9 | C74 | UE supports Emergency speech call and 1XRTT and EPS IMS emergency call (NOTE 2) |
| 19.1.3b | Void |  |  |  |
| 19.1.3c | Emergency call with emergency registration / Abnormal case / IM CN sends 503 Service Unavailable / UE performs emergency call via CS domain / UTRAN or GERAN | Rel-9 | C62 | UE supports Emergency speech call and (UTRAN or GERAN) and EPS IMS emergency call (NOTE 6) |
| 19.1.4 | Void |  |  |  |
| 19.1.5 | Emergency call with emergency registration / Emergency SIP signalling and media in parallel with an other ongoing IM CN subsystem signalling and media | Rel-9 | C72 | UE supports MTSI and MTSI speech and Communication Hold during emergency call and preconditions and E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.1.6 | Emergency call with emergency registration / Success / GIBA against a network with GIBA support only | Rel-9 | C124 | UE supports IMS security and GIBA and E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.3.1 | Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not | Rel-9 | C73 | UE supports MTSI and MTSI speech and preconditions and E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.3.2 | Non-UE detectable emergency call / IM CN sends 380 Alternative Service including emergency service URN and no emergency subservice type / Non-emergency IMS registration / UTRAN or GERAN | Rel-9 | C66 | UE supports MTSI and MTSI Speech and (UTRAN or GERAN) and EPS IMS emergency call (NOTE 2, 6) |
| […] |  |  |  |  |
| 19.3.2b | Non-UE detectable emergency call / IM CN sends a 380 with unavailable emergency service URN / UE performs normal call via CS domain / UTRAN or GERAN | Rel-9 | C66 | UE supports MTSI and MTSI Speech and emergency call and (UTRAN or GERAN) and EPS IMS emergency call (NOTE 6) |
| 19.3.2c | Non-UE detectable emergency call / IM CN sends a 380 with available emergency service URN / UE performs CS Emergency call via CS domain / UTRAN or GERAN | Rel-9 | C66 | UE supports MTSI and MTSI Speech and emergency call and (UTRAN or GERAN) and EPS IMS emergency call (NOTE 6) |
| 19.3.3 | Non-UE detectable emergency call / IM CN sends 380 Alternative Service / Emergency IMS registration | Rel-9 | C73 | UE supports MTSI and MTSI speech and E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.3.4 | Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired | Rel-9 | C73 | UE supports MTSI and MTSI speech and E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.4.1 | Emergency call without emergency registration / EPS / UE does not contain an ISIM or USIM | Rel-9 | C162 | UE supports E-UTRA and not UE category M1 and no USIM test execution and EPS IMS emergency call |
| 19.4.2 | Emergency call without emergency registration / EPS / UE contains an ISIM or USIM / UE is in state EMM-REGISTERED.LIMITED-SERVICE | Rel-9 | C64 | UE supports E-UTRA and not UE category M1 and EPS IMS emergency call |
| […] |  |  |  |  |
| 19.4.5 | Emergency call without emergency registration / UE credentials are not accepted | Rel-9 | C59 | UE supports E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.4.6 | Emergency call without emergency registration / Failure of registration / Rejected by 403 (Forbidden) | Rel-9 | C125 | UE supports IMS emergency call without registration after rejection of emergency registration and E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.4.7 | Emergency call without emergency registration / Failure of registration / against a network with GIBA support only | Rel-9 | C134 | UE supports IMS emergency call without registration after rejection of emergency registration that used IMS security. The UE does not support GIBA. UE supports IMS security and E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.5.1 | New initial emergency registration / UE obtains from the serving IP-CAN an IP address different than the IP address used for the emergency registration | Rel-9 | C59 | UE supports E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.5.6 | User-initiated emergency reregistration / UE has emergency related ongoing dialog | Rel-9 | C59 | UE supports E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.5.7 | User-initiated emergency reregistration / The user initiates an emergency call | Rel-9 | C59 | UE supports E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.5.8 | Void |  |  |  |
| 19.5.9 | In parallel emergency and non-emergency registrations | Rel-9 | C59 | UE supports E-UTRA and not UE category M1 and EPS IMS emergency call |
| 19.5.10 | Deregistration upon emergency registration expiration | Rel-9 | C59 | UE supports E-UTRA and not UE category M1 and EPS IMS emergency call |
| […] |  |  |  |  |
| Emergency Service over IMS / WLAN |  |  |  |  |
| G.19.1 | Emergency call with emergency registration / WLAN | Rel-11 | C121 | UE supports WLAN and IMS emergency call over WLAN |
| […] |  |  |  |  |
| Emergency Service over IMS / UE category M1 |  |  |  |  |
| J.19.1 | Emergency call with emergency registration / Success / Location information available / UE category M1 | Rel-13 | C132 | UE supports is capable of obtaining location information and UE category M1 and EPS IMS emergency call |
| J.19.2 | Emergency call with emergency registration / Success / Location information not available / UE category M1 | Rel-13 | C133 | UE supports UE category M1 and EPS IMS emergency call |

For 34.229-5 test cases:

TS 34.229-2 - Table 4.2: Applicability of TS 34.229-5 tests Conditions

| Conditions/Options |  |  |
| --- | --- | --- |
| […] |  |  |
| C12 | IF A.6a/2 AND A.18/5 AND A.12/27 AND [93] A.4.3.7-1/XX THEN R ELSE N/A | IMS security and NR and is capable of obtaining location information and IMS voice over NR |
| […] |  |  |
| C39 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND [93] A.4.3.7-1/XX THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and IMS voice over NR |
| C40 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.4/16 AND A.12/33 AND [93] A.4.3.7-1/XX THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and preconditions and Communication Hold during emergency call and IMS voice over NR |
| C41 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.4/16 AND [93] A.4.3.7-1/XX THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and preconditions and IMS voice over NR |

TS 34.229-2 - Table 4.2: Applicability of TS 34.229-5 tests

| Clause | Title | Release | Applicability | Comments |
| --- | --- | --- | --- | --- |
| […] |  |  |  |  |
| Emergency Calls |  |  |  |  |
| 10.1 | Emergency Call with emergency registration / Success / Location information available / 5GS | Rel-15 | C12 | UE supports IMS security and NR and is capable of obtaining location information and IMS voice over NR |
| 10.2 | Emergency Call with emergency registration / Success / Location information not available / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS voice over NR |
| 10.3 | Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS | Rel-15 | C40 | UE supports IMS security and NR MTSI and MTSI speech and preconditions and Communication Hold during emergency call and IMS voice over NR |
| 10.4 | Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not / 5GS | Rel-15 | C41 | UE supports IMS security and NR and MTSI and MTSI speech and preconditions and IMS voice over NR |
| 10.6 | Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS voice over NR |
| 10.9 | Emergency call without emergency registration / UE credentials are not accepted / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS voice over NR |
| 10.11 | Emergency call without emergency registration / Failure of registration / Rejected by 403 (Forbidden) / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS voice over NR |
| 10.12 | User-initiated emergency reregistration / UE has emergency related ongoing dialog / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS voice over NR |
| 10.13 | User-initiated emergency reregistration / User initiates an emergency call / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS voice over NR |
| 10.14 | In parallel emergency and non-emergency registrations / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS voice over NR |
| 10.15 | Deregistration upon emergency registration expiration / 5GS | Rel-15 | C39 | UE supports IMS security and NR and MTSI and MTSI speech and IMS voice over NR |

## 5.3	Detailed changes for IMS eCall

To be handled in the open Work Item NR_EIEI-UEConTest.