3GPP TSG-RAN5 Meeting #93-e	R5-216805

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1482 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MESSAGE for delivery report for MT SMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [TS23.040, 9.2.2.1a] TP-PI in SMS-DELIVER-REPORT has bits where TP-UDL and TP-DCS are optional as shown in 9.2.2.1a( ii)) [TS23.040, 9.2.3.27] The structure of the TP-PI is as follows: Octet 1: bit 7; bit 6; bit 5; bit 4; bit 3; bit 2; bit 1; bit 0. Extension bit; Reserved; Reserved; Reserved; Reserved; TP-UDL; TP-DCS; TP-PID. Therefore, UE can include TP-UDL, TP-DCS and TP-PID as these are optional.. |
| Summary of change: |  | TP-UDL, TP-DCS and TP-PID values are not checked |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5s211245(MT SMS Test cases, eg 9.4.NR5GC) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: TP-PID is also optional. Updated the message content check accordinlgy |  |  |  |  |  |  |  |  |

<Start of Change>

## A.7.2	MESSAGE for delivery report for MT SMS

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | MESSAGE |  | RFC 3428 [91] TS 24.341 [90] |
| Request-URI |  | same P-Asserted-Identity URI as received in A.7.1 |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by |  | not checked |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE |  |  |
| tag |  | any value |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same as P-Asserted-Identity URI received in A.7.1 |  | TS 24.341 [90] |
| tag |  | not present |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | any value (but different from the Call-ID values used in preceding requests of this test case) |  |  |
| In-Reply-to |  |  |  | RFC 3261 [15] |
| callid |  | The value of the Call-Id received in the original MT SMS |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | any value |  |  |
| method |  | MESSAGE |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/vnd.3gpp.sms |  |  |
| Content-Length |  | header shall be present if UE uses TCP to send this request |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |
| Message-body |  | RP-ACK message with RP-User Data including SMS-DELIVERY-REPORT: TP-MTI=‘00’B (SMS-DELIVERY-REPORT) TP-PI=‘00000XXX’B |  | TS 24.011 [92] |

NOTE 1:	X is not checked

<End of Change>