3GPP TSG-RAN5 Meeting #93-e	R5-216897r1

# Electronic Meeting, 08th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2065 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IE Table 4.6.3-141 - ReportConfigInterRAT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SRVCC_NR_to_UMTS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, To add default configuration for IE Table 4.6.3-141 - ReportConfigInterRAT |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Default configuration for IE Table 4.6.3-141 - ReportConfigInterRAT is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - "reportQuantityCell" is corrected to "reportQuantity". - All boolean are changed to lowercase. - Condition explaination for PERIODICAL is updated. - Editorial changes. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	ReportConfigInterRAT

Table 4.6.3-141: ReportConfigInterRAT (InterRAT-Thres, NR-Thres)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL OR PERIODICAL_UTRA |
| reportInterval | ReportInterval |  |  |
| reportAmount | infinity |  |  |
| reportQuantity SEQUENCE { |  | UE shall ignore the value(s) provided in reportQuantity if reportQuantityUTRA-FDD-r16 is configured. |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityUTRA-FDD-r16 | Not present |  |  |
| reportQuantityUTRA-FDD-r16 SEQUENCE { |  |  | PERIODICAL_UTRA |
| cpich-RSCP | true |  |  |
| cpich-EcN0 | false |  |  |
| } |  |  |  |
| includeCommonLocationInfo-r16 | Not present |  |  |
| includeBT-Meas-r16 | Not present |  |  |
| includeWLAN-Meas-r16 | Not present |  |  |
| includeSensor-Meas-r16 | Not present |  |  |
| } |  |  |  |
| eventTriggered SEQUENCE { |  |  | EVENT_B1 OR EVENT_B2 OR EVENT_B1_UTRA OR EVENT_B2_UTRA |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  | EVENT_B1 |
| b1-ThresholdEUTRA CHOICE { |  |  |  |
| rsrp | InterRAT-Thres | INTEGER (0..97) |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| Hysteresis | 0 (0 dB) | The actual value is field value * 0.5 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| eventB2 SEQUENCE { |  |  | EVENT_B2 |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | NR-Thres | INTEGER(0..127) |  |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | InterRAT-Thres | INTEGER (0..97) |  |
| } |  |  |  |
| reportOnLeave | FALSE |  |  |
| Hysteresis | 3 (1.5dB) | The actual value is field value * 0.5 dB |  |
| timeToTrigger | ms1024 |  |  |
| } |  |  |  |
| eventB1-UTRA-FDD-r16 SEQUENCE { |  |  | EVENT_B1_UTRA |
| b1-ThresholdUTRA-FDD-r16 CHOICE { |  |  |  |
| utra-FDD-EcN0-r16 | InterRAT-Thres |  |  |
| } |  |  |  |
| reportOnLeave-r16 | false |  |  |
| hysteresis-r16 | 0 |  |  |
| timeToTrigger-r16 | ms0 |  |  |
| } |  |  |  |
| eventB2-UTRA-FDD-r16 SEQUENCE { |  |  | EVENT_B2_UTRA |
| b2-Threshold1-r16 CHOICE { |  |  |  |
| rsrp | NR-Thres | INTEGER(0..127) |  |
| } |  |  |  |
| b2-Threshold2UTRA-FDD-r16 CHOICE { |  |  |  |
| utra-FDD-EcN0-r16 | InterRAT-Thres |  |  |
| } |  |  |  |
| reportOnLeave-r16 | false |  |  |
| hysteresis-r16 | 0 |  |  |
| timeToTrigger-r16 | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType | ssb |  |  |
| reportInterval | ms120 |  |  |
| reportAmount | r2 |  |  |
| reportQuantity SEQUENCE { |  | UE shall ignore the value(s) provided in reportQuantity if reportQuantityUTRA-FDD-r16 is configured. |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityUTRA-FDD-r16 | Not present |  |  |
| reportQuantityUTRA-FDD-r16 SEQUENCE { |  |  | EVENT_B1_UTRA OR EVENT_B2_UTRA |
| cpich-RSCP | false |  |  |
| cpich-EcN0 | true |  |  |
| } |  |  |  |
| includeCommonLocationInfo-r16 | Not present |  |  |
| includeBT-Meas-r16 | Not present |  |  |
| includeWLAN-Meas-r16 | Not present |  |  |
| includeSensor-Meas-r16 | Not present |  |  |
| } |  |  |  |
| reportCGI SEQUENCE { |  |  | CGI |
| cellForWhichToReportCGI | EUTRA-PhysCellId |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EVENT_B1 | Configuration of Event B1 on E-UTRA carrier |
| EVENT_B2 | Configuration of Event B2 on E-UTRA carrier |
| EVENT_B1_UTRA | Configuration of Event B1 on UTRA carrier |
| EVENT_B2_UTRA | Configuration of Event B2 on UTRA carrier |
| CGI | Configuration of CGI measurement |
| PERIODICAL | Configuration of periodical reporting on E-UTRA carrier |
| PERIODICAL_UTRA | Configuration of periodical reporting on UTRA carrier |

<End of modified section 1>