3GPP TSG-RAN5 Meeting #93-e	R5-216998r2

# Electronic Meeting, 08th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2559 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 8.1.5.8.1_Latency check |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, According to 38.331 cl. 5.3.7.2 UE will reset MAC entity and release its SpCell configuration during RRC re-establishment procedure. 38.331 cl.5.3.7.2: 1> if UE is not configured with conditionalReconfiguration: 2> reset MAC; 2> release spCellConfig, if configured;. So UE can't acquire UL resource via monitoring UL grant or sending SR during RRC re-establishment. Instead, UE will send PRACH preamble and use the UL grant provided in RAR for UL transmission (RRC message, RLC status report, ...). 38.321 cl.5.1.4: 1> else: 2> start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission; 2> monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.. However, UE is expected to monitor period DCI in setp 25A. It'll cause UE to fail the test. So we suggest: - change step 25A to monitoring RAR rather than period DCI. - For FR2 test, changing period DCI to RAR is still not enough. The reason is that prach-ConfigurationIndex is set to 149 by default according to 38.508-1 Table 4.6.1-28, i.e. the PO periodicity is 40ms according to 38.211 Table 6.3.3.2-4 . which means that UE may have no chance to send PRACH during RRC latency (10ms). We use 167 (corresponding to 10ms PO periodicity) instead. Furthermore, we have to extend ra-ResponseWindow to 10ms to avoid UE sending PRACH again during RRC latency. 2, According to 38.331 clause 5.3.7.2 upon initiation of the procedure, the UE shall: 1> stop timer T310, if running; 1> stop timer T312, if running; 1> stop timer T304, if running; 1> start timer T311; 1> stop timer T316, if running; 1> if UE is not configured with conditionalReconfiguration: 2> reset MAC; 2> release spCellConfig, if configured; 2> suspend all RBs, except SRB0; The ULInformationTransfer message is used for the uplink transfer of NAS or non-3GPP dedicated information. Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet). If SRB2 is suspended, the UE does not send this message until SRB2 is resumed For SRB2 is not resumed, UE can not transmite ULInformation. 3, The service request procedure should be complete. 4, When the message of UE capability information will be segmented when it is too big for default UL grant 4229bits. This will have a delay to receive he message. |
| Summary of change: |  | 1, Change period DCI in Step25A to RAR and accept PRACH between Step 25 and Step25A. 2, Change the PRACH configuration index for FR2 from 149 to 167. 3, Extend the ra-ResponseWindow in FR2 to 10ms. 4, Add step 26a1 and step 26a2. 5, Modify Table 8.1.5.8.1.3.2-3 step 9. 6, Modify the ul grant to the maximum grant allowed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - corrected wrong step number. - Note 1 and 3 are updated. 2nd revision: - Update the prose CR R5-216998 for reason 2, 3 and 4. - CATT is added as co-source. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.5.8.1	Processing delay / RRC_Idle to RRC_Connected / RRC_Inactive to RRC_Connected / Success / Latency check

8.1.5.8.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and has sent an RRCSetupRequest message }

ensure that {

when { UE receives RRCSetup message and after 10ms receives an UL grant }

then { UE successfully transmits RRCSetupComplete message }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receive a SecurityModeCommand message and after 5ms receives an UL grant }

then { UE successfully transmits SecurityModeComplete }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receive a RRCReconfiguration message to establish DRB that is not part of the current UE configuration and after 10ms receives an UL grant }

then { UE successfully transmits RRCReconfigurationComplete message }

}

(4)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry message and after 80ms receives an UL grant }

then { UE successfully transmits an UECapabilityInformation message }

}

(5)

with { UE in NR RRC_INACTIVE state and has sent an RRCResumeRequest message }

ensure that {

when { UE receives RRCResume message and after 6 or 10ms receives an UL grant depending on UE capability }

then { UE successfully transmits RRCResumeComplete message }

}

(6)

Void

(7)

with { UE in NR RRC_CONNECTED state and and has sent an RRCReestablishmentRequest message after detecting radio link failure on expiring of timer T310 }

ensure that {

when { UE receives RRCReestablishment message and after 10ms receives an UL grant }

then { UE successfully transmits RRCReestablishmentComplete message }

}

(8)

Void

(9)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives CounterCheck message and after 5ms receives an UL grant }

then { UE successfully transmists an CounterCheckResponse message }

}

8.1.5.8.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.3.4, 5.3.4.3, 5.3.5.3, 5.3.6.3, 5.3.7.4, 5.3.13.4, 5.6.1.3 and 12. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.3.4.3]

The UE shall:

…

1>	if the SecurityModeCommand message passes the integrity protection check:

…

2>	submit the SecurityModeComplete message to lower layers for transmission, upon which the procedure ends;

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

…

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

…

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

…

[38.331, clause 5.3.6.3]

Upon receiving the CounterCheck message, the UE shall:

…

1>	submit the CounterCheckResponse message to lower layers for transmission upon which the procedure ends.

[38.331, clause 5.3.7.4]

The UE shall set the contents of RRCReestablishmentRequest message as follows:

…

1>	submit the RRCReestablishmentRequest message to lower layers for transmission.

[38.331, clause 5.3.13.4]

The UE shall:

…

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

…

1>	submit the RRCResumeComplete message to lower layers for transmission;

…

[38.331, clause 5.6.1.3]

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr:

2>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr;

2>	include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4;

…

1>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 12]

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation). In case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], clause 8.6.3.

Figure 12.1-1: Illustration of RRC procedure delay

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC setup | RRCSetup | RRCSetupComplete | 10 |  |
| RRC re-establishment | RRCReestablishment | RRCReestablishmentComplete | 10 |  |
| RRC resume | RRCResume | RRCResumeComplete | 6 or 10 | Value=6 applies for a UE supporting reduced CP latency for the case of RRCResume message only including MAC and PHY configuration, reestablishPDCP and reestablishRLC for SRB2 and DRB(s), and no DRX, SPS, configured grant, CA or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRCResumeComplete and the data is transmitted over common search space with DCI format 0_0. In this scenario, the RRC procedure delay [ms] can extend beyond the reception of the UL grant, up to 7 ms. For other cases, Value = 10 applies. |
| RRC resume (scell addition) | RRCResume | RRCResumeComplete | 16 |  |
| Initial AS security activation | SecurityModeCommand | SecurityModeComplete/SecurityModeFailure | 5 |  |
| Other procedures |  |  |  |  |
| UE capability transfer | UECapabilityEnquiry | UECapabilityInformation | 80 |  |
| Counter check | CounterCheck | CounterCheckResponse | 5 |  |

8.1.5.8.1.3	Test description

8.1.5.8.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 2.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

-	Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

UE:

-	None.

Preamble:

-	The UE is in state RRC_IDLE using generic procedure parameter Connectivity (NR) according to TS 38.508-1 [4], clause 4.5.4.

8.1.5.8.1.3.2	Test procedure sequence

Table 8.1.5.8.1.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while column marked "T1", is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.5.8.1.3.2-1: Time instances of cell power level and parameter changes in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -96 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -88 |

Table 8.1.5.8.1.3.2-2: Time instances of cell power level and parameter changes in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | FFS |

Table 8.1.5.8.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a matched ng-5G-S-TMSI. | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 3A | The SS starts transmitting a periodic DCI from (10∙2μ+1+TDL) slots after step 3 to schedule PUSCH. (Note 1, Note 2) |  |  |  |  |
| 4 | Check: Does the UE transmit an RRCSetupComplete message within (10∙2μ+1+TDL+K2) slots after successful completion of step 3? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCSetupComplete | 1 | P |
| 5 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 5A | The SS starts transmitting a periodic DCI from (5∙2μ+1+TDL) slots after step 5 to schedule PUSCH. (Note 1, Note 2) |  |  |  |  |
| 6 | Check: Does the UE transmit a SecurityModeComplete message within (5∙2μ+1+TDL+K2) slots after successful completion of step 5? (Note 1, Note 2, Note 3) | --> | NR RRC: SecurityModeComplete | 2 | P |
| 7 | The SS transmits a UECapabilityEnquiry message. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 7A | The SS starts transmitting a periodic DCI with the maximum grant allowed by the carrier bandwidth from (80∙2μ+1+TDL) slots after step 7 to schedule PUSCH. (Note 1, Note 2) |  |  |  |  |
| 8 | Check: Does the UE transmit a UECapabilityInformation message within (80∙2μ+1+TDL+K2) slots after successful completion of step 7? (Note 1, Note 2, Note 3) | --> | NR RRC: UECapabilityInformation | 4 | P |
| 9 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB #n for the UE piggybacked with SERVICE ACCEPT message. | <-- | NR RRC: RRCReconfiguration NR 5GMM: SERVICE ACCEPT | - | - |
| 9A | The SS starts transmitting a periodic DCI from (10∙2μ+1+TDL) slots after step 9 to schedule PUSCH. (Note 1, Note 2) |  |  |  |  |
| 10 | Check: Does the UE transmit an RRCReconfigurationComplete message within (10∙2μ+1+TDL+K2) slots after successful completion of step 9? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCReconfigurationComplete | 3 | P |
| 11 | The SS transmits a CounterCheck message. | <-- | NR RRC: CounterCheck | - | - |
| 11A | The SS starts transmitting a periodic DCI from (5∙2μ+1+TDL) slots after step 11 to schedule PUSCH. (Note 1, Note 2) |  |  |  |  |
| 12 | Check: Does the UE transmit a CounterCheckResponse message within (5∙2μ+1+TDL+K2) slots after successful completion of step 11? (Note 1, Note 2, Note 3) | --> | NR RRC: CounterCheckResponse | 9 | P |
| 13 | Void | - | - | - | - |
| - | EXCEPTION: Steps 14a1 to 14a7 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 14a1 | IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 14a2 | Wait 1s to let UE enter RRC_INACTIVE state. | - | - | - | - |
| 14a3 | The SS transmits a Paging message including a matched ng-5G-S-TMSI. | <-- | NR RRC: Paging | - | - |
| 14a4 | The UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest | - | - |
| 14a5 | The SS transmits an RRCResume message for the UE | <-- | NR RRC: RRCResume | - | - |
| - | EXCEPTION: Steps 14a5Aa1 to 14a5Ab2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported | - | - | - | - |
| 14a5Aa1 | IF pc_reducedCP_Latency THEN the SS starts transmitting a periodic DCI from (6∙2μ+1+TDL) slots after step 14a5 to schedule PUSCH. (Note 1, Note 2) |  |  |  |  |
| 14a5Aa2 | Check: Does the UE transmit an RRCResumeComplete message within (6∙2μ+1+TDL+K2) slots after successful completion of step 14a5? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCResumeComplete | 5 | P |
| 14a5Ab1 | ELSE the SS starts transmitting a periodic DCI from (10∙2μ+1+TDL) slots after step 14a5 to schedule PUSCH. (Note 1, Note 2) |  |  |  |  |
| 14a5Ab2 | Check: Does the UE transmit an RRCResumeComplete message within (10∙2μ+1+TDL+K2) slots after successful completion of step 14a5? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCResumeComplete | 5 | P |
| 14a6-14a7 | Void | - | - | - | - |
| 14-21 | Void | - | - | - | - |
| 22 | The SS re-adjusts the SS levels according to row "T1" in table 8.1.5.8.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping | - | - | - | - |
| 23 | Wait 1s to let T310 expire | - | - | - | - |
| 24 | The UE transmits an RRCReestablishmentRequest message on Cell 2 | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 25 | The SS transmits an RRCReestablishment message | <-- | NR RRC: RRCReestablishment | - | - |
| 25AA | Check:Does the UE transmit an PRACH within 10ms after step25? | --> | PRACH | 7 | P |
| 25A | The SS shall transmit RAR from (10∙2μ+1+TDL) slots after step 25 to schedule PUSCH. (Note 1, Note 2) | <-- | RAR | - | - |
| 26 | Check: Does the UE transmit an RRCReestablishmentComplete message within (10∙2μ+1+TDL+K2) slots after successful completion of step 25? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCReestablishmentComplete | 7 | P |
| 26A | The SS transmits an RRCReconfigurationmessage to resume existing radio bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 26B | The UE transmit an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 27-35 | Void | - | - | - | - |
| Note 1: TDL is the delay in slots to the next available DL slot suitable for the transmission of a UL grant. Note 2: μ is the SCS index. μ = 0/1/2/3 for SCS = 15kHz/30kHz/60kHz/120kHz respectively. Note 3: K2 is the offset (in slots) between UL grant and the PUSCH it scheduled |  |  |  |  |  |

8.1.5.8.1.3.3	Specific message contents

Table 8.1.5.8.1.3.3-1: RRCReconfiguration (Step 9, Table 8.1.5.8.1.3.2-3)

| Derivation Path: TS 38.508-1[4], table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-SRB2-DRB(1,0) | TS 38.508-1 [4] Table 4.8.1-4 |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-SRB2-DRB(1,0) | TS 38.508-1 [4] Table 4.8.1-2B |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.1.3.3-2: CounterCheck (Step 11, Table 8.1.5.8.1.3.2-3)

| Derivation Path: TS 38.508-1[4], Table 4.6.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CounterCheck ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| counterCheck SEQUENCE { |  |  |  |  |
| drb-CountMSB-InfoList SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info { |  | 1 entry |  |  |
| drb-Identity[1] |  | DRB-Identity with Condition DRBn | entry 1 n is the index of the DRB established in step 9 of Table 8.1.5.8.1.3.2-3, which is allocated according to internal TTCN mapping |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.8.1.3.3-3: CounterCheckResponse (Step 12, Table 8.1.5.8.1.3.2-3)

| Derivation Path: TS 38.508-1[4], Table 4.6.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CounterCheckResponse ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| counterCheckResponse SEQUENCE { |  |  |  |  |
| drb-CountInfoList SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo { |  | 1 entry |  |  |
| drb-Identity[1] |  | DRB-Identity with Condition DRBn | entry 1 n is the index of the DRB established in step 9 of Table 8.1.5.8.1.3.2-3, which is allocated according to internal TTCN mapping |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.8.1.3.3-4: RRCRelease (Step 14a1, Table 8.1.5.8.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with Condition NR_RRC_INACTIVE

Table 8.1.5.8.1.3.3-5: RRCResume (Step 14a5, Table 8.1.5.8.1.3.2-3)

Derivation Path: TS 38.508-1[4],Table 4.6.1-17

Table 8.1.5.8.1.3.3-6: Void

Table 8.1.5.8.1.3.3-7: Void

Table 8.1.5.8.1.3.3-7A: Void

Table 8.1.5.8.1.3.3-8: Void

Table 8.1.5.8.1.3.3-9: Void

Table 8.1.5.8.1.3.3-10: Void

Table 8.1.5.8.1.3.3-11: SIB1 (Preamble, and all steps of Table 8.1.5.8.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| rach-ConfigGeneric SEQUENCE { |  |  |  |  |
| prach-ConfigurationIndex |  | 167 |  | FR2 |
| ra-ResponseWindow |  | sl40 |  | SCS60 |
|  |  | sl80 |  | SCS120 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of modified section 1>