3GPP TSG-RAN5 Meeting #93-e	R5-217097r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2167 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Switch/Power off procedure after EMERGENCY CALL RELEASED in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current Prose expects that upon reception of De-registration REQ the UE will either transmit the De-registration ACCEPT (Step 3a1) ot will do nothing (Step 3b1). This behavior is fine in situation where the UE has performed normal registration, and then has performed an emergency call establishment. However, for a situation like NR testcase 11.4.3, where the UE has only performed Registration for emergency, this is not fully applicable. Some devices in such situation(like NR testcase 11.4.3) will not consider the registration to be a “normal” one, and hence will transmit a 5GMM STATUS message upon reception of De-registration REQ. In other words, the UE may think that its not possible to De-register since the registration never happened. Moreover, the 24.50 cl 5.5.2.3 does not have any text that deals with a scenario where the UE is registered for emergency and has now received a De-registration REQ. This means that some UE implementations may consider that after the Registration for emergency, it is OK to expect a De-registration REQ from the SS, and thus transmit a De-registration Accept (38.523-3 cl 10.3.7 Step 3a1), but some may consider this to be a message received in incompatible protocol state and transmit a 5GMM STATUS message. Current Prose does not accommodate the 5GMM STATUS message scenario. This needs to be accommodated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new steps 3c1 – 3c3 to handle the 5GMM STATUS reception scenario. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN based on current prose can unfairly fail a conformant device |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-217097r1: Corrected the text of EXCEPTION after Step 2 to mention Step 3c3 |  |  |  |  |  |  |  |  |

### 10.3.7	Switch/Power off procedure after EMERGENCY CALL RELEASED in RRC_CONNECTED

Table 10.3.7-1: Switch/Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits a DEREGISTRATION REQUEST message with Deregistration type IE set to "re-registration not required". | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION REQUEST |
| 2 | Start T_Delay=5 sec. |  |  |
| - | EXCEPTION: Steps 3a1 – 3c3 describe optional behaviour that depends on the UE implementation. | - | - |
| 3a1 | The UE transmits a DEREGISTRATION ACCEPT message. | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION ACCEPT |
| 3a2 | Stop T_Delay. |  |  |
| 3a3 | The SS transmits an RRCRelease message | <-- | RRC: RRCRelease |
| 3b1 | T_Delay expires. (NOTE 1) | - | - |
| 3b2 | The SS locally releases the RRC connection. |  |  |
| 3c1 | The UE transmits a 5GMM STATUS message | --> | NR RRC: ULInformationTransfer 5GMM: 5GMM STATUS |
| 3c2 | Stop T_Delay. |  |  |
| 3c3 | The SS transmits an RRCRelease message | <-- | RRC: RRCRelease |
| 4 | Test procedure for Switch off / Power off in State DEREGISTERED as specified in TS 38.508-1 [5] subclause 4.9.6.4. | - | - |
| NOTE 1: The UE has performed local deregistration, so the SS shall also perform the deregistration procedure. |  |  |  |

Table 10.3.7-2: DEREGISTRATION REQUEST (Step 1, Table 10.3.7-1)

| Derivation Path: TS 38.508-1 [5], Table 4.7.1-14. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| De-registration type |  | ‘0001’B |  | Re-registration not required |  |  |  |